3GPP TSG-RAN2#127										        R2-2406909

Maastricht, Netherlands, August 19 – 23, 2024

Source: 			ZTE Corporation, Sanechips

Title: 	Initialization of DRX_SFN_COUNTER at handover

Agenda item:	7.5.3

Document for: Discussion and Decision

# Introduction

- At RAN2#126 it was agreed the UE will initialise the DRX_SFN_COUNTER to 1 if the drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of hyper frame and to 0 otherwise. The HO scenario was briefly discussed but was left as FFS. In this tdoc we discuss how this works for HO and propose a way forward.

# Discussion

Based on the agreement at RAN2#126, the UE will initialize the DRX_SFN_COUNTER to 1 or 0, based on whether drx-TimeReferenceSFN is received in the first half or the second half of hyper frame. Since the processing time is left to UE implementation, the UE will basically check this at the time when the processing of the received (re-)configuration message is completed. In case of HO, the target and the source cell may or may not be synchronized. If the source and target cells are synchronized then there seems to be no ambiguity. However, if the source and target are not synchronized, then there may be some ambiguity between the DRX_SFN_COUNTER value at the UE and the target gNB.

Observation 1: When the source and target cell are not synchronized, there may be an ambiguity in the initialization of the DRX_SFN_COUNTER value between the UE and network.

In order to solve this issue, we should clarify whether the SFN from source cell or target cell is used as reference for initialization of the DRX_SFN_COUNTER. Since the target cell may be unaware of the SFN from source cell and the drx-TimeReferenceSFN is set by the target cell, it is better to let the UE determine this based on the target cell SFN. i.e., the UE will initialize the DRX_SFN_COUNTER to 1 if the drx-TimeReferenceSFN is included in the first half of hyper frame (i.e. the SFN of the target cell would have been between 0 and 511) and to 0 otherwise. If this is agreeable, we propose to capture this as a note in MAC spec (see the CR in R2-2406918).

Proposal: In case of handover, the UE uses the target cell SFN as reference for initializing the DRX_SFN_COUNTER.

# Conclusion

In this contribution the following observations and proposals are made:

Observation 1: When the source and target cell are not synchronized, there may be an ambiguity in the initialization of the DRX_SFN_COUNTER value between the UE and network.

Proposal: In case of handover, the UE uses the target cell SFN as reference for initializing the DRX_SFN_COUNTER.

# References