3GPP TSG-RAN2#127 	R2-2406914

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda item:		8.7.2 (NR_XR_Ph3-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on Multi-modal support for XR

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed multi-modal awareness and made agreements as follows.

Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. Ask SA2 whether/what information could be provided to RAN Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows. Can consider while drafting the LS to indicate the potential benefits

In this contribution, we discuss potential enhancements for multi-modal service.

2.	Discussion

Enhancement based on Multi-modal awareness

According to TR 22.847 [1], for multi-modal traffic in XR service, synchronized transmission of multi-modal data from multiple QoS flows is essential, since the asynchronous transmission of multi-modal data degrades the XR user experience.

Table 5.1.6-2: Typical synchronization thresholds for immersive multi-modal VR applications synchronization threshold (note 1). audio-tactile; audio delay: 50 ms; tactile delay: 25 ms. visual-tactile; visual delay: 15 ms; tactile delay: 50 ms. NOTE 1: For each media component, “delay” refers to the case where that media component is delayed compared to the other.. .

In the last meeting, RAN2 agreed to support multi-modal awareness in RAN for UL and DL, and RAN2 sent LS to SA2 to ask whether/what information can be provided from SA2

For DL data, SA2 may discuss which multi-modal awareness information should be provided to RAN from the core network based on RAN2 LS. We think RAN2 can further discuss the enhancement for DL, if SA2 sends reply LS.

However, in case of UL data, we think that synchronized transmission should be ensured by the UE. This is because UL data is generated by the UE, and the UE can know the association between associated multi-modal data by UE implementation.

Proposal 1. For the enhancement based on multi-modal awareness, RAN2 considers the UL synchronized transmission by UE.

In order to ensure UL synchronized transmission, UL data from different QoS flows should meet the synchronization threshold. The synchronization threshold is defined in TS 22.261 [2] as follows:

synchronization threshold: A synchronization threshold can be defined as the maximum tolerable temporal separation of the onset of two stimuli, one of which is presented to one sense and the other to another sense, such that the accompanying sensory objects are perceived as being synchronous. NOTE 10: This definition is based on [41].

Considering the synchronization threshold, there are two options to ensure the synchronized transmission.

- Option 1. QoS flow level synchronized transmission

- Option 2. Packet level synchronized transmission

In our understanding, the difference between Option 1 and Option 2 is as follows:

In Option 1, each QoS flow is associated with each other, and different QoS flows are transmitted in synchronized manner, i.e., if one QoS flow is transmitted, another associated QoS flow should be transmitted within the synchronization threshold in time domain, but which packets are transmitted in each QoS flow depends on the UL grant size and packet size.

In Option 2, each UL packet of QoS flows is associated with each other, and UL packets from different QoS flow are transmitted in synchronized manner. If the first packet from a QoS flow is associated with the second packet from another QoS, these two packets from different QoS flow would be transmitted together for synchronization or within the synchronization threshold in time domain.

We think that Option 2 is more aligned with the concept of multi-modal service. This is because if UL packets are not synchronized, UL resource would be wasted due to transmission of outdated data, which leads to performance degradation of XR service.

Therefore, in order to meet the synchronization threshold between UL packets, RAN2 should consider packet level synchronized transmission to support the multi-modal service with inter-QoS flow dependency.

Proposal 2. For UL, RAN2 consider the synchronized transmission on packet level.

If synchronized transmission for the multi-modal service is supported on packet level, the gNB and the UE should know which packets in different QoS flows should be transmitted in synchronized manner, e.g., packet#A in QoS flow#1 should be synchronized with packet#B in QoS flow#2. In this sense, following information would be useful to support the multi-modal service with multi-modal inter-dependencies, e.g. synchronization and/or coordination requirements:

- Association information between QoS flows

- synchronization threshold; and

- Information of PDUs or PDU sets having strong dependencies.

With this information, RAN2 can discuss a mechanism to ensure that the packets from different QoS flows, which have strong dependency, can be transmitted in synchronized manner.

Proposal 3. RAN2 considers the following information for the support of the UL synchronized transmission on packet level:

- Association information between QoS flows

- synchronization threshold; and

- Information on PDUs or PDU sets having strong dependencies.

Power saving enhancement for Multi-modal service

According to WID of Rel-19 XR [3], in addition to method to handle multiple QoS flows with inter-QoS flow dependencies, the efficiency enhancement for capacity and power consumption needs to be studied for multi-modality as one of objective.

In Rel-18 XR, RAN2 studied XR-specific power saving to accommodate XR service characteristics (e.g. periodicity, multiple flows), and in RAN2#119bis-e, RAN2 agreed to consider RRC pre-configuration and switching of configurations for DRX, i.e., switching configuration between multiple DRX configurations.

However, in RAN#98-e, it was decided to narrow down the topic to DRX enhancement for non-integer periodicity, and RAN2 agreed to use one DRX configuration with non-integer DRX cycle. Thus, currently, the optimized DRX method considering different periodicities is not supported.

Meanwhile, as mentioned above, for multi-modal traffic in XR service, there are different types of data, and each type of data may require different periodicities.

In addition, according to TR 38.838 [4], it is beneficial for power saving to start onDuration aligning with each XR DL traffic arrival time. That is, there is power saving gain if DRX configuration is configured considering the traffic characteristics. The following table shows the result of power saving evaluation in TR 38.261 [2].

###### 8.3.3.1.1.1 DL+UL joint evaluation Table 8.3.3.1.1.1-1: Summary of FR1, DL+UL power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PSG (%), Note 1,4; Source. Mean (%); Range (%). DU; VR; 30; eCDRX; 11.64; 4.51 ~ 23.49; Source 18, Source 7, Source 16. CG; 30; eCDRX; 6; Source 7. AR (UL 1/2 streams); 30; eCDRX; 11.06; 4.6 ~ 20.77; Source 18. InH; VR; 30; eCDRX; 17.63; 7.23 ~ 25.12; Source 20, Source 18. 45; eCDRX; 25.64; 25.63 ~ 25.65; Source 20. CG; 30; eCDRX; 18.25; 18.23 ~ 18.26; Source 20. AR (UL 1/2 streams); 30; eCDRX; 12.23; 4.82 ~ 23.61%; Source 18. Note 1 : PSG was computed for the cases only with marginal loss in % of DL+UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see clause 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme.. ###### 8.3.3.1.1.2 DL-only evaluation Table 8.3.3.1.1.2-1: Summary of FR1, DL-only power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PSG(%), Note 1,4; Source. Mean (%); Range (%). DU; VR; 30; eCDRX; 15.70; 5.76 ~ 34.95; Source 9, Source 18, Source 7, Source 16. 45; eCDRX; 18.14; 9.72 ~ 27.26; Source 18. InH; VR; 30; eCDRX; 20.812; 9.36 ~ 29.43; Source 20, Source 18. 45; eCDRX; 19.96; 9.42 ~ 29.1; Source 20, Source 18. CG; 30; eCDRX; 26.38; 26.38; Source 20. UMa; VR; 30; eCDRX; 18.88; 10.05 ~ 29.06; Source 18. 45; eCDRX; 18.22; 9.86 ~ 27.33; Source 18. Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see clause 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme.. ###### 8.3.3.1.1.3 UL-only evaluation Table 8.3.3.1.1.3-1: Summary of FR1, UL-only power evaluation results for eCDRX Scen-arios; App; DL Bit rate (Mbps); PS scheme, Note 2; PS Gain (%), Note 1,4; source. Mean (%); Range (%). DU; AR UL 1 / 2 streams; 10.2; eCDRX; 25.56%; 19.89 ~ 32.02%; Source 18. InH; AR UL 1 / 2 streams; 10.2; eCDRX; 28.67%; 22.66 ~ 35.24%; Source 18. Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see clause 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme..

Observation 1. There is power saving gain if DRX configuration is configured considering the traffic characteristics.

According to WID of Rel-19 XR [3], multi-modal XR service can have multiple flows which may be configured to handle different periodicities of XR traffic. For example, Flow#1 is mapped to video frame and Flow#2 is mapped to audio frame. Then, the traffic periodicity of Flow#1 may be shorter than Flow#2. Thus, considering the Observation 1, in terms of power saving, it would be beneficial to apply different DRX configuration for each flow.

Observation 2. In terms of power saving, it would be beneficial to apply different DRX configuration for each flow.

However, in the current specification, it is only allowed to have two DRX groups, i.e., only two DRX configurations are allowed. In order to apply different DRX configuration for each flow, the current two DRX configurations may not be sufficient, and more than two DRX configurations should be able to be configured to support multiple flows of XR service. In our view, if each flow is allowed to be configured with different DRX configuration to fit their traffic characteristic, considerable power saving gain would be achieved. Thus, for power saving in XR, we think RAN2 should consider to allow more than two DRX configurations.

Proposal 4. RAN2 should consider to allow more than two DRX configuration.

Another point to be considered is that, XR characteristics of each flow may be changed during ongoing XR service, e.g. fps of video stream is increased. In this situation, if DRX configuration can be changed according to the changed characteristic of XR traffic, it is more beneficial for power saving.

Currently, however, DRX configuration can be changed by RRC, and it may not be sufficient to adapt quickly to the changed XR traffic characteristic during ongoing XR service. Thus, from power saving point of view, faster way to change the DRX configuration other than RRC signalling should be considered to apply DRX configuration immediately in response to change of XR traffic characteristic.

Proposal 5. RAN2 should consider how to apply DRX configuration immediately in response to change of XR traffic.

Capacity enhancement for Multi-modal service

As mentioned above, for multi-modal traffic in XR service, there are different types of data and each type of data is mapped to different flow.

In the current specification, the multiple CG configurations can be used to support multiple flows. Specifically, the multiple CGs can be configured or released by the RRC signalling for Type 1 Configured Grant operation.

However, for activation of SPS/type 2 CG, only one SPS/CG can be activated by L1 signalling. That is, if the multiple SPS/CG configurations need to be activated, multiple signal exchanges are required, causing a long latency.

Considering the XR traffic characteristics with multiple flows, it is possible that the traffic of multiple flows can occur simultaneously, or the traffic pattern can be changed for multiple flows. Thus, the simultaneous activation or deactivation of multiple SPS/type 2 CGs should be studied in order to efficiently activate or deactivate multiple SPS/CG configurations.

Proposal 6. RAN2 should consider a mechanism to activate/deactivate multiple SPS/CGs efficiently.

Furthermore, in XR service, when the UL data is transmitted, it is likely that the related DL data is also transmitted from the network, or vice versa. For example, if the UL data is periodically transmitted to Application server, the related DL data is periodically transmitted to response to the UL data. For this, CG and SPS may be configured and activated.

In above example, if no more the periodic UL data is expected, the related DL data is also not expected, and the network may deactivate both CG and SPS. In the current specification, CG and SPS activation/deactivation is performed independently. Thus, to deactivate both CG and SPS, the network sends signaling separately for CG deactivation and SPS deactivation, i.e., separate signaling are needed to deactivate both CG and SPS.

Then, if the periodic UL data occur again, the related DL data is also expected, so the network activates CG and SPS again by sending separate signaling.

In our view, this is inefficient in terms of signaling overhead. Thus, how to handle efficiently CG and SPS activation/deactivation should be considered in case of that both UL data and related DL data are expected in XR service. For example, CG and SPS is associated, both CG and SPS are activated simultaneously.

Proposal 7. RAN2 should consider how to handle efficiently CG and SPS activation/deactivation in case of that both UL data and related DL data are expected in XR service.

3.	Conclusion

In this document, we discuss enhancements based on multi-modal awareness and other enhancements., and made observations and proposals as follows.

Observation 1. There is power saving gain if DRX configuration is configured considering the traffic characteristics.

Observation 2. In terms of power saving, it would be beneficial to apply different DRX configuration for each flow.

Proposal 1. For the enhancement based on multi-modal awareness, RAN2 considers the UL synchronized transmission by UE.

Proposal 2. For UL, RAN2 consider the synchronized transmission on packet level.

Proposal 3. RAN2 considers the following information for the support of the UL synchronized transmission on packet level:

- Association information between QoS flows

- synchronization threshold; and

- Information on PDUs or PDU sets having strong dependencies.

Proposal 4. RAN2 should consider to allow more than two DRX configuration.

Proposal 5. RAN2 should consider how to apply DRX configuration immediately in response to change of XR traffic.

Proposal 6. RAN2 should consider a mechanism to activate/deactivate multiple SPS/CGs efficiently.

Proposal 7. RAN2 should consider how to handle efficiently CG and SPS activation/deactivation in case of that both UL data and related DL data are expected in XR service.

4.	Reference

- 3GPP TR 22.847	Study on supporting tactile and multi-modality	V18.2.0

- 3GPP TR 22.261	Service requirements for the 5G system; Stage 1	V19.6.0

- RP-240111 	WID on XR (eXtended Reality) for NR Phase 3	Nokia (Rapporteur)

- 3GPP TR 38.838	Study on XR (Extended Reality) Evaluations for NR	V17.0.0