3GPP TSG-RAN2#127 	R2-2406915

Maastricht, Netherlands, 19th – 23th Aug, 2024

Agenda item:		7.20.2 (NR_MIMO_evo_DL_UL-Core)

Source:	LG Electronics Inc.

Title: 	Remaining issue on STx2P PHR

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed STx2P PHR and made agreements as follows.

Remove the PH 2 for Type 2 PH reporting from the figures of the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE. When the MAC entity transmitting the PHR is not configured with twoPHRmode, for Rel-18 multi-panel PHR, apply current Rel-17 procedure that report one type 1 PH with possible wording changes in the current steps (e.g., clarify/modify “first transmission” considering simultaneous tx for multi panel case, etc) It’s up to NW implementation to make sure the current Rel-18 multi-entry PHR MAC CE format for STx2P can work (e.g., for dynamic BWP switch). RAN2 assumes the inter-node msg can be used to make sure the current Rel-18 multi-entry PHR MAC CE format for STx2P can work for DC, i.e., need RRC changes (e.g, by new parameter in inte-node msg) The following will be included in the post meeting email disc for RRC CR: Introduce a new parameter in inter-node msg, e.g., twoSRS-MultipanelScheme, (similar to twoSRS-PUSCH-Repetition-r17), that indicates whether the indicated serving cell is configured with multiple panel simultaneous uplink transmission schemes of multipanelSchemeSDM or multipanelSchemeSFN corresponding to two SRS resource sets configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'. The new parameter and twoSRS-PUSCH-Repetition-r17 are not configured together for a serving cell.

Based on above agreements, RAN2 had post e-mail discussion [1] to implement MAC CR, but some issues were newly raised and were not resolved.

In this contribution, we discuss remained issues in the post e-mail discussion

2.	Discussion

During post e-mail discussion [1], how to obtain PH value was discussed for the case where the physical layer provides two Type 1 PH values but the MAC entity can transmit one Type 1 PH value.

In detailed, if a serving cell is configured with multipanelScheme and a MAC entity this serving cell belongs to is configured with twoPHRMode, the physical layer provides two Type 1 PH values. In DC case, if the other MAC entity is not configured with twoPHRMode and transmits STxMP PHR MAC CE, this MAC entity cannot transmit two Type 1 PH value and one Type 1 PH value needs to be selected.

However, RAN2 had not discussed this cases and there is no description in RAN1 specification, so RAN2 postpone this issue.

For Rel-17 mTRP PUSCH repetition, there was a similar discussion. In RAN2#119-e, how to obtain PH value was discussed if the physical layer provides two Type 1 PH value for a serving cell based on twoPHRMode and mTRP PUSCH repetition but the MAC entity can transmit one Type PH value, and RAN2 concluded that the MAC entity obtains Type 1 PH value of the first real transmission, i.e., earlier real transmission, at the slot where PHR MAC CE is transmitted.

However, for Rel-18 STxMP, we think that there is a case where this behaviour cannot be inherited as it is. There are three cases with STxMP.

- Case#1: PUSCH transmission associated with first TCI-State and PUSCH transmission associated with second TCI-State are performed simultaneously in a same slot.

- Case#2: PUSCH transmission associated with first TCI-State is performed earlier than PUSCH transmission associated with second TCI-State in a same slot.

- Case#3: PUSCH transmission associated with second TCI-State is performed earlier than PUSCH transmission associated with first TCI-State in a same slot.

For Case#1, if sTx-2Pnael is configured, the simultaneous transmission on both panel is performed in a same slot and two Type 1 PH values associated with first TCI-State and second TCI-state are provided. Thus, the first real transmission cannot be defined and RAN2 has no knowledge of which Type 1 PH value should be reported.

Therefore, we think that it would be better to ask to RAN1 about which Type 1 PH value should be reported in Case#1, and during post e-mail discussion [1], companies also have a consensus that RAN2 sends LS to RAN1 to ask about this. In addition to PH value, Pcmax also can be asked.

Proposal 1. RAN2 sends LS to ask to RAN1 about followings.

- If PUSCH transmission associated with first TCI-State and PUSCH transmission associated with second TCI-State are performed simultaneously in a same slot, which Type 1 PH value and Pcmax value should be reported between Type 1 PH value and Pcmax value associated with the first TCI-State and the second TCI-State?

Draft LS is attached in Annex.

For Case#2 and Case#3, we think that the first real transmission can be defined. In Case#2, PUSCH transmission associated with first-TCI state is considered as the first real transmission, and in Case#3, PUSCH transmission associated with second-TCI state is considered as the first real transmission.

Therefore, we think that Rel-17 UE behaviour obtaining Type 1 PH value can be inherited for Rel-18 STxMP.

Proposal 2. The MAC entity transmitting PHR MAC CE obtains Type 1 PH value and Pcmax value of the first real transmission for following cases.

- PUSCH transmission associated with first TCI-State is performed earlier than PUSCH transmission associated with second TCI-State in a same slot.

- PUSCH transmission associated with second TCI-State is performed earlier than PUSCH transmission associated with first TCI-State in a same slot.

3.	Conclusion

In this document, we discuss how to obtain PH values for STxMP, and made proposals as follows.

Proposal 1. RAN2 sends LS to ask to RAN1 about followings.

- If PUSCH transmission associated with first TCI-State and PUSCH transmission associated with second TCI-State are performed simultaneously in a same slot, which Type 1 PH value and Pcmax value should be reported between Type 1 PH value and Pcmax value associated with the first TCI-State and the second TCI-State?

Proposal 2. The MAC entity transmitting PHR MAC CE obtains Type 1 PH value and Pcmax value of the first real transmission for following cases.

- PUSCH transmission associated with first TCI-State is performed earlier than PUSCH transmission associated with second TCI-State in a same slot.

- PUSCH transmission associated with second TCI-State is performed earlier than PUSCH transmission associated with first TCI-State in a same slot.

4.	Reference

- [Post126][204][MIMOevo] CR for TS 38.321 (Samsung)

5.	Annex

1. Overall Description:

According to 38.213 S7.7, for a serving cell configured with multipanelScheme, the physical layer provides two Type 1 PH value and two Pcmax value.

In the current MAC specification, if PHR is triggered and PHR MAC CE is generated, the MAC entity transmitting the PHR MAC CE obtains PHR information (e.g. PH values and Pcmax) for all activated serving cells, even in DC case.

In addition, twoPHRMode is configured per MAC entity and multipanelScheme and sTx-2panel are configured per serving cell. Thus, in DC case, there is a case where one MAC entity is configured with twoPHRMode and the other MAC entity is not configured with twoPHRMode.

In this case, if the one MAC entity generates PHR MAC CE and if a serving cell is configured with multipanelScheme, the one MAC entity can obtain two Type 1 PH value and two Pcmax value for the serving cell. If the other MAC entity generates PHR MAC CE, the other MAC entity can obtain one Type 1 PH value and one Pcmax value for the serving cell.

If the MAC entity transmitting PHR MAC CE is not configured with twoPHRMode and the MAC entity a serving cell belongs to is configured with twoPHRMode and the serving cell is configured with multipanelScheme,

- Question 1) Does the physical layer provide provides two Type 1 PH value and two Pcmax value for the serving cell?

- Question 2) If yes and if sTX-2Panel is configured and PUSCH + PUSCH transmission is performed simultaneously on both panel, which values should be obtained between two Type 1 PH value and two Pcmax associated with the first TCI-state and the second TCI-state for the serving cell?

2. Actions:

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to answer above questions in order to give RAN2 a clear guidance about the PHR MAC CE design for R18 STx2P.