3GPP TSG-RAN WG2 Meeting #127	R2-2406917

Maastricht, Netherlands, Aug 18th – 23rd, 2024

Agenda item:		7.3.2

Source:				Samsung

Title:					Clarification for SP CSI reporting on PUCCH

Document for:		Discussion & Decision

# 1 Introduction

This paper discusses an issue related to the status of the sub-configurations of a configured Semi-presistent CSI reporting on PUCCH upon (re-)configuration by upper layers and after reconfiguration with sync.

# 2 Discussion

### 2.1 SP CSI sub-configurations

In Rel-18, the network may activate and deactivate at least one of the configured sub-configuration of a configured Semi-Persistent CSI reporting on PUCCH of a Serving Cell by sending the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE. However, it is not sufficiently clear about the status of the sub-configurations of a configured Semi-presistent CSI reporting on PUCCH upon (re-)configuration by upper layers and after reconfiguration with sync. Hence, it is proposed to make it clear as suggested in proposal 1.

Proposal 1: All the sub-configurations, if any, of a configured Semi-presistent CSI reporting on PUCCH are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. Adopt text proposal 1 as provided.

Text Proposal 1:

### TS 38.321 V18.2.0 ### 5.18.6 Activation/Deactivation of Semi-persistent CSI reporting on PUCCH The network may activate and deactivate the configured Semi-persistent CSI reporting on PUCCH of a Serving Cell by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.16 or the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.80. The network may activate and deactivate at least one of the configured sub-configuration of a configured Semi-Persistent CSI reporting on PUCCH of a Serving Cell by sending the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE described in clause 6.1.3.80. For each Semi-persistent CSI reporting on PUCCH configuration configured with csi-ReportSubConfigToAddModList, the network may deactivate all configured sub-configurations by sending the SP CSI reporting on PUCCH Activation/Deactivation MAC CE. The configured Semi-persistent CSI reporting on PUCCH and its all sub-configurations, if any, are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. The MAC entity shall: 1> if the MAC entity receives an SP CSI reporting on PUCCH Activation/Deactivation MAC CE or an Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE on a Serving Cell: 2> indicate to lower layers the information regarding the SP CSI reporting on PUCCH Activation/Deactivation MAC CE or the Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

# 3 Conclusion

Request RAN2 to discuss and agree to the following proposal:

Proposal 1: All the sub-configurations, if any, of a configured Semi-presistent CSI reporting on PUCCH are initially deactivated upon (re-)configuration by upper layers and after reconfiguration with sync. Adopt text proposal 1 as provided.