3GPP TSG-RAN WG2 #127	R2-2406919

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.6.2

Source:	Ericsson

Title:	Important aspects regarding inter-CU LTM

Document for:	Discussion

# 1	Introduction

At RAN2#126, the following agreements on inter-CU LTM were made:

Agreements on scenarios:

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures:

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell Switch Completion:

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

This contribution outlines a number of aspects that should be discussed within the scope of Inter-CU LTM in Rel-19.

# 2	Discussion

### 2.1	LTM Reference Configuration

In Rel-18 LTM, RAN2 has agreed the support of a single reference configuration where a LTM candidate cell can either be a “delta configuration” of the configured reference configuration or a “complete configuration”, meaning that the reference configuration is either ignored (if configured) or not configured.

In LTM Rel-18, only one reference configuration is supported.

When it comes to Rel-19 LTM, we see no benefit to deviate from the same principle and allow the network to configure multiple reference configurations at the UE. In this case, if we allow the use of multiple reference configurations, the main drawbacks could be a more complex signaling design and a coordination at the network side which would be unnecessary (especially in adding/modifying/releasing such reference configurations). According to this, we propose:

Only one reference configuration can be configured for both intra-CU and inter-CU LTM.

### 2.3	Configuration of LTM CSI Resource Configuration

A similar discussion to that one of the LTM reference configuration can also be done for the LTM CSI resource configuration which is used to indicate to the UE which reference signals to measure for one or more configured LTM candidate cells.

In Rel-18 LTM, the CU is responsible to generate the LTM CSI reference configuration because of two main reasons:

- The CU knows the general situation of the selected LTM candidate cells because it is the node which receives the L3 measurements at the time of the LTM preparation.

- The CU also knows which are the SSBs configured for each cell belonging to a DU for which an F1 interface is setup (this information is available during the F1 setup procedure).

We understand that what has been done in Rel-18 can also be applied in inter-CU LTM as the reasons for having the CU to generate the LTM CSI resource configuration still apply for the case when there is the Xn interface instead of the F1. In fact, the source CU knows which are the SSBs configured for each cell belonging to a (candidate) CU for which an Xn interface is setup (this information is available during the Xn setup procedure). According to this, we propose:

As in Rel-18 LTM, in inter-CU LTM the source CU is responsible to generate the LTM CSI resource configuration. Details on the signalling are up to RAN3.

### 2.4	Early Sync Phase

At the RAN2#125bis meeting, these agreements on early sync were made:

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Further agreements where made at RAN2#126:

Early sync:

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

For the early DL synchronization, we see no difference in the procedure whether the early TCI state activation is done towards TCI which is part of an intra-CU or inter-CU LTM candidate cell configuration. The only difference would be that RAN3 need to introduce the necessary signaling over the Xn interface. According to this, we propose:

The existing Candidate Cell TCI States Activation/Deactivation MAC CE for the early TCI state activation is reused also for inter-CU LTM.

### 2.5	Execution

At the RAN2#125-bis meeting it was agreed that:

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

The above agreement suggests that the UE should keep its LTM candidate cell configurations after an inter-CU LTM cell switch as well as after an intra-CU LTM cell switch. The same as would apply for L3 handover (including conditional reconfiguration).

As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after an inter-CU LTM cell switch procedure, unless these are explicitly released by the network.

### 2.6	NR-DC aspects

The WID [2] includes the following objectives on NR-DC:

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

According to what is described in the WID, our understanding is that the following scenarios are supported for the configuration of LTM at the MN and SN:

- Intra-CU LTM at the MN and intra-CU at the SN  Supported

- Inter-CU LTM at the MN and intra-CU at the SN  Supported

- Intra-CU LTM at the MN and inter-CU at the SN  Supported

- Inter-CU LTM at the MN and inter-CU at the SN  Not Supported

This is also clear from the agreement took in the last RAN2 meeting:

Agreements on scenarios:

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells

Therefore, we propose the following:

The following scenarios for the configuration of LTM at the MN and SN are supported:

  - Intra-CU LTM at the MN and intra-CU LTM at the SN (already in Rel-18)

  - Intra-CU LTM at the MN and inter-CU LTM at the SN

  - Inter-CU LTM at the MN and intra-CU LTM at the SN

Even the configuration of LTM can be supported for the following scenarios, one needs to clarify at the time of the LTM execution what happen at the MN and the SN. This pretty much depends on whether an intra-CU LTM or an inter-CU LTM cell switch procedure is executed by the UE. According to this, we have the following understandings, which is somehow aligned also with the DC procedures standardized since Rel-15:

- If an intra-CU LTM cell switch is executed at the MN or SN, this does not impact the LTM configurations (whether is intra-CU or inter-CU) at the SN or MN.

- If an inter-CU LTM cell switch is executed at the MN, the LTM configurations at the SN are either released or reconfigured. In this case, the LTM configuration at the SN might not be kept as the security as the MN has changed.

- If an inter-CU LTM cell switch is executed at the SN, the LTM configurations at the MN are either released or reconfigured. In this case, the LTM configuration at the MN cannot be kept as SN has changed.

According to this, our proposal for the handling of the LTM configurations on when LTM is executed is as follows:

In case of LTM execution (intra-CU or inter-CU) at the MN or SN, the following handling of the LTM configurations are supported:

  - If an intra-CU LTM cell switch is executed at the MN or SN, this does not impact the LTM configurations (whether is intra-CU or inter-CU) at the SN or MN.

  - If an inter-CU LTM cell switch is executed at the MN, the LTM configurations at the SN are either released or reconfigured.

  - If an inter-CU LTM cell switch is executed at the SN, the LTM configurations at the MN are either released or reconfigured.

A final case that needs to be considered is when an SN is released and there are LTM candidate configurations at the MN which are built considering such SN (which now is not there anymore). In such a case, if the SN is released during an LTM cell switch procedure, this information should be propagated to all other candidate CU which have prepared an LTM candidate configuration which was relying on using the released SN. This will avoid a misalignment on the UE configuration (which does not have any SN configured) and a candidate LTM configuration (which expect the UE to have an SN). According to this, we propose:

In case an SN is released during an LTM cell switch procedure, all other candidate CU needs to be informed about the release of the SN. Details of the signalling are up to RAN3.

The legacy L3 PSCell change can be initiated either by the serving SN or by the MN. In case of inter-SN PSCell change there is always MN involvement and the SN RRCReconfiguration message, which is generated by the SN, may be sent to the UE via the MN. However, even if for the LTM configuration at the SN the RRCReconfiguration message (which include the SN configured LTM candidate configurations) can be delivered to the UE either directly by the SN or via the MN, the LTM cell switch execution and the early synchronization procedure are only executed by the SN by using either L1 (e.g., PDCCH order) or L2 (e.g., MAC CE) signaling.

The early synchronization (UL and DL) procedures and the LTM execution are only triggered at the UE by the serving DU in the SN (either via L1 or L2 signalling).

The MN is not allowed to trigger the early synchronization (UL and DL) procedures and the LTM execution at the SN.

According to this, in order to keep the solution simple and to avoid unnecessary tight coordination between MN and SN, a straightforward solution would be to only allow SN-initiated inter-LTM cell switch executions in Rel-19. Thus, we propose:

Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

### 2.7	Stage-2 Update

In the ANNEX we provide a TP to 38.300 which RAN2 can start to use as a baseline in its work on inter-CU LTM. It is written as an update of the existing Rel-18 intra-CU LTM procedure, extending it the inter-gNB mobility – using the same principle as L3 Handover and CHO is already described in 38.300. The TP is based on the existing RAN2 and RAN3 agreements.

According to this, we propose:

RAN2 to agree on the TP to 38.300 provided in the ANNEX.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In LTM Rel-18, only one reference configuration is supported.

Observation 2	The early synchronization (UL and DL) procedures and the LTM execution are only triggered at the UE by the serving DU in the SN (either via L1 or L2 signalling).

Observation 3	The MN is not allowed to trigger the early synchronization (UL and DL) procedures and the LTM execution at the SN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only one reference configuration can be configured for both intra-CU and inter-CU LTM.

Proposal 2	As in Rel-18 LTM, in inter-CU LTM the source CU is responsible to generate the LTM CSI resource configuration. Details on the signalling are up to RAN3.

Proposal 3	The existing Candidate Cell TCI States Activation/Deactivation MAC CE for the early TCI state activation is reused also for inter-CU LTM.

Proposal 4	As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after an inter-CU LTM cell switch procedure, unless these are explicitly released by the network.

Proposal 5	The following scenarios for the configuration of LTM at the MN and SN are supported:

a.	Intra-CU LTM at the MN and intra-CU LTM at the SN (already in Rel-18)

b.	Intra-CU LTM at the MN and inter-CU LTM at the SN

c.	Inter-CU LTM at the MN and intra-CU LTM at the SN

Proposal 6	In case of LTM execution (intra-CU or inter-CU) at the MN or SN, the following handling of the LTM configurations are supported:

b.	If an inter-CU LTM cell switch is executed at the MN, the LTM configurations at the SN are either released or reconfigured.

c.	In an inter-CU LTM cell switch is executed at the SN, the LTM configurations at the MN are either released or reconfigured.

Proposal 7	In case an SN is released during and LTM cell switch procedure, all other candidate CU needs to be informed about the release of the SN. Details of the signalling are up to RAN3.

Proposal 8	Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Proposal 9	RAN2 to agree on the TP to 38.300 provided in the ANNEX.

# 4	References

- 3GPP TS 38.300 v18.2.0 2024-06.

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN meeting #104, Shanghai, China, June 17-20, 2024

### ANNEX 	TP to 38.300 [1]

--------------------------------------------------------START TP to 38.300-------------------------------------------------------------

#### 9.2.3.5	L1/L2 Triggered Mobility

##### 9.2.3.5.1	General

LTM is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB may change UE serving cell by a cell switch command signalled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command. The LTM procedure can be used to reduce the mobility latency as described in Annex G.

When configured by the network, it is possible to activate TCI states of one or multiple cells that are different from the current serving cell. For instance, the TCI states of the LTM candidate cells can be activated in advance before any of those cells become the serving cell. This allows the UE to be DL synchronized with those cells, thereby facilitating a faster cell switch to one of those cells when cell switch is triggered. All the activated TCI states except those received in the cell switch command are deactivated upon LTM cell switch execution.

When configured by the network, it is possible to initiate UL TA acquisition (called early TA) procedure of one or multiple cells that are different from the current serving cells. If the cell has the same NTA as the current serving cells or NTA=0, early TA acquisition procedure is not required. The network may request the UE to perform early TA acquisition of a candidate cell before a cell switch. The early TA acquisition procedure is triggered by PDCCH order as specified in clause 9.2.6 or realized through UE-based TA measurement as configured by RRC. In the former case, the gNB/gNB-DU to which the candidate cell belongs calculates the TA value and sends it to the gNB/gNB-DU to which the serving cell belongs via gNB-CU. The serving cell sends the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch. In the latter case, the UE performs TA measurement for the candidate cells after being configured by RRC but the exact time the UE performs TA measurement is up to UE implementation. The UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command, if it does not include any valid TA value. The network may also send a TA value in the LTM cell switch command MAC CE without early TA acquisition.

Depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch. If the valid TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. In the case where UE-based TA measurement is configured, but no valid TA value is provided in the cell switch command, the UE applies the valid TA value by itself if available. The UE performs RACH-less LTM cell switch upon receiving the cell switch command whenever a valid TA value is available. If no valid TA value is available, the UE performs RACH-based LTM cell switch.

Regardless of whether the UE is configured for UE-based TA measurement for a certain candidate cell, it will still follow the PDCCH order, which includes performing a random access procedure towards one or more candidate cells. This also applies to the candidate cells for which the UE is capable of deriving TA values by itself. Additionally, regardless of whether the UE has already performed a random access procedure towards the candidate cells, it will still follow the UE-based measurement configuration if configured by the network.

For RACH-less LTM, the UE accesses the target cell using either a configured grant or a dynamic grant. The configured grant is provided in the LTM candidate configuration, and the UE selects the configured grant occasion associated with the beam indicated in the cell switch command. Upon initiation of LTM cell switch to the target cell, the UE starts to monitor PDCCH on the target cell for dynamic scheduling. Before RACH-less LTM procedure completion, the UE shall not trigger random access procedure if it does not have a valid PUCCH resource for triggered SRs.

The following principles apply to LTM:

-	Security keys are maintained upon an LTM cell switch;

-	Subsequent LTM is supported.

LTM supports both intra-gNB-DU and inter-gNB-DU mobility as well as inter-gNB mobility. LTM supports both intra-frequency and inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. LTM is supported only for licensed spectrum. The following scenarios are supported:

-	PCell change in non-CA scenario and non-DC scenario;

-	PCell and SCell(s) change in CA scenario;

-	Dual connectivity scenario: including PCell and MCG SCell(s) change and intra-SN PSCell and SCG SCell(s) change without MN involvement. LTM for simultaneous PCell and PSCell change is not supported.

While the UE has stored LTM candidate configurations the UE can also execute any L3 handover except for DAPS handover. In the RRC message which the UE applies for any L3 handover (except DAPS), LTM candidate configurations can be added/modified/released by the target cell.

##### 9.2.3.5.2	C-Plane Handling

Cell switch command is conveyed in a MAC CE, which contains the necessary information to perform the LTM cell switch.

The overall procedure for LTM is shown in Figure 9.2.3.5.2-1 below. Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion. The general procedure over the air interface is applicable to SCG LTM. Further details of SCG LTM can be found in TS 37.340 [21].

Figure 9.2.3.5.2-1. Signalling procedure for intra-AMF/UPF LTM

The procedure for LTM where neither the AMF nor the UPF changes is as follows:

1.	The UE sends a MeasurementReport message to the source gNB. The source gNB decides to configure LTM and initiates LTM preparation of one or more candidate cells belonging to one or more candidate gNBs.

2.	The source gNB decides to configure LTM and initiates LTM preparation.

3.	The source gNB issues a Handover Request message to the target gNB.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB prepares the LTM candidate configuration(s) and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB, which includes a transparent container to be sent to the UE as an RRC message including the LTM candidate configurations.

6.	The source gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

7.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the source gNB.

NOTE 1:	FFS: Preparations for UL and DL synchronization are performed between the source and target gNB with the LTM candidate cell(s).

8a.	The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command. The UE may activate and deactivate TCI states of LTM candidate cell(s), as triggered by the source gNB.

8b.	The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the source gNB. When UE-based TA measurement is configured, UE acquires the TA value(s) of the candidate cell(s) by measurement. UE performs early TA acquisition with the candidate cell(s) as requested by the network before receiving the cell switch command as specified in clause 9.2.6. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the target gNB sends the TA value, the associated CFRA resource information to the source gNB. The TA value of the candidate cell is indicated in the cell switch command. The UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

9-10. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

11.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 6) is applicable.

12-13.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available.

14.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of the Cell Switch command to the UE.

15. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID. It may also perform security key refresh when applicable.

16.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17-18.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target gNB in the target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

19-20.	The target gNB sends the HANDOVER SUCCESS message to the source gNB. to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message.

NOTE 2:	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

21.	Path switch procedure is performed between the target gNB and the AMF to trigger 5GC to switch the DL data path towards the target gNB and to establish an NG-C interface instance towards the target gNB.

22.	The target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the LTM cell switch.

The steps 8-22 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 5.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to intra-gNB-DU LTM, inter-gNB-DU LTM and inter-gNB LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].

##### 9.2.3.5.3	U-Plane Handling

After receiving an LTM cell switch command MAC CE, the UE performs MAC reset. Whether the UE performs RLC re-establishment and PDCP data recovery during cell switch is explicitly controlled by the network through RRC signalling.

--------------------------------------------------------END TP to 38.300---------------------------------------------------------------