3GPP TSG-RAN WG2 #127	R2-2406920

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.6.3

Source:	Ericsson

Title:	Important aspects regarding event triggered L1 measurements

Document for:	Discussion

# 1	Introduction

Previous meetings have listed a number of use cases to design the event based L1 measurements around, and we think it is very beneficial to start with the use cases to support, and then motivate events based on these use cases. For this reason, this contribution discusses use cases and events, as well as some other L1 measurement centric discussion points.

# 2	Discussion

This section provides a detailed view of the measurement-related enhancements to support the Event-triggered L1 measurement and reporting, as one of the main objectives described in the WID [1].

## 2.1	Use Case Definition

Up until and including RAN2#126 a few use cases have been agreed to use as basis for the definition of events. The agreed use cases and events are marked in green below in Figure 1. The suggested mapping between the use cases and events could be discussed but could be used as a baseline for the discussion in this paper.

Figure 1; Proposed and agreed use cases and events (yellow/grey = proposed, green = agreed)

One event that were discussed during RAN2#126 but never agreed was “Beam of serving cell becomes better than absolute threshold”. This event can be used for the network to know that it does not have to consider mobility procedures any longer, since the serving cell is above a defined threshold.

Adding the event “Beam of serving cell becomes better than absolute threshold” would enable the network to stop LTM and optionally release all LTM candidate configurations.

Add the event “Beam of serving cell becomes better than absolute threshold” to the L1 event-triggered reporting.

For the not already agreed use cases we propose to add “Release LTM Candidate Configuration” and “Early Sync deactivation of LTM candidate cells” to the supported ones to enable discussion of further events for this.

More use cases can be considered to define necessary events.

Add use cases “Release LTM Candidate Configuration” and “Early Sync management of LTM candidate cells” to the list of supported use cases for L1 event-triggered reporting.

Add the event “Beam of candidate cell becomes offset better than pre-activated beam of candidate cell” to the L1 event-triggered reporting.

As a secondary priority, add the event “Beam of candidate cell becomes worse than absolute threshold”

## 2.2	Beam level vs Cell level measurements

A further aspect to be addressed is whether the UE should perform measurements based on beam- or cell-level. In our view, for event-triggered L1 measurement reporting, beam-level measurements may suffice since the decision to trigger LTM is based on the network implementation. The network has a comprehensive overview of all beams based on the beam-level measurements in event-triggered L1 measurement reporting. However, cell-level measurement events might be necessary in certain scenarios, such as in Conditional LTM when the handover is triggered by the UE itself based on the Conditional LTM execution conditions. In such cases, we may postpone the use of events evaluated based on cell-level measurements when Conditional LTM will start.

The need of events evaluated based on cell-level measurements is discussed within the conditional LTM objective.

In the last RAN2 meeting it was agreed that the use of beam level measurements for the events evaluation is the baseline. However, it is still not clear what beams are used for the evaluation. There are several options that can be exploited, such as:

- Only the best beam is used for the event evaluation

- The best N beams are used for the event evaluation

- The beams above a threshold X are used for the event evaluation

- The currently activated beam(s) is used for the evaluation

However, we think that this topic is out of the RAN2 domain, and it would be to leave the decision to RAN1, which is the working group that has more knowledge about this. Therefore, we propose:

The decision on what beam(s) of the serving cell and neighbouring cell is used for event evaluation is left to RAN1. RAN2 to consider sending an LS about this.

## 2.3	Triggering for sending the measurement report

One open issue is whether a measurement report should be sent when the leaving condition is met. In L3 measurements, a UE sends a measurement report when the field reportOnLeave is configured and also when the leaving condition is met for a certain event. For L1, it would be to align as this information is particularly useful for the network to understand which of the configured candidate cell is not good anymore. This may lead the network to release such candidate cell or to disable certain feature such as the UL or DL early synchronization. Therefore, we propose:

The UE can be configured to send a L1 measurement report when the leaving condition is met for L1 event, similar to what is available for L3 measurements.

Another aspect which would be good to align with what we have for L3 is the sending of a measurement report periodically after an event criterion has been fulfilled. This is particularly useful for feature like LTM because it will guarantee the network to have very timely information from the UE before taking the decision on whether to trigger an LTM cell switch procedure or not. Thus, our proposal is:

The UE sends periodic L1 measurement reports after event-related measurement report has been sent. A report amount and report interval as specified, similar to what is available for L3 measurements.

## 2.4	Report content

The current LTM measurement report is a natural starting point for the discussion on the report content although adaptations for event-triggered reports need to be done. To begin with, it must be clear to the network what was the event that triggered the reporting. It appears reasonable that all configured events are associated with an event ID and that the measurement report includes the ID of the triggering event.

Event triggered LTM measurement report should include an identifier of the event and conditions that triggered the report.

Below we propose additional report content for each event type that has been agreed in RAN2.

LTM2: From the fact that the event has triggered, the network can conclude that the RSRP of the serving beam is below the configured threshold. It is however beneficial for the network to know the exact RSRP of the serving beam to understand the urgency and how fast the conditions are degrading:

For LTM2 the measurement report should include the RSRP of the serving beam.

LTM3: This event is triggered by conditions on two different resources: the candidate beam and the serving beam. The candidate beam identity is the target of a mobility action (presync or cell switch) and need to be included in the report. It can be beneficial to also include the serving beam RSRP to give the network more detailed understanding of the situation:

For LTM3 the measurement report should include an identifier of the candidate beam that triggered the event, the RSRP of the triggering beam, and the serving beam RSRP.

LTM4: This event is triggered when a candidate beam becomes better than an absolute threshold. The candidate beam identifier needs to be included in the report since it is the mobility target as well as the RSRP of the candidate beam.

For LTM4 the measurement report should include an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

LTM5: This event is triggered by conditions on two different resources: the serving beam which is worse than a first absolute threshold and a candidate beam that is better than a second absolute threshold. It is the combination of LTM2 and LTM4 and for the reasons already mentioned for those events, we think that the serving beam RSRP, an identifier of the candidate beam that triggered the event, and the RSRP of the triggering beam should be included in the measurement report when LTM5 is triggered.

For LTM5 the measurement report should include the serving beam RSRP, an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

The above observations are summarized in Table 1

| Event | Measurement report content |
| --- | --- |
| LTM2 | Serving beam RSRP |
| LTM3 | Serving beam RSRP + triggering beam (identifier, RSRP) |
| LTM4 | Triggering beam (identifier, RSRP) |
| LTM5 | Serving beam RSRP + triggering beam (identifier, RSRP) |

Table 1: Mandatory content of the event-driven measurement reports for the agreed events.

For all the mentioned measurement reports it should also be possible to include measurements on additional candidate beams, like in the Rel-18 measurement report format.

For all the defined LTM events it should be possible to include measurement on additional candidate beams that did not trigger the event in the measurement report, and the corresponding RSRP values.

According to our analysis, we believe that the information included in Table 1 should be taken into account when designing the content of the LTM CSI measurement report.

RAN2 to take into account the information in Table 1 when designing the content of the LTM CSI measurement report.

## 2.5	Which layer evaluates the L1 events

So far there has been a clear separation when it comes to performing measurements (on L1 and L3) and evaluating events (on L3). The L1 measurements are performed and measured on the physical layer while the L3 measurements are the competence of the RRC layer. When it comes to the event triggered L1 measurements, RAN2 would need to decide on which layer the criteria for each configured event are evaluated, and this may be done either on the RRC layer, MAC layer, or physical layer. Either way, this is something that needs to be discussed and decided by RAN2. Thus, we propose:

RAN2 shall be the WG to decide which layer shall evaluate the criteria for each L1 event configured.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Adding the event “Beam of serving cell becomes better than absolute threshold” would enable the network to stop LTM and optionally release all LTM candidate configurations.

Observation 2	More use cases can be considered to define necessary events.

Observation 3	For LTM2 the measurement report should include the RSRP of the serving beam.

Observation 4	For LTM3 the measurement report should include an identifier of the candidate beam that triggered the event, the RSRP of the triggering beam, and the serving beam RSRP.

Observation 5	For LTM4 the measurement report should include an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

Observation 6	For LTM5 the measurement report should include the serving beam RSRP, an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

Observation 7	For all the defined LTM events it should be possible to include measurement on additional candidate beams that did not trigger the event in the measurement report, and the corresponding RSRP values.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Add the event “Beam of serving cell becomes better than absolute threshold” to the L1 event-triggered reporting.

Proposal 2	Add use cases “Release LTM Candidate Configuration” and “Early Sync management of LTM candidate cells” to the list of supported use cases for L1 event-triggered reporting.

Proposal 3	Add the event “Beam of candidate cell becomes offset better than pre-activated beam of candidate cell” to the L1 event-triggered reporting.

Proposal 4	As a secondary priority, add the event “Beam of candidate cell becomes worse than absolute threshold”

Proposal 5	The need of events evaluated based on cell-level measurements is discussed within the conditional LTM objective.

Proposal 7	The UE can be configured to send a L1 measurement report when the leaving condition is met for L1 event, similar to what is available for L3 measurements.

Proposal 8	The UE sends periodic L1 measurement reports after event-related measurement report has been sent. A report amount and report interval as specified, similar to what is available for L3 measurements.

Proposal 9	Event triggered LTM measurement report should include an identifier of the event and conditions that triggered the report.

Proposal 10	RAN2 to take into account the information in Table 1 when designing the content of the LTM CSI measurement report.

Proposal 11	RAN2 shall be the WG to decide which layer shall evaluate the criteria for each L1 event configured.

# 4	References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024.

3GPP TS 38.331 NR RRC, v18.1.0, March 2024.