3GPP TSG-RAN WG2 #127	R2-2406921

# Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.6.3

Source:	Ericsson, Honor, Huawei, InterDigital Inc., MediaTek Inc., NEC, Nokia, NTT DOCOMO, Oppo, T-Mobile USA, Vivo, ZTE corporation

Title:	Discussion on which layer reports the event triggered L1 measurements

Document for:	Discussion

# 1	Introduction

In RAN2#125bis RAN2 briefly discussed how the UE should report the L1 measurements when these are triggered by a L1 event. The options that were discussed were basically two:

- Measurements are sent over UCI

- Measurements are sent in a MAC CE.

This contribution discusses and evaluates these options from different aspects.

# 2	Discussion

To evaluate these options discussed (UCI and MAC CE), we think it is important to consider aspects specifically needed for the event triggered L1 measurements. The aspects we think are important to consider are:

- Flexibility; Can the report container cater for different events and events of different sizes?

- Reliability; If an event is sent, will the solution ensure that the information will be received by the gNB?

- Resource Utilization; Apart from the information bits, will the selected container add overhead including any retransmissions?

- Timing; From the point when the event is generated, will there be a significant and/or predictable delay until the event information is received in the gNB?

- Complexity; Will the needed changes in the specification be large and impact on a broad scale?

With this in mind, let’s look closer on the different alternatives in the following chapters.

### 2.5.1	MAC CE

Since MAC CEs are sent inside a transport block, they are automatically protected by HARQ retransmissions and the requirements on reliability can then be considered as fulfilled. The network can then follow the paradigm from L3 mobility to only trigger once, since the HARQ retransmissions ensure that the report reaches the network.

If the event triggered L1 measurements are carried by a MAC CE, there is no need to pro-actively send the report multiple times as the HARQ retransmissions ensure that the report reaches the NW without unnecessary retransmissions.

MAC CE score high in reliability and resource utilization.

In addition to this, it would also be possible to define different MAC CEs for different events if one wants to have full flexibility in the report. If one MAC CE is introduced for all events, another solution would be to introduce which event has triggered the L1 report in the MAC CE payload information. On top of this, MAC CE will also give flexibility as the format may be designed in a way to handle different length and different information to be include.

Event triggered L1 measurements carried by a MAC CE has the benefit of high flexibility.

Finally, since a MAC CE is self-contained (the headers indicate the content of the MAC CE), it can be sent in any available UL resource. For example, if the UE has an ongoing UL transmission, for which it is provided an uplink scheduling grant for a new transmission, the UE can simply include a MAC CE on the PUSCH according to channel prioritization procedures within MAC. This means the timing of a MAC CE is only relying on the time to get an UL resource. Also, if the NW has already provided the UE with a grant of sufficient size, the UE can directly transmit the MAC CE. If no grant is available, the UE can send a scheduling request to the network to acquire a grant.

The scheduling of a MAC CE is simple, and timing is only depending on an UL grant.

Implementation complexity is considered as low for MAC CE since RAN2 only have to implement new MAC CE(s) and potentially channel priority changes for the logical channels.

### 2.5.2	UCI

UCI is sent outside the transport block. Thereby, it is not protected by HARQ: if UCI is missed, it is lost. However, it is important that an event triggered L1 measurement report reaches the NW and in order to support this, there are essentially two alternatives to achieve the desired reliability for UCI: i) the retransmissions of UCI are polled explicitly by the NW, or ii) the UE repeats the procedure as long as the event is fulfilled. Of these solutions, the main issue is that it becomes complicated to define when the UE should disregard of a measurement report.

The reliability of reports via UCI is questionable as the UE does not understand whether a report has been (correctly) received by the network. Thus, to fulfil reliability aspects, UCI would suffer from poor resource utilization.

On top of this, what is clear is that UCI reporting has very little flexibility. The NW knows the format of the UCI that it receives in a certain UL resource, since the UE is mandated to follow the RRC configuration provided in the CSI report configuration. The only exception to this rule is the CSI part 1 and part 2 that are used for large UCI reports, where the content of CSI part 1 impacts the size of CSI part 2. To achieve the desired flexibility of the UE-initiated reporting, a new UCI design philosophy is needed: to allow for varying UCI size, the CSI part 1 and part 2 paradigm could be reused, and some fields in UCI could determine how the NW should interpret the content, for example by indicating which event was triggered.

Reports via UCI provide much less flexibility than reports via MAC CE.

To transmit a report over UCI, the UE only has to include the format in PUCCH or PUSCH. However, in order to do that there has to be allocated resources for the UE. Thus, it can be seen in a similar way as a grant; in order to not create waste, the network has to allocate sufficient resources for the UE. This means that from a timing perspective UCI has similar attributes as MAC CE.

The scheduling of a UCI is simple, and timing is only depending on an UL resources as provided by the network.

### 2.5.3	Summary

Based on the discussions we can summarize the results in a table.

|  | MAC CE | UCI |
| --- | --- | --- |
| Flexibility |  |  |
| Reliability |  |  |
| Resource Utilization |  |  |
| Timing |  |  |
| Complexity |  |  |

Based on the selection criteria flexibility, reliability, resource utilization, timing and complexity, it is our understanding that reporting the event triggered L1 measurement via a MAC CE is the most efficient and reasonable option.

Delivering event triggered L1-measurements in a MAC CE fulfils all the needed requirements.

Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	If the event triggered L1 measurements are carried by a MAC CE, there is no need to pro-actively send the report multiple times as the HARQ retransmissions ensure that the report reaches the NW without unnecessary retransmissions.

Observation 2	MAC CE score high in reliability and resource utilization.

Observation 3	Event triggered L1 measurements carried by a MAC CE has the benefit of high flexibility.

Observation 4	The scheduling of a MAC CE is simple, and timing is only depending on an UL grant.

Observation 5	Implementation complexity is considered as low for MAC CE since RAN2 only have to implement new MAC CE(s) and potentially channel priority changes for the logical channels.

Observation 6	The reliability of reports via UCI is questionable as the UE does not understand whether a report has been (correctly) received by the network. Thus, to fulfil reliability aspects, UCI would suffer from poor resource utilization.

Observation 7	Reports via UCI provide much less flexibility than reports via MAC CE.

Observation 8	The scheduling of a UCI is simple, and timing is only depending on an UL resources as provided by the network.

Observation 9	Delivering event triggered L1-measurements in a MAC CE fulfils all the needed requirements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Event-triggered L1-measurements are reported by the UE to the network via MAC CE.