3GPP TSG-RAN WG2 Meeting #127	R2- 2406923

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	8.7.4

Source: 	Canon Research Centre France

Title:	Discussion on additional priority for delay aware LCP

Document for:	Discussion

# Introduction

In the RAN2#126 meeting at Fukuoka, RAN2 discussed the issue of time critical data being delayed by non time critical data having higher priority.

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description)

To solve this issue, it has been agreed to study a dual priority mechanism:

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

In this contribution we discuss how the dual priority mechanism shall be designed so that it maintains the fairness of LCP algorithm.

# Discussion

LCP is designed to achieve fairness. After LCH restrictions, LCP is performed in two steps. First, the granted resources are allocated to each LCH by priority order up to their bucket capacity. Then in a second step if there are remaining resources they are allocated to each channel by priority with no size limit.

In Fukuoka meeting it was decided to enhance LCP to resolve the delay issue for low priority time critical data. Also, in Fukuoka meeting it was decided to use an additional priority for a LCH in case there are time-critical data to be sent.

In our understanding, when this priority switching mechanism is implemented, each time an LCH has time critical data available for transmission, the additional priority will be used by the LCP algorithm. The additional priority being higher than the legacy priority, the time critical data will have a better chance to be selected by the LCP algorithm to be transmitted within the current configured grant.

That’s also means that high priority non time critical data have less chance to be transmitted within the current configured grant. And since the data are non time critical, it is acceptable to delay the transmission until further grant. However, the LCP fairness being broken, the deprioritized high priority traffic is now prone to starvation.

Observation 1: When time critical data are prioritized, non time critical data having higher priority could miss a transmission opportunity

This is particularly true in the case where the granted resources cannot accommodate for the bucket size of all channels. In that case it may happen that high priority channel cannot be allocated transmission opportunity corresponding to the allowed bucket size.

It is therefore important that the LCP fairness is being restored at a further grant when there are no more time critical data to handle.

Proposal 1: Restore LCP fairness to prevent high priority non time critical data starvation.

Fairness restoration may be left to gNB implementation, it would be up to the gNB to provide more resources at next grant to accommodate for the resources lost by the deprioritized data. However, priority switching is an autonomous decision by the UE, so there are no means for the gNB to trigger fairness restoration unless additional signalling is implemented.

Observation 2: gNB is not aware of LCH priority switching.

When a further grant is received by the UE, if there are no time critical data to be sent, the UE can compensate the previously lost resources for the high priority LCH.

Proposal 2: Fairness restoration is implemented at the UE as a resource compensation mechanism

To implement fairness restoration, the UE shall first record the resource prejudice inflicted to the deprioritized LCH when a grant is received and time critical data on another channel is prioritized.

Then when a further grant is received, if the granted resources cannot accommodate for the resources previously lost for the high priority channel, then the UE may redirect resources currently allocated to the lower priority channel to the higher priority channel as a compensation.

For example, if the received grant can accommodate for the bucket size of the two channels, then the UE may decide to allocate the bucket of the low priority channel, in addition to the bucket of the high priority channel to the high priority channel.

Proposal 3: Under certain conditions, redirect resources allocated to low priority channel to higher priority channels

# Conclusion

In this contribution, we conclude the following observations and proposals.

Observation 1: When time critical data are prioritized, non time critical data having higher priority could miss a transmission opportunity

Proposal 1: Restore LCP fairness to prevent high priority non time critical data starvation.

Observation 2: gNB is not aware of LCH priority switching.

Proposal 2: Fairness restoration is implemented at the UE as a resource compensation mechanism

Proposal 3: Under certain conditions, redirect resources allocated to low priority channel to higher priority channels

# References

- Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024