3GPP TSG RAN WG2 #127			R2-2406924

Maastricht, NL, August 19th – 23rd, 2024

Source:	Sharp

Title:	Discussion on RRM Measurement Prediction

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

# Introduction

In RAN#103, the study on AI/ML Mobility [1] was approved. Some of the objectives of the study include the following aspects.

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity trade-offs [RAN2]

The following agreements were made in RAN2#125bis with respect to RRM Measurement Prediction (AI 8.3.2).

Agreements 1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results - We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predicition. - The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction： Spatial-domain prediction： RSRP difference to the actual measurement Temporal prediction：RSRP difference to the actual measurement measurement reduction rate as one KPI - As a first step we will focus on measurement prediction accuracy. FFS whether and what system level performance evaluation is needed

Further in RAN2#126, the following agreements were made.

Agreements

- For the cell level measurement prediction, start with consider a fixed value for L3 filtering in simulation.   FFS which fixed value

- For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result

# Discussion

In the SID [1], it is considered that the measurement predictions consider both UE-sided models and network-sided models. As agreed in the previous meetings, several aspects including cell-level, beam-level temporal measurement predictions have already been considered in the study. As shown in Figure 1, RRM measurements in general are utilized as inputs to the configured AI/ML model to produce predictions or inferences for future RRM measurements.

Figure 1: Measurement prediction data based on past measurements

Irrespective of the type of measurements, since the measurements are performed at the UE, the following aspects need to be considered. It can be expected that the UE performing the measurement would have access to the measurement data earlier than the network, which needs to wait until the UE reports the current measurements. Further, since reporting more measurements than existing procedures results in additional signaling, it can be expected that the UE typically has more measurement data relevant to itself than the network. Irrespective of the measurement reporting configurations, the UE may also independently perform measurements if required, without reporting the results to the network. Given these factors, it may be beneficial to first focus the study on UE-sided model. Although UE may have lesser compute/energy resources than the network, UE-sided model can potentially provide benefits associated with reduced signaling and possibly also reduce latency associated with inferences of AI/ML mobility models. Therefore, it is proposed that the UE-sided models are prioritized in the initial stages of the study.

Observation 1: The UE performing measurements would have access to the measurement data earlier than the network, since the network only has access to measurements after the UE reports them.

Observation 2: Since the UE does not report all measurements to the network, it can be expected that the UE has access to more measurements than the network.

Proposal 1: RAN2 to prioritize UE-sided model for RRM measurement prediction.

Since it is agreed in the SID [1] that any decisions on handovers are made by the network, in case UE-sided AI/ML mobility models are used, it would be essential that the UE communicates some information on the model predictions to the network. Since predicted measurements inherently differ from legacy measurement results, it might require further considerations during the study on whether the existing measurement reporting frameworks are sufficient for UE-sided AI/ML models, even though the predicted results may be in the same format (RSRP) as the legacy measurements. This is required to enable the network to differentiate clearly between actual measurements and predicted measurements.

Observation 3: Measurement report predictions are inherently different from legacy (or actual) measurement results, even though the two entities might represent the same quantities (e.g. RSRP and expected RSRP).

Proposal 2: RAN2 to discuss the applicability of the legacy measurement reporting procedure to predicted measurement results in the case of UE-sided AI/ML Mobility models.

# Conclusion

In this contribution, we have the following observations:

Observation 1: The UE performing measurements would have access to the measurement data earlier than the network, since the network only has access to measurements after the UE reports them.

Observation 2: Since the UE does not report all measurements to the network, it can be expected that the UE has access to more measurements than the network.

Observation 3: Measurement report predictions are inherently different from legacy (or actual) measurement results, even though the two entities might represent the same quantities (e.g. RSRP and expected RSRP).

In this contribution, we have the following proposals:

Proposal 1: RAN2 to prioritize UE-sided model for RRM measurement prediction.

Proposal 2: RAN2 to discuss the applicability of the legacy measurement reporting procedure to predicted measurement results in the case of UE-sided AI/ML Mobility models.

# References

[1] RP-240082 Revised SID on AIML for mobility in NR.