3GPP TSG-RAN WG2 Meeting #127	R2-2406929

Maastricht, The Netherlands, 19 – 23 August, 2024

Agenda item :	7.24.1

Source: 	Huawei, HiSilicon

Title: 	Impact of SDT signalling optimization for partial context transfer on RAN2

Document for:	Discussion and Decision

# 1 Introduction

Based on an LS from RAN3 in [1], during RAN2#124 meeting, RAN2 agreed the following:

Agreements

1 	RAN2 agrees to introduce a resumeIndication in the RRCRelease message sent to terminate SDT procedure, allowing the UE to initiate the RRC resume procedure immediately after receiving this RRCRelease message and after performing cell selection.  Same UE requirements as receiving paging.

- This functionality is an optional UE capability with signalling.

- Send LS to RAN3 after the CR agreable

The corresponding RRC CR was agreed in [2].

RAN2 has now received another LS from RAN3 in [3] where RAN3 informs that they endorsed CRs for Ts 38.300 and TS 38.401 to adopt the agreed solution. In addition, RAN3 indicated they are considering another solution to handle the non-SDT arrival during an ongoing SDT session and they asked whether RAN2 saw any impact or issue with this solution. RAN2 discussed this topic initially during RAN2#126 meeting and agreed the following:

=> Reply to LS: - No specification impact (for Rel-17 and 18) - However, there are RAN2 issues, like data loss (explain the issue) and Uu signaling overhead due to security being cleared (explain that as from Rel-15 …)

However, during the “[POST126][033][SDT] LS reply to RAN3 (ZTE)” e-mail discussion, two additional issues were raised:

- It was indicated that even though it was previously agreed that sending RRCSetup in response to SDT request is allowed, RAN2 had never considered that this would happen in the middle of an ongoing SDT procedure. It was assumed RRCSetup would be sent immediately upon SDT request, before any data between the UE and the network would be exchanged.

- Currently, the UE does not perform MAC reset upon reception of RRCSetup message. It was indicated this may have an impact on subsequent data transmission.

In this contribution, we analyse these two aspects in more detail.

# 2 Discussion

As mentioned in the introduction section, RRCSetup was mainly anticipated to be used by the network right after the SDT request is received by the network. As we indicated in our previous contribution on this topic in [4], RRCSetup is used during fallback from RRC Resume procedure to RRC connection establishment procedure and it makes the UE release its security context, all of its data radio bearers and its entire RRC configuration (except default values) causing additional procedures to be triggered in both AS and upper layers. That is why the fallback to RRC connection establishment is supposed to be utilized by the network only in exceptional cases, i.e. in case the retrieval of the UE context from the anchor gNB is not possible. On the other hand, if the network would like to send it in the middle of the SDT procedure while the data is being exchanged, it causes service and data interruption due to the release of existing PDCP and RLC entities. This issue has been recognized before and RAN2 agreed to mention it in the reply LS to RAN3.

Observation 1: Even though sending RRCSetup in the middle of an ongoing SDT procedure was not anticipated by RAN2 before and while it leads to service interruption and data loss, specifications do not prevent a network from doing that.

However, since RAN2 assumed that there will be no data exchange in between receiving RRCResumeRequest and sending RRCSetup from the network so far, MAC layer is not reset at the UE upon receiving RRCSetup. In case of SDT, some data may have been already sent from the UE to the gNB and some data may still reside in the MAC buffer. At the same time, all RLC entities established from scratch and the RLC state variables are set to their initial values. Assuming that RLC AM mode is used, this may lead to a situation where the RLC entity at the gNB receives an RLC SDU with an unexpectedly high SN value. If the SN of the received PDU will fall outside the reception window, the RLC SDU will be discarded. However, if it falls inside the reception window, it will be placed in the reception buffer. In consequence, the RLC entity will assume there is a number of SDUs missing and may send a status report where all the SDUs below the received SN are marked as missing. This may in turn lead to unnecessary retransmissions of the SDUs which might be still under an ongoing transmission. Such situation may repeat until reaching the SN of the residual SDU. When the SN of the residual SDU is reached, the new SDU with this SN will be in turn discarded as a duplicate which leads to data loss which should not happen for the RLC AM mode.

Figure 1. Unnecessary retransmissions and data loss due to residual data in the MAC buffer in RLC AM mode

Similar situation can happen in case SDT is used together with RLC UM mode. However, in this situation the RLC window will be advanced automatically when residual RLC SDU is received. In consequence, this will lead to a loss of all RLC SDUs with SN lower than the SN of the residual RLC SDU.

Figure 1. Data loss due to residual data in the MAC buffer in RLC UM mode

These situations would be avoided if MAC was reset, but this is currently not the case. Even if it was decided to specify MAC reset, this requires the UE to perform RACH to get synchronized with the gNB which would cause a delay in the connection establishment, similar to the one where the UE is moved to RRC INACTIVE state first. Also, it would mean the solution cannot be applied to legacy UEs. Another potential approach could be to assume that the gNB, by implementation, is able to detect the residual data received from the UE and discard it. This way the effect on RLC reception procedures would be avoided.

Proposal 1: RAN2 to discuss if it is acceptable for the UE not to perform MAC reset upon RRCSetup reception during an ongoing SDT procedure when some data has already been exchanged.

Depending on the answer to the question raised in Proposal 1, the reply LS to RAN3 should include this aspect, on top of the previously agreed ones.

Proposal 2: In the reply LS to RAN3, RAN2 should mention the potential issue on the residual data in MAC and clarify whether or not it can be handled by the network implementation (depending on the outcome of RAN2 discussion).

# 3 Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: Even though sending RRCSetup in the middle of an ongoing SDT procedure was not anticipated by RAN2 before and while it leads to service interruption and data loss, specifications do not prevent a network from doing that.

Proposal 1: RAN2 to discuss if it is acceptable for the UE not to perform MAC reset upon RRCSetup reception during an ongoing SDT procedure when some data has already been exchanged.

Proposal 2: In the reply LS to RAN3, RAN2 should mention the potential issue on the residual data in MAC and clarify whether or not it can be handled by the network implementation (depending on the outcome of RAN2 discussion).

# 4 References

- R3-234589, LS on SDT signalling optimization for partial context transfer, RAN3

- R2-2313652, Introduction of RRCRelease with resume indication for SDT [SDT_ReleaseEnh], Huawei, HiSilicon, China Telecom, Qualcomm, CATT, Lenovo, Orange, Vodafone, CMCC, China Unicom

- R3-242198, Reply LS on SDT signalling optimization for partial context transfer, RAN3

- R2-2405649, Discussion on RAN3 LS on SDT signalling optimization for partial context transfer, Huawei, HiSilicon