3GPP TSG-RAN WG2 Meeting #127							      R2-2406934

Maastricht, The Netherlands, 19 – 23 August, 2024

Title: 	Discussion on functionality-based LCM for UE-sided model for BM

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

# Introduction

In RAN2 #126 meeting, we have the following agreements for the UE-side model for BM case. In this paper, we discuss the FFSs based on the [POST126][032] email discussion, and some other issues based on the RAN1 progress.

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

# Discussion

## Discussion based on [POST126][032] e-mail discussion

### Network/UE side additional conditions

Based on the discussion in [POST126][032] email discussion, the gNB needs to know the supported functionality information as well as the applicable functionality information from the UE before configuring the UE to do the measurements prediction for both proactive reporting and reactive reporting, as we discussed in the email discussion:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability signaling.

- Applicable functionalities refer to functionalities that the UE is ready to apply for model inference.

The network side additional conditions are needed to be known to the network as part of the functionality applicability reporting, to make sure the consistency across the training and inference for UE-sided model. The details of what needs to be considered for the NW-side additional conditions are currently discussed in RAN1. RAN1 had the following agreements for the network side additional conditions.

RAN1#117: Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study) RAN1#116bis: Agreements Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Details of NW-side additional conditions:

Based on above, RAN1 has proposed to introduce the associated IDs to represent the network side additional condition, which is under discussion. We think there is no need to repeat this discussion in RAN2, and RAN2 can assume NW-side additional condition as associated ID.

Proposal 1: The details of what needs to be considered for the NW-side additional conditions can be left to RAN1 discussion. RAN2 assumes NW-side additional condition as associated ID.

The overall procedure to align the training and the inference can be described as follows:

- First, the UE trains the model based on the dataset and associates it with the related associated ID based on network configuration.

- The UE reports the applicable functionality to the NW, which is associated with the NW-side additional condition identified by the associated ID.

- NW selects the applicable functionality and determines the inference configuration based on the NW-side additional condition.

Signaling of the associated IDs:

Based on the above procedure, the key issue is how the NW can know the relations between the reported applicable functionalities and the associated IDs. Based on the [POST126][032] email discussion, we have the following options:

- Option 1: Joint decision by UE and NW: UE reports the applicable functionalities and the related associated ID to the NW to make the final decision. UE-side additional conditions are checked by UE internally.

- Option 2: UE decision: UE decides the applicable functionalities based on NW-side additional conditions indicated by the NW and UE-side additional conditions internally known by UE.

- Option 3: NW decision (not preferred by majority companies, not considered in this paper)

For option 1, when the NW configures the UE to report the applicable functionalities, the UE will report all the linked associated IDs after checking the UE side additional conditions. Based on this information, the NW determines which applicable functionality can be used in the current NW-side additional condition. When the NW side additional condition changes, the NW can do an update based on the associated ID reported earlier. This option is reasonable, but in some situations can cause some signaling overhead in case the network is able to currently support only a very few NW-side additional conditions/associated IDs. For, example, if the UE has applicable functionality for associated ID #1, #2, and #3, but the current NW supports only ID#4, then the UE reporting is meaningless.

In this case, we think option 2 is useful so that the NW may indicate the interested associated IDs when requesting the applicable functionality reporting. The UE only needs to report the applicable functionalities associated with the indicated associated IDs. Compared with option 1, the UE has more information (i.e. associated ID), and thus it can do the filtering. This option allows to reduce some signalling in Uu.

Based on the above analysis, both option 1 and option 2 are feasible and the choice between them will depend on the NW implementation and current NW-side additional conditions. We think it can be left to NW implementation which option to use. If the NW indicates associated ID(s) to UE, the UE checks both UE-side additional conditions and the indicated associated ID for determination of applicable functionalities; otherwise, the UE only checks UE-side additional conditions and reports them together with the supported associated IDs.

Proposal 2: Both option 1 (Joint decision by UE and NW) and option 2 (UE decision) should be supported for applicable functionalities determination:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs. (Option 1)

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition. (Option 2)

Granularity of the associated IDs:

For the granularity of the associated IDs, RAN1 has the following working assumption.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Based on the above working assumption, associated IDs for one applicable functionality may not be the same from one cell to another cell. It means that when the UE moves from one cell to another cell, the UE needs to update the reporting. For example, first the UE is in cell 1, it reports the applicable functionality and its associated IDs #1, #2, and #3. When the UE moves to cell 2, it reports the applicable functionality and its associated IDs #5, #6, and #7. Associated IDs #1, #2, and #3 should be allocated and valid in cell 1, while associated IDs #5, #6, and #7 should be allocated and valid in cell 2.

Each time the UE changes the serving cell, the legacy applicable functionality information will be invalid. When connects in the new cell, the NW needs to trigger the applicable functionality for the new cell, or the UE needs to update and report the applicable functionality automatically. It means that each time the cell change happens, we need to go through the applicable functionality reporting and inference configuration once again in order to configure the proper functionality in the new cell, which can cause AI/ML operation interruption. We think that some mechanism are needed to guarantee the continuity of the AI/ML operation even the associated ID is allocated locally in one cell.

Observation 1: The associated ID may not be the same from one cell to another cell, which can cause the AI/ML operation interruption during the handover.

Proposal 3: As the associated ID is not globally unique, RAN2 should discuss how to guarantee the continuity of the AI/ML operation during mobility between the cells.

Based on above, the associated IDs may not be the same in different cells. It means the functionalities and the related associated IDs information should be reported upon cell change. Since the UE capability is semi-static, the associated IDs cannot be part of UE capability reporting, and should be in the applicable functionality reporting.

Proposal 4: As associated IDs may differ from cell to cell, they cannot be part of UE capability reporting.

### Proactive and reactive reporting

Based on the previous discussion, the proactive reporting reuses the existing UAI framework, i.e., the UE reports applicable functionalities based on the network configuration and when applicability of some functionality changes, the UE updates the information (Step 3). After the UE reports applicable functionalities, the network provides another configuration which is used by the UE for inference (Step 5). When it comes to the reactive reporting, as can be seen in Figure 1, the procedure is quite similar. The main potential difference mentioned by the companies lies in the contents of the configuration provided in Step 3 and whether Step 5 is needed.

(a)                                                                      (b)

Figure 1. Applicable functionality reporting: (a) proactive reporting, and (b) reactive reporting

It should be also noted that UAI framework was agreed to be used at least for proactive reporting while what signaling is used for reactive reporting is left FFS. In our opinion UAI is appropriate for both types of reporting. In fact, we think that the fact the UE reports applicable functionalities upon being configured to do so by the network, is exactly what reactive reporting means. Such behavior is a usual way of implementing UAI reporting for various features and we can follow this principle for applicable functionality reporting. This is already reflected by the proposal from post-meeting e-mail discussion:

Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

We believe that the yellow highlighted text already covers reactive approach while the blue highlighted text covers proactive reporting.

Observation 2: Applicable functionality reporting via UAI covers both reactive and proactive approach.

When it comes to the second bullet, we believe that NW-side additional condition should be part of UAI configuration provided from the network to the UE in Step 3. As we proposed in Proposal 4, the network may optionally signal associated ID so that the UE may filter the applicable functionalities.

Proposal 5: The network may include NW-side additional conditions in the UAI configuration provided to the UE for applicable functionality reporting, i.e. in OtherConfig.

It has been argued by some companies that the network should have an option of providing full inference configuration already in Step 3. This has been justified by the fact that RAN1 is considering to include associated ID within CSI configuration. It should be however noted that what RAN1 has in mind is the time when the network provides the CSI configuration for model training purposes, not for applicable functionality reporting. Whatever configuration is needed for the UE to determine applicable functionality can be part of UAI configuration and we do not see a need for the network to blindly provide full inference configuration already before applicable functionalities are reported by the UE. This causes unnecessary signaling overhead and makes the UE behavior unclear e.g. is the understanding the UE will automatically apply some of these configurations? How is this decided? How is this reported to the network? At the same time, we fail to see any benefits in specifying such option. We propose not to discuss such possibility unless further progress is reached in RAN1 which requires RAN2 to revisit this issue.

Proposal 6: RAN2 should assume that the network provides full inference configuration to the UE only once the applicable functionalities are reported by the UE. This can be revisited in case some RAN1 agreements require RAN2 to do so.

### Discussion on applicability/non-applicability

This clause discusses the FFS on the applicability/non-applicability of a functionality.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

In both proactive reporting and reactive reporting, the UE reports the applicable functionality based on the network request. From the UE point of view, the UE determines whether the requested functionality is available or applicable based on the following three cases.

- Case 1 (functionality is not available): there is no available model for the indicated functionality at the UE side.

- Case 2 (functionality is available but not applicable): model is available for the indicated functionality at the UE side, but the functionality is not applicable in the current condition. For example, in option 1 (Joint decision by UE and NW), the UE checks and determines functionality is not applicable in the current UE side addition condition. In option 2 (UE decision), the UE checks both UE side addition condition and the NW side addition condition, and determines functionality is not applicable.

- Case 3 (functionality is available and applicable): there is available model for the indicated functionality, and the functionality is applicable in the current UE side addition condition, or in the current UE side addition condition and the indicated NW side condition from the NW.

In the email discussion, we have two open issue:

- Issue 1: whether the UE needs to report the availability of the functionality

- Issue 2: whether the UE needs to report the non-applicability of the functionality

For the first issue, the main question is whether the network needs to know the availability of the functionality if the functionality is not applicable. We think it depends on whether this information can be somehow useful to the network, e.g., if the network knew the non-applicability is due to lack of the proper AIML model at the UE, based on such information the network could trigger the model training for this functionality.

Proposal 7: RAN2 to discuss whether the knowledge about the non-availability of the reported functionality can be used by the network, e.g. to trigger AIML model training.

For the second issue, it seems a common understanding that an applicable functionality as indicated by a UE may not remain applicable all the time. UE internal conditions and/or NW side conditions can impact the status of the functionality. Once an applicable functionality is activated, the functionality has to be deactivated if it becomes non-applicable, otherwise system performance will suffer degradation. Therefore, when the functionality has been previously reported as applicable and it becomes non-applicable, such information needs to be delivered to the network. In our understanding, this is covered by the following proposal from the e-mail discussion already:

Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

However, it should be noted that as opposed to the changes of NW side conditions, UE internal conditions are unseen and unpredicted from NW perspective. Even if UE could have the flexibility to timely indicate the status changes of the applicable functionality due to UE internal conditions, signaling overhead is introduced, especially for the cases where the changes among different functionality statuses are frequent. On the other hand, if status updates frequency is limited by the network, then there may be a delay in UE reporting a functionality as non-applicable. In this case, the NW may need to configure more resources for performance monitoring, but this will introduce additional resource overhead.

Observation 3: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

From network perspective, it is preferential to choose such functionalities for inference which are likely to be used for a longer time and which are not prone to frequent status changes, e.g. due to dynamic changes of UE-side conditions. To help the network in choosing proper configurations, it would be useful if on top of bare applicability information, the UE also provided also some kind of “stability of applicability” indication. For example, if the UE knows a certain inference operation is likely to exceed its processing power or that the model is only applicable for the current UE speed, the network should be aware of this. Such indication does not have to mention an explicit reason why the functionality status may change soon, but instead the UE could for example indicate the time during which the UE commits to keep the applicability of the functionality. Such information would allow the network to achieve a better tradeoff between signaling/resource overhead and system performance guarantee and would be helpful for performing appropriate functionality management. For example, if the applicable status can be maintained for a relatively long time, less resources for functionality monitoring are needed. If the applicable status cannot be maintained for a relative long time or cannot be determined reliably, the NW may decide not to activate the functionality, even though it is applicable for the time being, or the NW may still choose to activate the functionality, but configure more resources for performance monitoring.

Observation 4: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 8: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated.

### Discussion on initial state and activation/deactivation

This section is to address the FFSes from RAN2#126:

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

In the email discussion, we have the three options as shown in below.

From [POST126][032] email discussion, Q3-2: Option 1: The applicable functionality is activated by receiving configuration for applicable functionalities in Step 5 (if needed). If configuration is not provided by the network, it means the functionality is not activated. Option 2: The applicable functionality is automatically activated if it is included in applicable functionality reporting (assuming the network configuration received in Step 3 is directly applied and the functionality is activated). Option 3: A functionality is activated based on a field in RRCReconfiguration in Step 3 or Step 5 (indicating the functionality activation status), and additionally via L1/L2 based activation/deactivation signaling. L1/L2 based activation/deactivation signaling is up to RAN1.

We think option 1 can be baseline, since is the basic principle of the RRC configuration. This is already reflected by the following proposal from the e-mail discussion:

Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

The separation of inference configuration and activation/deactivation can be considered as an enhancement. RAN2 often uses the activation/deactivation signalling at lower layers, e.g. via MAC CE, to do configuration management and reduce the signalling overhead and latency, while the initial state can be configured in the RRC configuration. For the UE side model for BM case, RAN2 needs to discuss the motivation and benefit for introducing such additional activation/deactivation step. If the motivation and benefits are confirmed, RAN2 can discuss the details, e.g., the initial state configuration and signalling used for activation/deactivation (e.g., L1/L2 signalling). This may also depend on the discussion held in RAN1, so RAN2 may wait for more RAN1 progress before handling this.

Proposal 9:  Additional step of activation/deactivation of functionalities configured for inference via L1/L2 signalling may be first discussed by RAN1 and RAN2 may discuss this further after RAN1 agrees on benefits and motivation.

### Supported functionality reporting

In this section, we would like to discuss the granularity of the supported functionality reporting for beam management case. If the AI/ML model for beam management can generalize well, we think the supported/applicable functionality can be per use case. In the beam management use case, there are a variety of Set A and Set B combinations, and the capability for the Spatial-domain and Temporal-domain prediction are also different. Considering the UE capability limitation, e.g., on memory storage and processing resource, it is not easy to have a generalized functionality for beam management use cases. We think a functionality can be associated with a set of (or a group of) Set A/Set B combination(s) for BM-Case1 or BM-Case2.

Observation 5: A functionality for Beam management use case can be associated with one Set A/Set B combination or a group of Set A/Set B combinations for BM-Case1 or BM-Case2.

When the gNB wants to use the AI/ML model to help with the beam management, the gNB first checks the supported functionality via the UE capability transfer procedure, e.g., the UE reports the supported functionality based on the gNB enquiry. To reduce the signaling overhead, the supported functionality in the UECapabilityInformation can be associated with a group of Set A/Set B combination(s). Following are two examples for classrification in the UE Capability reporting.

- sSupported functionality 1: functionality for BM-Case1 with the 4-to-X beam prediction, in which X can be 8, 16, or 32. As an example, one sub-functionality can be for BM-Case1 with the 4-to-8 beam prediction.

- sSupported functionality 2: functionality for BM-Case2 with the X-to-Y beam prediction, the combination of (X, Y) can be (4, 8), (8, 16), or (16, 32). As an example, one sub-functionality can be for BM-Case2 with the 4-to-8 beam prediction.

Proposal 10: For UE-side model for beam management, it is proposed RAN2 to agree that the supported functionality reported in the UE capability can be associated with a group of Set A and Set B combinations for BM-Case1 or BM-Case2.

## Discussion on monitoring

For the UE-side AI/ML model for BM-Case1 and BM-Case2, RAN1 had the following progress for performance monitoring.

RAN1 progress For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

RAN1 is still discussing the requirements and solutions for monitoring. In option 1, RAN1 has agreed measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report, which can be in the L1 signaling. For Option 2, the UE-assisted performance metric and how to report are still FFSs. RAN2 can wait for RAN1 for more progress, and if RAN1 would like RAN2 to check some parts of the solution, RAN2 can have further discussions in future.

Observation 6: RAN1 is discussing the solutions for monitoring, and agreed that the L1-RSRP and/or RS index can be reported to the NW in option 1. It can be left to RAN1 to decide whether L1 signaing can be reused.

Observation 7: In option 2, the UE-assisted performance metric and how to report are FFSs, RAN 2 can wait for RAN1 to decide the content to report.

Proposal 11: RAN2 waits for RAN1 progress related to performance monitoring for the UE-side AI/ML model for BM-Case1 and BM-Case2.

# Conclusion

In this paper, our observations and proposals are summarized as follows:

Network/UE side additional conditions

Observation 1: The associated ID may not be the same from one cell to another cell, which can cause the AI/ML operation interruption during the handover.

Proposal 1: The details of what needs to be considered for the NW-side additional conditions can be left to RAN1 discussion. RAN2 assumes NW-side additional condition as associated ID.

Proposal 2: Both option 1 (Joint decision by UE and NW) and option 2 (UE decision) should be supported for applicable functionalities determination:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs. (Option 1)

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition. (Option 2)

Proposal 3: As the associated ID is not globally unique, RAN2 should discuss how to guarantee the continuity of the AI/ML operation during mobility between the cells.

Proposal 4: As associated IDs may differ from cell to cell, they cannot be part of UE capability reporting.

Proactive and reactive reporting

Observation 2: Applicable functionality reporting via UAI covers both reactive and proactive approach.

Proposal 5: The network may include NW-side additional conditions in the UAI configuration provided to the UE for applicable functionality reporting, i.e. in OtherConfig.

Proposal 6: RAN2 should assume that the network provides full inference configuration to the UE only once the applicable functionalities are reported by the UE. This can be revisited in case some RAN1 agreements require RAN2 to do so.

Applicability/non-applicability

Observation 3: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

Observation 4: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 7: RAN2 to discuss whether the knowledge about the non-availability of the reported functionality can be used by the network, e.g. to trigger AIML model training.

Proposal 8: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated.

Initial state and activation/deactivation

Proposal 9:  Additional step of activation/deactivation of functionalities configured for inference via L1/L2 signalling may be first discussed by RAN1 and RAN2 may discuss this further after RAN1 agrees on benefits and motivation.

Supported functionality reporting

Observation 5: A functionality for Beam management use case can be associated with one Set A/Set B combination or a group of Set A/Set B combinations for BM-Case1 or BM-Case2.

Proposal 10: For UE-side model for beam management, it is proposed RAN2 to agree that the supported functionality reported in the UE capability can be associated with a group of Set A and Set B combinations for BM-Case1 or BM-Case2.

Performance monitoring

Observation 6: RAN1 is discussing the solutions for monitoring, and agreed that the L1-RSRP and/or RS index can be reported to the NW in option 1. It can be left to RAN1 to decide whether L1 signaing can be reused.

Observation 7: In option 2, the UE-assisted performance metric and how to report are FFSs, RAN 2 can wait for RAN1 to decide the content to report.

Proposal 11: RAN2 waits for RAN1 progress related to performance monitoring for the UE-side AI/ML model for BM-Case1 and BM-Case2.