3GPP TSG-RAN WG2 Meeting #127					           R2-2406935

Maastricht, The Netherlands, 19 – 23 August, 2024

Agenda Item:		8.3.2.1

Source:			Huawei, HiSilicon

Title:			Simulation results for RRM measurement prediction

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#126 meeting, the agreement on the priority of each use cases of RRM measurement prediction were reached as follows:

Agreements Case number; Prioritization; Evaluation scenario combination; target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell.

According to the agreement, case 2/3/4 are considered as high priority. In this paper, we will provide some simulation results for RRM measurement prediction, which will cover cases 2/3/4. In addition, we will also provide the detailed simulation assumptions and model description.

# 2   Discussion

## 2.1 Simulation assumptions

The dataset is an important part of training AIML models. Since it is agreed to generate dataset by system simulation, the alignment of simulation assumptions is the basis for discussion. In this section, we list the simulation assumptions used in our system simulation in Table 1, including the common assumptions and the company selectable assumptions chosen by us.

Table 1. Summarized simulation assumptions

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model, |

In addition, other simulation configurations used to generate the dataset are listed in Table 2, including the number of UEs, UE placement and dataset splitting method.

Table 2. Other simulation configurations

| Parameter | Value for FR1 and FR2 |
| --- | --- |
| UE number | 20 in total |
| UE placement | Uniform by cell |
| Dataset splitting | Data of 19 UEs used for training, data of the remaining UE used for testing |

## 2.2 RRM measurement prediction results

To facilitate the discussion of the prediction results, a unified report template has been agreed in RAN2 last meeting. Therefore, we list our RRM measurement prediction results in the format of the report template case by case, in which the average RSRP difference and the corresponding speed value are marked in the same colour. For each UE speed, we tried different number of epochs and obtain multiple different results, in which one epoch refers to the process of the AIML algorithm traversing the entire training dataset once.

### 2.2.1 Results of case 2 (FR1 intra-frequency temporal domain case B)

| Report parameters |  | Huawei |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | FR1: 30, 60, 90 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 8 |
|  | Number of RX beams | 8 |
|  | Measurement reduction rate(50%,…Note2) | 50% |
|  | Observation window (Note3) | N/A |
|  | Prediction window (Note3) | N/A |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 60000 |
|  | Testing | 1000 |
| AIML model input/output | Model input (Note 5) | Best L1 beam level RSRP, UE location |
|  | Model output(Note 6) | Best L1 beam level RSRP |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.03 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | Number of epochs; L3 Average RSRP diff. 200; 1.12/2.12/4.04. 500; 1.14/2.24/3.83. 2000; 1.14/2.30/4.23. |
|  | Other optional KPIs (e.g., L1 beam-level RSRP difference,) | Number of epochs; L1 Average RSRP diff. 200; 2.43/5.57/11.35. 500; 2.50/5.84/11.10. 2000; 2.61/6.02/13.30. |

### 2.2.2 Results of case 3 (FR1 to FR1 inter-frequency (frequency domain))

| Report parameters |  | Huawei |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | FR1: 30, 60, 90 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 8 |
|  | Number of RX beams | 8 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | N/A |
|  | Prediction window (Note3) | N/A |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 30000 |
|  | Testing | 500 |
| AIML model input/output | Model input (Note 5) | Best L1 beam level RSRP of serving cell, UE location |
|  | Model output(Note 6) | Best L1 beam level RSRP of inter-frequency neighbour cell |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.03 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | Number of epochs; L3 Average RSRP diff. 200; 3.09/2.69/6.58. 500; 2.61/2.18/5.97. 2000; 2.28/2.67/5.92. |
|  | Other optional KPIs (e.g., L1 beam-level RSRP difference,) | Number of epochs; L1 Average RSRP diff. 200; 3.54/4.04/9.14. 500; 4.17/4.74/9.51. 2000; 4.07/5.11/10.26. |

### 2.2.3 Results of case 4 (FR2 intra-frequency temporal domain case A)

| Report parameters |  | Huawei |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | FR2: 60, 90, 120 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 64 |
|  | Number of RX beams | 8 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | 800ms |
|  | Prediction window (Note3) | 400ms |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 60000 |
|  | Testing | 1000 |
| AIML model input/output | Model input (Note 5) | L1 beam level RSRP, UE location |
|  | Model output(Note 6) | L1 beam level RSRP |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.04 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | Number of epochs; L3 Average RSRP diff. 200; 10.03/10.47/11.47. 300; 10.15/11.15/12.18. 500; 10.83/13.94/12.43. |
|  | Other optional KPIs (e.g., L1 beam-level RSRP difference,) | Number of epochs; L3 Average RSRP diff. 200; 11.90/12.74/14.36. 300; 12.68/14.92/15.66. 500; 14.18/15.38/16.40. |

### 2.2.4 Analysis of the results

The impact of UE speed on the results is as expected, and the performance for 90km/h is much worse than that for 30km/h. Besides, it is observed based on the results above that the results in FR1_to_FR1 cases are better than that in FR2_to_FR2 case. This is because the channel has greater fading and changes more drastically at high frequency, and the effect of noise is amplified when the signal quality is low, making it difficult for the AIML model to learn the actual channel characteristics. One possibility is to consider “easier” simulation assumptions to check whether AIML performs better, e.g. companies could try evaluating models assuming a smaller inter-site distance or a shorter prediction window for simulation.

Proposal 1: RAN2 should check how AIML performs for FR2 with smaller inter-site distance (e.g. 100 m) or a shorter prediction window.

In addition, it is noted that the number of epochs has an impact on the performance of the model, because few epochs will lead to undertraining, while too many epochs result in overfitting. We think it would be useful if companies shared their assumptions on this parameter.

Proposal 2: It is recommended for companies to provide their assumption on the number of epochs together with the prediction results.

Furthermore, companies provide the predicted performance at present, but it is unclear what will be the effect on mobility performance. Therefore, how to translate these results into handover performance needs to be discussed.

Proposal 3: It is recommended to perform system level simulations in order to understand how the prediction results translate into handover performance.

### 2.2.5 Comparison of the results

Since companies may use different configurations to generate datasets, which may result in different final prediction results, the effects of these configurations should be excluded for a better study of the AIML model. Therefore, we propose to classify simulation results according to the simulation assumptions adopted by the companies and then analyse the prediction results.

Proposal 4: It is necessary to classify the results based on the simulation assumptions adopted by the companies before analysing them.

By comparing the prediction results of the companies with the same simulation assumptions, we can analyse the impact of each training parameter (e.g. dataset size, additional input, etc.) on the prediction performance. In addition, we can analyse the impact of the simulation parameters on the prediction performance, by comparing the prediction results of the companies with different simulation assumptions.

Proposal 5: It is recommended to analyse the RRM measurement prediction results in the following way:

- Analyse the impact of model training parameters on performance by comparing the results of companies with the same simulation assumptions.

- Analyse the impact of simulation parameters on performance by comparing the results of companies with different simulation assumptions.

# 3   Conclusions and recommendations

In this paper, we discuss RRM measurement prediction with following proposals:

Proposal 1: RAN2 should check how AIML performs for FR2 with smaller inter-site distance (e.g. 100 m) or a shorter prediction window.

Proposal 2: It is recommended for companies to provide their assumption on the number of epochs together with the prediction results.

Proposal 3: It is recommended to perform system level simulations in order to understand how the prediction results translate into handover performance.

Proposal 4: It is necessary to classify the results based on the simulation assumptions adopted by the companies before analysing them.

Proposal 5: It is recommended to analyse the RRM measurement prediction results in the following way:

- Analyse the impact of model training parameters on performance by comparing the results of companies with the same simulation assumptions.

- Analyse the impact of simulation parameters on performance by comparing the results of companies with different simulation assumptions.