3GPP TSG-RAN WG2 Meeting #127					           R2-2406936

Maastricht, The Netherlands, 19 - 23 August, 2024

Agenda Item:		8.3.2.2

Source:			Huawei, HiSilicon, China Telecom

Title:			Discussion on other aspects related to RRM prediction

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#125bis meeting [1], some agreements on RRM measurement prediction and simulation were reached as follows:

Agreements 1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results - We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal prediction. - The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction： Spatial-domain prediction： RSRP difference to the actual measurement Temporal prediction：RSRP difference to the actual measurement measurement reduction rate as one KPI - As a first step we will focus on measurement prediction accuracy. FFS whether and what system level performance evaluation is needed

Agreements to start evaluations - FR1-to-FR1 - Focus on intra-frequncy in time domain prediction for the purpose of measurement reduction - Study inter-frequency scenario in terms of which scenarios can be studied without requiring new channel model and also resolving any simulation assumptions (if possible). - FR2-to-FR2 - Focus on intra-frequency - Perform evaluation both in time and spatial domain

In RAN2#126 meeting [2], some agreements on RRM measurement prediction and simulation were reached as follows:

Agreements on simulation assumptions 1 For 2nd study goal i.e. to enhance handover performance, evaluation exercise will focus on FR2 to FR2 intra-frequency scenario. 2 For the evaluation exercise for 2nd study goal, RAN2 should assume that there is no reduction in measurement overhead 3 Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases 4 for RRM sub case 1, it is up to company to report L1 RSRP difference 5 Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured 6 For RRM sub case 1 and 3, it is up to company’s implementation whether L1 filtering is applied for input L1 beam level measurement. Companies are expected to report what L1 filtering they use in their simulation. (skipped)

Agreements 1 average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged (skipped)

=> We will start the evaluation with measurements prediction accuracy and model complexity. We can discuss system performance after we see which scenarios have good measurements prediction accuracy.

Agreements 1 For the cell level measurement prediction, start with consider a fixed value for L3 filtering in simulation. FFS which fixed value 2 For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result

In this paper, we will discuss some remaining issues for RRM measurement prediction.

# 2   Discussion

## 2.1 Beam level measurement prediction

According to the green highlighted agreement [1], we should also consider RRM measurement prediction for L3 beam level measurements. L3 beam level measurements are normally reported by the UE together with cell level measurements as they are useful for the gNB when deciding the RA resource to access the target cell. Hence, it is important to consider them when discussing AIML aided mobility measurement predictions, e.g., to achieve complete measurement reduction for UE power saving.

Observation 1: L3 beam level measurements are normally reported by the UE together with cell level measurements as they are useful for the gNB when deciding the RA resource to access the target cell. It is important to consider them when discussing AIML aided mobility measurement predictions.

What is more, they can be easily integrated into the studied AIML framework. For example, if we consider RRM Sub-use case 1 where L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated, then we can reuse exactly the same model, but only generate L3 beam level measurement results instead of cell level results at the end. For other sub-use cases we only need to change the training dataset to L3 beam level measurement results compared with cell level measurement prediction. For beam level measurement predictions, we can therefore distinguish similar three sub-use cases as for cell level measurements, i.e.:

- Case 1: To predict L1 beam level measurement results, then generate L3 beam level measurement results based on the predicted L1 beam level measurement results

- Case 2: To directly predict L3 beam level measurement results based on L3 beam level measurement results

- Case 3: To directly predict L3 beam level measurement results based on L1 beam level measurement results

In terms of simulation assumptions and studied KPIs, the agreements reached on cell level measurement prediction can be fully reused on L3 beam level measurement prediction. For the measurement accuracy, the only difference will be that we will have to refer to RSRP difference of beam-level measurement instead of cell level measurement. Also, we need to agree on the value of filter coefficient for deriving L3 beam level result. Therefore, we propose the following:

Proposal 1: In addition to cell level measurements, RAN2 should also evaluate the accuracy of L3 beam level measurement predictions by reusing the simulation assumptions made for cell level measurement prediction and using L3 beam level RSRP difference as a KPI. L3 beam filter coefficient should be aligned between the companies, e.g. a value of 4 can be used.

Proposal 2: For L3 beam level measurement prediction model, the companies may choose to simulate one of the following cases:

- Case 1: To predict L1 beam level measurement results, then generate L3 beam level measurement results based on the predicted L1 beam level measurement results

- Case 2: To directly predict L3 beam level measurement results based on L3 beam level measurement results

- Case 3: To directly predict L3 beam level measurement results based on L1 beam level measurement results

## 2.2 System performance KPI

In RAN2#125bis meeting [1], there is an FFS to discuss whether and what system level performance evaluation is needed. And in RAN2#126 meeting [2], it was agreed that system performance can be discussed after we see which scenarios have good measurements prediction accuracy. As can be seen in our companion paper in [3], it is possible to obtain results with satisfactory prediction accuracy. In some scenarios, the accuracy tends to be worse, e.g. the RSRP difference is higher than 3dB. However, we still do not know whether and what impact such level of an error has on the handover performance which is the essence of this study. In our opinion, the following items related to system performance KPI should be considered in the future work:

- Handover success rate

- Handover ping-pong rate

When it comes to handover success rate and handover ping-pong rate, in our understanding the goal is not to worsen these KPIs while reducing the measurement effort, at least for FR1 case. The main gain expected from the measurement prediction is to decrease the measurement efforts which translates into other benefits as mentioned above, e.g. signalling overhead or UE power consumption reduction. However, it should not be expected that handover success rate will be improved at the same time. Furthermore, in many of the current network deployments handover success rate is already very satisfactory in FR1 case.

Proposal 3: During the study, RAN2 can consider additionally the following system performance KPIs for RRM measurement prediction: handover success rate and handover ping-pong rate.

## 2.6 Comparison between UE-sided model and NW-sided model

According to the current SID, the RRM measurement prediction can be performed both at the UE side and NW side. The pros and cons for UE-sided model and NW-sided model should be therefore analysed by RAN2. We give a preliminary analysis on this issue as shown in Table 1.

Table 1: Pros and Cons for UE-sided model and NW-sided model

|  | NW-sided model | UE-sided model |
| --- | --- | --- |
| Pros | - Saves signaling overhead (the NW can predict some measurement based on other measurement reporting) - Improves throughput (no need for measurement gap at UE side) - UE power saving - Less complexity for LCM procedure (e.g. training, model/functionality handling). It depends on the progress of R19 AI for air interface | - Improve throughput (measurement gap at UE side may be reduced) - UE power saving (To be confirmed as UE performs less measurements but on the other hand needs to use more computational power) |
| Cons | - If NW-sided model needs some information (e.g. for inference) from UE side, it may need UE reporting of such information in Uu interface. | - There may be more complexities for LCM procedure for UE-sided model than NW sided model. |

Proposal 4: RAN2 to study the pros and cons for NW-sided model and UE-sided model respectively.

# 3   Conclusion

In this paper, we discuss RRM measurement prediction with the following proposals:

Observation 1: L3 beam level measurements are normally reported by the UE together with cell level measurements as they are useful for the gNB when deciding the RA resource to access the target cell. It is important to consider them when discussing AIML aided mobility measurement predictions.

Proposal 1: In addition to cell level measurements, RAN2 should also evaluate the accuracy of L3 beam level measurement predictions by reusing the simulation assumptions made for cell level measurement prediction and using L3 beam level RSRP difference as a KPI. L3 beam filter coefficient should be aligned between the companies, e.g. a value of 4 can be used.

Proposal 2: For L3 beam level measurement prediction model, the companies may choose to simulate one of the following cases:

- Case 1: To predict L1 beam level measurement results, then generate L3 beam level measurement results based on the predicted L1 beam level measurement results

- Case 2: To directly predict L3 beam level measurement results based on L3 beam level measurement results

- Case 3: To directly predict L3 beam level measurement results based on L1 beam level measurement results

Proposal 3: During the study, RAN2 can consider additionally the following system performance KPIs for RRM measurement prediction: handover success rate and handover ping-pong rate.

Proposal 4: RAN2 to study the pros and cons for NW-sided model and UE-sided model respectively.

# 4	Reference

- R2-2404102	Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China

- (Draft) Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan (https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_126/Report)

- R2-2406935	Simulation results for RRM measurement prediction	Huawei, HiSilicon