3GPP TSG-RAN WG2 Meeting #127					           R2-2406937

Maastricht, The Netherland, 19 – 23 August, 2024

| Agenda Item: | 8.3.4.1 |
| --- | --- |
| Source: | Huawei, HiSilicon |
| Title: | Discussion on simulation assumptions and evaluation methodology for RLF failure prediction |
| Document for: | Discussion and Decision |

# 1   Introduction

According to RAN2#126 meeting, the following agreements have been achieved:

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this paper, we will further discuss the RLF specific methodology and simulation assumptions.

# 2   Discussion

## 2.1 RLF specific simulation assumptions

The alignment of simulation assumption is the basis for discussion on this topic. Since RAN2 has discussed these in detail for the case of RRM measurement prediction, we should reuse the conclusions as much as possible to reduce the workload. Table 1 lists the simulation assumptions for FR2, which is prioritized for RLF prediction.

Table 1. Summarized simulation assumptions for FR2

| Parameter | Value for FR2 |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UE rotation, Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Other assumptions are not precluded. |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Other assumptions are not precluded |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200 m |
| BS Antenna height | 10m |
| UE Antenna height | 1.5 m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Companies report which of the following models they used: wrap round model, circle-bouncing model, boundary-terminated model |

Proposal 1: Reuse the simulation assumptions listed in Table 1.

According to the agreement, RAN2 will focus on RLF due to T310 expiry, which is modelled in 36.839 in the following way:

- Start T310 when CQI<Qout;

- Keep T310 running if CQI<Qin;

- RLF is declared when the T310 expires.

This process is presented in Figure 1, which is excerpted from TR 36.839.

Figure 1: A handover failure is declared for case 1.

The above process shows that Qin and Qout are key factors for RLF modelling. Therefore, RAN2 needs to reach a consensus on their values. The recommended values in TR 36.839 can be reused:

Excerpted from TR 36.839 When a UE tracks RLFs according to TS 36.300 [3], Qout is monitored with a 200ms window and Qin is monitored with a 100ms window (as specified in TS 36.133 [2]). Both windows are updated once per frame, i.e. once every 10 ms with the measured wideband CQI value. The RLF and HO failure modelling related parameters are shown in the table 5.2.1.3.1 below: Table 5.2.1.3.1: The parameters for determine the RLFs and the PDCCH failures. Items; Description. Qout; -8 dB. Qin; -6 dB. T310; 1s (the default value in 36.331). N310; 1. T311; Not used for calibration (since RLF recovery is not simulated in the calibration). N311; 1.

Proposal 2: The recommended values in TR 36.839 can be reused, including:

- Monitoring windows of Qout and Qin are 200ms and 100ms, respectively;

- The sample period is 10ms;

- The values of Qout and Qin are -8dB and -6 dB, respectively.

## 2.2 Methodology for RLF prediction

According to the agreement, there are two ways to predict RLF using an AI/ML model, i.e. direct prediction and indirect prediction. For direct prediction method, the historical RLF events should be collected with the measurement results to generate the dataset. However, RLF does not always happen, which means less data can be collected. In addition, the direct prediction method is to infer whether RLF will occur in the future based on the measurement results in the past, which implies that the AI/ML model needs to learn the modelling method of RLF, which undoubtedly increases the difficulty of prediction. On the contrary, the indirect prediction method has an advantage of offering more data samples for training the AI/ML model. In addition, the principle of predicting a future result by using the historical measurement result is similar to that of RRM measurement prediction. Therefore, some previous work of RRM measurement prediction can be reused (e.g. model architecture, training method, etc.), thereby reducing workload.

Proposal 3: For RLF prediction, RAN2 can start to evaluate the simulations using indirect prediction as a baseline.

Since the output of the model is the predicted L1 measurement result in the indirect prediction method, it is simple to calculate whether and when an RLF will occur according to the model output. Further, when the model makes multiple predictions with a sliding window, a time period in which an RLF occurs can be obtained. Therefore, expected RLF time or time period can be considered as the RLF prediction result for the indirect prediction method.

Proposal 4: Expected RLF time or time period can be considered as the RLF prediction result for the indirect prediction method.

## 2.3 Template of simulation results

A unified report template has been agreed in RAN2#126 meeting, and the simulation template table has been captured in A.1 of TR 38.744. As mentioned in section 2.2, the indirect prediction method is similar to the RRM measurement prediction, thus this template can also be used for RLF prediction.

Proposal 5: The simulation template table in A.1 of TR 38.744 can be reused for indirect RLF prediction at least.

# 3   Conclusion

Proposal 1: Reuse the simulation assumptions listed in Table 1 as much as possible.

Proposal 2: The recommended values in TR 36.839 can be considered as the baseline, including:

- Monitoring windows of Qout and Qin are 200ms and 100ms, respectively;

- The sample period is 10ms;

- The values of Qout and Qin are -8dB and -6 dB, respectively.

Proposal 3: For RLF prediction, RAN2 can start to evaluate the simulations using indirect prediction as a baseline.

Proposal 4: Expected RLF time or time period can be considered as the RLF prediction result for the indirect prediction method.

Proposal 5: The simulation template table in A.1 of TR 38.744 can be reused for indirect RLF prediction at least.