3GPP TSG-RAN WG2 Meeting #127	R2-2406939

Maastricht, NL, 16 – 20 August, 2024

Agenda Item:	8.7.4

Source: 	Canon Research Centre France

Title:	Discussion on Delay status report

Document for:	Discussion and decision

# Introduction

During RAN2#126 in Fukuoka the following agreements were recorded in [1] (the number of reported values was left for further study):

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

# Discussion

During last meeting it was agreed to enhance delay status reporting with multiple pairs of data volume and remaining time.

In this paper we discuss possible triggering enhancement with regards to the multiple pairs of data volume and remaining time, and whether an indication of PDU Set Importance (PSI) needs to be included in DSR.

## Trigger

According to R18 DSR, a single pair of data volume and remaining time is allowed in one DSR for each LCG. Only data which have not been reported in prior DSR can trigger a new DSR (2).

For one LCG, for each LCH of the LCG, a DSR is triggered when the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG.

Considering a situation in a DSR comprising multiple pairs, with each pair corresponding to a different remaining time threshold, we should decide which threshold is used as a trigger for the DSR.

To keep the design simple, we can use a single threshold to trigger the DSR, for example the lowest remaining time among the different pairs can be used by default. Of course, the network can configure one of the pair thresholds to be used as a trigger.

Proposal 1: The remaining time associated with at least one data volume can be used as a trigger for the DSR.

Since there are multiple pairs, we should be able to report some volumes multiple times if their remaining time changes below a second threshold. Otherwise, it will be impossible for the gNB to track the progress of the UE TX buffer filing. Indeed, some pairs may have new data volumes that need to be reported.

Thus, it is important that the DSR can be triggered if at least one of the pairs has new data which have passed the corresponding remaining time threshold.

Proposal 2: If there are data which have passed a second threshold for one of the already reported pairs then a new DSR shall be triggered

As some pairs could have data that have already been reported in a previous DSR, we should add a data novelty indication for each pair. This allows the gNB to keep track of the progress of each pair in volume. For example, if already reported data are now below a lower threshold (call it “th2”) and some new arrived data are already below “th2”, then two pairs must be reported and the new one shall be distinguishable from the old one.

Proposal 3: Define a data novelty indication for each pair.

Since an already reported data volume will have a smaller remaining time it should be useful to identify each pair so that the gNB can track the remaining time evolution of a given data volume. If a pair is identified then it will not be necessary to add the remaining time information when the novelty flag is false.

Proposal 4: Define a pair identification based on the first reported data volume.

## PSI

PDU Set importance indication is used for congestion management. Except the congestion condition, PSI has no use for the network. Thus, it is unlikely that there would be any benefits to include PSI indication in the DSR. Indeed, most of the time it will not help the network to make scheduling decisions.

Proposal 5: Do not include PSI indication in the DSR

# Conclusion

According to the analysis in section 2, it is proposed:

## Trigger

Proposal 1: The remaining time associated with at least one data volume can be used as a trigger for the DSR.

Proposal 2: If there are data which have passed a second threshold for one of the already reported pairs then a new DSR shall be triggered.

Proposal 3: Define a data novelty indication for each pair.

Proposal 4: Define a pair identification based on the first reported data volume.

## PSI

Proposal 5: Do not include PSI indication in the DSR

# References

- Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024

- NR; Medium Access Control (MAC) protocol specification, (Release 18)