3GPP TSG-RAN WG2 Meeting #127	R2- 2406940

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	8.7.5

Source: 	Canon Research Centre France

Title:	Discussion on RLC AM Enhancements

Document for:	Discussion

# Introduction

In the RAN2#126 meeting at Fukuoka, RAN2 discussed how to avoid unnecessary retransmission by RLC AM and reached the following agreement:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

Tx initiated approach was clarified by following agreement:

For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side

Rx initiated approach was also clarified:

For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

Also, during the same meeting, timely retransmission by RLC was discussed and the following agreements were reached:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In this contribution we discuss both how to avoid unnecessary retransmission (2.1), and how to ensure timely RLC retransmission (2.2).

# Discussion

## Avoid unnecessary retransmission

### TX approach

Generally, TX side has the knowledge of PDU Set information. PDCP layer is already able to monitor each PDU Set delay budget and to maintain remaining time indication for each PDU Sets.

Inter layer information circulation about PDU Sets is already implemented between MAC layer and PDCP layer. DSR triggering in MAC relies on PDU Set remaining time information provided by PDCP layer.

Observation 1: Necessary mechanisms for obsolete SDU detection by RLC transmitter are already available.

When PDCP transmit entity discards obsolete SDUs, it sends a discard notification to the PDCP receiving entity.

Observation 2: Discard notification and corresponding RX side processing exists at PDCP layer.

Proposal 1: Use PDCP PDU Set discard mechanism as baseline for RLC TX initiated PDU Set discard.

### RX Approach

Contrary to TX side, RX side has no knowledge of PDU Sets at any layer. This is because PDU Sets information, including PDU Set boundary and sequence numbers, are not included inline.

How RX side can determine that an SDU is obsolete is therefore a difficult issue.

In [2], the authors suggest that the RX side sends a fake ack when an SDU failed to be received before being obsolete. To define SDU obsolescence the authors suggest to use a dedicated timer starting a discard timer for each received SDU. In [3] and [4], the authors suggest to use the t-Reassembly timer as SDU obsolescence indication.

Observation 3: RX approach seems based on t-Reassembly timer as PDU obsolescence indication.

However, t-Reassembly timer is used as a trigger to send a nack indication to the TX side to prompt SDU retransmission. To our understanding that would mean that retransmission would only be achieved through autonomous retransmission in that case.

Proposal 2: Adopt RX approach only if autonomous retransmission is adopted.

## Timely RLC retransmission

### Autonomous retransmission

One way to reduce the retransmission delay is to anticipate the retransmission of PDUs with unknown status. The unknown status is indicative of the PDU having been transmitted to the receiver but not acknowledged as being received or detected lost by the receiver.

This enables earlier and more efficient retransmission of some PDUs, which thereby minimizes the overhead when compared to systematic retransmission.

Proposal 3: Consider autonomous retransmission of PDUs with unknown status.

The overhead may be further reduced if the autonomous retransmission of PDUs with unknown status is not systematically triggered. For example, the autonomous retransmission of PDUs having an unknown status may occur following the transmission of all PDUs of a PDU Set.

Proposal 4: Further study the criteria for triggering autonomous retransmission of PDUs with unknown status.

### Enhanced status report

In [4], the authors suggest to speed-up the transmission of status report by the receiving side by the introduction of new triggers, for example, in [5], the authors suggest using a timer-based trigger.

However, while triggering status report either periodically or based on PDB may improve the transmission responsiveness it will not be sufficient in all cases. For example, when a status report timer elapses while the t-Reassembly timer is still running, there will be no nack information sent to the transmitter.

Observation 4: increasing the status report frequency is not useful when the t-Reassembly timer is still running

t-Reassembly timer can be reduced (or even set to 0), which increases the responsiveness of the RLC receiver once the sequence number gap is detected. But reducing the t-Reassembly timer increases the number of false missing detections, which can then increase the overhead across the network.

Observation 5: Enhanced status report costs extra resources in downlink.

In [6], the authors propose to discard the prohibit timer to insure timely transmission of the status report. But the prohibit timer is set by the network for reasons that are independent from the XR application itself, and therefore we think that it should not be ignored when set.

Proposal 5: Do not alter the prohibit timer when set by the network.

Proposal 6: Do not enhance the status report.

### Enhanced polling

The maximum polling frequency can be configured as one polling bit every 4 packets. But polling frequency cannot match the PDU Sets boundaries because PDU Set boundaries are variable.

In [8], the authors suggest to reduce the t-PollRetransmit timer below 5 ms.

Observation 6: Increasing the polling frequency can result in a large number of non-useful status reports being sent.

In [5] and [7], the authors suggest initiating polling by the transmitter based on remaining time information.

However, if the polling bit triggers a status report based on PDB, but the t-Reassembly timer is still running, there will be no nack information sent back to the transmitter.

Observation 7: Autonomous retransmission is necessary to overcome the delay induced by the t-Reassembly timer.

# Conclusion

In this contribution, we conclude the following observations and proposals.

## Avoid unnecessary retransmission

### TX approach

Observation 1: Necessary mechanisms for obsolete SDU detection by RLC transmitter are already available.

Observation 2: Discard notification and corresponding RX side processing exists at PDCP layer.

Proposal 1: Use PDCP PDU Set discard mechanism as baseline for RLC TX initiated PDU Set discard.

### RX Approach

Observation 3: RX approach seems based on t-Reassembly timer as PDU obsolescence indication.

Proposal 2: Adopt RX approach only if autonomous retransmission is adopted.

## Timely RLC retransmission

### Autonomous retransmission

Proposal 3: Consider autonomous retransmission of PDUs with unknown status.

Proposal 4: Further study the criteria for triggering autonomous retransmission of PDUs with unknown status.

### Enhanced status report

Observation 4: increasing the status report frequency is not useful when the t-Reassembly timer is still running

Observation 5: Enhanced status report costs extra resources in downlink.

Proposal 5: Do not alter the prohibit timer when set by the network.

Proposal 6: Do not enhance the status report.

### Enhanced polling

Observation 6: Increasing the polling frequency can result in a large number of non-useful status reports being

Observation 7: Autonomous retransmission is necessary to overcome the delay induced by the t-Reassembly timer.

# References

- Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024

- R2-2404596, Discussion on RLC AM enhancements, Huawei, HiSilicon

- R2-2404293, RLC AM enhancements for XR, Nokia, Nokia Shanghai Bell

- R2-2404197, RLC AM retransmission enhancements, Xiaomi

- R2-2404850, RLC re-transmission enhancements for XR, ITRI

- R2-2405002, RLC enhancements for XR, InterDigital

- R2-2405445, Solutions for RLC AM retransmission enhancement, TCL

- R2-2405032, Discussion on RLC Retransmission Enhancements in XR, CMCC