3GPP TSG-RAN WG2 Meeting #127	R2-2406941

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT functionalities

Document for: Discussion and Decision

# 1	Introduction

In RAN2#126 meeting, the following agreements on functionality were achieved:

RAN2#126 agreements on functionality: - Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. - Legacy NR BSR/SR is not needed for A-IoT communication. - FFS whether further indication of device message size/status is needed - AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

Based on the current RAN2 progress and focusing on the issues without conclusions, this contribution discusses data transmission, information visible to reader, multiplexing, message size reporting and AS scheduling information.

# 2	Discussion

## 2.1	General aspects and high-level principle for the study item

Considering that a sampling frequency offset (SFO) for A-IoT could be like 105 PPM (10% timing error), as analysed in [1], it is not possible for A-IoT devices to have time domain slot alignment. Meanwhile, an A-IoT system without time domain alignment has been under study by RAN1. Some timing acquisition signals for both device-to-reader and reader-to-device transmission need to be studied to address the timing acquisition issue.

R1#116 agreement At least the following time domain frame structure is studied for A-IoT R2D and D2R transmission. - For R2D transmission, - A R2D timing acquisition signal (e.g. R2D preamble) is included at least for timing acquisition and for indicating the start of the R2D transmission in time domain. - For D2R transmission, - A D2R timing acquisition signal (e.g. D2R preamble) is included at least for timing acquisition and for indicating the start of the D2R transmission in time domain. - FFS other necessary component(s), e.g. midamble, postamble, periodic sync signal, control fields, guard period

Since the timing accuracy of A-IoT device could be extremely low, a device cannot work well in an accurately synchronous system by using legacy timing methods in 3GPP. Therefore, unlike NR, the new interface for Ambient IoT would be an asynchronous system, similar to RFID.

Observation 1:	The Ambient IoT should be an asynchronous system.

## 2.2	Data transmission

In RAN2#125-bis, it is agreed that SDAP, PDCP and RLC layer are not supported:

RAN2#125-bis agreement: - SDAP is not supported for UP protocol stack. - PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 discussion) and any other really needed functionalities. - RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. - RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2

Based on the discussion in SA2 and the solutions captured in SA2 TR, there would be an A-IoT specific upper layer above AS layers. For data transmission procedure, RAN2 can assume that there will be at least an upper layer, which is decided by SA2, to carry A-IoT data, e.g. an application and/or NAS-like layer. After the A-IoT data is delivered from the upper layer, it can be transmitted just as one container/payload in AS lower layer, i.e. an AS layer SDU.

Additionally, as captured in RAN2#125-bis agreement, it is not necessary to support RRC layer, see below:

RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

According to the agreement, a new AS protocol on top of A-IoT MAC layer or A-IoT MAC layer can be used to carry the upper layer data if there is no RRC layer. Which layer is used to carry the container can be decided in SI later phase or WI phase. Then, to the AS layer, the e.g. NAS-like layer message is just one SDU to be encapsulated in e.g. A-IoT MAC layer.

Proposal 1:	For A-IoT, RAN2 assumes there will be the upper layer(s), which is to be decided by SA2 (e.g. an application layer and/or NAS-like layer). For A-IoT data transmission, the upper layer data (SDU) is transmitted as the payload in AS layer.

## 2.3	Information visible to reader

In RAN2#125-bis meeting, there were some discussions on what information can be visible to reader and the following FFS was given to allow companies to analyse further.

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

First of all, the number of devices should be visible to the reader, e.g. at least an indication of whether the service is triggered for a single device or multiple devices. As discussed above, the A-IoT paging identifier is provided by CN to select one or multiple devices. Since RAN2 agreed to support both contention-free access and contention-based access, it is necessary to have such information for the reader to trigger different types of random access procedures (contention-free access or contention-based access). If only one device is triggered to access, contention-free access can be used. When multiple devices are triggered, contention-based access procedure will be applied. In such case, the (rough) number of devices to be triggered can be also provided to reader from CN. It is helpful for the reader to allocate the appropriate amount of resources for the targeted devices. For example, if 100 devices are triggered by the service, the reader could allocate about 100 access occasions (or a few more) for these devices to access by being aware of the number of devices.

The service type (i.e. inventory and/or command) could be another useful information for the reader. Assuming that the command procedure follows the inventory procedure, the service type information can help the reader to know whether there is a following command procedure after the inventory procedure. If the service type indicates inventory-only, the reader will trigger/indicate another device to report device ID after receiving the device ID from one device. If the service type indicates an inventory and command combined procedure, the reader will wait for the following command message after receiving a device ID and sending the device ID to CN.

Proposal 2:	RAN2 assumes that the following information can be visible to the reader and useful for AS procedure:

- Whether single or multiple devices are triggered; and the (rough) number of devices to be triggered, if multiple devices are triggered;

- The service types (i.e. inventory and/or command).

## 2.4	Multiplexing

In existing Uu MAC layer, MAC layer supports multiplexing of data from multiple logical channels, and multiplexing of data from logical channel and MAC CE. With respect of multiplexing of data from multiple logical channels, RAN2#126 agreed that “Multiple “AIoT logical channels” for upper layer data are not supported”. Based on the agreement, it is obvious that multiplexing of data from different A-IoT logical channels is not needed for A-IoT. With respect of multiplexing of upper layer data, i.e. MAC SDU, and control information, i.e., MAC CE, although it may cause additional A-IoT device complexity, it is feasible for A-IoT devices to support such multiplexing if needed.

Observation 2:	From A-IoT device complexity perspective, it is feasible to support the multiplexing (e.g. MAC SDU and MAC CE, if any), while it may cause additional device complexity.

## 2.5	Message size information

In RAN2#126 meeting, the following agreements on functionality were achieved:

RAN2#126 agreements on functionality: - Legacy NR BSR/SR is not needed for A-IoT communication - FFS whether further indication of device message size/status is needed

To better schedule the next D2R message, a reader needs to know the (rough) D2R message size, especially when the size of D2R message is variable due to the following factors:

- For inventory use cases, the size of D2R message (e.g. device ID) may be not unified. As described in SA1 TR 22.840, for inventory, the size of a D2R message is primarily determined by the length of EPC (Electronic Product Code). The EPC is a unique identifier for an item that can be used in supply chain management systems to track and manage products, particularly in RFID (Radio Frequency Identification) systems where EPCs are often used. The EPC length can vary from 96 to 496 bits according to EPC Tag Data standard [2], and directly impacts the information capacity about the product. For example, longer EPC can include more details like manufacturer and product type, and can also be used to associate a larger number of products.

- For command use cases, the size of D2R message (e.g. response to different commands) is also variable. As described in SA1 TR 22.840, in different scenarios, D2R message size varies due to the different stored information. In RFID systems, specific memory banks or variable data can be indicated by a Read Command due to the flexibility and customization options.

As analysed above, the D2R message size, e.g., the device ID or the response to command, is variable. Hence, it is useful for the reader to be aware of the D2R message size through device report or CN indication:

- CN indication: As an alternative approach, CN can indicate the expected data size of the following D2R transmission to the reader when sending DL command. Then the reader can schedule the corresponding D2R message according to the CN’s indication, e.g., indicating TBS together with transmitting the DL command. Compared with device reporting-based approach, this can eliminate the need of two additional steps (e.g., BSR-like and scheduling MAC CEs) during the command procedures.

- Device report: The device can indicate the (rough) size of the next D2R data to be transmitted. As for command use cases, a device can know the data size to be transmitted only after receiving the command message. In such case, at least one interaction between device and reader is needed for scheduling the next D2R message. For this purpose, two additional steps are needed between R2D command and D2R response, i.e., one for message size reporting (e.g., via BSR-like MAC CE) and one for scheduling message for D2R response (e.g., scheduling MAC CE). This will increase the additional signalling overheads and reduce the transmission efficiency. Furthermore, size granularity of message size reporting impacts the D2R transmission overhead. For example, a more accurate message size is better for scheduling by reader, but takes more overheads.

Therefore, if the CN can provide message size information for both inventory and command use cases, it will be sufficient.

Observation 3:	There are two alternatives for the reader to better schedule suitable TBS for D2R transmission,

- Alternative 1 (seems sufficient): CN can provide the information on the (rough) expected data size of following D2R transmission; and/or

- Alternative 2: In D2R, the device can indicate the (rough) size of the next D2R data to be transmitted (e.g. device ID size).

## 2.6	AS scheduling

### 2.6.1	AS temporary ID for scheduling

In RAN2#125-bis, it is proposed to discuss whether a radio network temporary identifier (e.g. C-RNTI like) is needed for A-IoT device. This RNTI-like ID concept can be used to indicate that the data transmission is targeted to a specific UE in case other UE decodes the packet. Similarly, this mechanism can be also reused by A-IoT device. However, the RNTI-like design should be in RAN1 scope. By using this, the device can know that the data is transmitted to itself (not others) in physical layer without the need of decoding the data in MAC layer.

Observation 4:	RAN2 assumes that some AS scheduling temporary ID (RNTI like) may be needed to indicate the specific target device for data transmission/reception. It is to be discussed/designed in physical layer.

With respect of the generation/allocation of the AS temporary ID, it is more reasonable to reuse a current random ID for random access or leave the reader to generate the temporary ID for device(s) considering different scenarios. Specifically:

- For contention-based case, RAN2#126 agreed that for 4 step RA “A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).” It is simpler to reuse this existing ID than to introduce a new temporary ID, which can reduce the complexity of device implementation (e.g. on random generation). Therefore, after contention resolution, the device can reuse the existing random ID in Msg1 as the temporary ID for further R2D reception and scheduling.

- For contention-free case, for better scheduling, reader can generate and allocate an AS temporary ID to each device at the beginning, so that the device can directly use the allocated temporary ID for R2D reception. In addition, if different devices performing contention-free access generate the AS temporary ID by themselves, it may occur “an AS temporary ID conflict” between different devices, which leads to the case where a device incorrectly receives data of another device. Therefore, it is more reasonable for the reader to generate and allocate the AS temporary ID for each device in this case.

Observation 5	The AS scheduling temporary ID can be generated by the device and confirmed by the reader, or can be directly allocated by the reader, e.g.:

- In CBRA, the random ID in Msg1 can be reused, after reader addresses the contention;

- In CFRA, the reader allocates a device specific AS scheduling temporary ID.

### 2.6.2	Scheduling information

In legacy NR system, PDCCH can carry DL control information for UL data transmission. For A-IoT, in RAN1 agreement, for R2D, only one physical channel, i.e. PRDCH, is supported:

RAN1#117 agreement: For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH

In RAN2 agreement, the A-IoT MAC layer is under study:

RAN2#125-bis agreements: - PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 discussion) and any other really needed functionalities. - SDAP is not supported for UP protocol stack. - RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. - RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

From RAN2 respective, the L1 R2D control/scheduling information (if defined) can be put into A-IoT MAC payload, which is similar to legacy MAC CE. This can simplify the physical layer design, considering that only one R2D physical channel is supported. There should be no difference whether the R2D control information is specified in MAC layer or PHY layer.

Proposal 3:	RAN2 considers that the A-IoT MAC layer signalling can be the solution to carry R2D scheduling/control information.

## 2.7	Reader initiated paging

### 2.7.1	Proximity determination

In RAN SID, proximity determination was proposed to be studied as shown below:

RP-240826: - Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects).

As clarified in the RAN SID, the proximity determination aims to determine whether BS/intermediate UE and ambient IoT device are near each other or not [2].

In RAN1#116-bis, RAN1 agreed that proximity determination is not based on device side measurements. Therefore, a potential method to support the proximity determination is to rely on reader implementation. Hence, from RAN2 perspective, no spec impact is foreseen in order to support it with the current agreed AS procedure.

RAN1#116-bis agreement[2]: Proximity determination based on device side measurements is not considered.

Observation 6a:	RAN1 agreed “Proximity determination based on device side measurements is not considered.”

Observation 6b:	To support the proximity determination, the current RAN2 A-IoT paging can already trigger the devices to perform D2R transmission. Then, it is up to the reader implementation to determine whether the devices are near or far. No further new AS procedure is needed for proximity determination purpose.

From E2E procedure perspective, the service request initiating the proximity determination should be an upper layer request (either from core network or from server). Otherwise, it is not clear how the reader will use the proximity determination result or where to report the proximity determination result. Considering SA2 and SA3 will take RAN SID into account during their study, RAN2 can assume the proximity determination is triggered by core network/server behind the core network.

SA3 SID: 1. Identify security threats introduced by AIoT services for use cases captured in TS 22.369, for topologies captured in RP-234058, and for architecture captured in TR 23-700-13. 2. Identify security requirements to address the identified threats. 3. Develop potential solutions that fulfill the security requirements, taking into account AIoT device constraints agreed upon in other 3GPP working groups.

SA2 SID: For this study, the types of devices, traffic assumptions and connectivity topologies are used from the RAN study item (please see RP-234058).

Observation 6c:	RAN2 assumes the proximity determination use case is initiated/triggered by the core network/server behind the core network (e.g. by the inventory request).

Proposal 4:	RAN2 considers that there is no additional RAN2 impact to support the proximity (i.e. reusing the current procedure for inventory and command).

### 2.7.2	Periodical paging

For periodical paging issue, the reader may do periodic paging transmission based on the CN request, under RAN3 discussion. However, this periodical transmission relies on the implementation of the reader, which is transparent to the A-IoT devices.

Observation 7:	Based on the CN request, it is up to reader implementation to do the periodic paging transmission, which is transparent to devices.

## 2.8	Permanent vs. temporary paging identifier and inventory reporting

Apart from the permanent ID, there are also proposals to use the temporary ID as paging identifier. However, even if the temporary ID is to be supported, there are some cases when the device is not assigned with the temporary ID yet. For instance, when the device powers on, it does not have a temporary ID assigned. It can be only triggered by paging with a permanent ID (e.g. group ID or device ID) to perform A-IoT random access. Therefore, the permanent ID should be at least supported regardless of whether the temporary ID can be supported.

Observation 8:	The permanent ID for paging should be at least supported, since there are cases when device is not assigned with the temporary ID yet (even if temporary ID can be supported).

As for the temporary ID, companies have a view that temporary IDs, being shorter, cut down on reporting overhead during inventories compared to using permanent IDs. However, the impact of overhead issue can be weakened or handled without using temporary IDs.

When an A-IoT device receives an A-IoT Paging message for inventory or command purposes from a reader, it compares the provided group ID or mask against its own device ID. Based on this comparison, the A-IoT device determines if it has been selected to respond. If there is only a single or few devices to be paged, the overhead issue is not a problem.

In addition, considering the group ID/mask in the A-IoT paging message is a part of the device ID, the A-IoT device can only report the remaining part of the device ID by removing the redundant group ID/mask part (i.e. “truncated device ID”). In this case, the overhead can be significantly reduced.

Some companies may have some concerns about the potential privacy issue of using a permanent device ID in the A-IoT paging message without encryption. If necessary, the core network can use the group ID/mask instead of including the single device ID when paging a device (and filter out the other reported device ID at CN). In this manner, it can protect the privacy of the device ID effectively, because the ID of a specific device is not exposed during the paging process.

Observation 9: The temporary paging identifier is not needed:

- For the overhead issue:

  - The group ID/mask will be used for the inventory of multiple devices. If there is a single or very few devices to be paged, the overhead issue is not a problem.

  - For device ID reporting overhead, some truncated device ID can be reported by removing the redundant mask part.

- For potential device ID privacy issue, the network can choose to use group ID, instead of just including the single device ID in paging message.

Also, there are some disadvantages of introducing the temporary ID. The device needs to store an additional temporary ID in addition to the permanent ID, which causes extra memory and increases the cost and the power consumption of the device.

In addition, in order to use temporary ID during the inventory process and avoid flushing all the temporary information when the device powers off, the device needs to store the temporary ID in the non-volatile memory. Then the device needs to implement write operations for storing temporary ID in case it may power off later. In the specification of UHF RFID, the mandatory size of a data block to be written into the memory per writing operation (by the “write” command) is only 16 bits and it needs large latency to complete one write operation [3]. Consequently, it is recommended to avoid frequent writing operation to the memory of device in the design.

Observation 10:		The drawbacks of introducing temporary paging identifier include:

- It causes additional memory usage in the device side;

- It requires frequent write operation to the device memory.

## 2.9	Paging reachability

Given the RAN2 125bits meeting agreement, we hereby clarify how the paging reachability can be supported.

RAN2 assumes that the device will not support tracking/RAN area update procedure.

For DO-DTT and DT traffic types, how to ensure the A-IoT device reachability from CN point of view needs to be considered, i.e. to find the suitable reader to reach some A-IoT device(s). As a potential implementation solution, the CN can store some history information of the association between A-IoT devices and readers, e.g. based on the previous location report or the information from OAM/server. Then, the CN can know which reader to send the A-IoT service request in order to reach one/a group of/all A-IoT devices. Additionally, this paging reachability issue is under the scope of SA2, without RAN2 impact.

In the last meeting, RAN3 agree to use the network locating report procedure to support this use case.

Use cases for locating an AIoT device: - Find an appropriate “reader” close to the A-IoT device;

Observation 11a:	In order to know which readers can serve one/a group of/all A-IoT devices, the CN can store some history information of the association between A-IoT devices and the readers, e.g. based on the previous location report or the information from OAM/server.

In last meeting, some companies proposed to support to page unidentified devices, but from device side, to remember/identify whether it has responded to a reader requires more standard work and also has impact to device hardware implementation, e.g. memory.

Observation 11b:	It is difficult for A-IoT device to determine whether it is “unidentified/unknown” device or “new coming” device. There should be no device side specific operation to support the paging reachability.

# 3	Conclusions

This contribution makes the following proposals:

Observation 1:	The Ambient IoT should be an asynchronous system.

Observation 2:	From A-IoT device complexity perspective, it is feasible to support the multiplexing (e.g. MAC SDU and MAC CE, if any), while it may cause additional device complexity.

Observation 3:	There are two alternatives for the reader to better schedule suitable TBS for D2R transmission,

- Alternative 1 (seems sufficient): CN can provide the information on the (rough) expected data size of following D2R transmission; and/or

- Alternative 2: In D2R, the device can indicate the (rough) size of the next D2R data to be transmitted (e.g. device ID size).

Observation 4:	RAN2 assumes that some AS scheduling temporary ID (RNTI like) may be needed to indicate the specific target device for data transmission/reception. It is to be discussed/designed in physical layer.

Observation 5:	The AS scheduling temporary ID can be generated by the device and confirmed by the reader, or can be directly allocated by the reader, e.g.:

- In CBRA, the random ID in Msg1 can be reused, after reader addresses the contention;

- In CFRA, the reader allocates a device specific AS scheduling temporary ID.

Observation 6a:	RAN1 agreed “Proximity determination based on device side measurements is not considered.”

Observation 6b:	To support the proximity determination, the current RAN2 A-IoT paging can already trigger the devices to perform D2R transmission. Then, it is up to the reader implementation to determine whether the devices are near or far. No further new AS procedure is needed for proximity determination purpose.

Observation 6c:	RAN2 assumes the proximity determination use case is initiated/triggered by the core network/server behind the core network (e.g. by the inventory request).

Observation 7:	Based on the CN request, it is up to reader implementation to do the periodic paging transmission, which is transparent to devices.

Observation 8: The permanent ID for paging should be at least supported, since there are cases when device is not assigned with the temporary ID yet (even if temporary ID can be supported).

Observation 9: The temporary paging identifier is not needed:

- For the overhead issue:

  - The group ID/mask will be used for the inventory of multiple devices. If there is a single or very few devices to be paged, the overhead issue is not a problem.

  - For device ID reporting overhead, some truncated device ID can be reported by removing the redundant mask part.

- For potential device ID privacy issue, the network can choose to use group ID, instead of just including the single device ID in paging message.

Observation 10: The drawbacks of introducing temporary paging identifier include:

- It causes additional memory usage in the device side;

- It requires frequent write operation to the device memory.

Observation 11a:	In order to know which readers can serve one/a group of/all A-IoT devices, the CN can store some history information of the association between A-IoT devices and the readers, e.g. based on the previous location report or the information from OAM/server.

Observation 11b:	It is difficult for A-IoT device to determine whether it is “unidentified/unknown” device or “new coming” device. There should be no device side specific operation to support the paging reachability.

Data transmission

Proposal 1:	For A-IoT, RAN2 assumes there will be the upper layer(s), which is to be decided by SA2 (e.g. an application layer and/or NAS-like layer). For A-IoT data transmission, the upper layer data (SDU) is transmitted as the payload in AS layer.

Information visible to reader

Proposal 2:	RAN2 assumes that the following information can be visible to the reader and useful for AS procedure:

- Whether single or multiple devices are triggered; and the (rough) number of devices to be triggered, if multiple devices are triggered;

- The service types (i.e. inventory and/or command).

Scheduling information

Proposal 3:	RAN2 considers that the A-IoT MAC layer signalling can be the solution to carry R2D scheduling/control information.

Proximity

Proposal 4:	RAN2 considers that there is no additional RAN2 impact to support the proximity (i.e. reusing the current procedure for inventory and command).

# 4	References

- R1-2401767, Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things) in NR, Ad-Hoc Chair (Huawei).

- EPC Tag Data Standard, version 2.0.0, available at https://ref.gs1.org/standards/tds/2.0.0/