3GPP TSG-RAN WG2 Meeting #127	R2-2406943

Maastricht, Netherlands, 19th August – 23th August, 2024

Agenda Item:	8.2.5

Source:	Huawei, HiSilicon

Title:	A-IoT Topology 2 issues

Document for: Discussion and Decision

# 1	Introduction

In the SID, two topologies are included in the general scope as follows:

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

RAN2 has been focusing on Topology 1 in previous meetings, and agreed the stage 2 procedures supporting inventory and/or DL command use cases, based on which RAN2 can start to analyse the Topology 2 specific issues to support those use cases.

In this contribution, we summarize the Topology 2 related progress in SA2 and RAN3, and provide our views on the Topology 2 essential issues and corresponding RAN2 impact.

# 2	Discussion

## 2.1	Overall procedure in Topology 2

As shown in Figure 2.1-1, the overall procedure in order to perform A-IoT service in Topology 2 can be summarized into the following steps:

- Step 1: Intermediate UE performs authorization procedures with CN, and CN will provide the authorization information to BS. This step should be similar to existing authorization procedure defined for sidelink/ProSe, and should be up to SA2 and RAN3.

- Step 2: For an inventory and/or command service, the proper intermediate UE will be selected by CN and/or BS by implementation or based on some assistance information (e.g. location). This issue is being discussed already in RAN3 and SA2. From RAN2 perspective, there seems not much assistance information can be provided to the BS except UE capability. And usually how to decide target is determined by RAN3, e.g. target cell selection in HO procedure, target relay UE selection in inter-gNB path switch. In this sense, the UE reader selection can be left to RAN3 and SA2 in the SI phase, and RAN2 can be involved later in case Uu impact is identified in WI phase.

- Step 3: Once a UE is selected as reader, it needs to have some A-IoT configuration to perform A-IoT Uu procedures. Here the configuration is mainly A-IoT radio resource.

- Step 4: As reader, the intermediate UE performs the same overall AS procedures between A-IoT device and reader in Figure 6.2.1-1 in TR 38.769.

Figure 2.1-1 Overall A-IoT E2E procedure in Topology 2

With the above understanding on the procedure, it is proposed:

Proposal 1:	In Topology 2, the intermediate UE authorization is up to SA2/SA3 and RAN3.

Proposal 2:	In Topology 2, the UE reader selection is up to RAN3 and SA2.

The figure does not include the details how the intermediate UE receives the service request from CN, e.g. before or after step 3, via CP/UP higher layer or via AS layer, which are highly related to the E2E protocol stack design. We will first discuss the protocol stack in clause 2.2, and then the Uu part details in clause 2.3.

## 2.2	Protocol stacks (progress in SA2 and RAN3)

In current SA2 TR 23.700-13 v0.4.0, several Topology 2 solutions are captured, including solutions #3/4/11/12/21/22/30/32/33, the system architecture and end-to-end protocol stack are listed in the Annex.

The main differences among the above solutions are whether there is an end-to-end layer terminated between the intermediate UE and A-IoT control node in the core network, and which layer it is. This determines how the intermediate UE receives A-IoT paging related info from core network, and forwards devices’ A-IoT data between the A-IoT devices and core network. For instance,

- in solution #3/4/22/32, there is an end-to-end layer between the intermediate UE and core network which is the upper layer(s) above PDU layer, as shown in Figure 2.2-1;

- in solution #4/11/12/21/30, the end-to-end layer between the intermediate UE and core network is the NAS layer or new CP protocol layer above NAS layer, as shown in Figure 2.2-2;

- in solution #33, there is no direct end-to-end protocol layer between the intermediate UE and the core network for A-IoT services. Instead, the Uu RRC layer is used to forward the devices’ A-IoT NAS messages/A-IoT data, as shown in Figure 2.2-3.

Figure 2.2-1 PDU layer based solution, i.e. UP solution

Figure 2.2-2 NAS layer based solution, i.e. CP solution

Figure 2.2-3 RRC layer based solution

Similar protocol stacks are also proposed and under-discussion in RAN3. From RAN2 perspective, PDU based solutions and NAS based solution can be counted as UE “upper layer-based solution”, considering that PDU layer, NAS layer, new layers above PDU layer/NAS layer are all upper layers from RAN2 perspective as shown in Figure 2.2-4. Therefore, RAN2 can further study both options (i.e. upper layer based solution and RRC layer based solution) for the Uu impact.

Observation 1:	SA2 TR Solution #3/4/11/12/21/22/30/32 for Topology 2 can be summarized as the upper layer based solution from RAN2 perspective.

Figure 2.2-4 Upper layer based solution

Proposal 3:	For Topology 2 protocol stacks, RAN2 considers the upper layer based solution in Figure 2.2-4 (i.e. A-IoT device's upper layer data is carried by the UE reader's upper layer) and the RRC layer based solution in Figure 2.2-3 for further study.

## 2.3	RAN2 issues specific to Topology 2

### 2.3.1	Data forwarding via NR Uu interface for inventory/command use cases

From data forwarding perspective, we can observe the solutions above PDU layer or based on NAS layer have minimum RAN2 impact, and the major work is in SA2 (maybe CT as well). For instance, in solution #3/4/22/32, the information/data above PDU layer for A-IoT services are considered as normal data of the intermediate UE, and in solution #4/11/12/21/30, the info/data in NAS layer or above NAS layer for A-IoT services are considered as NAS messages of the intermediate UE. In both cases, the A-IoT data/info are agnostic to RAN. For the RRC layer based solution, the info/data for A-IoT services is to be embedded in RRC messages, and the BS needs to receive service request from core network and send the information to the intermediate UE via dedicated Uu RRC message. Therefore, the major work is supposed to be in RAN2 scope, meanwhile the CN impact could be minimized.

Proposal 4:	In upper layer based solution, the data forwarding for inventory (and command) has minor RAN2 impact, which can be left to SA2 study.

Then, to be more specific, in the RRC based solution the Uu procedure can be:

- BS sends the A-IoT related information (e.g. paging identifier) to intermediate UE via Uu RRC message based on received A-IoT service request.

- The intermediate UE generates A-IoT paging message based on the received RRC message.

- The intermediate UE performs the overall AS procedures between A-IoT device and reader in Figure 6.2.1-1 in TR 38.769 as in Topology 1.

- The intermediate UE forwards the A-IoT data via Uu RRC message.

The above general procedure can be captured in TR as well.

Proposal 5:	In RRC based solution, the data forwarding for inventory (and command) includes the additional NR Uu impact as follow, and can be captured in TR for Topology 2:

BS sends the A-IoT related information (e.g. paging identifier) to intermediate UE via Uu RRC message based on received A-IoT service request.

The device’s upper layer data is forwarded between the intermediate UE and BS, via Uu RRC messages.

### 2.3.2	A-IoT radio resource management

In the SID, it is clear that the spectrum for A-IoT Uu interface is FR1 licensed spectrum in FDD. Therefore, no matter which end-to-end protocol stack option is to be used, the basic principle should be that the radio resource in A-IoT Uu interface should be controlled and allocated by the macro BS. The same issue has been discussed in SL/ProSe feature, and the general solution is that when the UE performs registration, the core network will authorize the UE, and if the UE is allowed to perform the specific service, the core network will pass the corresponding authorization info to the BS. The BS can allocate radio resource to the authorized UE when needed. This general procedure can be taken as the baseline for A-IoT Topology 2, regardless the end-to-end protocol stack option. For instance, the BS can indicate the intermediate UE reader about the time to start and to stop using the A-IoT radio resources. There may be some RAN1 involvement, e.g. how to define the radio resource. This could be related to the common issue in Topology 1 which is related to the basic PHY design. Thus, it’s better to be left to WI phase.

Proposal 6a:	In Topology 2, the radio resources used in A-IoT Uu interface should be allocated and controlled by the BS (in any protocol stack option).

Proposal 6b:	The BS can indicate to the authorized intermediate UE on when to start and to stop using the allocated A-IoT radio resources.

### 2.3.3	UE reader’s RRC state

To perform the inventory/command procedure, intermediate UE should be in RRC_CONNECTED state to receive service request from CN. After it collects device IDs/A-IoT data from the devices, it needs to forward the data to CN via Uu, so it’s better to stay in RRC_CONNECTED state. From network perspective, there is no need/motivation to release the UE readers to RRC_IDLE/RRC_INACTIVE state when they are involved in the on-going A-IoT services.

Proposal 7a:	As baseline, the intermediate UE should be in RRC_CONNECTED, during inventory/command procedures.

Then if an idle/inactive UE is selected as UE reader, it needs to be transited to RRC_CONNECTED state. There are two options under-discussion in SA2 and RAN3: UE reader is selected by CN or by RAN. For selection by CN option, the selected UE readers can be informed to the BS (RAN2 can wait for RAN3 and SA2). For the selection by BS option, CN assisting information to the BS for UE reader selection can be left to RAN3 and SA2 (RAN2 can wait). Therefore, in SI phase, there is no big RAN2 impact if idle/inactive UE is selected as UE reader.

Observation 2:	If idle/inactive UE is selected by the CN or RAN, the UE can be paged by the legacy paging procedure (no big RAN2 impact foreseen).

Companies may also consider to allow idle/inactive UE to perform A-IoT procedure with the A-IoT devices. For any NR Uu data to transmit for A-IoT purpose, it can trigger the UE reader to perform the state transition and data transmission using NR as baseline (e.g. similar to SDT in RRC_INACTIVE). RAN2 may need to discuss whether and how to use the A-IoT radio resource allocated by the BS in UE reader’s IDLE/INACTIVE state. Further, if the idle/inactive intermediate UE performing A-IoT procedure is allowed, the cell (re)selection of UE reader can follow the NR procedure as baseline (corner cases, if any, can be checked in WI phase). Such kind of stage-3 details can be considered as FFS, if needed, and left to WI design.

Observation 3:	If there is strong interest to support the idle/inactive intermediate UE performing A-IoT procedure, it can be checked in WI phase on the spec impact and feasibility.

In our understanding, there is no motivation for the BS implementation to not maintain the UE reader in RRC_CONNECTED state, if there is the ongoing A-IoT procedure.

Proposal 7b:	RAN2 assumes that the intermediate UE is not supposed to be idle/inactive state, when the UE reader is performing the A-IoT procedure.

### 2.3.4	Others

There might be some other stage-3 issues specific to Topology 2, which can be considered in WI phase.

Observation 4:	Following UE reader operation details and optimization can be considered as stage3 phase issues (no impact on the feasibility):

- Network may need to indicate the support for UE reader (for access control);

- UE reader may need to indicate its intention to operate as intermediate UE reader to the network, when entering the RRC_CONNECTED state;

- UE reader specific NR Uu capability signalling;

- Optimization handling specific to UE reader handover/RLF recovery, during the ongoing A-IoT procedure.

# 3	Conclusions

This contribution makes the following proposals:

Observation 1:	SA2 TR Solution #3/4/11/12/21/22/30/32 for Topology 2 can be summarized as the upper layer based solution from RAN2 perspective.

Observation 2:	If idle/inactive UE is selected by the CN or RAN, the UE can be paged by the legacy paging procedure (no big RAN2 impact foreseen).

Observation 3:	If there is the strong interest to support the idle/inactive intermediate UE performing A-IoT procedure, it can be checked in WI phase on the spec impact and feasibility.

Observation 4:	Following UE reader operation details and optimization can be considered as stage3 phase issues (no impact on the feasibility):

- Network may need to indicate the support for UE reader (for access control);

- UE reader may need to indicate its intention to operate as intermediate UE reader to the network, when entering the RRC_CONNECTED state;

- UE reader NR Uu capability signalling;

- Optimization handling specific to UE reader handover/RLF recovery, during the ongoing A-IoT procedure.

Overall procedure in Topology 2

Proposal 1:	In Topology 2, the intermediate UE authorization is up to SA2/SA3 and RAN3.

Proposal 2:	In Topology 2, the UE reader selection is up to RAN3 and SA2.

Protocol stacks

Proposal 3:	For Topology 2 protocol stacks, RAN2 considers the upper layer based solution (i.e. A-IoT device's upper layer data is carried by the UE reader's upper layer) and the RRC layer based solution for further study.

Upper layer based protocol stack

RRC layer based protocol stack

RAN2 issues specific to Topology 2

- Data forwarding via NR Uu

Proposal 4:	In upper layer based solution, the data forwarding for inventory (and command) has minor RAN2 impact, which can be left to SA2 study.

Proposal 5:	In RRC based solution, the data forwarding for inventory (and command) includes the additional NR Uu impact as follows, and can be captured in TR for Topology 2:

BS sends the A-IoT related information (e.g. paging identifier) to intermediate UE via Uu RRC message based on received A-IoT service request.

The device’s upper layer data is forwarded between the intermediate UE and BS, via Uu RRC messages.

- A-IoT radio resource management

Proposal 6a:	In Topology 2, the radio resources used in A-IoT Uu interface should be allocated and controlled by the BS (in any protocol stack option).

Proposal 6b:	The BS can indicate to the authorized intermediate UE on when to start and to stop using the allocated A-IoT radio resources.

- UE reader’s RRC state

Proposal 7a:	As baseline, the intermediate UE should be in RRC_CONNECTED, during inventory/command procedures.

Proposal 7b:	RAN2 assumes that the intermediate UE is not supposed to be idle/inactive state, when the UE reader is performing the A-IoT procedure.

# 4	Annex-Topology 2 solutions cited from SA2 TR 23.700-13 v0.4.0

| Solution No. | System architecture and end-to-end protocol stack copied from SA2 TR 23.700-13 v0.4.0 |
| --- | --- |
| Solution #3: Lightweight Ambient IoT system | Figure 6.3.1.4-2: Ambient IoT system architecture for Topology 2 Figure 6.3.1.6.2-2: Protocol stack between AIoT Device and Application Function for Topology 2 |
| Solution #4: Simplified system for AIoT | Figure 6.4.1.4.1-1: The example of reference architecture which supporting topology 1 & 2 Figure 6.4.1.5-2: protocol stack example-A for Topology2 Figure 6.4.1.4.1-3: The example of reference architecture for alternative 2 Figure 6.4.1.4.2-1 An example of reference architecture for discovering UE reader via control plane and AIoT device related data delivery via UE reader user plane Figure 6.4.1.5-2: protocol stack example-B for Topology2 |
| Solution #11: 5GC support for AF to communicate with AIoT devices | Figure 6.11.1-1 5GS enhancement to support AIoT Devices for Topology 2 |
| Solution #12: UE Function Delegation in Intermediate Node for AIoT Device | Figure 6.12.1-1: UFDM in AIoT Capable UE for AIoT devices |
| Solution #21: Architecture enhancement to support AIoT services for topology 2 | Figure 6.21.1-1: System Architecture to support AIoT Services for Topology 2 Figure 6.21.1-2: Protocol stack to support AIoT Services for Topology 2 |
| Solution #22: AF Based Solution for Topology 2 | Figure 6.22.1.1-1: System Architecture of AF Based Solution Figure 6.22.1.2-1: Protocol Stack for AF Based Solution (over user plane) |
| Solution #30: Support AIoT Devices using AIOTF for Topology 2 | Figure 6.30.1.1-1: System Architecture of Supporting AIoT Devices using AIOTF for Topology 2 Figure 6.30.1.2-1: Protocol Stack for Supporting AIoT Devices using AIOTF for Topology 2 |
| Solution #32: AIoT communication for connectivity topology 2 and AIoT data transmission in the UP of PDU Session | Figure 6.32.2.1-1: Assumed architecture for AIoT services using topology 2 and and AIoT data transmission in the UP of a PDU Session Figure 6.32.2-1: Protocol Stack for AF Based Solution (over user plane) |
| Solution #33: Solution for different Ambient IoT Device Types | Figure 6.33.1.3-1: Reference Architecture to support Ambient IoT Figure 6.33.1.4-1: Protocol Stack to support Ambient IoT |