3GPP TSG-RAN WG2 Meeting #127														                 R2-2406951

Masstricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	7.6.2

Source:	MediaTek Inc.

Title:	Discussion on RRC Corrections to IoT NTN

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss some RRC corrections for Rel-18 IoT NTN enhancement.

# Discussion

## 2.1 Model of T390 expiry

The following text in 36.331 i20 5.3.3.21 implies that at the moment of indication that the GNSS position has become out-of-date, the UE checks if timer T390 expires (marked with yellow). However T390 was just started as mentioned in the above statement (marked with blue). Checking for T390 expiry should not be done upon indication that the GNSS position has become out-of-date.

#### 5.3.3.21 UE actions upon indication of out-of-date GNSS position Upon indication that the GNSS position has become out-of-date while in RRC_CONNECTED, the UE shall: 1> if the UE does not support performing GNSS position fix in RRC_CONNECTED and ul-TransmissionExtensionEnabled is not configured: 2> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'; 1> else if ul-TransmissionExtensionEnabled is configured: 2> if timeAlignmentTimer is configured to be infinity: 3> start timer T390 with the timer value set to ul-TransmissionExtensionValue; 3> restart timer T390 with the timer value set to ul-TransmissionExtensionValue upon indication from lower layers to extend the UL transmission; 2> else: 3> start timer T390 with the timer value set to the remaining time of timeAlignmentTimer; 3> restart timer T390 upon indication from lower layers to extend the UL transmission, with the timer value set to the configured value of timeAlignmentTimer; 2> if timer T390 expires and no indication of network triggered GNSS measurement has been received from lower layer: 3> if gnss-AutonomousEnabled is configured: 4> perform GNSS measurement using autonomous gaps as specified in clause 5.5.9; 3> else: 4> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'; 1> else if ul-TransmissionExtensionEnabled is not configured and no indication of network triggered GNSS measurement is received from lower layer: 2> if gnss-AutonomousEnabled is configured: 3> perform GNSS measurement using autonomous gaps as specified in clause 5.5.9; 2> else: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

Since this is about T390 expiry rather than the UE action when GNSS position is outdated, we suggest moving the text about T390 expiry to a new standalone clause.

Proposal 1: Moving the text about T390 expiry in 5.3.3.21 to a new standalone clause.

The text proposal is provided below.

#### 5.3.3.21 UE actions upon indication of out-of-date GNSS position Upon indication that the GNSS position has become out-of-date while in RRC_CONNECTED, the UE shall: 1> if the UE does not support performing GNSS position fix in RRC_CONNECTED and ul-TransmissionExtensionEnabled is not configured: 2> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'; 1> else if ul-TransmissionExtensionEnabled is configured: 2> if timeAlignmentTimer is configured to be infinity: 3> start timer T390 with the timer value set to ul-TransmissionExtensionValue; 3> restart timer T390 with the timer value set to ul-TransmissionExtensionValue upon indication from lower layers to extend the UL transmission; 2> else: 3> start timer T390 with the timer value set to the remaining time of timeAlignmentTimer; 3> restart timer T390 upon indication from lower layers to extend the UL transmission, with the timer value set to the configured value of timeAlignmentTimer; 1> else if ul-TransmissionExtensionEnabled is not configured and no indication of network triggered GNSS measurement is received from lower layer: 2> if gnss-AutonomousEnabled is configured: 3> perform GNSS measurement using autonomous gaps as specified in clause 5.5.9; 2> else: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'. #### 5.3.3.x T390 expiry The UE shall: 1> if timer T390 expires and no indication of network triggered GNSS measurement has been received from lower layer: 2> if gnss-AutonomousEnabled is configured: 3> perform GNSS measurement using autonomous gaps as specified in clause 5.5.9; 2> else: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'; ## 7.3 Timers ### 7.3.1 Timers (Informative) Timer; Start; Stop; At expiry. T390 NOTE1; Upon GNSS validity duration expiry if ul-TransmissionExtensionEnabled is configured.; Upon leaving RRC_CONNECTED, or upon reception of network triggered GNSS measurement, or upon initiating the connection re-establishment procedure, or upon indication that a new GNSS position becomes valid during available idle periods in RRC_CONNECTED.; Perform the actions as specified in 5.3.3.x..

## 2.2 T390 expires during GNSS measurements using available idle periods

In the RAN2#126 meeting, RAN2 has agreed that T390 is stopped after successful GNSS position fix during C-DRX inactive time. However, the UE behaviour when the T390 expires during the UE autonomouly GNSS measurement in C-DRX inactive time is not specified. In this case, we think UE should be able to continue GNSS measurement using autonomous gap or restart a new GNSS measurment.

Proposal 2: If T390 expires and a GNSS measurement during the available idle period is in progress, it is up to UE implementation whether to continue the current GNSS measurement using autonomous gaps or start a new GNSS measurement.

The text proposal is provided below (marked with yellow).

#### 5.3.3.x T390 expiry The UE shall: 1> if timer T390 expires and no indication of network triggered GNSS measurement has been received from lower layer: 2> if gnss-AutonomousEnabled is configured: 3> perform GNSS measurement using autonomous gaps as specified in clause 5.5.9; NOTE: If T390 expires and a GNSS measurement during the available idle period is in progress, it is up to UE implementation whether to continue the current GNSS measurement using autonomous gaps or start a new GNSS measurement. 2> else: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

## 2.3 ASN.1 Error in CarrierFreqList of SIB32

Currently, the carrier frequency list of SIB32 use the IE ARFCN-ValueEUTRA. The EARFCN used in IoT NTN starts with 228501 according to 36.102 i50, but the max value of ARFCN-ValueEUTRA is 65535. Therefore, ARFCN-ValueEUTRA-r9 should be used instead of ARFCN-ValueEUTRA in the CarrierFreqList of SIB32.

-- REF ASN.1

ARFCN-ValueEUTRA ::=				INTEGER (0..maxEARFCN)

ARFCN-ValueEUTRA-v9e0 ::=			INTEGER (maxEARFCN-Plus1..maxEARFCN2)

ARFCN-ValueEUTRA-r9 ::=				INTEGER (0..maxEARFCN2)

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

Two differnet TP is show below.

Alt-1: Clean solution with NBC change:

### 6.3.1 System information blocks omit #### – SystemInformationBlockType32 The IE SystemInformationBlockType32 contains satellite assistance information for prediction of discontinuous coverage. SystemInformationBlockType32 is only signalled in a NTN cell. SystemInformationBlockType32 information element -- ASN1START SystemInformationBlockType32-r17 ::= SEQUENCE { satelliteInfoList-r17 SatelliteInfoList-r17 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ satelliteInfoList-v1800 SatelliteInfoList-v1800 OPTIONAL -- Need OR ]] } SatelliteInfoList-r17 ::= SEQUENCE (SIZE (1..maxSat-r17)) OF SatelliteInfo-r17 SatelliteInfoList-v1800 ::= SEQUENCE (SIZE (1..maxSat-r17)) OF CarrierFreqList-v1800 SatelliteInfo-r17 ::= SEQUENCE { satelliteId-r17 INTEGER (0..255), serviceInfo-r17 SEQUENCE { tle-EphemerisParameters-r17 TLE-EphemerisParameters-r17 OPTIONAL, -- Need OR t-ServiceStart-r17 TimeOffsetUTC-r17 OPTIONAL -- Need OR }, footprintInfo-r17 SEQUENCE { referencePoint-r17 SEQUENCE { longitude-r17 INTEGER (-131072..131071), latitude-r17 INTEGER (-131072..131071) } OPTIONAL, -- Need OR elevationAngles-r17 SEQUENCE { elevationAngleRight-r17 INTEGER (-14..14), elevationAngleLeft-r17 INTEGER (-14..14) OPTIONAL -- Need OP } OPTIONAL, -- Need OR radius-r17 INTEGER (1..256) OPTIONAL -- Need OR } } CarrierFreqList-v1800 ::= SEQUENCE (SIZE (1..maxFreq)) OF ARFCN-ValueEUTRA-r9 -- ASN1STOP

Alt-2: BC change:

(This change is not as straightforward as the NBC change.)

### 6.3.1 System information blocks omit #### – SystemInformationBlockType32 The IE SystemInformationBlockType32 contains satellite assistance information for prediction of discontinuous coverage. SystemInformationBlockType32 is only signalled in a NTN cell. SystemInformationBlockType32 information element -- ASN1START SystemInformationBlockType32-r17 ::= SEQUENCE { satelliteInfoList-r17 SatelliteInfoList-r17 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ satelliteInfoList-v1800 SatelliteInfoList-v1800 OPTIONAL -- Need OR ]], [[ satelliteInfoList-v18x0 SatelliteInfoList-v18x0 OPTIONAL -- Need OR ]] } SatelliteInfoList-r17 ::= SEQUENCE (SIZE (1..maxSat-r17)) OF SatelliteInfo-r17 SatelliteInfoList-v1800 ::= SEQUENCE (SIZE (1..maxSat-r17)) OF CarrierFreqList-v1800 SatelliteInfoList-v18x0 ::= SEQUENCE (SIZE (1..maxSat-r17)) OF CarrierFreqList-v18x0 SatelliteInfo-r17 ::= SEQUENCE { satelliteId-r17 INTEGER (0..255), serviceInfo-r17 SEQUENCE { tle-EphemerisParameters-r17 TLE-EphemerisParameters-r17 OPTIONAL, -- Need OR t-ServiceStart-r17 TimeOffsetUTC-r17 OPTIONAL -- Need OR }, footprintInfo-r17 SEQUENCE { referencePoint-r17 SEQUENCE { longitude-r17 INTEGER (-131072..131071), latitude-r17 INTEGER (-131072..131071) } OPTIONAL, -- Need OR elevationAngles-r17 SEQUENCE { elevationAngleRight-r17 INTEGER (-14..14), elevationAngleLeft-r17 INTEGER (-14..14) OPTIONAL -- Need OP } OPTIONAL, -- Need OR radius-r17 INTEGER (1..256) OPTIONAL -- Need OR } } CarrierFreqList-v1800 ::= SEQUENCE (SIZE (1..maxFreq)) OF ARFCN-ValueEUTRA CarrierFreqList-v18x0 ::= SEQUENCE (SIZE (1..maxFreq)) OF ARFCN-ValueEUTRA-v9e0 -- ASN1STOP ### 6.7.1 General NB-IoT message structure -- ASN1START NBIOT-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::= BEGIN IMPORTS RRCConnectionReestablishmentReject, SecurityModeCommand, SecurityModeComplete, SecurityModeFailure, AdditionalSpectrumEmission, ARFCN-ValueEUTRA, ARFCN-ValueEUTRA-r9, ARFCN-ValueEUTRA-v9e0, CarrierFreqsGERAN, CellGlobalIdEUTRA, CellIdentity, C-RNTI, DedicatedInfoNAS, DRB-Identity, GNSS-PositionFixDuration-r18, GNSS-ValidityDuration-r17, InitialUE-Identity, IntraFreqExcludedCellList, IntraFreqNeighCellList, I-RNTI-r15, LocationInfo-r10, maxAccessCat-1-r15, maxBands, maxExcludedCell, maxCellInter, maxCellIntra, maxFBI2, maxFreq, maxMultiBands, maxNrofS-NSSAI-r15, maxPageRec, maxPLMN-r11, maxSAI-MBMS-r11, maxSat-r17, maxSIB, maxSIB-1, MBMS-SAI-r11, MBMS-SAI-List-r11, MBMSSessionInfo-r13, NeighSatelliteInfoList-r18, NextHopChainingCount, NG-5G-S-TMSI-r15, PagingUE-Identity, PLMN-Identity, PLMN-IdentityList2, P-Max, PowerRampingParameters, PreambleTransMax, PhysCellId, Q-OffsetRange, Q-QualMin-r9, Q-RxLevMin, ReestabUE-Identity, RegisteredAMF-r15, RegisteredMME, ReselectionThreshold, ResumeIdentity-r13, RRC-TransactionIdentifier, RSRP-Range, S-NSSAI-r15, S-TMSI, SatelliteId-r18, SatelliteInfoList-r17, SatelliteInfoList-v1800, ServingSatelliteInfo-r17, ServingSatelliteInfo-v1820, SetupRelease, ShortMAC-I, SystemInformationBlockType16-r11, SystemInfoValueTagSI-r13, T-Reordering, T-ReorderingExt-r17, TimeAlignmentTimer, TimeSinceFailure-r11, TimeOffsetUTC-r17, TMGI-r9, TrackingAreaCode, TrackingAreaCode-5GC-r15, UAC-AC1-SelectAssistInfo-r15, DataInactivityTimer-r14 FROM EUTRA-RRC-Definitions; -- ASN1STOP #### 6.7.3.1 NB-IoT System information blocks omit #### – SystemInformationBlockType32-NB The IE SystemInformationBlockType32-NB contains satellite assistance information for prediction of discontinuous coverage. SystemInformationBlockType32-NB is only signalled in a NTN cell. SystemInformationBlockType32-NB information element -- ASN1START SystemInformationBlockType32-NB-r17 ::= SEQUENCE { satelliteInfoList-r17 SatelliteInfoList-r17 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ satelliteInfoList-v1800 SatelliteInfoList-v1800 OPTIONAL -- Need OR ]], [[ satelliteInfoList-v18x0 SatelliteInfoList-v18x0 OPTIONAL -- Need OR ]] } -- ASN1STOP

Proposal 3: RAN2 to discuss the ASN.1 issue of EARFCN value in carrierFreqList of SIB32.

# Conclusion

Proposal 1: Moving the text about T390 expiry in 5.3.3.21 to a new standalone clause.

Proposal 2: If T390 expires and a GNSS measurement during the available idle period is in progress, it is up to UE implementation whether to continue the current GNSS measurement using autonomous gaps or start a new GNSS measurement.

Proposal 3: RAN2 to discuss the ASN.1 issue of EARFCN value in carrierFreqList of SIB32.