3GPP TSG-RAN WG2 Meeting #127	R2-2406954

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SSB SCell operation

WID/SID:	Netw_Energy_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we will discuss the aspects related to the on-demand SSB SCell operation which is one of the objectives of the R19-network energy savings work item

# 2	On-demand SSB SCell Operation

## 2.1	Scenarios and signalling

RAN1 has made following agreements regarding on-demand SSB:

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE FFS: Whether the RRC is newly introduced or existing RRC is reused

Based on the agreements in RAN1#117, RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission. Our current understanding is these signaling are applicable to Scenario#2, Scenario#2A as depicted in the figure 1 below, where:

- For Scenario#2: the on-demand SSB transmission can be indicated by gNB at the time instance T1 (when UE receives SCell configuration via RRC message) or during the time period between T1 and T2. From RAN1 discussion it is not clear whether this would include scenario where SCell is directly activated when configured.

- For Scenario#2A: the on-demand SSB transmission can be indicated by gNB at the time instance T2 (when UE receives SCell activation via MAC CE command). From RAN1 discussion it is not clear whether this would include scenario where SCell is directly activated when configured.

For the RAN2 discussion we prefer to consider Scenario#2 and Scenario#2A for both Case#1(without always-on SSB) and Case#2(with always-on SSB).

Figure 1: Scenarios and indication for on-demand SSB transmission.

RAN2 could possibly start the discussion on how the on-demand SSB configurations that can be provided to the UE to support these scenarios.

Observation 1: During the last RAN1 meeting, RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

During the RAN1#117, it has also been agreed to provide the on-demand SSB configuration via RRC signaling and may include at least the parameters below

- Frequency of the on-demand SSB

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

- Periodicity of the on-demand SSB

Also, there are more than one SSB transmission patterns supported in RAN1 as listed below

- Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

- Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

- Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

- Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

- Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

Depending on the SSB pattern to be transmitted, we believe the configurations may have to include additional parameters for example,

- The periodicity parameter is required for all the on-demand transmission options.

- Number of SSB bursts is required for the Option 3

- time duration for which on-demand SSB are transmitted for Option 2 and Option 4

- alternate or a second periodicity for Option 4.

- and may also require a corresponding SMTC configuration.

Some of the patterns for on-demand SSB may be as indicated in the Figure 2.

Figure 2: On-demand SSB transmission patterns.

In general, on-demand SSB configuration may be provided via the Serving Cell Configuration. As per our understanding, the set of parameters required for the on-demand SSB operation may be kept minimal. As indicated in Figure 3 two options may be considered.

- the minimal set consisting of mandatory parameters that can work on top of the legacy SSB configuration.

- the full set of parameters that would replace the legacy SSB configuration.

Figure 3: On-demand SSB configuration structure

Proposal 1 : RAN2 to discuss the on-demand SSB configuration and consider the possibility of optimizing the configuration.Discuss the possibility of defining a minimal set that may work on top of the legacy configuration or a full set of parameter that replace the legacy configuration.

It may be beneficial to configure one or more patterns for on-demand SSB.

SSB-Pattern-Config :: SEQUENCE {

pattern-id INTEGER (1..M)

ssb-Periodicity-r19 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1},

activeDuration ENUMERATED {sf5, sf10, sf20, sf40, sf80, sf160, sf320, sf640} OPTIONAL,

numberOfBurst INTEGER (1..N)

ssb-Frequency ARFCN-ValueNR,

subcarrierSpacing SubcarrierSpacing,

ssb-PositionsInBurst CHOICE {

shortBitmap BIT STRING (SIZE (4)),

medium Bitmap BIT STRING (SIZE (8)),

longBitmap BIT STRING (SIZE (64))

} OPTIONAL,

ss-PBCH-BlockPower INTEGER (-60..50) OPTIONAL,

}

Further on the signaling, RAN1 has agreed to support MAC-CE signaling to indicate the on-demand SSB transmissions. As per our understanding, the existing SCell activation may be used as an indication to start the on-demand SSB transmission when the required configurations are available via RRC if it is assumed the on-demand SSB is always implicitly activated upon SCell activation as long as configured for the SCell without multiple patterns. If that is not the case (e.g. not only for SCell activation or there might be multiple patterns), the network might need to explicitly indicate the on-demand SSB transmission activation/pattern and for this purpose, a new MAC-CE could be used. Whether explicit deactivation is also needed is still unclear yet in RAN1 which would impact the MAC CE design as well. Coexistence with legacy TRS also needs to be considered.

Besides, the MAC-CE design also depends on which of the following scenarios are to be supported (e.g. if it is always only for SCell activation), which are still under discussion in RAN1:

- In Scenario#2, on-demand SSB configuration is provided at time T1, and between time T1 and T2 a new MAC-CE is used to trigger one of the SSB patterns configured.

- In Scenario#2A, on-demand SSB configuration is provided at time T1, and at time T2, the legacy MAC-CE based SCell activation command is sent, this can be used as a reference for on-demand SSB transmission

- In Scenario#2A, on-demand SSB configuration is provided at time T1, and at time T2, a new MAC-CE is sent to both trigger the on-demand SSB transmission and also SCell activation

In summary, the following details are needed for MAC CE design for OD-SSB:

- Whether it is only for SCell activation or it can be activated independently from SCell activation;

- If only upon activation, whether it is implicitly activated upon SCell activation whenever configure or it requires explicit indication if it can be activated for some SCells but deactivated for other SCells even if they are configured with OD-SSB;

- Whether there are multiple patterns;

- Whether it needs explicit deactivation indication or it is timer/counter based without explicit deactivation.

Since those are under discussion in RAN1, it makes sense to wait for more input before we start the MAC CE design to avoid duplicated discussions in both WGs.

Proposal 2 : More details is needed from RAN1 for the MAC-CE design to trigger on-demand SSB .

On L3 measurements based on on-demand SSB, RAN1 thinks that RAN2 should lead the discussions. Our understanding is that on-demand SSB based L3 measurements and reporting may depend on the on-demand SSB pattern.

It is required to align the SMTC window with the on-demand SSB transmission. The signaling used for on-demand SSB indication may provide the reference for the STMC. E.g time T slots after which SSB transmission may start would have impact to the SMTC window. The SMTC configuration may be explicitly configured or derived based on the on-demand SSB configuration.

Figure 4: on-demand SSB and SMTC

Proposal 3: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.

# 3	Conclusion

Observation 1: During the last RAN1 meeting, RRC and MAC-CE based signaling have been agreed to indicate on-demand SSB transmission at least for Scenario#2 and Scenario#2A for both Case#1 and Case#2.

Proposal 1 : RAN2 to discuss the on-demand SSB configuration and consider the possibility of optimizing the configuration.Discuss the possibility of defining a minimal set that may work on top of the legacy configuration or a full set of parameter that replace the legacy configuration.

Proposal 2 : More details is needed from RAN1 for the MAC-CE design to trigger on-demand SSB .

Proposal 3: RAN2 to discuss the configurations for SMTC along with the on-demand SSB configuration.