3GPP TSG-RAN WG2 Meeting #127	R2-2406955

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SIB1 for Idle/Inactive mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

The release 19 work item on network energy saving (RP-234065) includes the following objective:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In this contribution, we will discuss the issues related to the on-demand SIB1 for idle/inactive mode UEs. And in our companion contribution, we will discuss the issues on-demand SSB SCell operation and adaptation of common signal/channel transmissions in [1] and [2], respectively.

Agreements on on-demand SIB1:

Overall procedure:

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

When the UE requests on-demand SIB1:

- The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

Based on the outcome of RAN2#125bis meeting, the following agreements were made:

Then in the RAN2#126 following additional agreements were achieved:

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

And following proposals were postponed:

Proposal 9b (modified): After the UE camps on NES cell, UE can acquire WUS configuration from the NES cell.

Proposal 8: SIB1 validity is supported, i.e. if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1.

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

In this paper we try to discuss further details of the OD-SIB1 in Scenario 1 and provide thoughts on Scenario 3.

# 2	Non standalone

## 2.1	Non-standalone scenarios

Regarding the on-demand SIB1 (OD-SIB1) operation of the NES cell, the following cases are agreed for further study from RAN1 #117:

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

-	Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

-	Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

| Figure 1 – Scenario 1a - RAN1 Case 2 (Option 1+B+X) | Figure 2 – Scenario 1b (with SIB1 from Cell A) - RAN1 Case 3 (Option 2+B+Y) |
| --- | --- |

Feasibility of scenario 1a- RAN1 case 2 from RAN1 perspective is agreed. Then, the motivation and benefit to consider another non-standalone case are not clear. Below, we study case 3 and provide the pros and cos for this case.

For case 3, the following aspects are a limiting the benefits of this case:

- More NW signaling is needed for case 3 comparing to case 2. In addition to WUS configuration exchange, SIB1 payload and SIB1 update to be exchanged between Cell A and NES cell. More signaling overhead over backhaul F1-AP and/or Xn interface is needed with respect to case 2.

- Case 3 requires UE requesting on-demand SIB1 to send PRACH towards Cell A. Hence, higher PRACH resource utilization at Cell A with respect to case 2 is expected. Case 2 allows offloading of the Cell A and, importantly, increases the utilization of the NES cell with no/limited NES cell’s energy consumption increase for PRACH monitoring (since the NES cell should monitor anyway for PRACH transmissions of UEs, which have already acquired the SIB1).

- SIB1 update issue: due to exchange between nodes in case 3, it is hard to maintain an updated version of SIB1 of NES cell transmitted by Cell A. However, in case 2, this problem does not exist as NES cell is transmitting its own SIB1.

In addition, the NES gain of case 3 is marginal with respect to case 2. In R1-2405162 it has been summarized that ‘Case 3 design can provide 2% of network energy saving gain over design Case 2 for the SIB1 transmission rate of 30%’.

Due to limited TUs allocated for OD-SIB1 operation, RAN1 to down-select the cases above to one case. Based on the discussion above, Case 2 has more advantages with respect to Case 3 and should thus be selected moving forward.

Proposal 1: RAN2 to down-select the cases listed below to one case.

- Case 2: Scenario 1a: Option 1+B+X where:

    - UE obtains WUS configuration from Cell A.

    - UE sends UL WUS, e.g., Msg1, to NES cell.

    - UE receives OD-SIB1 from NES cell.

In May RAN2 we had discussion:

Case 3 in RAN1 (P1: R2-2404226: Samsung)

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

- Postponed.

For case 3, as discussed above, more NW signalling, more PRACH resource utilization and SIB1 update aspects are drawbacks for this case with respect to case 2.

Observation 1: case 3 has more drawback than case 2. There is no clear benefit to specify two cases for non-standalone scenarios.

Proposal 2: RAN2 to consider Case 2, i.e. scenario 1a, only for normative work.

## 2.2 Baseline operation for scenario 1

- UE obtains the WUS configuration from cell A,

  - PCI+Frequency (at least) used to associate configuration to a cell.

  - Cell A may provide configuration for multiple cells.

  - Multiple Cell A(s) may provide WUS configuration for same cell.

  - At least configuration is provided by cell A SIB(s). in addition to cell A SIB (s), RRCRelease may provide WUS configuration. This makes sense to us as there is likely scenarios that cell starts to use OD-SIB1 and releases UES to IDLE/INACTIVE – It seems quite good idea not to mandate reselection to Cell A and then back to NES Cell

- Once UE (re)selects to cell for which UE has WUS configuration and NW indicates it is utilizing OD-SIB1 procedure - UE may request OD-SIB1. Legacy UE should bar this kind of cell but OD-SIB1 supporting UE should not.

  - Request is assumed to be preamble transmission

    - We don’t combine this with direct request for moving to RRC_CONNECTED i.e. UE will first acquire SIB1 and after that resume/request to be moved to RRC_CONNECTED

  - RAN2 assumes UE will receive RAR

  - Assuming NW detects OD-SIB1 request it will provide SIB1 to UE. It has not been discussed in RAN2 whether SIB1 provision is:

    - via OD-SIB1 specific transmission AND/OR

    - NW resumes regular SIB1 transmisssion

- If SIB1 reception is successful

  - UE may camp on the cell will apply regular operations – paging reception, SI change modification monitoring etc…

  - It was left for further consideration how UE determines validity of SIB1

  - RAN2 also agreed that UE needs reacquire SIB1 in case of SIB change notification (legacy behaviour).

- If not successful

  - As baseline upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. Without modification to specification if UE has not been able to acquire SIB1 it will bar the cell for up to 300 seconds

As we can see RAN2 has established baseline operation for scenario 1 but of course there are still various aspects that need to be handled.

### 2.2.1	Validity of SIB1/MIB

As pointed out in above summary point 3b/c consider how UE may determine SIB1 validity for the cell. In release 15 this is solved by UE always acquiring SIB1 whenever it reselects to a cell (UE cannot assume 3 hour validity time) – see 38.331 section 5.2.2.2.1:

Thus, in the legacy UE will always reacquire MIB/SIB1 whenever (re)selecting to a cell – this will naturally ensure that SIB1 is always valid.

Observation 2: In legacy operation UE always reacquires MIB/SIB1 whenever (re)selecting a cell.

This can be easily also way to ensure SIB1 is valid in OD-SIB1 operation. Of course, this could cause some additional OD-SIB1 requests thus we think this could be good baseline for SIB1 validity ensurement together with earlier RAN2 agreement that in case of SIB change notification UE will always reacquire SIB1:

Proposal 3:  We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

Proposal 4: Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement)

### 2.2.2	Reselection control

It would not be desirable that legacy UEs or UEs that do not support OD-SIB1 feature would try to perform cell reselection on a NES cell that is operating in OD-SIB1 mode. These UEs would not find SIB1, e.g. based on MIB information, and would have to apply barring to the cell, and would need to look for another suitable cell to camp. This would cause unnecessary UE power consumption and delays. In order to avoid this, it would be desirable for the network to set the cell reselection parameters in such a way that such cell reselection attempts are prevented. One way to realize this is to set the reselection priorities in such a way to enable that. In order to have different reselection priorities per UE, one could use dedicated priorities but those have the drawback of extensive signaling in RRCRelease as well as that not all networks/UEs may support dedicated priorities. Also, only way to update priorities is then by moving UE to CONNECTED state and that is rather cumbersome as it is expected that OD-SIB1 status of cells may change in time (e.g. to provide SIB1 if load in the area becomes large). Thus it would be better to have some control that is specific to NES UEs.

Proposal 5: RAN2 should support to have OD-SIB1 specific reselection control.

### 2.2.2	About UE identification of a NES cell with on-demand SIB1 operation

RAN1 #116bis agreed about the following for UE identification of NES cell with on-demand SIB1.

Agreement

RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options:

-	Option 1: By WUS configuration

-	Option 2: By PBCH payload of NES cell

For option 1, UE camping on Cell A can obtain WUS configuration and identify a NES cell based on cell identity carried on WUS config.

For option 2:

- Scenario that requires UE to identify the NES cell by PBCH payload:

This would help when UE first re-selects a NES cell before re-selecting/camping on Cell A. However, this scenario can be avoided based on reselection control discussion provided in previous section. NW can set the reselection priorities in such a way to enable UE to re-select Cell A before re-selecting NES cell. UE can camp firstly on Cell A and option 1 can be used to obtain WUS configuration.

One drawback of this method is that UE would need to check OD-SIB1 status of the cell everytime before it requests OD-SIB1 i.e. reselecting out of the cell where it is camping to know whether cell is supporting OD-SIB1 at the moment.

- Impact to legacy UE

To avoid any confusion to legacy UEs, we should assume that the NES cell’s MIB/PBCH shall indicate ‘No SIB1’ by using the existing MIB/PBCH’s IEs

Proposal 6: NES cell’s MIB should indicate ‘No SIB1’ for legacy UE.

### 2.2.3	Switching between regular mode and on-demand SIB1 mode

As the MIB cand indicates “no SIB1” then how could NW move between regular SIB1/OD-SIB1 operation.

One possibility is basically completely turn off / on the cell whenever mode of operation changes, but this is costly and takes time. Thus, it would be desirable if the moving between operations can be done faster and more fluently.

Moving from regular SIB1 operation to OD-SIB1 operation needs further thinking as legacy UEs may be camping in the cell when operation mode changes. Let assume few scenarios:

Legacy UE camping while NW stops SIB1 transmission and starts OD-SIB1. As long as UE stays camped, UE can use SIB1 until there is SI modification notification. In this scenario UE will try reacquire SIB1 possibly without reading MIB but naturally SIB1 reception will fail and UE consider cell as bared. And if legacy UE acquire MIB then it will find that SIB1 is not broadcast and it will bar the cell. So UE will recover.

Observation 4: Legacy UE will tolerate moving from regular SIB1 transmission to OD-SIB1 while camping on cell even if UE does not reacquire MIB while camping on the cell

Moving from OD-SIB1 operation to regular SIB1 operation is possible as no legacy UE is camping in the cell and RAN2 can define any (if any) required procedures to support this..

Observation 5: As no legacy UE is operating on OD-SIB1 cell then it should be possible to move from OD-SIB1 to regular SIB1 mode without impacting legacy UEs.

To avoid any mismatch between UE stored cell operation mode and current NES cell operation mode, UE should OD-SIB1 operation mode of NES cell before requesting for OD-SIB1 in order to avoid UE doing RACH for OD-SIB1 in a cell not utilizing OD-SIB1.

Observation 6: UE should update/acquire OD-SIB1 mode of cell before requesting OD-SIB1 in the cell

RAN2 has not yet agreed how UE gets the information whether cell is operating in WUS mode thus we consider here these two scenarios:

- If UE gets this information from other cell (e.g. from WUS configuration received in cell A) then UE would need to reselect out of the cell in order to know cell is operating in on -demand SIB1 mode or then we should have some way of indicating validity of WUS configuration in the cell (see 2.1.4) i.e. UE needs to apply latest WUS configuration for the cell before requesting OD-SIB1

- If the information ofr OD-SIB1 operation mode is in MIB then UE would need to reacquire MIB1 before utilizing OD-SIB1 procedure.

Proposal 7: Rel 19 UEs should acquire OD-SIB1 mode of the cell before applying OD-SIB1 procedure in order to avoid UE requesting PRACH for OD-SIB1 in  a cell not using OD-SIB1.

If rerequesting OD_SIB1 occurs often then forcing reselection to cell providing WUS config is something to be avoided but likely The UE rerequesting OD-SIB1 probably does not occur very often thus it might be sufficient to just identify the cell being in OD-SIB1 mode by providing information together with WUS config in cell A.

Proposal 8: UE will identify cell being in OD-SIB1 mode by WUS configuration received from cell A.

### 2.2.4	RAR response handling

RAN1 consider dedicated PRACH resources for SIB1 request. Dedicated PRACH resources are not referring to UE-specific PRACH resources but rather PRACH resources dedicated to on-demand SIB1 request.

For PRACH resources for UL WUS, RAN2 should consider on-demand SI procedure as starting point. For on-demand SI, the network may configure the UE with dedicated RA resources in SI-RequestConfig within SI-Scheduling Info IE in SIB1. For on-demand SIB1, the dedicated RA resources could be part of the WUS configuration. Any of time, frequency and/or PRACH preamble resources may be used as dedicated PRACH resources to allow full flexibility to the network.

We need to highlight the impact of designing dedicated PRACH resources to RAR response. If common PRACH preamble is used for all UEs requesting on-demand SIB1 operation, this can allow a RAR response that can be interpreted/decoded by all UEs. Thus, a UE intended to transmit UL WUS for requesting SIB1, can monitor if any RAR response is sent by network to check if on-demand SIB1 is already requested by other UEs. However, if dedicated ROs, time/frequency resources are considered as dedicated PRACH resources and it is up to UE to select a preamble, RAR response is a dedicated response that can be decoded only by UE requesting on-demand SIB1.

Observation 7: Designing dedicated PRACH resources for SIB1 request assuming a common preamble for requesting SIB1 used by all UEs. It allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

It might be difficult though with current RAR monitoring based on RA-RNTI of the PRACH occasions as the UE would need to monitor all the potential RA-RNTIs of PRACH for OD-SI before WUS transmission. Common RA-RNTI would be required to enable this.

Observation 8: To enable UEs monitoring on-demand SIB1 without transmitting UL WUS, may require configuring a common RNTI.

### 2.2.5	WUS configuration and validity

In RAN2#126 we had following discussion:

Proposal 9b (modified): After the UE camps on NES cell, UE can acquire WUS configuration from the NES cell.

- Postponed.

[Session chair]: Understand the scenario is the UE reselected from cell A to NES cell. While UE stays in the NES cell, the UE may need to re-acquire SIB1. Then the question is how the UE gets WUS configuration? E.g. performs cell reselection to cell A and reacquires WUS configuration or reacquire from NES cell itself? [Rakuten]: If NW tries to transmit SIB1 always to guarantee 3 hours invalidity, how NW can have energy saving? [Nokia]: Understand NES cell does not transmit SIB1, then how its SIB1 can be transmitted to cover this concern?

As described in 2.1.1 in our view while camping on NES cell operating on OD-SIB1 mode UE gets indications whether SIB1 is updated and UE would then request OD-SIB1 from the cell based on the configuration it has. With existing agreements we can only provide WUS configuration via SIB of cell A (and possible in RRCRelease) thus UE would need to somehow know whether the WUS configuration it has is still valid.

Same issue lies in the case UE acquires WUS configuration from cell A and reselecting immediately or later to NES cell – how does UE know the WUS configuration is still valid? We could consider following options:

- Time limitation for validity after which UE needs reacquire WUS configuration in order to use OD-SIB1 procedure. But with this there is always chance that NW needs to update WUS configuration while validity time is still running.

We have some sort of value tag on WUS configuration and UE can check validity from the NES cell using OD-SIB1 mode. This would require transmission of value tag somewhere else than SIB1 (as that is not transmitted on the cell). Having this in MIB is challenging and likely it is quite difficult to have any new signal associated to MIB with space for value tag.

Observation 9: It seems quite difficult to avoid UE never using outdated WUS configuration unless we can have some value tag added in MIB/new signal.

So question is that how severe the problem is if UE uses old WUS configuration. If it is small then likely no extra solutions are needed but e.g. similar time limitation as for SIB (3 hours) we use for WUS configuration. But if the problem is severe then it might be that OD-SIB1 procedure is quite difficult to realize. In case UE uses old WUS configuration it will fail to receive SIB1 and will bar the cell and obtain new configuration from cell A. Based on this from technical perspective it would be best if one could have method for UE to check whether the used WUS configuration is valid or not.

Proposal 9: RAN2 to consider a value tag in WUS configuration and a value tag in MIB. RAN1 needs to be consulted if this is feasible from their point of view.

# 5	Conclusions:

Proposal 1: RAN2 to down-select the cases listed below to one case.

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

Observation 1: case 3 has more drawback than case 2. There is no clear benefit to specify two cases for non-standalone scenarios.

Proposal 2: RAN2 to consider Case 2, i.e. scenario 1a, only for normative work.

Observation 2: In legacy operation UE always reacquires MIB/SIB1 whenever (re)selecting a cell.

Proposal 3:  We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

Proposal 4: Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement)

Proposal 5: RAN2 should support to have OD-SIB1 specific reselection control.

Proposal 6: NES cell’s MIB should indicate ‘No SIB1’ for legacy UE.

Observation 4: Legacy UE will tolerate moving from regular SIB1 transmission to OD-SIB1 while camping on cell even if UE does not reacquire MIB while camping on the cell

Observation 5: As no legacy UE is operating on OD-SIB1 cell then it should be possible to move from OD-SIB1 to regular SIB1 mode without impacting legacy UEs.

Observation 6: UE should update/acquire OD-SIB1 mode of cell before requesting OD-SIB1 in the cell

Proposal 7: Rel 19 UEs should acquire OD-SIB1 mode of the cell before applying OD-SIB1 procedure in order to avoid UE requesting PRACH for OD-SIB1 in  a cell not using OD-SIB1.

Proposal 8: UE will identify cell being in OD-SIB1 mode by WUS configuration received from cell A.

Observation 7: Designing dedicated PRACH resources for SIB1 request assuming a common preamble for requesting SIB1 used by all UEs. It allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

Observation 8: To enable UEs monitoring on-demand SIB1 without transmitting UL WUS, may require configuring a common RNTI.

Observation 9: It seems quite difficult to avoid UE never using outdated WUS configuration unless we can have some value tag added in MIB/new signal.

Proposal 9: RAN2 to consider a value tag in WUS configuration and a value tag in MIB. RAN1 needs to be consulted if this is feasible from their point of view.