3GPP TSG-RAN WG2 Meeting #127	R2-2406956

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.5.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On Adaptation of common signal/channel transmissions for NES

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss aspects related to the potential adaptations of the common signal/channel transmissions.

# 2	Adaptation of SSB in time domain

During the previous RAN2 meeting, there were no agreements on this topic. Pending agreements from the RAN1 meetings are

- RAN1#117 meeting: Two options of SSB adaptation were described:

  - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active].

Agreement​ For adaptation of SSB in time-domain, Option 1 is supported​ - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)​ - Note: Using Option 2 to realize Option 1 is not precluded​ - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]​ - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities​ - FFS: Details including applicable scenarios ​ FFS: Support of Cell DTX for connected mode UEs for SSB​

- RAN1#116bis meeting

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode Note: Impact to idle/inactive UEs shall be minimized

As per the RAN1#117 agreement, Option 1 is supported, while using Option 2 to realize Option 1 is not precluded. At the same time, scenarios where Option 1 and/or Option 2 of SSB adaptation can be applied is an FFS discussion. In our view, RAN1 should clarify first in which scenarios Option 1 or Option 2 are useful to achieve network energy saving (NES). So far, it is still unclear what are the targeted scenarios for SSB adaptation in time domain. RAN2 can then provide input.

Proposal 1: RAN2 to wait for RAN1 to clarify scenarios for SSB adaptation in the time domain where Option 1 or Option 2 can provide network energy saving before providing input.

In any case, it is important to highlight when defining SSB adaptation scenarios and solutions for Rel-19 NES capable devices, it should be ensured that no negative impact is imposed to legacy UEs, as noted in the Rel-19 WID’s objective. Therefore, adaptation of SSB transmissions should not result in dropping of any SSBs that legacy UEs expect to receive.

Proposal 2: Adaptation of SSB transmissions should not result in dropping of any SSBs that legacy UEs expect to receive.

If the cell is associated with a CD-SSB, the cell can be used for cell selection/reselection procedures, e.g., to provide initial access to legacy UEs [TS 38.300]. In that case, there is a limitation from the specifications [TS 38.213] that, for initial cell selection, a legacy UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames, i.e., 20ms. A cell with this setting is typically a coverage cell, which may also be used as a PCell for UEs in RRC connected state. Adapting the SSB periodicity in such a cell to a value higher than 20ms for NES purposes may have a negative impact to legacy UEs attempting to perform initial cell selection, as they may not be able to find the cell. Therefore, the SSB periodicity in a cell providing initial access to legacy UEs can be set up 20ms to avoid the impact on legacy UEs, but this may reduce the energy saving potential.

Observation 1: The SSB periodicity of a cell providing initial access to legacy UEs can be set up to 20ms to avoid the impact on legacy UEs, but this may reduce the energy saving potential.

To enable larger NES gains, Rel-19 NES capable UEs could be made capable to access a cell providing initial access that uses a relaxed SSB periodicity of e.g., 40ms. From our point of view, a discussion on the implications of this approach can be considered.

Proposal 3: Discuss implications of relaxing the default SSB periodicity value of 20ms for the initial access procedure of Rel.19 NES-capable UEs.

# 3		PRACH Adaptation in time domain

As per the latest RAN1#117 meeting outcome, it was agreed to support at least the PRACH adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any). It was also noted that NES-capable UEs can use both additional PRACH resources and PRACH resources configured for legacy UEs.

In our view, the agreed PRACH adaptation based on additional PRACH resources should be applied for both 4-step RACH and 2-step RACH (if configured in the cell). Particularly for 2-step RACH, further clarifications and considerations would be needed. For instance, it is not clear whether additional PUSCH resources for NES-capable shall be considered for MsgA payload transmission or the same PUSCH as configured for legacy UEs shall be considered.

Proposal 4: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

There are two types of PRACH that can be configured:

- PRACH associated with CBRA configured by cell-specific signaling.

- PRACH associated with CFRA configured by UE-dedicated signaling.

The agreed PRACH adaptation shall at least target CBRA procedures. PRACH adaptation associated with CFRA procedures shall be studied as well, if time allows.

Proposal 5: Study at least PRACH adaptation solution(s) that are applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

The likelihood that cells are designed to only serve R19 UEs is low; thus, the PRACH adaptation should be applicable to cells that support both legacy and Rel-19 (NES-capable) UEs. Moreover, in order to avoid any impact on legacy UEs as noted in the WID, and to enable the initial access for all UEs before being able to identify whether they are legacy or NES-capable UEs, the cell should support both legacy and NES-capable UEs.

Proposal 6: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

Observation 2: The configuration of a PRACH configuration index for the additional PRACH resources different from the PRACH configuration index for the legacy resource offers more flexibility to the NW and can help accommodating scheduler needs. However, the possible overlaps between sub-sets of ROs would need to be further studied in this case.

Observation 3: Densification of RO in the system frames where legacy ROs are located offers more compact monitoring occasion in time and/or frequency, which may provide longer BS sleeping opportunities and to achieve better energy saving, with no impact on legacy UEs. However, the possible impact in terms of latency and energy consumption for handling other UL channels, e.g., RRC_CONNECTED PUSCH and PUSCH, would need to be further studied in this case.

Observation 4: The versatility of the existing framework may not be enough to cover all the needs a NW scheduler may have in this context, irrespective of whether the creation of the additional ROs is obtained through densifying ROs or increasing the number of system frames in which ROs are created.

Observation 5: The configuration of the PRACH configuration index for the additional PRACH resources as the legacy resource allows the network energy consumption for PRACH reception to occur mainly (or only) during system frames where legacy ROs are located and, at the same time, ensures that PRACH resources are not overlapping in time and/or frequency.

Proposal-7: RAN2 to wait for RAN1 to conclude on whether to consider same or different PRACH configuration indices for NES-capable UEs. Implications on RAN2 are FFS.

### Signaling aspects

As per the latest RAN1#117 meeting outcome, it was agreed the configuration of additional PRACH resources is provided by semi-static signalling. For the (de)-activation of these additional PRACH resources, different options can be considered depending on how frequent the adaptation of PRACH resources is to be applied to the cell load changes over time and the number of NES-capable UEs in the cell supporting the dynamic adaptation of the PRACH resources.

A common signalling mechanism shall be considered for UEs in RRC Idle/Inactive and RRC connected state.

Moreover, high layer signalling, e.g., SIB1 could be considered, by indicating the configuration of the additional PRACH resources and whether they are activated or not. Alternatively, a more dynamic approach would be to use L1 signalling and more precisely to reuse the existing DCIs, e.g., paging information (paging DCI) which targets UEs in all RRC states.

Proposal 8: RAN2 to study the following for PRACH adaptation:

- whether to consider the same signaling for UEs in RRC Idle/Inactive and RRC connected state.

- whether to consider higher-layer semi-static signaling, e.g., SIB1,

- whether RAN1 to consider an L1-based signalling. If supported, existing DCIs, e.g., paging DCI, should be used.

# Adaptation of paging occasions

At RAN2 #126 the adaptation of paging occasions was not treated. At RAN2 #125bis the following was agreed:

1. From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle. 2. For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. 3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution we compare the options a and b. The starting point is the illustration in Figure 4.

The legacy approach (top drawing in Figure 4) operates using a certain paging frame (PF) interval, e.g. 80 ms or 160 ms, and 1/2/4 paging occasions (PO) per PF. Furthermore, the network may configure RACH resources near the PFs to facilitate the UE can perform RACH after being paged. In terms of reference signals, the SSBs will be evenly distributed, e.g. using 20 ms, while the release 17 feature of TRS for RRC Idle/Inactive UEs may facilitate reference signal availability tailored to the PF interval.

The option a (middle drawing in Figure 4) implies a set of paging frames are bundled. In our view, this means the set of bundled paging frames occur with a short paging frame interval, while the bundles occur according to a longer bundle interval. Similar to the legacy approach, the RACH resources can be allocated near the PFs, but in option a the RACH resources may also be allocated in a bundled fashion to support the potential increase in paging load in the time period of the bundle. It is worth noting that this may result in overlapping of paging and RACH activities, which is beneficial for the network energy saving due to the shorter total active time.

The option b (lower drawing in Figure 4) implies the paging frame interval is extended. To compensate for the longer interval additional POs may be allocated per PF (orange color in the drawing). Furthermore, it may be necessary to also allocate additional RACH resources and TRS to support the additional POs.

Figure 4 Legacy paging approach and potential adaptations a and b.

Based on the analysis of the Figure 4 we provided a set of observations in Table 1.

Table 1 Comparison of paging adaptation options a and b.

|  | Option a (bundled PFs) | Option b (extended PF interval) | Observations |
| --- | --- | --- | --- |
| Specification impact | Impact on 38.331 and 38.304 to define the bundle window | Additional T and ns values in 38.331. The 38.304 equations are unchanged | Option b is simpler to implement in specification |
| PO resources | Legacy number of POs per PF and legacy number of PFs per paging cycle | Increased number of POs per PF, but less PFs per paging cycle | No difference between a and b i.e. also no delay difference, but the distribution of POs in option b may negatively impact the network sleeping opportunities. |
| RACH resources | RACH resources related to one PF can time-wise overlap with next PF/POs in the bundle window | RACH resources are distributed according to the extended PF interval | The number of RACH resources is the same for both options, but in option b they do not overlap with other NW signaling and thus have higher energy cost |
| Reference signal resources (SSB/TRS) | Reference signals (TRS) can be tailored to the bundle window start | Reference signals (TRS) are distributed according to the extended PF interval | In option b more reference signals (TRS) are needed and they will not overlap with other NW signaling and thus have higher energy cost |

In summary, there will be no delay difference between the two options, but option b is simpler to implement in the specification. However, from network energy saving point of view, the option a is advantageous because it allows the RACH procedure to overlap in the time domain with the bundled paging frames/occasions and furthermore requires less TRS, or at least enable the TRS to also overlap with other transmissions. In addition, the option b, where the interval between PFs is increased and compensated by more POs per PF may result in a significant distribution of POs over time, depending on the configured search space, which could impact the network’s sleeping opportunities negatively.

Observation 6: Option b is simpler to implement in specification than option a for paging adaptation.

Observation 7: There is no paging delay difference between a and b, but the distribution of POs in option b may negatively impact the network sleeping opportunities.

Observation 8: The number of RACH resources is the same for both options, but in option b they do not overlap with other NW signaling and thus have higher energy cost

Observation 9: In option b more reference signals (TRS) are needed and they will not overlap with other NW signaling and thus have higher energy cost

Based on these we propose RAN2 shall prioritize to specify option a (bundle paging frames) for paging adaptation.

Proposal 9: RAN2 to prioritize to specify option a (bundle paging frames) for paging adaptation.

RAN2 furthermore discussed how to handle legacy UEs. In our view, RAN2 can discuss how the specific paging resources are available to legacy UEs and Rel-19 NES UEs. For example, if a set of PFs are bundled (=option a), the legacy UE will not be aware of the bundling, but only the legacy PF(s) which potentially fall within the bundle period. Thus, it could be an option to keep such legacy PF(s), with corresponding legacy PO(s), available for the legacy UEs. The Rel-19 NES UEs which map to the legacy PF(s) and legacy PO(s) within the bundle could thus monitor the same paging information as the legacy UEs, while Rel-19 NES UEs which map to other PF(s) and PO(s) (i.e. PF(s) and PO(s) not configured/understood by legacy UEs) shall monitor the corresponding paging information.

Observation 10: Legacy UEs can monitor legacy PF(s) and legacy PO(s) which occur within a set of bundled PFs.

Likewise, for option b, the legacy UEs will only be able to monitor the PF(s) and PO(s) which coincides with a legacy value of N, while the remaining (i.e. new) PF(s) and PO(s) that are mapped based on a new N (e.g. T/64) will only be monitored by Rel-19 NES UEs.

Observation 11: Legacy UEs can monitor legacy PF(s) and legacy PO(s) based on legacy value of N and not PF(s) and PO(s) based on a new N, when the new PF(s) and PO(s) do not coincide with the legacy PF(s) and PO(s).

In summary, the Rel-19 NES UEs can use legacy PF(s) and PO(s), while legacy UEs can not use Rel-19 PF(s) and PO(s) based on option a or b. Alternatively, the Rel-19 NES UEs can also be configured not to use the legacy PF(s) and PO(s), but it would complicate the configuration and mapping of PF(s) and PO(s) without any obvious benefit.

Proposal 10: Rel-19 NES UEs can use legacy PF(s) and PO(s) in addition to Rel-19 PF(s) and PO(s), while legacy UEs cannot use Rel-19 PF(s) and PO(s) based on option a or b.

Confining the paging occasions in the time domain is not supposed to increase the paging latency, but rather shift the paging occasions to save network energy. In the legacy specification, the UE can be configured with the cell specific or UE specific DRX (paging) cycle. The UE specific DRX cycle is useful for enhancing UE energy saving or for reducing paging latency, by configuring a cycle longer or shorter, respectively, than the cell specific cycle. RAN2 has not yet discussed if & how the paging occasion confinement will impact the UEs configured with a UE specific DRX cycle. In our view, there are scenarios such as ultra-reliable low-latency communication where a shorter UE specific DRX cycle is required to fulfil the latency requirements.

Proposal 11: RAN2 to discuss if and how the paging adaptation enhancement will impact UEs configured with a UE specific DRX cycle.

## PRACH Adaptation in time domain in conjunction with adaptation of paging occasions

The PRACH load level will inherently increase if the paging occasions are confined in time domain. This is because the UEs will have to be paged during such confined time period, thereby causing the UEs to respond to the paging also in a confined time interval.

Proposal 12: RAN2 to study PRACH adaptation in time domain to spread the PRACH load when paging occasion adaptation is used.

# Conclusion

Proposal 1: RAN2 to wait for RAN1 to clarify scenarios for SSB adaptation in the time domain where Option 1 or Option 2 can provide network energy saving before providing input.

Proposal 2: Adaptation of SSB transmissions should not result in dropping of any SSBs that legacy UEs expect to receive.

Observation 1: The SSB periodicity of a cell providing initial access to legacy UEs can be set up to 20ms to avoid the impact on legacy UEs, but this may reduce the energy saving potential.

Proposal 3: Discuss implications of relaxing the default SSB periodicity value of 20ms for the initial access procedure of Rel.19 NES-capable UEs.

Proposal 4: Clarify whether further considerations are needed for the 2-step PRACH adaptation e.g., the resource used by the UE to transmit MsgA.

Proposal 5: Study at least PRACH adaptation solution(s) that are applied to CBRA. PRACH adaptation applied to CFRA shall also be investigated, if time allows, however no CFRA-specific PRACH adaptation solutions should be considered.

Proposal 6: PRACH adaptation should be applicable to cells that support both legacy and Rel-19 UEs.

Observation 2: The configuration of a PRACH configuration index for the additional PRACH resources different from the PRACH configuration index for the legacy resource offers more flexibility to the NW and can help accommodating scheduler needs. However, the possible overlaps between sub-sets of ROs would need to be further studied in this case.

Observation 3: Densification of RO in the system frames where legacy ROs are located offers more compact monitoring occasion in time and/or frequency, which may provide longer BS sleeping opportunities and to achieve better energy saving, with no impact on legacy UEs. However, the possible impact in terms of latency and energy consumption for handling other UL channels, e.g., RRC_CONNECTED PUSCH and PUSCH, would need to be further studied in this case.

Observation 4: The versatility of the existing framework may not be enough to cover all the needs a NW scheduler may have in this context, irrespective of whether the creation of the additional ROs is obtained through densifying ROs or increasing the number of system frames in which ROs are created.

Observation 5: The configuration of the PRACH configuration index for the additional PRACH resources as the legacy resource allows the network energy consumption for PRACH reception to occur mainly (or only) during system frames where legacy ROs are located and, at the same time, ensures that PRACH resources are not overlapping in time and/or frequency.

Proposal-7: RAN2 to wait for RAN1 to conclude on whether to consider same or different PRACH configuration indices for NES-capable UEs. Implications on RAN2 are FFS.

Proposal 8: RAN2 to study the following for PRACH adaptation:

Observation 6: Option b is simpler to implement in specification than option a for paging adaptation.

Observation 7: There is no paging delay difference between a and b, but the distribution of POs in option b may negatively impact the network sleeping opportunities.

Observation 8: The number of RACH resources is the same for both options, but in option b they do not overlap with other NW signaling and thus have higher energy cost

Observation 9: In option b more reference signals (TRS) are needed and they will not overlap with other NW signaling and thus have higher energy cost

Proposal 9: RAN2 to prioritize to specify option a (bundle paging frames) for paging adaptation.

Observation 10: Legacy UEs can monitor legacy PF(s) and legacy PO(s) which occur within a set of bundled PFs.

Observation 11: Legacy UEs can monitor legacy PF(s) and legacy PO(s) based on legacy value of N and not PF(s) and PO(s) based on a new N, when the new PF(s) and PO(s) do not coincide with the legacy PF(s) and PO(s).

Proposal 10: Rel-19 NES UEs can use legacy PF(s) and PO(s) in addition to Rel-19 PF(s) and PO(s), while legacy UEs cannot use Rel-19 PF(s) and PO(s) based on option a or b.

Proposal 11: RAN2 to discuss if and how the paging adaptation enhancement will impact UEs configured with a UE specific DRX cycle.

Proposal 12: RAN2 to study PRACH adaptation in time domain to spread the PRACH load when paging occasion adaptation is used.