3GPP TSG-RAN WG2 Meeting #127	R2-2406958

Maastricht, Netherlands, 19-23 August 2024

Agenda item:	8.8.4

Source:	Toyota ITC

Title:	Discussions on MBS in Rel-19 NTN

Document for:	Discussion and Decision

# 1	Introduction

A Work Item for defining Rel-19 Phase 3 enhancements to Non-Terrestrial Networks (NTN) was approved at RAN#102, and revised at RAN#104 [1]. One of the associated objectives was the following:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

For NR NTN, MBS provides advantages compared to TN in terms of coverage and resource optimization.

This document provides some thoughts and proposals on MBS in NTN in the Rel-19.

# 2	Discussion

## 2.1	Agreements from RAN2#126

The following agreements were reached at RAN2#126 in [2]:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

With additional points below:

Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

## 2.2	UE outside of the intended service area

Establish and release MRB(s) associated with a service limited to an intended service area when not located within, or exiting the intended service area:

We believe that for battery consumption gains, the UE should not monitor MCCH/MTCH when outside the intended service area. In addition, the monitoring of unwanted MCCH/MTCH may raise security issues. Hence, we believe that the UE shall not establish MRB(s) associated with a service limited when it is not located within the area. The UE shall release its established MRB(s) associated with a service limited to an intended service area when exiting the area.

Observation 1: The UE should not monitor MCCH/MTCH when outside the intended service area, for power consumption and security reasons.

Proposal 1: The UE shall not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area.

Proposal 2: The UE shall release its established MRB(s) associated with a service limited to an intended service area when exiting the area.

Predictable UE behaviour:

The two additional points related with the agreements in 2.1 above reflect that it is up to UE implementation how to detect that it is not in the intended service area. It is unsure up to which extend this sentence should be understood, however we believe that the way how the UE knows whether it is in the intended service area or not should be predictable. Indeed, the agreement 5 in 2.1 above indicates that “When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)”. It seems inconsistent that the UE may know information on the MBS session and intended service area, but that how the UE detects whether it is in the intended service area or not would be left to UE implementation.

In addition, one of the objectives of the WI in [1] is:

“Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area.”

It would therefore also appear inconsistent to specify means to indicate the intended service area to the UE, but to then to leave up to UE implementation how/whether to use this.

Observation 2: How the UE detects whether or not it is in the intended service area should be predictable, hence it cannot be left up to UE implementation.

Proposal 3: A way for the UE to detect whether it is in the intended service area or not will be provided to the UE by the standards, in a predictable way.

## 2.3	Ubiquitous coverage for e.g. digital radio

Digital radio is one of the services that could make use of MBS Rel-19 NTN. This brings the need for ubiquitous coverage with low data rates.

Observation 3: One use case for MBS Rel-19 NTN is ubiquitous coverage with low data rates, e.g. to fulfil digital radio.

## 2.4	Service continuity

Service continuity is an important requirement for MBS Rel-19 NTN. This includes ensuring that the service remains available when the visible satellite changes.

In addition, at RAN2-125bis meeting, the following was agreed in 4 below (other agreements were removed for concision):

RAN2-125bis agreements:

  - (…)

4. The intended service area can cover the area of more than one NTN cells (or portions thereof)

  - (...)

Therefore, it is important to also consider service continuity where the UE changes cell within the service area.

Observation 4: Service continuity is an important requirement for MBS Rel-19 NTN. This includes ensuring that the service remains available when the visible satellite changes and also ensuring service continuity when the UE changes cell within the service area.

At the RAN2-125bis meeting, some companies mentioned that satellite movement in NGSO deployments could cause the cell(s) serving a service area to constantly change, for example in [3]. This appears to be validated by the RAN2-125bis agreement 1 below, where it was agreed to not restrict the work to any satellite constellation type:

RAN2-125bis agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

  - (…)

This was confirmed by the RAN2-126 agreement below from [2]:

RAN2-126bis agreements:

1. For MBS broadcast service, both EFC and EMC are supported.

2-5. (…)

In addition, it was mentioned at RAN2-125-bis that if there is a need to support earth-moving cells where a gNB coming into an MBS service area dynamically receives/releases Broadcast Session, there would be a need to define network signalling for the cell identifiers associated with the MBS service area to change over time, with associated RAN3 signalling impact. See for example [4].

The position of the satellite is known in advance. Therefore, to avoid S1 or X2 over-signalling and associated resource impact, one way to achieve this would be to use configurations planned in advance. The cell identifiers / service area association configuration may be configured in advance, and become active or inactive with time.

Observation 5: For satellites that move compared to the earth (i.e. non-geostationary), which are under the scope of RAN2-126 agreement, the cell identifiers – service area association, may change constantly over time.

Observation 6: For satellites that move compared to the earth (i.e. non-geostationary), to avoid frequent network signalling, one way to solve the frequent change of service area and cell identifiers – service area association change would be to use configurations planned in advance, where the cell identifiers / service area association may be configured in advance, and become active or inactive with time.

Proposal 4: For satellites that move compared to the earth (i.e. non-geostationary), to avoid frequent network signalling, configurations planned in advance are used, where the cell identifiers / service area association is configured in advance, and become active or inactive with time.

Proposal 5: RAN2 to send an LS to RAN3 about necessary change required to update the cell identifiers / service area association. This makes use of configurations planned in advance are used, where the cell identifiers / service area association is configured in advance, and become active or inactive with time.

# 3	Conclusion

In this document the following observations and proposals have been made:

Observation 1: The UE should not monitor MCCH/MTCH when outside the intended service area, for power consumption and security reasons.

Observation 2: How the UE detects whether or not it is in the intended service area should be predictable, hence it cannot be left up to UE implementation.

Observation 3: One use case for MBS Rel-19 NTN is ubiquitous coverage with low data rates, e.g. to fulfil digital radio.

Observation 4: Service continuity is an important requirement for MBS Rel-19 NTN. This includes ensuring that the service remains available when the visible satellite changes and also ensuring service continuity when the UE changes cell within the service area.

Observation 5: For satellites that move compared to the earth (i.e. non-geostationary), which are under the scope of RAN2-126 agreement, the service area, or cell identifiers – service area association, may change constantly over time.

Observation 6: For satellites that move compared to the earth (i.e. non-geostationary), to avoid frequent network signalling, one way to solve the frequent change of service area and cell identifiers – service area association change would be to use configurations planned in advance, where the cell identifiers / service area association may be configured in advance, and become active or inactive with time.

Proposal 1: The UE shall not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area.

Proposal 2: The UE shall release its established MRB(s) associated with a service limited to an intended service area when exiting the area.

Proposal 3: A way for the UE to detect whether it is in the intended service area or not will be provided to the UE by the standards, in a predictable way.

Proposal 4: For satellites that move compared to the earth (i.e. non-geostationary), to avoid frequent network signalling, configurations planned in advance are used, where the cell identifiers / service area association is configured in advance, and become active or inactive with time.

Proposal 5: RAN2 to send an LS to RAN3 about necessary change required to update the cell identifiers / service area association. This makes use of configurations planned in advance are used, where the cell identifiers / service area association is configured in advance, and become active or inactive with time.

# References

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3. 3GPP TSG RAN Meeting #104 Shanghai, China, 17-20 June 2024.

- R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan. ETSI MCC.

- R2-2403320, RAN2-125bis meeting, “Support for broadcast service in non-terrestrial networks”, InterDigital.

- R2-2403306, RAN2-125bis meeting, “On MBS Service Area Signalling in Rel-19 NTN”, Nokia.

- R2-2402355, RAN2-125bis meeting, “Discussion on support of broadcast service via NR NTN”, CATT, China Broadnet.