3GPP TSG RAN WG2 Meeting #127	R2-2406960

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.3

Source:	CMCC

Title:	Discussion on A-IoT paging

Document for:	Discussion

Introduction

In RAN2#126 meeting, The following progress are made for A-IoT paging, shown as:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Then in this contribution, we would like to provide our views on remaining issues for A-IoT paging between reader (gNB or intermediate UE) and A-IoT device(s).

Discussion

## 2.1 Background （RFID design）

In radio-frequency identification (RFID) [1], the signaling interaction between reader and tag(s) in the inventory and access process is shown in Figure 1.

Figure 1: Illustration of an inventory and access process in RFID

From Fig.1, it can be found that the paging-like functionality in RFID is achieved with the Select command and Query command. The reader groups the tags with the Select command and identify the number of available slots to targeted tags with the Query command. Specifically, the reader can set the selected flags/inventoried flags of a group of tags with Select command. Subsequently, the Query command is transmitted by the reader, tags with matched selected flag/inventoried flag and session will randomly select a slot to response between 0 and 2Q-1 in which Q is contained in Query command. In other words, it can be considered that the explicit time-domain resources is provided to tags with Query command.

Observation 1: In RFID, the explicit indication of time-domain resources is provided to tags with Query command.

## 2.2 A-IoT paging

### 2.2.1 Message content

2.2.1.1  Identifier of target A-IoT device(s)

In RAN2#126 meeting, it has been agreed that [2]:

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

It has been agreed that single device ID, group ID and no device ID can be contained in A-IoT paging message, while whether multiple device IDs can be contained need to be confirmed by SA2. For the case that multiple device IDs are contained in A-IoT paging, we have the following observations on the aspects other than use case. If an A-IoT paging containing multiple device IDs is supported, the maximum number of device IDs that can be included is influenced by the length of the device ID and the maximum amount of data that can be transmitted at once in the physical layer. Besides, the case that multiple deive IDs contained in an A-IoT paging may increase the length of A-IoT paging message and thus, increase the energy consumption for an A-IoT device receiving the whole A-IoT paging message before determining whether the A-IoT paging is target to it. While, the case that multiple device IDs contained in an A-IoT paging may slao bring some benefits. For example, compared with transmitting multiple A-IoT paging each with single device ID, transmitting an A-IoT paging with multiple device IDs may reduce the signaling overhead to some extent, especially when performing the same operation on them.

Observation 2: Containing multiple device IDs in an A-IoT paging message may increase the energy consumption for an A-IoT device receiving the whole A-IoT paging message before determining whether the A-IoT paging is target to it.

Observation 3: Containing multiple device IDs in an A-IoT paging message may reduce the signaling overhead, especially when performing the same operation on those devices.

2.2.1.2  Identifier of trigger cause

In RAN2#126 meeting, it is agreed that:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

It is agreed that the baseline procedures of “inventory only” and “inventory and command” case are both initiated by A-IoT paging transmission. For the procedure of “Command only” case, it can also be initiated by A-IoT paging transmission. From our point of view, the A-IoT paging can be not only one message but also a set of messages. For example, during the procedure of “command only” case, an A-IoT paging message whose content similar to A-IoT paging message for “inventory only” and “inventory and command” procedure is transmitted to A-IoT devices to identify target devices and indicate D2R resources for response at first. Then a subsequent A-IoT paging which contains the command (Write, Read, etc.) is transmitted. In this way, it is beneficial for device monitoring A-IoT paging because the first A-IoT paging message are always in similar formats even for different procedures. However, compared to sending only one A-IoT paging, sending a set of A-IoT paging messages may slightly increase signaling overhead, considering that the physical layer may add preamble, CRC, etc. separately.

With the consideration that A-IoT devices may conduct different follow-up processes according to different trigger cause of A-IoT paging. For example, if the A-IoT paging is used to inventory, the target device(s) perform random access. While, if a read command is sent as an A-IoT paging, the target device(s) may transmit the respond of read command directly. Therefore, an identifier should be contained in A-IoT paging for target A-IoT device(s) identifying different trigger causes (e.g. inventory, read, write, etc.).

Observation 4: For the procedure of “command only” case, transmitting an A-IoT paging message whose content similar to “inventory only” and “inventory and command” procedure at first may be beneficial for device monitoring.

Proposal 1: An A-IoT paging may include one message or a set of messages.

Proposal 2: In A-IoT paging, an identifier should be contained for target A-IoT devices identifying the trigger cause (e.g. inventory, read, write, etc.).

2.2.1.3  Identifier of multiple access method

In RAN1#116-bis meeting, it has been agreed that [2]:

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

It can be seen that both TDMA and FDMA of D2R transmissions are agreed to be studied by RAN1, while whether CDMA is feasible and necessary needs further discussion. If at least two multiple access methods (such as: TDMA, FDMA, etc.) are supported, one identifier which marks it may be contained in A-IoT paging. Whether the identifier is explicitly or implicitly contained in A-IoT paging needs further discussion.

Proposal 3: A-IoT paging can indicate the multiple access method of target devices (e.g. TDMA, FDMA, CDMA, etc.). FFS through explicit or implicit parameter

2.2.1.4  Identifier of D2R transmission resources

In RAN2#126 meeting, it has been agreed that:

Agreement - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

Based on the study in RAN1, the D2R transmission resources may include time-domain (such as access occasion) and/or frequency-domain (such as the frequency-shift) and/or code-domain resources. From our point of view, the D2R transmission resources can be shared D2R resources or dedicated D2R resources, in which dedicated D2R resources represents the access resources assigned to specific Ambient IoT device, while shared D2R resources denote the common resources which should be shared by all paged Ambient IoT devices. In other words, Ambient IoT devices with dedicated D2R resources can access/respond without competition, while Ambient IoT devices with shared D2R resources will compete with each other and may collide when responding. Note that in order to allocate dedicated D2R resources, one device ID or a list of device IDs (if multiple device IDs containing in an A-IoT paging is supported) should be contained in A-IoT paging. While, for the A-IoT paging targeted to one A-IoT device, it is not necessary to contain explicit indication of D2R transmission resources. Note that the allocation of D2R resources is related to the number of paged devices and some other factors. For example, if ten devices are targeted, at least ten access occasions should be allocated to avoid collisions as soon as possible if TDMA is applied for D2R transmission. With the consideration that the radio resources should be allocated by gNB, then the number of target A-IoT devices should also be known by gNB. Otherwise, the gNB should always estimates the device number, which may bring larger overhead and decrease the efficiency. While note that for paging all A-IoT devices, the procedure of estimating the number of devices which is given in our another papaer [3] may be necessary. With the number of target A-IoT devices, gNB can allocated D2R resources for A-IoT radio efficiently. To this end, RAN2 can assume that the number of paged devices can be known by gNB through explicit parameter from CN or estimated by gNB. Besides, the type of A-IoT paging message transmitted from CN (e.g. inventory, read, write, etc.) should also be visible to gNB to assistant resources allocation of A-IoT radio more efficiently.

Proposal 4: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 5: There may be no explicit indication of D2R transmission resources when paging a single device.

Proposal 6: RAN2 assume that the number of paged devices can be known by gNB through explicit parameter from CN or estimated by gNB. FFS how to estimate the number of devices

Proposal 7: The type of A-IoT paging message transmitted from CN (e.g. inventory, read, write, etc.) should be visible to gNB. FFS whether other parameters should also be known by gNB

2.2.1.4 Other identifier

What’s more, in NR, the paging triggered by CN is broadcasted among all cells within the corresponding TAI list, while the paging triggered by RAN is broadcasted among all cells within the RAN Notification Area (RNA). For Ambient IoT, it has been approved that RAN2 assumes that the device will not support tracking/RAN area update procedure. To improve the efficiency of A-IoT paging, it is beneficial for gNB or cN to maintain an association relationship between A-IoT devices and associated reader/area. For example, a command or inventory message can be sent to the reader associated with the target device which sends an A-IoT paging to target device as an initial attempt. If the response of target device is not received, that is, A-IoT paging failure occurs, more readers can be selected to send A-IoT paging. With the consideration of support the association, re-association, etc. efficiently, there may also be the identifier of the reader or something like A-IoT paging area contained in A-IoT paging. For example, when a reader ID is contained in A-IoT paging, the A-IoT device can identify its association relationship with reader by recording reader ID. Then when an A-IoT device finds the reader ID contained in A-IoT paging is different from the reader ID saved, it can perceive its association relationship with reader changes. With that, the gNB-reader can inventory the A-IoT device(s) whose association relationship with reader changes by transmitting specific A-IoT paging instead of always inventorying all A-IoT devices.

What’s more, when different types of random access procedure (e.g. 2-step CBRA, 3-step CBRA, CFRA, etc.) are supported in Ambient IoT system, an identifier which indicates the type of random access may be needed. Note that CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated. However, for random access procedures that cannot rely on implicit indications in A-IoT paging for differentiation, such as 2-step CBRA and 3-step CBRA, an identifier is necessary.

Observation 5: It is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated area to improve the efficiency of A-IoT paging.

Proposal 8: To improve the efficiency of A-IoT paging, an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging.

Proposal 9: When multiple random access types (e.g. 2-step CBRA, 3-step CBRA etc.) are supported, an identifier indicating the type of random access may be contained in A-IoT paging.

### 2.2.2 Initiation

From our point of view, an A-IoT paging can be initiated by core network to inventory A-IoT device(s) or send read or write command to the A-IoT device(s). As for reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system saved in reader side which is not in RAN2 scope. For example, if the association relationship is saved in gNB-reader, then it may trigger an A-IoT paging to update the association relationship between reader and A-IoT device(s). To this end, RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

Observation 6: For reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system saved in reader side.

Proposal 10: RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

### 2.2.3 Transmitting behavior

In RAN1#117, it has been agreed as following [4]:

Agreement For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH

It is clear that PRDCH is the only physical channel for R2D, thus, the A-IoT paging is transmitted on PRDCH.

Proposal 11: The A-IoT paging is transmitted on PRDCH by a reader to Ambient IoT devices.

In RAN2#125-bis meeting, it has been approved that:

- RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

From our point of view, the introduction of a new AS protocol on top of A-IoT MAC layer is not necessary but increasing the complexity and power consumption of A-IoT devices. To this end, A-IoT paging is transmitted in A-IoT MAC layer.

Proposal 12: A-IoT paging is transmitted in A-IoT MAC layer.

### 2.2.4 Monitoring behavior and determination of transmission/access occasion

In RAN2#126 meeting, it is agreed that:

- From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

In RAN1#117 meeting, it is agreed that:

Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission.

It is clear that both monitoring behavior and the determination of transmission/access occasion are closely related to the timing accuracy of A-IoT devices which is still studying in RAN1. Therefore, for monitoring behavior, we can assume the device can receive as long as there is enough energy until more progress or agreements are made by RAN1. For determining the corresponding D2R transmission/access occasion after A-IoT paging, we can assume there is R2D clock signal, which is similar to QueryRep in RFID, helps A-IoT devices count the time until more progress or agreements are made by RAN1.

Observation 7: Both monitoring behavior and the determination of transmission/access occasion are closely related to the timing accuracy of A-IoT devices which is still studying in RAN1.

Proposal 13: For determining the corresponding D2R transmission/access occasion after A-IoT paging, RAN2 assume there is R2D clock signal, which is similar to QueryRep in RFID until more progress in RAN1.

### 2.2.5 A-IoT paging procedure

Based on above analysis, the general A-IoT paging procedure can be given in Fig. 2.

Figure 2: General A-IoT paging procedure

To be specific, the CN initiates an A-IoT paging and transmits it to reader. Note that the detailed procedure between gNB-reader or intermediate UE and CN are not analyzed in this paper. After the available A-IoT radio resources is configured, the reader transmits an A-IoT paging (trigger cause, device ID(s)/group ID/no ID, dedicated/shared D2R resources etc.) in A-IoT MAC layer.

Proposal 14: The A-IoT paging procedure in A-IoT radio shown in Figure 2 can be considered as a baseline for RAN2.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on paging for Ambient IoT:

Observation 1: In RFID, the explicit indication of time-domain resources is provided to tags with Query command.

Observation 2: Containing multiple device IDs in an A-IoT paging message may increase the energy consumption for an A-IoT device receiving the whole A-IoT paging message before determining whether the A-IoT paging is target to it.

Observation 3: Containing multiple device IDs in an A-IoT paging message may reduce the signaling overhead, especially when performing the same operation on those devices.

Observation 4: For the procedure of “command only” case, transmitting an A-IoT paging message whose content similar to “inventory only” and “inventory and command” procedure at first may be beneficial for device monitoring.

Observation 5: It is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated area to improve the efficiency of A-IoT paging.

Observation 6: For reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system saved in reader side.

Observation 7: Both monitoring behavior and the determination of transmission/access occasion are closely related to the timing accuracy of A-IoT devices which is still studying in RAN1.

Proposal 1: An A-IoT paging may include one message or a set of messages.

Proposal 2: In A-IoT paging, an identifier should be contained for target A-IoT devices identifying the trigger cause (e.g. inventory, read, write, etc.).

Proposal 3: A-IoT paging can indicate the multiple access method of target devices (e.g. TDMA, FDMA, CDMA, etc.). FFS through explicit or implicit parameter

Proposal 4: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 5: There may be no explicit indication of D2R transmission resources when paging a single device.

Proposal 6: RAN2 assume that the number of paged devices can be known by gNB through explicit parameter from CN or estimated by gNB. FFS how to estimate the number of devices

Proposal 7: The type of A-IoT paging message transmitted from CN (e.g. inventory, read, write, etc.) should be visible to gNB. FFS whether other parameters should also be known by gNB

Proposal 8: To improve the efficiency of A-IoT paging, an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging.

Proposal 9: When multiple random access types (e.g. 2-step CBRA, 3-step CBRA etc.) are supported, an identifier indicating the type of random access may be contained in A-IoT paging.

Proposal 10: RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

Proposal 11: The A-IoT paging is transmitted on PRDCH by a reader to Ambient IoT devices.

Proposal 12: A-IoT paging is transmitted in A-IoT MAC layer.

Proposal 13: For determining the corresponding D2R transmission/access occasion after A-IoT paging, RAN2 assume there is R2D clock signal, which is similar to QueryRep in RFID until more progress in RAN1.

Proposal 14: The A-IoT paging procedure in A-IoT radio shown in Figure 2 can be considered as a baseline for RAN2.

Reference

- EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID, v2.0.1.

- “RAN1 Chair’s Notes”, RAN1#116-bis, Changsha, Hunan Province, China.

- “Further consideration on Ambient IoT random access”, RAN2#127, Maastricht, Netherlands, CMCC.

- “RAN1 Chair’s Notes”, RAN1#117, Fukuoka City, Fukuoka, Japan.