3GPP TSG RAN WG2 Meeting #127	R2-2406961

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.5

Source:	CMCC

Title:	Discussions on topology 2 for A-IoT

Document for:	Discussion

Introduction

In RAN#103 meeting, a revised SID on solutions for Ambient IoT (Internet of Things) in NR has been approved [1]. The following RAN2-led objectives have been identified within the general scope described in the SID:

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor

Then in this contribution, we would like to provide our considerations on Ambient IoT with topology 2.

Discussion

### 2.1 General

In revised SID [1], it has been stated that the overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT. To this end, RAN2 should manage to design a harmonized air interface for A-IoT radio between reader and A-IoT device(s) among topology 1 and topology 2. Besides, for the topology 2, the air interface between intermediate UE and gNB should also be studied. From our point of view, NR Uu should be the air interface between intermediate UE and gNB. Besides, the impact on legacy NR Uu interface and procedures should be minimized as much as possible.

Proposal 1: RAN2 should design a harmonized air interface of A-IoT radio as much as possible for topology 1 and 2.

Proposal 2: Reuse NR Uu as the air interface between intermediate UE and gNB, the impact on legacy NR Uu interface and procedures should be minimized as much as possible.

### 2.2 Selection of intermediate UE

For the selection of intermediate UE, it may be conducted by gNB or CN. To be specific, for the intermediate UE selection dominated by the CN, the A-IoT capable UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as an intermediate UE. While, for the intermediate UE selection dominated by gNB, the A-IoT capable UE in RRC_INACTIVE and RRC_CONNECTED state can be selected as an intermediate UE. For an intermediate UE, before it sends inventory or command, it should be configured with available A-IoT radio resources by gNB which is detailed in section 2.2. Thus, the intermediate UE should be in RRC_CONNECTED state at least when receiving the inventory or command and the A-IoT radio resource configuration. If a UE in RRC_IDLE is selected as an intermediate UE, a CN-initiated paging should be transmitted to transform it to RRC_CONNECTED. And if a UE in RRC_INACTIVE is selected as an intermediate UE, a RAN-initiated paging is transmitted. Note that after intermediate UE receiving A-IoT inventory/command message and the A-IoT radio resource configuration from network side, UE can be released to RRC_IDLE or RRC_INACTIVE or maintained in RRC_CONNECTED during subsequent procedure in A-IoT radio.

Observation 1:  For the intermediate UE selection dominated by the CN, the A-IoT capable UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as an intermediate UE.

Observation 2:  For the intermediate UE selection dominated by gNB, the A-IoT capable UE in RRC_INACTIVE and RRC_CONNECTED state can be selected as an intermediate UE.

Proposal 3: At least RRC_CONNECTED and RRC_INACTIVE UE can be selected as an intermediate UE.

Proposal 4: The intermediate UE should be in RRC_CONNECTED state at least when receiving the inventory or command and the A-IoT radio resource configuration.

Proposal 5: After intermediate UE receiving A-IoT inventory or command message and A-IoT radio resource configuration from network side, UE can be released to RRC_IDLE or RRC_INACTIVE or maintained in RRC_CONNECTED during subsequent procedure in A-IoT radio.

### 2.2 Resource allocation for intermediate UE

In revised SID, it is clarified in general scope that:

- B.Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - C. FR1 licensed spectrum in FDD.

Considering that the spectrum used by A-IoT link between intermediate UE and A-IoT device is licensed spectrum and the intermediate UE is under network control, gNB should be responsible for allocating and scheduling the A-IoT radio transmission resources. In other words, the intermediate UE can communicate with A-IoT device(s) only after it is configured with available A-IoT radio resource. The detailed methods may include:

- Option 1: The intermediate UE receives the configuration of A-IoT radio resources from gNB without request.

- Option 2: The intermediate UE requests the A-IoT radio resources from gNB.

Proposal 6: The A-IoT radio transmission resources used by intermediate UE should be allocated by gNB.

Proposal 7: For an intermediate UE, the methods to obtain A-IoT radio resources include:

- Option 1: The intermediate UE receives the configuration of A-IoT radio resources from gNB without request..

- Option 2: The intermediate UE requests the A-IoT radio resources from gNB.

### 2.3 Signaling/data transmission procedure

In 3GPP TR 23.700-13 v0.4.0, some solutions for Topology 2 has been captured by SA2. In solution #33, NR Uu RRC layer is used to forward the NAS messages/data between intermediate UE and CN, which is called RRC-based solution in this paper. In solution #4/11/12/21/30, the NAS layer is the end-to-end layer between intermediate UE and CN, which is called NAS-based solution in this paper. From our point of view, the signaling/data transmission procedure between intermediate UE and gNB are analyzed as following:

### 2.3.1 RRC-based solution

Considering that the signaling or data interacted between gNB and intermediate UE is RRC message, the protocol stack and corresponding procedure can be given in Figure 1 and Figure 2, respectively.

Figure 1: Protocol stack of Topology 2 with RRC-based solution

Figure 2: Procedure of RRC-based solution

- If the intermediate UE is not in RRC_CONNECTED state, a paging is transmitted to transfer it to RRC_CONNECTED state.

- The gNB receives an inventory or command message transmitted from CN in NG/XX-AP interface.

- The gNB generates and transmits a RRC message which contains inventory or command to intermediate UE.

- The intermediate UE transmits an A-IoT paging to A-IoT device(s) and conduct the corresponding procedure indicated in the received RRC message.

- A-IoT device transmits its response to intermediate UE.

- The intermediate UE transmits a RRC message with device response to gNB.

- The gNB transmits a message with device response to CN in NG/XX-AP interface.

Note: The configuration of A-IoT radio resource can be transmitted to intermediate without request before step 3, e.g. in step 2.

Observation 3: For RRC-based solution, the gNB or intermediate UE generates and transmits a RRC message which contains command or response.

Proposal 8: For Topology 2 protocol stack, RAN2 can capture Figure 1 as the baseline of the RRC-based solution.

Proposal 9: RAN2 can capture the procedure of RRC-based solution shown in Figure 2 as a baseline.

### 2.3.2 NAS-based solution

In this case, CN interacts with intermediate UE using NAS message directly. The protocol stack and corresponding procedure can be given in Figure 3 and Figure 4, respectively.

Figure 3: Protocol stack of Topology 2 with NAS-based solution

Figure 4: Procedure of RRC-based solution

- If the intermediate UE is not in RRC_CONNECTED state, a paging is transmitted to transfer it to RRC_CONNECTED state.

- The intermediate UE receives a NAS message which contains service notification from CN.

- The intermediate UE request the A-IoT radio resources from gNB. FFS details

3.The intermediate UE transmits the A-IoT paging to A-IoT device(s) and conduct the corresponding procedure indicated in the received RRC message.

4.A-IoT device transmits its response to intermediate UE.

5. The intermediate UE transmits a NAS message which contains the device response to CN.

Observation 4: For NAS-based solution, the CN or intermediate UE transmits a NAS message which contains command or response.

Observation 5: For NAS-based solution, the intermediate UE needs to request the A-IoT radio resources from gNB.

Proposal 10: For Topology 2 protocol stack, RAN2 can capture Figure 3 as the baseline of the NAS-based solution.

Proposal 11: RAN2 can capture the procedure of NAS-based solution shown in Figure 4 as a baseline.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some considerations for topology 2 as following:

Observation 1:  For the intermediate UE selection dominated by the CN, the A-IoT capable UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state can be selected as an intermediate UE.

Observation 2:  For the intermediate UE selection dominated by gNB, the A-IoT capable UE in RRC_INACTIVE and RRC_CONNECTED state can be selected as an intermediate UE.

Observation 3: For RRC-based solution, the gNB or intermediate UE generates and transmits a RRC message which contains command or response.

Observation 4: For NAS-based solution, the CN or intermediate UE transmits a NAS message which contains command or response.

Observation 5: For NAS-based solution, the intermediate UE needs to request the A-IoT radio resources from gNB.

Proposal 1: RAN2 should design a harmonized air interface of A-IoT radio as much as possible for topology 1 and 2.

Proposal 2: Reuse NR Uu as the air interface between intermediate UE and gNB, the impact on legacy NR Uu interface and procedures should be minimized as much as possible.

Proposal 3: At least RRC_CONNECTED and RRC_INACTIVE UE can be selected as an intermediate UE.

Proposal 4: The intermediate UE should be in RRC_CONNECTED state at least when receiving the inventory or command and the A-IoT radio resource configuration.

Proposal 5: After intermediate UE receiving A-IoT inventory or command message and A-IoT radio resource configuration from network side, UE can be released to RRC_IDLE or RRC_INACTIVE or maintained in RRC_CONNECTED during subsequent procedure in A-IoT radio.

Proposal 6: The A-IoT radio transmission resources used by intermediate UE should be allocated by gNB.

Proposal 7: For an intermediate UE, the methods to obtain A-IoT radio resources include:

- Option 1: The intermediate UE receives the configuration of A-IoT radio resources from gNB without request..

- Option 2: The intermediate UE requests the A-IoT radio resources from gNB.

Proposal 8: For Topology 2 protocol stack, RAN2 can capture Figure 1 as the baseline of the RRC-based solution.

Proposal 9: RAN2 can capture the procedure of RRC-based solution shown in Figure 2 as a baseline.

Proposal 10: For Topology 2 protocol stack, RAN2 can capture Figure 3 as the baseline of the NAS-based solution.

Proposal 11: RAN2 can capture the procedure of NAS-based solution shown in Figure 4 as a baseline.

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, Maastricht, Netherlands, CMCC, Huawei, T-Mobile USA.