3GPP TSG RAN WG2 Meeting #127	R2-2406962

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on random access in SBFD

Document for:	Discussion

Introduction

In revised WID[1], the following objectives are given for random access in SBFD, shown as:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

Then in this contribution, we would like to provide our views on RAN2 impacts to support random access in SBFD.

Discussion

## 2.1 General

In RAN1#116 meeting, the following working assumption and agreements are made:

Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

In RAN1#116-bis meeting, the working assumption in RAN1#116 is confirmed. In RAN1#117, it is further agreed that:

Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

To summarize, RAN1 support the random access in SBFD symbols for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states. From RAN2’s perspective, RAN2 confirms supporting the random access of SBFD aware UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

Proposal 1: RAN2 confirms supporting the random access of SBFD aware UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

There are different CBRA and CFRA events and corresponding PRACH resource indication/configuration signallings as summarized in Table 1.

Table 1. RA events and corresponding PRACH resource indication/configuration signallings

| RA event | CBRA PRACH resource indication/configuration signalling | CFRA PRACH resource indication/configuration signalling |
| --- | --- | --- |
| Initial access from RRC_IDLE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| RRC Connection Re-establishment procedure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | - CBRA resource configured by RACH-ConfigCommon - PDCCH order trigger | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| SR failure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Request by RRC upon synchronous reconfiguration (e.g. handover) | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by RACH-ConfigDedicated |
| RRC Connection Resume procedure from RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| To establish time alignment for a secondary TAG | - N/A | - CFRA resource configured by RACH-ConfigCommon - PDCCH order indicating the used PRACH resource and preamble |
| Request for Other SI | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by SI-RequestConfig or SI-RequestConfigRepetition |
| Beam failure recovery | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource, preamble and root sequence configured by BeamFailureRecoveryConfig |
| SDT in RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| Early UL synchronization with an LTM candidate cell | - N/A | - CFRA resource configured by EarlyUL-SyncConfig - PDCCH order trigger |
| RACH-based LTM cell switch | - CBRA resource configured by RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger | - CFRA resource configured by RACH-ConfigDedicated or RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger |

From Table 1, it can be observed that there are multiple events which can trigger random access procedures, what’s more, different RA procedures (CBRA or CFRA) may be triggered by different RA events. For RAN2, whether supporting all RA events for SBFD aware UEs in R19 needs to be discussed. If not all RA events are supported, which RA event is not supported in R19 SBFD needs to be studied as well. In fact, the supported RA events are related to the supported random access procedures. For example, CFRA with 2-step RA type is only supported for handover in NR, then if handover is not supported in SBFD, CFRA with 2-step RA type is not supported in SBFD accordingly.

Observation 1: Multiple different events can trigger random access procedures. For different RA events, different RA procedure (CBRA or CFRA) may be triggered.

Proposal 2: RAN2 is asked to study whether supporting all RA events for SBFD aware UEs conducting SBFD operation in R19.

In RAN1, it has been agreed that for SBFD aware UEs in RRC_CONNECTED state, 4-step CBRA and 4-step CFRA is supported, while whether supporting 2-step CBRA and 2-step CFRA needs further discussion. For 2-step RACH, it brings smaller latency compared with 4-step RACH. However, the introduction of SBFD can already reduce latency since the ROs in SBFD symbols are also valid for SBFD aware UEs. Therefore, we do not think that studying 2-step RACH is urgent for RAN2. Furthermore, it can also be found that CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration signallings for different RA events. With the consideration that the discussion in RAN1’s discussion is mainly based on RACH-ConfigCommon and CFRA PRACH resource can be configured by many different signallings, RAN2 can start the studying from 4-step CBRA for SBFD aware UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state at first. After the research has reached a certain level, RAN2 can extend some or all of the applicable agreements to other random access procedures to be studied, and focus on their unique design.

Observation 2: In RAN1, it has been agreed that for SBFD aware UEs in RRC_CONNECTED state, 4-step CBRA and 4-step CFRA is supported, while whether supporting 2-step CBRA and 2-step CFRA needs further discussion.

Observation 3: The introduction of SBFD can reduce the latency of 4-step RACH, thus it is not urgent for RAN2 to study 2-step RACH.

Proposal 3: RAN2 can start the studying from 4-step CBRA for SBFD aware UEs.

## 2.2 	Configuration of RACH resource

In RAN1#117 meeting, it has been agreed that [2]:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Agreement Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

In RAN1, RACH configuration Option 1 with Alt 1-1 and Option 2 has been agreed for SBFD aware UEs in RRC_CONNECTED state. From our point of view, above agreements can also be extended to SBFD aware UEs in RRC_IDLE and RRC_INACTIVE. Considering that only CBRA resources can be configured to  SBFD aware UEs in RRC_IDLE state by system information, RAN2 should support configuring 4-step CBRA resource by system information. In summary, 4-step CBRA resource for SBFD aware UEs can be configured at least by system information.

Proposal 4: 4-step CBRA resource for SBFD aware UEs can be configured at least by system information.

For detailed configuration methods, there are two possible solutions. One solution is to reuse the Rel-17 Feature Combination related random access design framework, that is, SBFD is considered as a new feature. According to current NR design, featureCombinationPreamblesList can be configured in rach-ConfigCommon or AdditionalRACH-Config-r17. Therefore, for RACH configuration Option 1 with Alt 1-1, the RACH configuration can be provided with featureCombinationPreamblesList in rach-ConfigCommon. For RACH configuration Option 2, the additional RACH configuration can be provided with featureCombinationPreamblesList in AdditionalRACH-Config-r17. In terms of the random access procedure, the SBFD feature will be taken in consideration when conducting selection of the set of Random Access resources for a SBFD aware UE. It can be expected that a SBFD aware UE will prioritize selecting Random Access resources available for SBFD feature.

Apart from this, in current NR design, 2-step RA and 4-step RA are configured in different IEs. Specifically, 2-step RA is configured in msgA-ConfigCommon-r16, while 4-step RA is configured in rach-ConfigCommon. Similar to this, the other solution is to introduce a new IE named rach-ConfigCommonSBFD to configure the 4-step CBRA resources for SBFD aware UEs. To this end, the 4-step CBRA configuration for SBFD is configured in rach-ConfigCommon when using RACH configuration Option 1 with Alt 1-1. For 4-step CBRA configuration Option 2, the RACH configuration for SBFD is configured in rach-ConfigCommonSBFD. An example is shown as:

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF  RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

additionalRACH-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF AdditionalPCIIndex-r17

OPTIONAL,  -- Need N

rsrp-ThresholdMsg1-RepetitionNum2-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum4-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum8-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

preambleTransMax-Msg1-Repetition-r18     ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}      OPTIONAL   -- Cond Msg1Rep1

]]

[[

rach-ConfigCommonSBFD-r19            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

In terms of the random access procedure, RAN2 should discuss how a SBFD aware UE selects Random Access resources between legacy random access resources and additional random access resources for SBFD aware UEs.

Observation 4: There are two possible solutions to configure 4-step CBRA resources for SBFD aware UEs:

- Solution 1: SBFD is considered as a new feature and the Rel-17 Feature Combination related random access framework is reused.

- Solution 2: Parallel to rach-ConfigCommon, a new IE (e.g. rach-ConfigCommonSBFD) is introduced.

Proposal 5: RAN2 considers following solutions for 4-step CBRA configuration:

- Solution 1: SBFD is considered as a new feature and the Rel-17 Feature Combination related random access framework is reused.

- Solution 2: Parallel to rach-ConfigCommon, a new IE rach-ConfigCommonSBFD is introduced. FFS how to select Random Access resources between legacy random access resources and additional random access resources for SBFD aware UEs

- FFS other solutions

Based on the progress in RAN1, it has been agreed that enabling both RACH configuration Option 1 with Alt 1-1 and RACH configuration Option 2 at the same time for a UE is not supported. If each SBFD aware UE can support only one RACH configuration Option, that is, either RACH configuration Option 1 with Alt 1-1 or Option 2, no additional work is required in RAN2. Otherwise, RAN2 should discuss how a SBFD aware UE identify which RACH configuration Option is enabled. From network configuration perspective, an easy way to indicate this is introducing a flag which indicates either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2 is activated at the moment. In fact, the flag mentioned above may not be an explicit flag. For example, if RACH configuration Option 2 is provided in RACH resource configuration signalling, then RACH configuration Option 2 is considered as enabled and RACH configuration Option 1 with Alt 1-1 is disabled. Taking Solution 2 above as an example, if rach-ConfigCommonSBFD is present, then rach-ConfigCommon configures legacy ROs only, in which legacy ROs denote the ROs in uplink or flexible symbols, which are available for both SBFD aware and non-SBFD aware UEs. Note that, to indicate the SBFD aware UEs in all three RRC states, the flag mentioned above should be introduced in system information (e.g. SIB1, etc.).

Observation 5: If each SBFD aware UE support only one RACH configuration Option, that is, either RACH configuration Option 1 with Alt 1-1 or Option 2, no additional work is required to indicate which RACH configuration Option is enabled in RAN2.

Proposal 6: To indicate which RACH configuration Option is enabled, there are two cases:

- Case 1: If each SBFD aware UE can support only one RACH configuration Option, no additional work is required.

- Case 2: If there are SBFD aware UE(s) which supports two RACH configuration Options, a flag can be introduced. FFS the flag is explicit or implicit

Proposal 7: If needed, a flag which indicating which RACH configuration Option is enabled can be introduced in system information (e.g. SIB1, etc.).

## 2.3 	Early indication

The gNB can identify a SBFD UE which transmits Msg1 in an additional RO since additional RO is in SBFD symbol. However, considering that legacy ROs are valid for both SBFD aware UEs and non-SBFD UEs, one issue needs to be discussed by RAN2 is whether to support early indication of SBFD aware UEs, which can be achieved by either Msg1-based or Msg3-based early indication.

- Msg1-based early indication

If Msg1-based early indication is supported, dedicated preamble(s) can be allocated to SBFD aware UEs. With that, gNB can identify a SBFD aware UE based on the preamble received and thus schedule subsequent uplink transmission (e.g. Msg3, etc.) in SBFD symbol(s) for SBFD aware UE, which brings benefits. However, if some dedicated preambles are allocated to SBFD aware UEs, accordingly, the number of preambles which can be used by non-SBFD aware UEs decreases, thus cause the degradation of PRACH capacity of non-SBFD aware UEs.

If Msg1-based early indication is not supported, the PRACH capacity of non-SBFD aware UEs will not be reduced, but gNB can not schedule the subsequent transmission (e.g. Msg3, etc.) in SBFD symbol(s) earlier for SBFD aware UE.

- Msg3-based early indication

- If Msg3-based early indication is supported, dedicated LCID(s) can be allocated to SBFD aware UEs. With that, gNB can identify a SBFD aware UE based on the logical channel ID in Msg3 and thus schedule subsequent uplink transmission in SBFD symbol(s) for SBFD aware UE which brings benefits. Considering using reserved LCID(s) in present NR, there is almost no influence on the SBFD aware UE itself and other UEs. However, compared with Msg1-based early indication, gNB identifies whether the UE is an SBFD aware UE later, thus can only schedule uplink transmission(s) after Msg3 to transmit on the SBFD symbol(s).

- From our point of view, RAN2 is proposed to support Msg3-based early indication for SBFD aware UEs, because it brings little influence to both SBFD aware UE(s) and non-SBFD aware UE(s). For Msg1-based early indication, whether it is supported depends on RAN1’s discussion. While, it should also be noted that if SBFD is considered as a new feature and the RANCH configuration for SBFD aware UEs is provided with featureCombinationPreamblesList, then Msg1-based early indication can be supported easily.

Observation 6: The gNB can identify a SBFD UE which transmits Msg1 in an additional RO which is in SBFD symbol.

Observation 7: If Msg1-based early indication is supported, gNB can identify a SBFD aware UE early and thus schedule its subsequent transmission (e.g. Msg3) in SBFD symbol(s), but may reduce the PRACH capacity of non-SBFD aware UEs.

Observation 8: If Msg1-based early indication is not supported, the PRACH capacity of non-SBFD aware UEs will not be reduced, but gNB can not schedule its subsequent transmission (e.g. Msg3) in SBFD symbol(s).

Observation 9: With Msg3-based early indication, there is almost no influence on the SBFD aware UE itself and other UEs. However, gNB identifies whether the UE is an SBFD aware UE a little later compared with using Msg1-based early indication.

Observation 10: If SBFD is considered as a new feature and the RANCH configuration for SBFD aware UEs is provided with featureCombinationPreamblesList, then Msg1-based early indication can be supported easily.

Proposal 8: RAN2 is proposed to support Msg3-based early indication for SBFD aware UEs. Whether supporting Msg1-based early indication depends on RAN1.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: Multiple different events can trigger random access procedures. For different RA events, different RA procedure (CBRA or CFRA) may be triggered.

Observation 2: In RAN1, it has been agreed that for SBFD aware UEs in RRC_CONNECTED state, 4-step CBRA and 4-step CFRA is supported, while whether supporting 2-step CBRA and 2-step CFRA needs further discussion.

Observation 3: The introduction of SBFD can reduce the latency of 4-step RACH, thus it is not urgent for RAN2 to study 2-step RACH.

Observation 4: There are two possible solutions to configure 4-step CBRA resources for SBFD aware UEs:

- Solution 1: SBFD is considered as a new feature and the Rel-17 Feature Combination related random access framework is reused.

- Solution 2: Parallel to rach-ConfigCommon, a new IE (e.g. rach-ConfigCommonSBFD) is introduced.

Observation 5: If each SBFD aware UE support only one RACH configuration Option, that is, either RACH configuration Option 1 with Alt 1-1 or Option 2, no additional work is required to indicate which RACH configuration Option is enabled in RAN2.

Observation 6: The gNB can identify a SBFD UE which transmits Msg1 in an additional RO which is in SBFD symbol.

Observation 7: If Msg1-based early indication is supported, gNB can identify a SBFD aware UE early and thus schedule its subsequent transmission (e.g. Msg3) in SBFD symbol(s), but may reduce the PRACH capacity of non-SBFD aware UEs.

Observation 8: If Msg1-based early indication is not supported, the PRACH capacity of non-SBFD aware UEs will not be reduced, but gNB can not schedule its subsequent transmission (e.g. Msg3) in SBFD symbol(s).

Observation 9: With Msg3-based early indication, there is almost no influence on the SBFD aware UE itself and other UEs. However, gNB identifies whether the UE is an SBFD aware UE a little later compared with using Msg1-based early indication.

Observation 10: If SBFD is considered as a new feature and the RANCH configuration for SBFD aware UEs is provided with featureCombinationPreamblesList, then Msg1-based early indication can be supported easily.

Proposal 1: RAN2 confirms supporting the random access of SBFD aware UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state.

Proposal 2: RAN2 is asked to study whether supporting all RA events for SBFD aware UEs conducting SBFD operation in R19.

Proposal 3: RAN2 can start the studying from 4-step CBRA for SBFD aware UEs.

Proposal 4: 4-step CBRA resource for SBFD aware UEs can be configured at least by system information.

Proposal 5: RAN2 considers following solutions for 4-step CBRA configuration:

- Solution 1: SBFD is considered as a new feature and the Rel-17 Feature Combination related random access framework is reused.

- Solution 2: Parallel to rach-ConfigCommon, a new IE rach-ConfigCommonSBFD is introduced. FFS how to select Random Access resources between legacy random access resources and additional random access resources for SBFD aware UEs

- FFS other solutions

Proposal 6: To indicate which RACH configuration Option is enabled, there are two cases:

- Case 1: If each SBFD aware UE can support only one RACH configuration Option, no additional work is required.

- Case 2: If there are SBFD aware UE(s) which supports two RACH configuration Options, a flag can be introduced. FFS the flag is explicit or implicit

Proposal 7: If needed, a flag which indicating which RACH configuration Option is enabled can be introduced in system information (e.g. SIB1, etc.).

Proposal 8: RAN2 is proposed to support Msg3-based early indication for SBFD aware UEs. Whether supporting Msg1-based early indication depends on RAN1.

Reference

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104,  Huawei.

- “RAN1 Chair’s Notes”, RAN1#117, Fukuoka City, Fukuoka, Japan.