3GPP TSG-RAN WG2 Meeting #127        	                       R2-2406964

Maastricht, NL, August 19th-23rd, 2024

Agenda item:	8.1.3

Source:	CMCC

Title:	Discussion on NW side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In previous meeting [1,2], RAN2 discussed the NW side data collection and achieved the following agreements.

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

Agreements

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

This contribution will discuss the mechanisms for NW side data collection.

- Discussion

- 2.1 Beam management with gNB-sided model

- In last meeting, RAN2 agreed to take immediate MDT as baseline for OAM-centric data collection. The legacy immediate MDT procedure is illustrated in Fig.1. It is noted that OAM will check user consent for data collection firstly, and then MDT configuration will be sent to gNB.

- Figure 1. Basic procedure for immediate MDT

- However, for gNB-centric approach, data collection is initiated and terminated by itself, gNB has no user consent. For beam management use case, based on RAN1 LS reply on data collection [3], the contents of data collection for model training are L1-RSRPs and/or beam-IDs. In legacy mechanism, these data are reported by legacy CSI reporting with no user consent. For model training, RAN2 can discuss whether the collected data should to be authorized by user consent.

| LCM purpose | UE-side/NW-side models | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | UE-side, NW-side | L1-RSRPs and/or beam-IDs | See Note 1 for L1-RSRPs | Relaxed |  |
| <omitted> |  |  |  |  |  |
| Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed. <omitted> |  |  |  |  |  |

- Proposal 1: For gNB-centric approach for beam management use case, RAN2 to discuss whether the collected data for model training (e.g. L1-RSRPs and/or beam-IDs) should to be authorized by user consent.

- In last meeting, RAN2 agreed to report multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. And it was agreed to support periodical reporting. The basic procedure for enhanced immediate MDT is illustrated in Fig.2. But FFS how to handle case when single RRC message is not sufficient.

- Figure 2. Basic procedure for enhanced immediate MDT

- For the data size, based on the content highlighted above, the maximum typical size would be up to ~500bits for BM Case 1 (Spacial domain prediction). For BM case 2, the payload size is not fixed which depends on the predicted time instance and history measurement time instance. We take BM Case 1 as example, there can be ~144 samples (9kbyte/500bits) per RRC reporting. Based on TR 38.843 [4], the typical time duration to generate one sample is 40ms, then the reporting cycle 5760ms (144*40ms) can be supported. Considering the UL RRC segmentation is up to 16, the reporting period can be extended to 92160ms (5760ms*16) ≈ 1.5min.

- Observation 1: For BM Case 1, one single RRC message can support ~144 samples reporting, and the corresponding reporting cycle will be 5760ms. Considering the UL RRC segmentation is up to 16, the reporting period can be extended to 92160ms≈1.5min.

- As specified in TS 38.331 [5], the minimum of report interval is 120ms, and the maximum is 30min. For the FFS how to handle case when single RRC message is not sufficient, we understand that it can be avoided by the network implementation, e.g. configure a suitable report interval which guarantees single RRC message is sufficient for collected data, so it is unnecessary to introduce additional enhancements for this issue.

- – ReportInterval The IE ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1) when reportType is set to either eventTriggered, periodical, cli-EventTriggered or cli-Periodical. Value ms120 corresponds to 120 ms, value ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on. ReportInterval information element -- ASN1START -- TAG-REPORTINTERVAL-START ReportInterval ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30 } -- TAG-REPORTINTERVAL-STOP -- ASN1STOP

- Proposal 2: For the case when single RRC message is not sufficient, it can be avoided by NW implementation, e.g. configure a suitable report interval which guarantees single RRC message is sufficient for collected data, so it is unnecessary to introduce additional enhancements.

- 2.2 Positioning with gNB-sided model (case 3a)

- Figure 3. Positioning with gNB-sided model (case 3a)

- During the R18 study, the entity for gNB-sided model training can reside at gNB or OAM. Based on the following RAN1 agreement, the label and its related data (e.g., time stamp) can be generated by at least LMF. Thus, the NRPPa should be enhanced to transmit the label and related data from LMF to gNB for model training.

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

- Proposal 3: For positioning with gNB-sided model (case 3a), enhance NRPPa to transmit the data (e.g. the label and its related data) from LMF to gNB for model training.

- In last meeting, RAN1 agreed that the measurement and its related data (e.g., timestamp) are generated by TRP/gNB for training. We understand that there may be some impacts on Xn interface but it is up to RAN3 discussion.

RAN1#117 Agreement - For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB.

- Proposal 4: For positioning with gNB-sided model (case 3a), there may be some impacts on Xn interface if the measurement and its related data (e.g., timestamp) are generated by TRP/gNB for training, but it is up to RAN3 discussion.

- Based on the following RAN1 agreement, the measurement for case 3a is based on the existing UL SRS and no new SRS will be introduced. Therefore, the legacy UL SRS transmitting and measurement will be reused, which has no additional impacts on Uu interface.

RAN1#116 Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE.

- Proposal 5: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface.

- 2.3 Positioning with LMF-sided model (case 2b and 3b)

- During the R18 study, the entity for LMF-sided model training resides at LMF and the LMF is the termination point for training data. Therefore the LMF-centric data collection should be considered, i.e. LMF configures the UE/gNB to initiate/terminate the data collection procedure.

- Figure 4. Positioning with LMF-sided model (case 2b and 3b)

For the content of collected data for LMF-sided model training, at least measurements (corresponding to model input, e.g. timing, power, and/or phase info) and location coordinates as model output are needed based on RAN1 reply LS [3]. During the R19 discussion, RAN1 has achieved the following agreements on the collected data for LMF-sided model.

RAN1#116 Agreement - For AI/ML based positioning case 2b and 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time RAN1#116bis Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope.

- In general, the training data can be generated by the UE (case 2b) or gNB (case 3b) and terminated at LMF. Hence, it is straight forward to extend the LPP and NRPPa protocol to collect data for LMF-sided model training. However, the details of training data are still under RAN1 discussion. Thus, we can wait for RAN1’s conclusion on the details of training data. In addition, RAN3 work will be needed if the enhancements on NRPPa are identified.

- Proposal 6: For positioning with LMF-side model (case 2b), extend the LPP protocol to collect data from UE to LMF for model training. The details are based on RAN1’s conclusion.

- Proposal 7: For positioning with LMF-side model (case 3b), extend the NRPPa protocol to collect data from gNB to LMF for model training. The details are based on RAN1’s conclusion.

# 3	Conclusion

Here are the proposals for NW side data collection.

Beam management with gNB-sided model

- Proposal 1: For gNB-centric approach for beam management use case, RAN2 to discuss whether the collected data for model training (e.g. L1-RSRPs and/or beam-IDs) should to be authorized by user consent.

- Observation 1: For BM Case 1, one single RRC signaling can support ~144 samples reporting, and the corresponding reporting cycle will be 5760ms. Considering the UL RRC segmentation is up to 16, the reporting period can be extended to 92160ms≈1.5min.

- Proposal 2: For the case when single RRC message is not sufficient, it can be avoided by NW implementation, e.g. configure a suitable report interval which guarantees single RRC message is sufficient for collected data, so it is unnecessary to introduce additional enhancements.

Positioning with gNB-sided model (case 3a)

- Proposal 3: For positioning with gNB-sided model (case 3a), enhance NRPPa to transmit the data (e.g. the label and its related data) from LMF to gNB for model training.

- Proposal 4: For positioning with gNB-sided model (case 3a), there may be some impacts on Xn interface if the measurement and its related data (e.g., timestamp) are generated by TRP/gNB for training, but it is up to RAN3 discussion.

- Proposal 5: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface.

Positioning with LMF-sided model (case 2b and 3b)

- Proposal 6: For positioning with LMF-side model (case 2b), extend the LPP protocol to collect data from UE to LMF for model training. The details are based on RAN1’s conclusion.

- Proposal 7: For positioning with LMF-side model (case 3b), extend the NRPPa protocol to collect data from gNB to LMF for model training. The details are based on RAN1’s conclusion.

# 4	Reference

- Draft_R2_125bis_Meeting_Report_v2

- Draft_R2_126_Meeting_Report_v2

- R2-2311720 Reply LS on Data Collection Requirements and Assumptions (R1-2310681)

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification