3GPP TSG-RAN WG2 Meeting #127        	                       R2-2406965

Maastricht, NL, August 19th-23rd, 2024

Agenda item:	8.1.4

Source:	CMCC, China Unicom, NTT DOCOMO, ZTE, OPPO

Title:	Discussion on UE side data collection

WID/SID:	NR_AIML_air

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed the understandings on the potential solutions for UE side data collection for training, and achieved the following agreements.

Agreements

- Capture table below in TR (Rapporteur will clean up and refine and review over email)

- Capture the privacy concerns from different stakeholders as informative annexes in the TR. All the options (1a, 1b, 2, 3) should respect those privacy concerns. Details are up to TR rapporteur.

| Aspects | 1a) OTT (3GPP Transparent) | 1b) The server for training data collection for UE-side models (3GPP non-transparent) | 2. Transfer via Core Network | 3. Transfer via OAM |
| --- | --- | --- | --- | --- |
| First termination entity | OTT server | The server for data collection for UE-side model training | Inside the CN (e.g., LMF) | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE-> CN ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE->gNB->OAM-> Server for data collection for UE-side model training/OTT server |
| UP/CP tunnel | UP tunnel | UP tunnel | CP tunnel (provided the data volume remains within the NAS signalling capacity) FFS: UP tunnel | CP tunnel (provided the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Has controllability FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Control granularity by NW | NA, the OTT server can directly request data from the UE. | Example: per PDU sessions | NAS procedure, FFS impact to other layers | RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility FFS: meaning of ‘partial/partially’ SAME for OPTION 3 | Full visibility for standardized data content. Partial visibility for partially standardized data content No standardized visibility FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | NA | SA2, SA3, RAN2 | SA2, SA3, RAN3 RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. (Subject to refinement and modification) - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. (Subject to refinement and modification, the scope does not exclude additional requisites, such as the ability to modify the collected data.) - Note 3: For Solution 1b, 2/3, the following options are identified to realize the different levels of data content visibility if different levels of data content visibility to MNO are considered. FFS on the data content visibility via SLA. - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No visibility for non-standardized data content. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. |  |  |  |  |

In this contribution, we will focus on the remaining FFS in the table and give the potential recommendation for RAN#105.

- Discussion

## 2.1 Clarification on full controllability

From the perspective of operator, full controllability means all the procedures as well as the control information, e.g. configuration for data collection, are both understandable and managed by MNO. However, the existing definition of full cntrollability only covers the procedures management. And we think MNO can do adjustment (e.g. update the data collection configuration) anytime without any SLA with the third party. So it should be defined within 3GPP domain for full controllability. Otherwise, it’s not full controllability from 3GPP point of view even if full controllability may be achieved via SLA with third party, anyway SLA is out of 3GPP scope.

- Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed.

Let’s give an example for easy understanding. In legacy of 3GPP spec, TCE node within OAM domain can trigger MDT data collection task, all the procedures as well as the control info, e.g. data collection configuration, involved in the procedures for MDT data collection are fully understandable and managed by MNO itself, i.e. no component is invisible to MNO no matter we’re discussing which part of the data collection procedure. So we think the definition of full controllability should be given within 3GPP domain as any definition out of 3GPP scope is not our focus.

Observation 1: In current 3GPP spec, full controllability on data collection means all the procedures as well as the control info, e.g. data collection configuration, involved in the procedures for data collection are understandable and managed by MNO, MNO can do adjustment anytime without any SLA with third party.

Based on above analysis, we have the following definition revision for full controllability:

Proposal 1: RAN2 to revise the full controllability definition as follows:

Note 1: Full controllability on data collection means all the procedures as well as the control info, e.g. data collection configuration, involved in the procedures for data collection are understandable and managed by MNO, MNO can do adjustment anytime without any SLA with third party.

## 2.2 Clarification on full data visibility

For the current definition of data visibility, we think it is too broad. In our view, when we discuss something in 3GPP, the definition should consider 3GPP spec as the baseline. Even for data out of 3GPP scope, the following definition is still applicable, which is not our intention from 3GPP point of view.

- Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data).

For example, in current EVEX data collection framework, a few data collection record types (e.g., ApplicationSpecificRecord) are not visible to the operator and are not standardized, but MNO is still aware of purpose of the data content. Due to the fact this kind of data is not standardized, MNO cannot process/modify the content of such data.

Observation 2: Currently, the definition on visibility of data content is too broad. Even for data out of 3GPP scope, the above definition is still applicable, which is not our intention from 3GPP point of view.

To better understand the data visibility concept in 3GPP domain, we can give another example, UE can report RSRP value based on RRM framework as specified in TS 38.331 [2], each RSRP index can refer to an actual value based on the mapping table in RAN4 spec TS 38.133 [3].

TS 38.331 – RSRP-Range The IE RSRP-Range specifies the value range used in RSRP measurements and thresholds. For measurements, integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [14]. For thresholds, the actual value is (IE value-156) dBm, except for the IE value 127, in which case the actual value is infinity. RSRP-Range information element -- ASN1START -- TAG-RSRP-RANGE-START RSRP-Range ::= INTEGER(0..127) -- TAG-RSRP-RANGE-STOP -- ASN1STOP

TS 38.133 Table 10.1.6.1-1: SS-RSRP and CSI-RSRP measurement report mapping Reported value; Measured quantity value (L3 SS-RSRP) and CSI-RSRP; Measured quantity value (L1 SS-RSRP and CSI-RSRP); Unit. RSRP_0; SS-RSRP<-156; Not valid; dBm. RSRP_1; -156≤ SS-RSRP<-155; Not valid; dBm. RSRP_2; -155≤ SS-RSRP<-154; Not valid; dBm. RSRP_3; -154≤ SS-RSRP<-153; Not valid; dBm. RSRP_4; -153≤ SS-RSRP<-152; Not valid; dBm. RSRP_5; -152≤ SS-RSRP<-151; Not valid; dBm. RSRP_6; -151≤ SS-RSRP<-150; Not valid; dBm. <Omitted>.

Based on 3GPP spec above, we can know that the RSRP data format is defined within 3GPP spec, so the receiver, e.g. NG-RAN, can decode this data type. On the other hand, the meaning of each RSRP index is also defined within 3GPP spec, so the receiver can process, e.g. modify/use, the data type after decoding. So when we’re talking about data visibility in 3GPP domain, the data should have clear data format definition within 3GPP spec and the meaning of each value of the data should also be clearly defined within 3GPP spec. Otherwise, there is no visibility from 3GPP perspective.

Observation 3: If all the properties of a data are clearly defined in 3GPP spec (in this case, there is no SLA between MNO and third party), it’s obvious that MNO has full visibility of such kind of data.

On the other hand, it should be also pointed out that 3GPP spec is not the only way to achieve full visibility without SLA involvement. ROHC feature is a typical use case introduced by 3GPP PDCP spec. Although ROHC feature is used within 3GPP system, the detailed definitions, i.e. all the properties of ROHC feature, are given in standardized spec out of 3GPP:

[7] IETF RFC 5795: "The RObust Header Compression (ROHC) Framework". [8] IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP and uncompressed". [9] IETF RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095". [10] IETF RFC 6846: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)". [11] IETF RFC 5225: "RObust Header Compression (ROHC) Version 2: Profiles for RTP, UDP, IP, ESP and UDP Lite".

From MNO point of view, MNO still has full visibility of any data using ROHC as the ROHC feature is also standardized without SLA.

Observation 4: If all the properties of a data are clearly defined in non-3GPP spec, i.e. standardized spec out of 3GPP (in this case, there is also no SLA between MNO and third party), we still can say that MNO has full visibility of such kind of data.

Another example is Uplink Data Compression, which is defined partially in IETF and another part in 3GPP. The DEFLATE Compressed Data Format is defined in IETF RFC 1951. 3GPP took IETF specification as reference and then defined the remaining part in TS 38.323 [4], e.g., UDC header, UDC feedback, pre-defined dictionary, checksum error handling. With the two specifications, UDC is clearly specified and is fully visible for MNO.

[19] IETF RFC 1951: "DEFLATE Compressed Data Format Specification version 1.3".

Observation 5: If part of the data properties are clearly defined in 3GPP spec while the remaining part of data properties are clearly defined in standardized spec out of 3GPP (in this case, there is also no SLA between MNO and third party), we still can say that MNO has full visibility of such kind of data.

Based on Observation 3-5, we can summarize that full data visibility can be achieved by MNO via the following three ways without SLA with third party:

- Way1: All the properties of a data are clearly defined in 3GPP spec.

- Way2: All the properties of a data are clearly defined in standardized spec out of 3GPP.

- Way3: Part of the data properties are clearly defined in 3GPP spec while the remaining part of data properties are clearly defined in standardized spec out of 3GPP.

Observation 6: Without SLA with third party, the following ways can be used by MNO to achieve full data visibility:

- Way1: All the properties of a data are clearly defined in 3GPP spec.

- Way2: All the properties of a data are clearly defined in standardized spec out of 3GPP.

- Way3: Part of the data properties are clearly defined in 3GPP spec while the remaining part of data properties are clearly defined in standardized spec out of 3GPP.

Based on the analysis above, we would like to revise the data visibility definition as the following:

Note 2: Full visibility of data content signifies the capability of the MNO to decode and process, e.g. modify/use, the data using 3GPP spec and/or standardized spec out of 3GPP without SLA with third party.

Proposal 2: RAN2 to revise the definition of full visibility as follows:

Note 2: Full visibility of data content signifies the capability of the MNO to decode and process, e.g. modify/use, the data using 3GPP spec and/or standardized spec out of 3GPP without SLA with third party.

## 2.3 FFS for Option 1b

Based on the latest TP [5] for TR 38.843, the definition of Option 1b is given as the following:

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 3GPP specification impact is expected.

- Clarification 1: AI/ML-specific Data Transfer Path

Currently, the following AI/ML-specific Data Transfer Path of Option 1 was captured in Table:

- UE ->Server for data collection for UE-side model training/OTT server (Note 4)

But based on above definition, UE will always transfer the training data to the server for data collection for UE-side model training (inside the MNO) first, whether further forward training data to the OTT server (outside the MNO) from the server for data collection for UE-side model training is optional. In this sense, the following update is needed:

- UE ->Server for data collection for UE-side model training [->OTT server] (Note 4)

Proposal 3: For Option 1b, update ‘UE ->Server for data collection for UE-side model training/OTT server (Note 4)’ into ‘UE ->Server for data collection for UE-side model training [->OTT server] (Note 4)’.

- Clarification 2: Controllability of MNO on data transfer

Based on the understanding of Proposal 1, for Option1b, the level of controllability for training data collection depends on whether MNO has full control of the server for data collection for UE-side model training (inside the MNO) from 3GPP spec perspective.

- If the server inside MNO is maintained by a trusted third party via SLA, to protect privacy for some collected data, the control info (e.g. data collection configuration/instructions) may not be fully visible to MNO, of course, which part of control info is visible to MNO can be achieved via SLA (usually SLA is out of 3GPP scope), in this case, full controllability may not be achieved.

- If the server inside MNO is maintained by MNO itself without any unknown parts, full controllability may be possible.

Observation 7: For Option1b, the level of controllability for training data collection depends on whether MNO has full control of the server for data collection for UE-side model training (inside the MNO) from 3GPP spec perspective.

- If the server inside MNO is maintained by a trusted third party via SLA, to protect privacy for some collected data, the control info, e.g. data collection configuration/instructions may not be fully visible to MNO, of course, which part of control info is visible to MNO can be achieved via SLA (usually SLA is out of 3GPP scope), in this case, full controllability may not be achieved.

- If the server inside MNO is maintained by MNO itself without any unknown parts, full controllability may be possible.

Based on observation 7 above, we have the following proposal:

Proposal 4: For Option 1b, update ‘Controllability can be achieved. FFS: level of controllability.’ into ‘Full controllability (if the server for data collection for UE-side model training is controlled by MNO without SLA), or partial controllability (if the server for data collection for UE-side model training is controlled by MNO with SLA negotiated with third party)’.

- Clarification 3: Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3)

Based on the understanding of Proposal 2, we will evaluate what kind of visibility can be achieved for Option1b. Let’s take Event Exposure (EVEX) data collection framework as a reference as clarified in [5], in which it states that they are considering EVEX as an example for Option 1b. The key Observations and Proposals in [6] are given below for information on data visibility of Option 1b:

Observation 5: Considering the EVEX as one of the examples for solution 1b), a few of the data collection record types (e.g., ServiceExperienceRecord, PerFlowServiceExperienceInfo, LocationRecord, CommunicationRecord, PerformanceDataRecord, etc.) are fully visible to the operator and are standardized. Observation 6: Considering the EVEX as one of the examples for solution 1b), a few data collection record types (e.g., ApplicationSpecificRecord) are not visible to the operator and are not standardized. Proposal 2: In solution 1b, the MNO can have ‘Full Visibility’, ‘Partial Visibility’, or ‘No Visibility’ depending on data collection record types. Proposal 3: The data collection solution 1b) may result in standard defined and non-standard defined data collection record types.

After some survey on the corresponding spec, we got totally different observations than above. Let’s give the details and evidence one by one.

Firstly, we need to discuss in which background 3GPP introduces EVEX framework, we can see the description in [7]:

4 Reference architecture for data collection and reporting 4.1 General Clause 6.2.8 of TS 23.288 [4] envisages a set of high-level procedures by which data is collected by a Network Data Analytics Function (NWDAF) from UE Application(s) via an intermediary Application Function. This clause defines a generic reference architecture for data collection and reporting that satisfies those procedures, including the logical functions involved and the logical reference points between them. The intermediary Application Function envisaged in [4] is here named the Data Collection AF. It is intended that this reference architecture be instantiated in domain-specific ways to suit the needs of different features of the 5G System. The reference architecture may be instantiated separately in different slices of a network. The services defined in the present document may be exposed to parties outside the trusted domain via the NEF, as defined in clause 4.7.1. The Data Collection AF may support CAPIF [8] to provide APIs to other applications (i.e. API invokers), as defined in clause 4.7.2. 4.2 Functional entities for data collection and reporting Figure 4.2-1 below shows the reference architecture for data collection and reporting using reference point notation. NOTE: The Data Collection AF may be deployed outside the trusted domain, in which case the services it exposes to API invokers are mediated by the NEF. The logical relationships denoted by the reference points are unaffected by such deployment choices. Figure 4.2-1: Reference architecture for data collection and reporting in reference point notation

Based on above description, we know that EVEX data collection framework is mainly used to collect data from UE APP layer. From MNO point of view, APP layer data is usually transparent to MNO, so it’s still acceptable that the data content reported by ApplicationSpecificRecord is non-standardized.

Observation 8: EVEX data collection framework is mainly used to collect data from UE APP layer. From MNO point of view, APP layer data is usually transparent to MNO, so it’s still acceptable that the data content reported by ApplicationSpecificRecord is non-standardized.

Apart from APP layer data, EVEX data collection framework can also be used to collect some MNO visible data type, some typical examples are given in [8]:

Table 6.3.1-2 specifies data types re-used from other specifications by the Ndcaf_DataReportingProvisioning service operations, including a reference to their respective specifications. Table 6.3.1-2: Externally defined data types used by Ndcaf_DataReportingProvisioning service operations Data type; Comments; Reference. AfEvent; Identifies a type of event.; TS 29.517 [5]. ApplicationId; Identifies the reporting application.; TS 29.571 [12]. DateTime; A point in time, expressed as an ISO 8601 [25] date and time.. DurationSec; A period of time, expressed in seconds.. Double. Float. Int32. Int64. Uint16. Uint32. Uint64. Uinteger.

From MNO point of view, MNO is aware that some types of data, e.g. DateTime/DurationSec, can be collected via EVEX framework as the high level description of these data are given in 3GPP spec [8], but this does not necessarily means that MNO has full visibility for all these data with MNO awareness. As the detailed definition of these Externally defined data types above is usually given in another spec, we need to dig deeper for further clarification. Let’s take TS 29.571 [9] as an example:

5.5.2 Simple Data Types This clause specifies common simple data types. Table 5.5.2-1: Simple Data Types Type Name; Type Definition; Description. Omit something here. PacketDelBudget; integer; Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP TS 23.501 [8])), expressed in milliseconds. Minimum = 1.. PacketDelBudgetRm; integer; This data type is defined in the same way as the "PacketDelBudget" data type, but with the OpenAPI "nullable: true" property.. PacketErrRate; string; String representing Packet Error Rate (see clause 5.7.3.5 and 5.7.4 of 3GPP TS 23.501 [8]), expressed as a "scalar x 10-k" where the scalar and the exponent k are each encoded as one decimal digit. Pattern: '^([0-9]E-[0-9])$' Examples: Packer Error Rate 4x10-6 shall be encoded as "4E-6". Packer Error Rate 10-2 shall be encoded as "1E-2".. PacketErrRateRm; string; This data type is defined in the same way as the "PacketErrRate" data type, but with the OpenAPI "nullable: true" property.. PacketLossRate; integer; Unsigned integer indicating Packet Loss Rate (see clauses 5.7.2.8 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in tenth of percent. Minimum = 0. Maximum = 1000.. PacketLossRateRm; integer; This data type is defined in the same way as the "PacketLossRate" data type, but with the OpenAPI "nullable: true" property.. AverWindow; integer; Unsigned integer indicating Averaging Window (see clause 5.7.3.6 and 5.7.4 of 3GPP TS 23.501 [8]), expressed in milliseconds. Minimum = 1. Maximum = 4095. Default = 2000...

Take PacketDelBudget as example, the data description states that Unsigned integer indicating Packet Delay Budget (see clauses 5.7.3.4 and 5.7.4 of 3GPP TS 23.501 [10])), expressed in milliseconds. Minimum = 1. One may think that MNO has full visibility of this type of data in 3GPP spec domain as the data description has a reference to 3GPP spec TS 23.501, but actually different MNO may have different coding rule to represent each value of the delay budget parameter because TS 23.501 spec only gives the value range of packet delay budget, the coding rule is vendor specific based on offline engineering:

5.7.4 Standardized 5QI to QoS characteristics mapping Standardized 5QI values are specified for services that are assumed to be frequently used and thus benefit from optimized signalling by using standardized QoS characteristics. Dynamically assigned 5QI values (which require a signalling of QoS characteristics as part of the QoS profile) can be used for services for which standardized 5QI values are not defined. The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.4-1. Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping 5QI Value; Resource Type; Default Priority Level; Packet Delay Budget (NOTE 3); Packet Error Rate; Default Maximum Data Burst Volume (NOTE 2); Default Averaging Window; Example Services. 1; GBR; 20; 100 ms (NOTE 11, NOTE 13); 10-2; N/A; 2000 ms; Conversational Voice. 2; (NOTE 1); 40; 150 ms (NOTE 11, NOTE 13); 10-3; N/A; 2000 ms; Conversational Video (Live Streaming).

Parameter PacketDelBudget is just one example, if you look at other parameters defined in [9], the logic is quite similar, i.e. data description field just gives the value range of the defined data type, the detailed coding rule is based on offline engineering (i.e. not specified, different MNO may have different coding rule for the same data type defined in the spec TS 29.571 [9]). But for parameters, e.g. RSRP, introduced in Uu interface, both data description field and coding rules are clearly specified in 3GPP spec, which can be considered as the baseline understanding on full visibility from MNO point of view.

Observation 9: In EVEX data collection framework, even for data type with MNO awareness, e.g. non-APP layer data, standardized spec alone still cannot guarantee full visibility of data content as the coding rule still relies on offline engineering. If the coding rule is defined by the third party, cross-vendor SLA is still needed for MNO to achieve full visibility of data content.

Based on Observation 8 and Observation 9, we can observe that, in EVEX data collection framework, no matter we’re talking about data type without MNO awareness, e.g. APP layer data, or data type with MNO awareness, e.g. non-APP layer data, full visibility of data content is not naturally achieved from spec (i.e. 3GPP spec and/or non-3GPP spec) point of view. Considering EVEX is a typical example of Option 1b, the same judgment can be also applicable for Option 1b. But it should be noted that full visibility of data content may still be achieved via SLA for Option 1b, this is anyway possible in theory.

Proposal 5: For Option 1b, RAN2 understands full visibility of data content is not naturally achieved from spec (i.e. 3GPP spec and/or non-3GPP spec) point of view.

## 2.4 FFS for Option 2

- Clarification 1: UP/CP tunnel

During last RAN2 meeting, it’s still unclear whether UP tunnel can be used to transmit collected data for Option 2.

Before we give the judgement, we can review what we have done for positioning data collection. Before R18, i.e. R17 and earlier release, LMF can only collect positioning data via LPP message and LPP message is carried via CP tunnel, i.e. NAS signaling. When it comes to R18, SA2 had introduced UP tunnel-based solution to carry LPP message. In this sense, we don’t find the strong reason to not consider UP tunnel for Option 2.

Another point is whether to consider LPP, we understand CP/UP tunnel concept is more generic, which can cover the LPP requirement, so there is no need to emphasize LPP in the table.

Based on above, we have the following proposal:

Proposal 6: For Option 2, update ‘CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP’ into ‘CP tunnel (provided that the data volume remains within the NAS signalling capacity per message) or UP tunnel (no explicit data volume limitation)’.

- Clarification 2: Protocol layer for data transfer

If CP tunnel is used for Option 2, RAN2 already agreed that NAS layer will be used to transmit collected data. For UP tunnel case, we understand the situation is quite similar as the positioning data transmission method introduced in R18, i.e. NAS layer will be used to transmit collected data. Thus, we have the following proposal:

Proposal 7: For Option 2, update ‘NAS layer for CP tunnel FFS: the protocol layer for UP tunnel’ into ‘NAS layer for both CP and UP tunnel’.

- Clarification 3: Network control level

For Option 2, it’s obvious that NAS procedure is at least involved for network control, but AS procedure may be optionally involved, we take positioning use case as an example, although LPP message is carried via UL NAS signaling, but the PRS related configuration is configured via DL RRC signaling, so for some cases, AS layer may be involved to control the data collection procedure. Of course, the involvement has some dependency on use case.

Based on above, we have the following proposal:

Proposal 8: For Option 2, update ‘Via NAS procedure FFS: impact to other layers’ into ‘Via NAS procedure and optionally via AS procedure’.

- Clarification 4: Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3)

For Option 2, the basic requirement is that the first termination of data collection is CN entity which is for sure under MNO control. It’s nature that MNO needs to have full visibility of data content within CN entity, so at least full visibility for standardized data content should be the baseline for Option 2. As for other level of visibility, e.g. partial visibility and no visibility, we don’t think it is acceptable from MNO point of view as partial visibility and no visibility of data content will not be considered as trustworthy data. More addition, partial visibility and no visibility of data content is covered by Option 1b, we need to have clear boundary between Option 1b and Option 2, so full visibility with standardized data content should be the baseline for Option 2.

Observation 10: For Option 2, partial visibility and no visibility of data content is not considered as partial visibility and no visibility of data content is already achieved by Option 1b, clear boundary between Options can make our discussion easier.

Proposal 9: For Option 2, RAN2 understands that full visibility with standardized data content is the baseline.

## 2.5 FFS for option 3

- Clarification 1: UP/CP tunnel

During last RAN2 meeting, it’s still unclear whether UP tunnel can be used to transmit collected data for Option 3.

In legacy, the data, e.g. MDT data, collected by OAM is all using CP tunnel in Uu interface, no AS data is collected by OAM via UP tunnel. We think that CP tunnel is enough for now.

Proposal 10: For Option 3, update ‘CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel’ into ‘CP tunnel (provided that the data volume remains within the RRC signalling capacity per message)’.

- Clarification 2: Protocol layer for data transfer

RAN2 already agreed that RRC layer will be used to transmit collected data for CP tunnel. For UP tunnel case, we understand this is not applicable.

So we have the following proposal:

Proposal 11: For Option 3, update ‘RRC layer for CP tunnel FFS: the protocol layer for UP tunnel’ into ‘RRC layer for CP tunnel’.

- Clarification 3: Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3)

We think the analysis for Option 2 is also valid for Option 3, in short, we have the following proposal:

Proposal 12: For Option 3, RAN2 understands that full visibility with standardized data content is the baseline.

- Clarification 4: Involved WGs

Whether SA2 impact is involved is FFS, we don’t think RAN2 alone can give the clear answer at this early stage, anyway this is just an initial evaluation, the safer way is to keep it for now.

Proposal 13: For Option 3, update ‘RAN2, RAN3, SA3, SA5, FFS SA2’ into ‘RAN2, RAN3, SA3, SA5, SA2’.

## 2.6 Updated Table

Based on the analysis in subclause 2.1 to 2.5, the table can be updated as the following:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Aspects | 1a) OTT (3GPP Transparent) | 1b) The server for training data collection for UE-side models (3GPP non-transparent) | 2. Transfer via Core Network | 3. Transfer via OAM |
| --- | --- | --- | --- | --- |
| First termination entity | OTT server | The server for data collection for UE-side model training | Inside the CN (e.g., LMF) | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE-> CN ->Server for data collection for UE-side model training[->OTT server] (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE->gNB->OAM-> Server for data collection for UE-side model training/OTT server |
| UP/CP tunnel | UP tunnel | UP tunnel | CP tunnel (provided the data volume remains within the NAS signalling capacity per message), or UP tunnel (no explicit data volume limitation) | CP tunnel (provided the data volume remains within the RRC signalling capacity per message) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for both CP tunnel and UP tunnel | RRC layer for CP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Full controllability (if the server for data collection for UE-side model training is controlled by MNO without SLA), or partial controllability (if the server for data collection for UE-side model training is controlled by MNO with SLA negotiated with third party) | Full controllability (Note 1) | Full controllability (Note 1) |
| Control granularity by NW | NA, the OTT server can directly request data from the UE. | Example: per PDU sessions | NAS procedure, optionally via AS procedure | RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | Full visibility of data content is not naturally achieved from spec (i.e. 3GPP spec and/or non-3GPP spec) point of view. | Full visibility for standardized data content | Full visibility for standardized data content |
| Involved WGs | NA | SA2, SA3, RAN2 | SA2, SA3, RAN3 RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability on data collection means all the procedures as well as the control info, e.g. data collection configuration, involved in the procedures for data collection are understandable and managed by MNO, MNO can do adjustment anytime without any SLA with third party - Note 2: Full visibility of data content signifies the capability of the MNO to decode and process, e.g. modify/use, the data using 3GPP spec and/or standardized spec out of 3GPP without SLA with third party. - Note 3: For Solution 1b, 2/3, the following options are identified to realize the different levels of data content visibility if different levels of data content visibility to MNO are considered. FFS on the data content visibility via SLA. - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No visibility for non-standardized data content. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. |  |  |  |  |

Proposal 14: RAN2 to endorse the above Table as the baseline for data collection for UE-sided model training.

## 2.7 Potential recommendation for RAN#105

As we analyzed in subclause 2.3, EVEX data collection framework is intended to collect data from UE APP layer, so no data visibility for APP layer records is still acceptable from MNO point of view as APP layer data is usually transparent to MNO.

But when it comes to collect data from UE AS layer, e.g. AS data for AI model training, the story is totally different. As AS data is usually generated under 3GPP spec, and MNO is the manager of 3GPP communication system, so it’s quite strange to say that MNO may be ruled out from touching the data content collected from UE AS layer. If AS layer data is collected via Option 1a, i.e. implementation-based solution, the data collection procedure is usually transparent to MNO as MNO is usually not aware of the data content transmitted via application layer, but if we want to introduce a 3GPP-visible way, i.e. Option 1b/2/3, to collect UE AS layer data, MNO should play an important role to manage the data collection procedure, not only from data collection configuration management perspective, but also from data content management point of view. This is the legacy logic we usually follow when we introduce new feature into 3GPP system.

Observation 11: In R19, RAN2 are discussing to collect UE data from AS layer, e.g. for AI model training purpose, the story is totally different than EVEX. As AS layer data is usually generated under 3GPP spec, and MNO is the manager of 3GPP communication system, so it’s quite strange to say that MNO may be ruled out from touching the data content collected from UE AS layer.

One may argue that some type of data collected from UE AS may involve product privacy, so this aspect should be taken into account when we design/evaluate a data collection framework. This is a good angle, but we should not forget that data privacy issue is only one aspect we need to consider, from MNO perspective, trustworthy issue can be another aspect that should be addressed and this aspect is extremely important for MNO as MNO should be responsible to avoid any data leakage to third party, this is not only a requirement from MNO but also a fundamental regulation requirement given by the government.

Observation 12: Data privacy issue is only one aspect 3GPP needs to consider when designing/evaluating a data collection framework, from MNO perspective, trustworthy issue can be another aspect that should be addressed and this aspect is extremely important for MNO as MNO should be responsible to avoid any data leakage to third party, this is not only a requirement from MNO but also a fundamental regulation requirement given by the government.

Based on above analysis, we understand if 3GPP visible data collection framework is needed, Option 2 or Option 3 should be the baseline as these options meet the fundamental requirements, e.g. data trustworthy issue, to manage a 3GPP network from MNO perspective. Whether we need both options or only one of them, this anyway can be decided during the normative work phase.

On the other hand, some vendors may have concern on data privacy for Option 2 and Option 3, we think this concern is also valid from business point of view. But we understand Option 1b is not the only solution to address data privacy concern, it’s also possible for Option 2 and Option 3. The next step is to take data privacy into account when we design the details of Option 2 and Option 3, anyway, data privacy issue will not be missed for Option 2 and Option 3, we think this is a compromise between MNO and chip vendor.

Observation 13: Option 1b is not the only solution to address data privacy concern, it’s also possible for Option 2 and Option 3 which depends on the solution details.

Proposal 15: From RAN2 point of view, Option 2 and Option3 is recommended for data collection for UE-sided model training.

Proposal 16: Even if Option 2 and Option 3 is recommended by RAN2, data privacy issue will be addressed for Option 2 and Option 3 when designing the solution details.

# 3	Conclusion

Here are the observations and proposals for UE side data collection.

Clarification on full controllability

Observation 1: In current 3GPP spec, full controllability on data collection means all the procedures as well as the control info, e.g. data collection configuration, involved in the procedures for data collection are understandable and managed by MNO, MNO can do adjustment anytime without any SLA with third party.

Proposal 1: RAN2 to revise the full controllability definition as follows:

Note 1: Full controllability on data collection means all the procedures as well as the control info, e.g. data collection configuration, involved in the procedures for data collection are understandable and managed by MNO, MNO can do adjustment anytime without any SLA with third party.

Clarification on full data visibility

Observation 2: Currently, the definition on visibility of data content is too broad. Even for data out of 3GPP scope, the above definition is still applicable, which is not our intention from 3GPP point of view.

Observation 3: If all the properties of a data are clearly defined in 3GPP spec (in this case, there is no SLA between MNO and third party), it’s obvious that MNO has full visibility of such kind of data.

Observation 4: If all the properties of a data are clearly defined in non-3GPP spec, i.e. standardized spec out of 3GPP (in this case, there is also no SLA between MNO and third party), we still can say that MNO has full visibility of such kind of data.

Observation 5: If part of the data properties are clearly defined in 3GPP spec while the remaining part of data properties are clearly defined in standardized spec out of 3GPP (in this case, there is also no SLA between MNO and third party), we still can say that MNO has full visibility of such kind of data.

Observation 6: Without SLA with third party, the following ways can be used by MNO to achieve full data visibility:

- Way1: All the properties of a data are clearly defined in 3GPP spec.

- Way2: All the properties of a data are clearly defined in standardized spec out of 3GPP.

- Way3: Part of the data properties are clearly defined in 3GPP spec while the remaining part of data properties are clearly defined in standardized spec out of 3GPP.

Proposal 2: RAN2 to revise the definition of full visibility as follows:

Note 2: Full visibility of data content signifies the capability of the MNO to decode and process, e.g. modify/use, the data using 3GPP spec and/or standardized spec out of 3GPP without SLA with third party.

FFS on Option 1b

Proposal 3: For Option 1b, update ‘UE ->Server for data collection for UE-side model training/OTT server (Note 4)’ into ‘UE ->Server for data collection for UE-side model training [->OTT server] (Note 4)’.

Observation 7: For Option1b, the level of controllability for training data collection depends on whether MNO has full control of the server for data collection for UE-side model training (inside the MNO) from 3GPP spec perspective.

- If the server inside MNO is maintained by a trusted third party via SLA, to protect privacy for some collected data, the control info, e.g. data collection configuration/instructions may not be fully visible to MNO, of course, which part of control info is visible to MNO can be achieved via SLA (usually SLA is out of 3GPP scope), in this case, full controllability may not be achieved.

- If the server inside MNO is maintained by MNO itself without any unknown parts, full controllability may be possible.

Proposal 4: For Option 1b, update ‘Controllability can be achieved. FFS: level of controllability.’ into ‘Full controllability (if the server for data collection for UE-side model training is controlled by MNO without SLA), or partial controllability (if the server for data collection for UE-side model training is controlled by MNO with SLA negotiated with third party)’.

Observation 8: EVEX data collection framework is mainly used to collect data from UE APP layer. From MNO point of view, APP layer data is usually transparent to MNO, so it’s still acceptable that the data content reported by ApplicationSpecificRecord is non-standardized.

Observation 9: In EVEX data collection framework, even for data type with MNO awareness, e.g. non-APP layer data, standardized spec alone still cannot guarantee full visibility of data content as the coding rule still relies on offline engineering. If the coding rule is defined by the third party, cross-vendor SLA is still needed for MNO to achieve full visibility of data content.

Proposal 5: For Option 1b, RAN2 understands full visibility of data content is not naturally achieved from spec (i.e. 3GPP spec and/or non-3GPP spec) point of view.

FFS on Option 2

Proposal 6: For Option 2, update ‘CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP’ into ‘CP tunnel (provided that the data volume remains within the NAS signalling capacity per message) or UP tunnel (no explicit data volume limitation)’.

Proposal 7: For Option 2, update ‘NAS layer for CP tunnel FFS: the protocol layer for UP tunnel’ into ‘NAS layer for both CP and UP tunnel’.

Proposal 8: For Option 2, update ‘Via NAS procedure FFS: impact to other layers’ into ‘Via NAS procedure and optionally via AS procedure’.

Observation 10: For Option 2, partial visibility and no visibility of data content is not considered as partial visibility and no visibility of data content is already achieved by Option 1b, clear boundary between Options can make our discussion easier.

Proposal 9: For Option 2, RAN2 understands that full visibility with standardized data content is the baseline.

FFS on Option 3

Proposal 10: For Option 3, update ‘CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel’ into ‘CP tunnel (provided that the data volume remains within the RRC signalling capacity per message)’.

Proposal 11: For Option 3, update ‘RRC layer for CP tunnel FFS: the protocol layer for UP tunnel’ into ‘RRC layer for CP tunnel’.

Proposal 12: For Option 3, RAN2 understands that full visibility with standardized data content is the baseline.

Proposal 13: For Option 3, update ‘RAN2, RAN3, SA3, SA5, FFS SA2’ into ‘RAN2, RAN3, SA3, SA5, SA2’.

Updated Table

Proposal 14: RAN2 to endorse the above Table as the baseline for data collection for UE-sided model training.

Potential recommendation for RAN#105

Observation 11: In R19, RAN2 are discussing to collect UE data from AS layer, e.g. for AI model training purpose, the story is totally different than EVEX. As AS layer data is usually generated under 3GPP spec, and MNO is the manager of 3GPP communication system, so it’s quite strange to say that MNO may be ruled out from touching the data content collected from UE AS layer.

Observation 12: Data privacy issue is only one aspect 3GPP needs to consider when designing/evaluating a data collection framework, from MNO perspective, trustworthy issue can be another aspect that should be addressed and this aspect is extremely important for MNO as MNO should be responsible to avoid any data leakage to third party, this is not only a requirement from MNO but also a fundamental regulation requirement given by the government.

Observation 13: Option 1b is not the only solution to address data privacy concern, it’s also possible for Option 2 and Option 3 which depends on the solution details.

Proposal 15: From RAN2 point of view, Option 2 and Option3 is recommended for data collection for UE-sided model training.

Proposal 16: Even if Option 2 and Option 3 is recommended by RAN2, data privacy issue will be addressed for Option 2 and Option 3 when designing the solution details.

# 4	Reference

- Draft_R2_126_Meeting_Report_v2

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification

- TS 38.133, NR; Requirements for support of radio resource management

- TS 38.323, NR; Packet Data Convergence Protocol (PDCP) specification

- R2-240XXXX Report of [POST125bis][020][AI/ML PHY] UE side data collection (Mediatek)

- R2-2405716 On UE Side Data Collection  Qualcomm Incorporated

- TS 26.531, Data Collection and Reporting; General Description and Architecture

- TS 26.532, Data Collection and Reporting; Protocols and Formats

- TS 29.571, 5G System; Common Data Types for Service Based Interfaces; Stage 3

- TS 23.501, System architecture for the 5G System (5GS); Stage 2