3GPP TSG-RAN WG2 Meeting #127        	                       R2-2406966

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda item:	8.3.2.2

Source:	CMCC

Title:	Discussion on other aspects related to RRM measurement prediction

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

In RAN2#115bis and RAN#116 meeting, RAN2 discussed the sub use case and KPI for RRM measurement prediction, and achieved the following agreements [1,2].

Agreements 1 For cell level measurement prediction model, at least consider the following cases: Case 1: To predict beam level results, then generate cell level results based on the predicted beam results; Case 2: To directly predict cell level results based on cell level results. Case 3: To directly predict cell level results based on beam level results 2 We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. 3 For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predicition. 4 The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction： Spatial-domain prediction： RSRP difference to the actual measurement Temporal prediction：RSRP difference to the actual measurement measurement reduction rate as one KPI 5 As a first step we will focus on measurement prediction accuracy. FFS whether and what system level performance evaluation is needed Agreements => We will start the evaluation with measurements prediction accuracy and model complexity. We can discuss system performance after we see which scenarios have good measurements prediction accuracy.

In this contribution, we will further discuss on RRM measurement specific sub use cases and relevant evaluation metrics/KPIs to evaluate.

- Discussion

- 2.1 Sub use cases for L3 beam level prediction

During the previous meetings, RAN2 discussed L3 cell level prediction sub use cases and simulation assumptions. As per SID [3], we think L3 beam level results are also needed for L3 mobility.

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

In legacy handover procedure, the network may also configure the UE to report the measurement information per beam. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as illustrated in Figure 1 [4]. The reported measurements per beam will be used for network to select a specific beam of target cell.

Figure 1: Measurement Model [4]

Observation 1: L3 beam level results are used for network to select a specific beam of target cell for L3 mobility.

Therefore, RAN2 is suggested to study the L3 beam level results prediction in this SID.

Proposal 1: RAN2 is suggested to study the L3 beam level results prediction.

For L3 beam level results prediction, the similar sub-use case as L3 cell level results prediction can be considered based on the above analysis, i.e.

Case 1: To predict L1 beam level results, then generate L3 beam level results based on the predicted L1 beam results

Case 2: To directly predict L3 beam level results based on L3 beam level results

Case 3: To directly predict L3 beam level results based on L1 beam level results

Figure 2. Sub use cases for L3 beam level results prediction

Proposal 2: For L3 beam level results prediction, at least the following sub use cases are considered:

Case 1: To predict L1 beam level results, then generate L3 beam level results based on the predicted L1 beam results

Case 2: To directly predict L3 beam level results based on L3 beam level results

Case 3: To directly predict L3 beam level results based on L1 beam level results

In addition, similar to L3 cell level results prediction, temporal domain, spatial domain and frequency domain prediction can be applied for L3 beam level results prediction.

Proposal 3: The temporal domain, spatial domain and frequency domain prediction can be applied for L3 beam level results prediction.

- 2.2 Evaluation metrics/KPIs

RAN2 agreed that the 1st study goal of evaluation is to reduce measurement overhead, and the following definitions were agreed in last meeting for measurement reduction rate:

- Measurement reduction rate in temporal domain (MRRT):

MRRT= skipped measurement time instances / total measurement time instances

- Measurement reduction rate in spatial domain (MRRS):

MRRS = skipped beams to be measured/ total beams to be measured

For frequency domain prediction, we think measurement gaps reduction rate is also an important metric for evaluation, and the following definition can be considered:

- The measurement gaps reduction rate (MGRR) is:

- MGRR = skipped gaps to be measured / total gaps to be measured

Proposal 4: For measurement reduction rate of frequency domain prediction, the following definition is considered:

- The measurement gaps reduction rate (MGRR) is:

- MGRR = skipped gaps to be measured / total gaps to be measured

In addition, we think the best cell prediction accuracy can be also considered as a metric for the prediction accuracy of cell level measurement prediction, which refers to the percentage of "the actual best cell based on the measurement results is the best cell based on the predicted results", because the network would select one target cell for handover procedure.

Proposal 5: The best cell prediction accuracy can be considered as a metric for the prediction accuracy of cell level measurement prediction.

- Best Cell prediction accuracy (%): the percentage of "the actual best cell based on the measurement results is the best cell based on the predicted results"

In last meeting, RAN2 agreed that we will start the evaluation with measurement prediction accuracy and model complexity, and we can discuss system performance after we see which scenarios have good measurements prediction accuracy. However, we think that system performance is also important for evaluation. RAN2 agreed the following evaluation scenario where the target study goal of case 2,3,5,6 is to reduce the measurement overhead. But we think the measurement reduction should keep the mobility performance from degrading, for example, to keep the HO successful rate above a threshold (e.g. 95%) or keep the degrading of HO successful rate below a threshold (e.g. 3%).

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Observation 2: The measurement reduction should keep the mobility performance from degrading, for example, to keep the HO successful rate above a threshold (e.g. 95%) or keep the degrading of HO successful rate below a threshold (e.g. 3%).

In addition, we understand that the measurement prediction accuracy does not always exhibit a direct positive correlation with handover performance. That is, the high measurement prediction accuracy (e.g. RSRP difference to the actual measurement is less than 1dBm) does not inherently guarantee the superior handover performance. Furthermore, even when the measurement prediction accuracy is less than optimal, the handover performance can still be satisfactory.

Observation 3: The measurement prediction accuracy does not always exhibit a direct positive correlation with handover performance.

On the other hand, the target study goal for FR2-FR2 prediction is to enhance handover performance. Thus, it is straightforward to evaluate the handover performance in this scenario, which cannot be reflected by the measurements prediction accuracy directly.

Therefore, we think that the handover performance should be evaluated. As listed in SID [3], Ping-pong HO, HOF/RLF, Time of stay, Handover interruption etc., can be considered. In addition, the HO successful rate can be also considered. And RAN2 can discuss the baseline threshold for evaluation.

Proposal 6: RAN2 consider the performance metrics/KPIs (e.g. Ping-pong HO rate, HO failure rate, RLF rate, short stay rate, HO successful rate), and discuss the baseline threshold for evaluation, such as the HO successful rate above a threshold (e.g. 95%) or the degrading of HO successful rate below a threshold (e.g. 3%).

Furthermore, the existing Handover Failure Modelling, RLF modelling and Ping-pong Modelling in TR 36.839 [5] can be reused as the baseline model for statistic of Handover failure rate, RLF rate, Ping-Pong HO rate and short stay rate, etc.

Proposal 7: The existing Handover Failure Modelling, RLF modelling and Ping-pong Modelling in TR 36.839 can be reused as the baseline model for statistic of the system level performance.

# 3	Conclusion

Here are the observations and proposals for AI/ML based RRM measurement prediction.

Sub use cases for L3 beam level prediction

Observation 1: L3 beam level results are used for network to select a specific beam of target cell for L3 mobility.

Proposal 1: RAN2 is suggested to study the L3 beam level results prediction.

Proposal 2: For L3 beam level results prediction, at least the following sub use cases are considered:

Case 1: To predict L1 beam level results, then generate L3 beam level results based on the predicted L1 beam results

Case 2: To directly predict L3 beam level results based on L3 beam level results

Case 3: To directly predict L3 beam level results based on L1 beam level results

Proposal 3: The temporal domain, spatial domain and frequency domain can be applied for L3 beam level results prediction.

Evaluation metrics/KPIs

Proposal 4: For measurement reduction rate of frequency domain prediction, the following definitions is considered:

- The measurement gaps reduction rate (MGRR) is:

- MGRR = skipped gaps to be measured / total gaps to be measured

Proposal 5: The best cell prediction accuracy can be considered as a metric for the prediction accuracy of cell level measurement prediction.

- Best Cell prediction accuracy (%): the percentage of "the actual best cell based on the measurement results is the best cell based on the predicted results"

Observation 2: The measurement reduction should keep the mobility performance from degrading, for example, to keep the HO successful rate above a threshold (e.g. 95%) or keep the degrading of HO successful rate below a threshold (e.g. 3%).

Observation 3: The measurement prediction accuracy does not always exhibit a direct positive correlation with handover performance.

Proposal 6: RAN2 consider the performance evaluation metrics/KPIs (Ping-pong HO rate, HO failure rate, RLF rate, short stay rate, HO successful rate), and discuss the baseline threshold for evaluation, such as the HO successful rate above a threshold (e.g. 95%) or the degrading of HO successful rate below a threshold (e.g. 3%).

Proposal 7: The existing Handover Failure Modelling, RLF modelling and Ping-pong Modelling in TR 36.839 can be reused as the baseline model for statistic of the system level performance.

# 4	Reference

- Draft_R2_125bis_Meeting_Report_v2

- Draft_R2_126_Meeting_Report_v2

- TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2

- RP-234055, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR 36.839 Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks