# 3GPP TSG-WG2 Meeting #127              	   R2-2406967

# Maastricht, Netherlands, Aug 19 - 23, 2024

# Source: 			CMCC

# Title:  Discussion on IoT NTN Store and Forward

# Agenda item:	8.9.2

# Document for: 	Discussion

Introduction

In the last RAN2#126 meeting, the following agreements [2] were approved as below:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

Based on the discussions and conclusions in the last two meetings, we are continuing to discuss some RAN2 related aspects in S&F operation.

Discussion

Architecture

In the last two meetings, RAN2 discuss S&F operation with the assumption that only eNB onboard. After June meeting, SA2 already have some conclusions for S&F architecture solutions which support both MME-split architecture and full CN onboard architecture shown as the following description in 23.700-29 [1]:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

The following option is agreed for supporting Store and Forward operation with a Split MME architecture with the following (informative) principles:

The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles:

Thus, from RAN2 point of view, both MME-split architecture and full CN onboard architecture should be supported as well. However, considering there is not so much work for full CN onboard architecture in RAN2 (S&F operation with full CN onboard architecture is mainly focused on CN side), we think it’s better to discuss S&F operation mainly based on MME-split architecture.

Proposal 1: It is proposed to support both MME-split architecture and full CN onboard architecture.

Proposal 2: It is proposed to take MME-split architecture as baseline in RAN2 discussion.

As for whether supporting single satellite architecture or multiple satellites architecture, RAN2 had some early discussion in RAN2#125bis meeting and it seemed that single satellite architecture had some controversies on whether supporting single satellite architecture. In the latest SA2 conclusion [1], SA2 have some related conclusions as below:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

b)	Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN.

c)	Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

From SA2 conclusion, we can see that when the network rejects UE initial access, the network response with cause value including: 1) Timer. When the timer expires, the UE can attempt to the current/another satellite which means there is one possibility that only one satellite in S&F operation serving for the UE; 2) The list of satellite IDs, the list in the rejection message is optional which can be illustrated as if there is no other available satellite in S&F operation, then no list is needed for only one satellite ID. The above two points can clearly illustrate that SA2 understand there could exist single satellite architecture scenario in S&F operation and SA2 do not exclude single satellite architecture.

Observation 1: SA2 consider S&F operation is suitable for both single satellite architecture and multiple satellite architecture.

From RAN2 point of view, since both architectures are considered in SA2 conclusion, we should not exclude the single satellite architecture out of scope. Besides, considering some countries or areas are still in the early stage of NGSO satellite deployment, it is hard to provide multi satellites serving mode for UE in S&F operation. Thus, it is meaningful to consider single satellite architecture in the early deployment stage.

Proposal 3: It is proposed that RAN2 support both single satellite architecture and multiple satellites architecture in S&F operation.

For the ISL issue, there is no clear conclusion for ISL in SA2 conclusion for now and still under discussion. At least, SA2 had not excluded ISL in S&F operation. Besides, supporting ISL can help to pass UE related informaiton between satellites directly without sending the information to the ground and let ground network send the information back to the other satellite whose procedure has huge latency. ISL architecture is especially beneficial for the initial access procedure which can significantly reduce latency. Thus, we think RAN2 shall not exclude ISL architecture as far now and wait for SA2 conclusion.

Proposal 4: It is proposed that RAN2 shall not exclude ISL and can wait for SA2 conclusion.

Access control

- For the question whether the legacy UE can camp on the cell in S&F operation, in the last RAN2#126 meeting, a conclusion is given as “RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN” [2]. According to lasted SA2 conclusion [1], we find that SA2 has defined new initial attach reject value as below: 1) A new information indication; 2) Wait timer; 3) The list of Satellite IDs (optional).

- 23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

- When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes:

- a)	A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network.

- b)	Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN.

- c)	Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

- The legacy UE can not recognize these new indications including in Reject message. Thus when the legacy UE makes initial attach in the cell with S&F operation, it will receive the reject message, regard this cell as not available and try to detach from the cell. Thus, instead of letting legacy UE send RA request and reject it, it is better to announce in the SIB message that legacy UEs and Rel-19 UE not supporting S&F operation should be barred and not allowed to camp on. In this way, both UE and satellite can save lots of signalling overhead and power consumption. Thus, we suggest to revise the last meeting agreement to “legacy UEs should be barred by legacy cellBarred and cellBarredNTN”

- Observation 2: Legacy UEs and Rel-19 UE not supporting S&F operation can not recognize the new attach or TAU reject message.

- Proposal 5: It is proposed that legacy UEs should be barred by legacy cellBarred and cellBarredNTN.

- If Proposal 5 is agreeable, legacy UEs or Rel-19 UEs not supporting IoT-NTN need to be barred from the cell in S&F operation. Similar to NTN/ATG, we can add an IE in SIB1 (e.g. cellBarredSF-r19) using one bit to indicate if UEs supporting S&F operation are allowed to camp on. In other words, the UEs in S&F operation could access to the cell only if the IE is present and set to notBarred.

- Proposal 6: It is proposed to add a new barring indication in SIB1 message (e.g. cellBarredSF-r19).

- During the initial attach procedure, if UE is rejected with new reject message including wait timer, SA2 give the conclusion about the UE behaviour during the wait time as below:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

3)	How the UE process this information is up to UE implementation and during the wait timer the UE can search for another terrestrial or satellite PLMN to get normal service.

- From SA2 conclusion, it is clear to see that UE can still search for terrestrial or normal satellite cell during the wait time. Thus, in our opinion, even UE is in the S&F operation, it can still have normal function and make the real time data transmission (not limited to Store and Forward mode only). Meanwhile, if there is one normal IoT NTN cell is available and accessible for UE to camp on, UE in S&F operation can try to camp on the normal NTN cell to get service.

- Proposal 7: UEs supporting S&F operation can still access to the normal IoT NTN cell.

- Proposal 8: During the initial attach procedure,, if there is an available and accessible normal IoT NTN cell before the wait timer expires, UEs in S&F operation can try to camp on.

UP solution

- In the RAN2#125bis meeting, RAN2 gave a conclusion on CP and UP solution as “The S&F satellite operation is applied to both CP solution and UP solution (for the UP solution pending on SA2 conclusions on the architecture)” [3]. About the UP solution, SA2 has already made the following conclusion:

23.700-29 Clause 8.2	Conclusion for KI #2: Support of Store and Forward Satellite operation

3)	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

- From the conclusion, we can see that SA2 has agreed that legacy CP and UP solutions can be used for data traffic in S&F operation

- Observation 3: SA2 confirm that legacy CP and UP procedure can be used for data traffic in S&F operation.

- Thus, from RAN2 point of view, it is nature to follow SA2 conclusion that legacy CP and UP solution can be reused.

- Proposal 9: It is proposed that RAN2 confirm legacy CP and UP solution can be reused for data traffic in S&F operation.

Wait timer during the suspension

In the initial attach procedure, SA2 has defined a wait timer to tell UE how long it should wait before re-attempting. Similarly, before connection suspension, the satellite also needs to tell UE about the waiting timer which is the estimated time before next data transmission. When the satellite is moving away, the service link is going to be unavailable and the data transmission between UE and eNB will be suspended in S&F operation. An wait time needs to be given from the satellite to UE to indicate the interruption time before the service link is broken. In this way, UE could know the estimated suspension time for next data transmission occasion. Also the estimated suspension time is useful for UE to estimate the paging occasion which means UE does not need to monitor all the time and can skip the irrelevant paging since UE knows when the next satellite will come.

- Proposal 10: It is proposed to introduce a wait timer from satellite to UE to indicate the connection interruption time before the suspension.

Conclusions

Observation 1: SA2 consider S&F operation is suitable for both single satellite architecture and multiple satellite architecture.

- Observation 2: Legacy UEs and Rel-19 UE not supporting S&F operation can not recognize the new attach or TAU reject message.

- Observation 3: SA2 confirm that legacy CP and UP procedure can be used for data traffic in S&F operation.

- Architecture

Proposal 1: It is proposed to support both MME-split architecture and full CN onboard architecture.

Proposal 2: It is proposed to take MME-split architecture as baseline in RAN2 discussion.

Proposal 3: It is proposed that RAN2 support both single satellite architecture and multiple satellites architecture in S&F operation.

Proposal 4: It is proposed that RAN2 shall not exclude ISL and can wait for SA2 conclusion.

- Access control

- Proposal 5: It is proposed that legacy UEs should be barred by legacy cellBarred and cellBarredNTN.

- Proposal 6: It is proposed to add a new barring indication in SIB1 message (e.g. cellBarredSF-r19).

- Proposal 7: UEs supporting S&F operation can still access to the normal IoT NTN cell.

- Proposal 8: During the initial attach procedure,, if there is an available and accessible normal IoT NTN cell before the wait timer expires, UEs in S&F operation can try to camp on.

- UP solution

- Proposal 9: It is proposed that RAN2 confirm legacy CP and UP solution can be reused for data traffic in S&F operation.

- Wait timer during the suspension

- Proposal 10: It is proposed to introduce a wait timer from satellite to UE to indicate the connection interruption time before the suspension.

References

- 3GPP TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

- 3GPP Chair_Notes, R2-126 NR-NTN-IoT-NTN (Sergio) EOM

- 3GPP Chair_Notes, R2-125bis NR-NTN-IoT-NTN (Sergio) EOM