3GPP TSG-RAN WG2 Meeting #127	R2-2406968

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.3

Source:	CMCC

Title:	Discussion on measurement event evaluation

Document for:	Discussion

Introduction

Measurements related enhancements for purpose of supporting LTM is one of the objectives of Rel-19 NR mobility enhancements Phase 4 [1]. Layer 3 mobility uses layer 3 measurement reporting which supports UE evaluated events for triggering of measurement reports and reduces signalling overhead compared to periodic measurement reporting. Such event triggering is not supported by the L1 measurements that are used for LTM mobility. Therefore, in order to reduce the signaling overhead of obtaining measurement reports in LTM, it is necessary to introduce event triggered L1 measurement reporting.

According to WID of NR mobility enhancements Phase 4, RAN2 needs to specify necessary components to support event triggered L1 measurement reporting:

RAN #102: RP-234036 <WI>: - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

Discussion

Event definition and Measurement RS configuration

Based on the agreements of RAN2#126, beam level events Event LTM2/3/4/5 are supported. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

RAN2#126:

Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events:

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In our opinion, Event LTM2 can be used to activate/deactivate LTM measurement resources, that is, when Event LTM2 is satisfied, LTM measurement resources are activated, and when it is not satisfied, it can be considered that the serving cell is good and therefore LTM measurement resources are deactivated; Event LTM3/4/5 can be used for LTM switch and early sync, and the above events can already meet the requirements. In addition, considering that the event type that triggers the report should be carried in the measurement report, which will affect the SR overhead, the Event definition should be as concise as possible, so we think that Event LTM1 is not needed.

Proposal 1: Considering that the event type that triggers the report should be carried in the measurement report, which will affect the SR overhead, the Event definition should be as concise as possible, so Event LTM1 is not needed.

In RAN2#126, Apple's Proposal 8 mentioned: the beam corresponding to the indicated TCI state of serving cell is used for the event evaluation. In our understanding, this is the conclusion in the current RAN1 MIMO enhancement, that is, the RS of the current beam is the QCL RS in the indicated TCI state. However, for mobility, the best beam of the serving cell should be considered in event evaluation, and the best beam is not necessarily the current beam of the UE. Similarly, for the candidate cell, the best beam measured by the UE should also be used in event evaluation. In addition, another problem is that if the RS configuration of the current beam is obtained by implicit indication (e.g. indicated by TCI state(s)), then in the current protocol, the QCL RS is often TRS (especially sub 6GHz). However, the current protocol does not allow the use of TRS to measure L1-RSRP, and RAN1 is still discussing a solution to this. Based on the above discussion, we believe that the best beam of the serving cell and neighboring cell measured by the UE should be used for event evaluation.

Proposal 2: The best beam of the serving cell and neighboring cell measured by the UE should be used for event evaluation.

For RS measurement for the new beam for Event LTM2/3/4/5, combined with the discussion in RAN1#117, the following two possible solutions are considered:

- Option 1(explicit manner): A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE.

- Option 2: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list.

In our opinion, at least Option 1 should be supported. As for Option 2, we need to discuss how to handle the case that only one TRS is configured in the indicated TCI state.

Proposal 3: For RS measurement for the new beam for Event LTM2/3/4/5, at least Option 1 should be supported.

Proposal 4: For Option 2, RAN2 discuss how to handle the case that only one TRS is configured in the indicated TCI state, and maybe need to waiting for RAN1 conclusion.

Beam or cell level measurement

In addition to beam level events, for the input of the L1 event evaluation, cell-level measurement results should also be considered. In the existing L3 measurement, the cell quality needs to be filtered by Layer 3, as shown in Fig. 1. Therefore, we should first reach a consensus on LTM cell-level measurement, which is based on the consolidation of multiple beams of a cell but does not pass through L3 filtering.

Figure 1 L3 Measurement Model

Proposal 5: RAN2 need to have common understanding on cell-level measurement, which is based on the consolidation of multiple beams of a cell but does not pass through L3 filtering.

Specifically, if only beam level events are considered, it may cause a ping-pong problem between two CUs, and there will also be frequent measurement reporting, which will significantly increase the overhead. Additionally, in addition to handover execution purpose, it is also important to consider event triggered L1 LTM measurement report for the early sync purpose. Therefore, cell level Event LTM3a/4a/5a should be introduced:

- Event LTM3a: Candidate cell becomes amount of offset better than serving cell;

- Event LTM4a: Candidate cell becomes better than absolute threshold;

- Event LTM5a: Serving cell becomes worse than absolute threshold1 AND candidate cell becomes better than another absolute threshold2.

For cell level measurement results, there are two options:

- Option 1: Similar to L3 measurements, beam specific measurements are consolidated to derive cell quality;

- Option 2: N beams all meet the corresponding thresholds, and then Event LTM3a/4a/5a is considered to be satisfied.

Proposal 6: To avoid frequent measurement reports and ping-pong problems, cell level Event LTM3a/4a/5a should be introduced:

- Event LTM3a: Candidate cell becomes amount of offset better than serving cell;

- Event LTM4a: Candidate cell becomes better than absolute threshold;

- Event LTM5a: Serving cell becomes worse than absolute threshold1 AND candidate cell becomes better than another absolute threshold2.

Proposal 7: For cell level measurement results, RAN2 considers the following two options:

- Option 1: Similar to L3 measurements, beam specific measurements are consolidated to derive cell quality;

- Option 2: N beams all meet the corresponding thresholds, and then Event LTM3a/4a/5a is considered to be satisfied.

Based on the above beam and cell level event definitions, in order to avoid ping-pong problems and high overhead of frequent measurement reports, events can be applied according to different purposes (e.g. early sync purpose and switch execution purpose). For example, when Event LTM3/4/5 is satisfied, the UE does not send a measurement report immediately, but can perform UE-based TA acquisition; further, when Event LTM3/4/5 is satisfied M times, the UE reports a measurement report, and the network can choose to trigger PDCCH ordered TA acquisition; and when the cell level Event LTM3a/4a/5a is satisfied, the UE immediately reports a measurement report, and the network can consider sending a cell switch command. Mechanisms like this can avoid frequent measurement reports and ping-pong problems.

Observation 1: Beam level and cell level events can play different roles. For example, beam-level events can assist in early sync, and cell-level events can assist in cell switching.

L1 Filtering, Hysteresis, Offset

For L1 Filtering, when evaluating beam level events, the best beam may change within a filtering time window. RAN2 should discuss how to deal with this problem.

Proposal 8: RAN2 discuss how to handle the best beam may change within a filtering time window.

For Hysteresis and Offset, if cell level events are introduced, then cell specific Hysteresis and Offset should be supported.

Proposal 9: If cell level events are introduced, then cell specific Hysteresis and Offset can be supported.

Conclusions

In this contribution, we analyse for measurement event evaluation, following are the observations and proposals.

Observations:

Observation 1: Beam level and cell level events can play different roles. For example, beam-level events can assist in early sync, and cell-level events can assist in cell switching.

Proposals:

Proposal 1: Considering that the event type that triggers the report should be carried in the measurement report, which will affect the SR overhead, the Event definition should be as concise as possible, so Event LTM1 is not needed.

Proposal 2: The best beam of the serving cell and neighboring cell measured by the UE should be used for event evaluation.

Proposal 3: For RS measurement for the new beam for Event LTM2/3/4/5, at least Option 1 should be supported.

Proposal 4: For Option 2, RAN2 discuss how to handle the case that only one TRS is configured in the indicated TCI state, and maybe need to waiting for RAN1 conclusion.

Proposal 5: RAN2 need to have common understanding on cell-level measurement, which is based on the consolidation of multiple beams of a cell but does not pass through L3 filtering.

Proposal 6: To avoid frequent measurement reports and ping-pong problems, cell level Event LTM3a/4a/5a should be introduced:

- Event LTM3a: Candidate cell becomes amount of offset better than serving cell;

- Event LTM4a: Candidate cell becomes better than absolute threshold;

- Event LTM5a: Serving cell becomes worse than absolute threshold1 AND candidate cell becomes better than another absolute threshold2.

Proposal 7: For cell level measurement results, RAN2 considers the following two options:

- Option 1: Similar to L3 measurements, beam specific measurements are consolidated to derive cell quality;

- Option 2: N beams all meet the corresponding thresholds, and then Event LTM3a/4a/5a is considered to be satisfied.

Proposal 8: RAN2 discuss how to handle the best beam may change within a filtering time window.

Proposal 9: If cell level events are introduced, then cell specific Hysteresis and Offset can be supported.

References

- RP-234036 New WID: NR mobility enhancements Phase 4;