3GPP TSG-RAN WG2 Meeting #127	R2-2406969

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.6.4

Source:	CMCC

Title:	Discussion on measurement reporting

Document for:	Discussion

Introduction

Measurements related enhancements for purpose of supporting LTM is one of the objectives of Rel-19 NR mobility enhancements Phase 4 [1]. Layer 3 mobility uses layer 3 measurement reporting which supports UE evaluated events for triggering of measurement reports and reduces signalling overhead compared to periodic measurement reporting. Such event triggering is not supported by the L1 measurements that are used for LTM mobility. Therefore, in order to reduce the signaling overhead of obtaining measurement reports in LTM, it is necessary to introduce event triggered L1 measurement reporting.

According to WID of NR mobility enhancements Phase 4, RAN2 needs to specify necessary components to support event triggered L1 measurement reporting:

RAN #102: RP-234036 <WI>: - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

Discussion

Contents in measurement report

Based on the agreements of RAN1#117, RAN1 made the following agreement on the contents in measurement report:

Agreement

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported.

- RRC can enable or disable whether current beam is always reported

  - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported

        - Note: The reported current beam is NOT counted in the N reported beams.

  - When disabled by RRC, N beams are reported.

RAN1 agreed to support at least beam ID + L1-RSRP and RRC can enable or disable whether current beam is always reported. In our understanding, this is reasonable and is also aligned with legacy reporting framework. Meanwhile, as in RAN2 mobility discussion, cell ID is also needed but can be further studied whether the cell ID in L1 measurement report is explicit or implicit. Besides, the information for the triggered event can be reported to inform the NW which event is triggered. In addition, the cell-level measurement results can be included to help the NW select the target candidate cell more comprehensively. if cell level events is introduced, the cell-level measurement results can be carried in the measurement report.

Proposal 1: The content of the L1 measurement report triggered by the event can include at least the following information:

- L1-RSRP and Beam ID

- Cell ID, explicitly or implicitly

- Information of the triggered event

- cell-level measurement results, if cell level events is supported

In L3 measurement, the network configures the parameters absThreshSS-BlocksConsolidation or absThreshCSI-RS-Consolidation as the beam selection threshold. When the UE performs measurement reporting, the network can also configure the parameter maxNrofRS-IndexesToReport to define the maximum number of reported beams to save resources. In L1 measurement, if cell-level measurement results are introduced, similar methods can also be considered to reduce the number of reported beams.

Proposal 2: The network can reduce reporting overhead by configuring the beam selection threshold and/or maximum number of reports for L1 measurements.

MAC CE or UCI for measurement reporting

There are two options to carry the L1 measurement reporting to the NW:

- Option 1: UCI

- Option 2: MAC CE

The current L1 measurement report is carried by UCI, which is transmitted by PUCCH or PUSCH. Pre-allocated PUCCH resources are reserved by the NW for the periodic L1 measurement reporting. In the discussion of UE-initiated/event-driven beam management at RAN1,  it has been agreed to use L1 UCI signalling to carry the measurement report. In last RAN1 meeting, RAN1 made the following working assumption on the signalling used for MIMO L1 measurement reporting:

Working Assumption

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

From the unified signaling design perspective, RAN2 should at least not exclude carrying measurement reports via L1 UCI siganling.

Proposal 3: From the unified signaling design perspective, RAN2 at least does not exclude carrying measurement reports via L1 UCI siganling.

For mode-A, UE first sends a 1-bit SR, and NW configures a resource for a second UL channel to carry beam report through DCI. Considering that RAN1 has only agreed to one event so far, 1 bit is no problem. However, RAN2 has discussed and approved 4 beam level events, and may further increase them in the future. Therefore, 1-bit SR cannot indicate the event that triggers the report, resulting in NW being unable to flexibly configure the resource size of the second UL channel according to different triggering events. For mode-B, NW will pre-allocated periodic resources to carry beam report for UE, and 1-bit SR may be enough. So from the perspective of RAN2, if considering supporting mode-A, it should support multi-bit indication in the first PUCCH and inform RAN1 about the conclusion.

Proposal 4: From the perspective of RAN2, if mode A is supported, multiple-bit indication can be supported in the first PUCCH and inform RAN1 about the conclusion.

However, for the event-triggered L1 measurement reporting, the L1 measurement report is only triggered when the UE determines that a specific event is met. So one of the benefits for the event-triggered reporting is to save the UL transmission resources for the reporting. But mode-A needs to send SR to request resources first, then NW configures resources through DCI, and finally UE sends L1 measurement report, which will bring very large delay. Mode-B, which pre-allocates UL resources, is less efficient on the resources usage, because when the UE does not send L1 measurement report, resources are wasted. At the same time, how much resources to pre-allocate is also a problem. Finally, if subsequent LTM and conditional LTM are taken into account, how mode-B pre-configures UL resources will also face challenges. Generally speaking, as the physical-layer uplink resource to carry the MAC CE is not pre-allocated or fixed, MAC CE-based reporting would not cause any wastage of resource and can have more/flexible payload than L1 signaling. Therefore, RAN2 should support the MAC CE solution and send an LS to RAN1.

Proposal 5: RAN2 support to use MAC CE as the UL signalling to transmit measurement report and send an LS to RAN1.

Conclusions

In this contribution, we analyse for measurement reporting, following are the proposals.

Proposals:

Proposal 1: The content of the L1 measurement report triggered by the event can include at least the following information:

- L1-RSRP and Beam ID

- Cell ID, explicitly or implicitly

- Information of the triggered event

- cell-level measurement results, if cell level events is supported

Proposal 2: The network can reduce reporting overhead by configuring the beam selection threshold and/or maximum number of reports for L1 measurements.

Proposal 3: From the unified signaling design perspective, RAN2 at least does not exclude carrying measurement reports via L1 UCI siganling.

Proposal 4: From the perspective of RAN2, if mode A is supported, multiple-bit indication can be supported in the first PUCCH and inform RAN1 about the conclusion.

Proposal 5: RAN2 support to use MAC CE as the UL signalling to transmit measurement report and send an LS to RAN1.

References

- RP-234036 New WID: NR mobility enhancements Phase 4;