3GPP TSG RAN WG2 #127			R2-2406970

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	CMCC

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda item:	8.4.3

Document for:	Discussion

- Introduction

Rel-19 NR LP-WUS/WUR WI was approved in [1], which states that In RRC IDLE/INACTIVE modes, it's observed that significant UE power saving gain (up to more than 90%) is obtained by using LP-WUS/WUR to trigger UE MR paging monitoring compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied. We will discuss on the following objectives in this contribution.

- For IDLE/INACTIVE modes

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- Discussion

2.1 Clarification of terms

At the last meeting, companies were somewhat vague about what measurement relaxation and measurement offloading meant, which made discussion of these issues very inefficient. In our view, measurement relaxation means that both MR and LR perform measurements, but MR can measure at a large r period to achieve power saving; Measurement offloading then means that the serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required.

Proposal 1: RAN2 clarifies the meaning of a term explicit. Measurement relaxation means that both MR and LR perform measurements, but MR can measure at a larger period to achieve power saving; Measurement offloading then means that the serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required.

2.2 Measurement relaxation/offloading of serving cell

When within the LP-WUS coverage, the UE only monitors using LP-WUR and can use it to perform measurements through LP-SS. Since RRM measurements in IDLE/INACTIVE state are mainly used for cell (re)selection, MR measurement relaxation/offloading of serving cell should have a premise that it cannot affect the cell (re)selection performance of UE.

Proposal 2: Serving cell measurement relaxation/offloading of MR cannot affect the performance of cell (re)selection by UE.

Measurement offloading

According to the conclusions of RAN2#126 and RAN4#111, at least measurement offloading is supported:

RAN2#126  Agreement

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR  and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

RAN4#111  Agreement

- Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Fully offloading case | Off | Off: FFS the condition and the details | ON |

Therefore, RAN2 should first focus on discussing the conditions for offloading serving cell measurements. For the entry of measurement offloading in the serving cell, an RSRP threshold can be defined. When the RSRP measurement result of MR is greater than the threshold (if UE also has LR measurement result at this time, it can be evaluated jointly, that is, when both MR And LR are greater than the threshold), UE will perform measurement offloading. Furthermore, for the purpose of simplifying the procedures, this threshold can be considered in conjunction with the threshold for LP-WUS monitoring. That is, the two thresholds can be the same or offset by a predetermined value.

Proposal 3: The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered if it is on at this time.

Proposal 4: The RSRP threshold can be considered in conjunction with the threshold for LP-WUS monitoring. That is, the two thresholds can be the same or offset by a predetermined value.

Proposal 5: The exit condition is based on the LR measurement results.

In the 'Fully offloading case' agreed by RAN4, MR serving and neighboring cell measurement are stopped. In our understanding, in order not to affect the existing cell (re)selection process in TS 38.304, this means that the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements. Otherwise, the MR cannot stop measuring neighboring cells. As shown in Figure 1

Figure 1 measurement offloading to LR

Proposal 6: According to the 'Fully offloading case' agreed by RAN4, the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements.

Considering that the latency of enabling MR is as high as hundreds of milliseconds, we should be very cautious about offloading serving cell measurements to LR. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state.

Proposal 7: UEs with high-mobility do not perform serving cell measurement offloading.

When the UE stops measurement offloading and turns on MR again, the delay between these two actions can be as high as hundreds of milliseconds, which is unprecedented. So RAN2 can discuss how to deal with the measurement loss during the hundreds of milliseconds required to restart MR. For example, LR will not completely stop measuring until MR is turned back on

Proposal 8: RAN2 discuss how to deal with the measurement loss during the hundreds of milliseconds required to restart MR.

measurement relaxation

If it is assumed that the MR needs to perform neighboring cell measurements and cannot be completely shut down, then the MR's measurement of the serving cell will not result in high additional power consumption. So MR measurement relaxation is also an option.

Similarly, an RSRP threshold can be predefined. In addition, since the MR is not turned off, the mobility of the UE may not need to be considered in this case.

Furthermore, measurement relaxation and offloading can be viewed as two gradual steps. That is, the thresholds for measurement relaxation and offloading are defined separately, where the offloading threshold is greater than the relaxation threshold, as shown in Figure 2.

Figure 2 measurement relaxation + measurement offloading with two gradual steps

Proposal 9: Following two options are considered for the measurement relaxation of serving cell:

- Option 1: The UE performs measurement relaxation when a certain RSRP threshold is met.

- Option 2: The UE first performs measurement relaxation when a certain RSRP threshold is met, and then performs measurement offloading when another greater RSRP threshold is met

2.3 Measurement relaxation of neighboring cell

In last meeting, we agreed that the RRM measurement of the neighboring cell can only be performed by MR. Therefore, the neighbor cell measurement relaxation mechanism in Release 16 can be reused, which includes two scenarios: not at cell edge and low mobility, respectively.

According to TS 38.133, if MR performs serving cell measurement offloading to LR, and the offloading threshold is greater than SnonIntraSearchP, it will not affect the existing R16 measurement relaxation mechanism for equal or lower priority neighboring cells because the measurement will not be turned on at this time. But for higher priority cells, the corresponding protocol in TS 38.133 is as follows:

TS 38.133  Measurements for UE fulfilling low mobility and not-at-cell edge criterion

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-frequency layers of higher, equal or lower priority at least every 1 hour.

- When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=60.

Therefore, for not-at-cell edge criterion, if the offloading threshold is greater than SnonIntraSearchP, the UE can implicitly determine whether the R16 measurement relaxation condition is met based on whether the serving cell performs measurement offloading. However, when the UE performs serving cell measurement offloading, it needs to use the LR measurement results to evaluate the low mobility criterion of the higher priority cells.

Observation 1: If MR performs serving cell offloading to LR, and the offloading threshold is greater than SnonIntraSearchP, it will not affect the existing R16 measurement relaxation mechanism for equal or lower priority neighboring cells.

Observation 2: If MR performs serving cell offloading to LR, and the offloading threshold is greater than SnonIntraSearchP, it will not affect the not-at-cell edge criterion of the existing R16 measurement relaxation mechanism for higher priority neighboring cells, but the LR measurement results need to be used to evaluate the low mobility criterion.

If the threshold does not meet the above conditions, or MR and LR are used together for serving cell measurement, it will affect the existing R16 measurement relaxation mechanism, as shown in Figure 3. That is, when evaluating the relaxation conditions of neighboring cell measurements in R16, it may be necessary to use the measurement results of LR.

Figure 3 SnonIntraSearchP>Threshold>SSearchThresholdP

In this case, all higher, equal or lower priority neighboring cells need to use LR measurements to evaluate the R16 measurement relaxation criterion.

Observation 3: If the serving cell measurement offloading threshold is less than SnonIntraSearchP, or both MR and LR doing measurement, all higher, equal or lower priority neighboring cells need to use LR measurement results to evaluate the R16 measurement relaxation criterion.

Proposal 10: RAN2 discusses support for using LR-based serving cell measurement results to evaluate R16 measurement relaxation criterion.

- Conclusions

In this contribution, we discussed the LP-WUS measurement relaxation/offloading in RRC_IDLE/INACTIVE mode, and following are the observations and proposals.

Observations:

Observation 1: If MR performs serving cell offloading to LR, and the offloading threshold is greater than SnonIntraSearchP, it will not affect the existing R16 measurement relaxation mechanism for equal or lower priority neighboring cells.

Observation 2: If MR performs serving cell offloading to LR, and the offloading threshold is greater than SnonIntraSearchP, it will not affect the not-at-cell edge criterion of the existing R16 measurement relaxation mechanism for higher priority neighboring cells, but the LR measurement results need to be used to evaluate the low mobility criterion.

Observation 3: If the serving cell measurement offloading threshold is less than SnonIntraSearchP, or both MR and LR doing measurement, all higher, equal or lower priority neighboring cells need to use LR measurement results to evaluate the R16 measurement relaxation criterion.

Proposals:

Proposal 1: RAN2 clarifies the meaning of a term explicit. Measurement relaxation means that both MR and LR perform measurements, but MR can measure at a larger period to achieve power saving; Measurement offloading then means that the serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required.

Proposal 2: Serving cell measurement relaxation/offloading of MR cannot affect the performance of cell (re)selection by UE.

Proposal 3: The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered if it is on at this time.

Proposal 4: The RSRP threshold can be considered in conjunction with the threshold for LP-WUS monitoring. That is, the two thresholds can be the same or offset by a predetermined value.

Proposal 5: The exit condition is based on the LR measurement results.

Proposal 6: According to the 'Fully offloading case' agreed by RAN4, the threshold for offloading measurements to the LR is not less than the threshold for starting neighboring cell measurements.

Proposal 7: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 8: RAN2 discuss how to deal with the measurement loss during the hundreds of milliseconds required to restart MR.

Proposal 9: Following two options are considered for the measurement relaxation of serving cell:

- Option 1: The UE performs measurement relaxation when a certain RSRP threshold is met.

- Option 2: The UE first performs measurement relaxation when a certain RSRP threshold is met, and then performs measurement offloading when another greater RSRP threshold is met

Proposal 10: RAN2 discusses support for using LR-based serving cell measurement results to evaluate R16 measurement relaxation criterion.

- References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC (moderator, RAN VC)