3GPP TSG RAN WG2 Meeting #127	R2-2406971

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.4

Source:	CMCC

Title:	Discussion on MBS broadcast service for NR NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this contribution, we would like to provide some further considerations on the MBS broadcast service for NTN system.

Discussion

## 2.1 How to provide service area information

- We agreed the following alternatives to include the service area information in last meeting:

- Alt. 1: SIB20

- Alt. 2: SIB21

- Alt. 3: MBSBroadcastConfiguration

- From our perspective, per the current MBS broadcast mechanism, MCCH carries the MBSBroadcastConfiguration message which indicates the MBS broadcast sessions that are provided in the cell as well as the corresponding scheduling related information for these sessions. Optionally, the MBSBroadcastConfiguration message may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. The configuration information required by the UE to receive MCCH is provided in SIB1 and SIB20. Additionally, system information may provide information related to service continuity of MBS broadcast in SIB21.

- If including service area information in SIB20, it could help UE to determine whether need to receive MBSBroadcastConfiguration based on the service area information. And if in SIB21, it is beneficial for service continuity (e.g mobility enhancements, whether cell reselection or handover is needed). Then MBSBroadcastConfiguration including mbs-SessionInfoList which provides the configuration of each MBS session provided by MBS broadcast in the current cell, considering we have another agreement that when intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details), therefore, using MBSBroadcastConfiguration could provide both service area information and association between service area and MBS session.

- Proposal 1: It is proposed to include service area information in MBSBroadcastConfiguration, and meanwhile provide the association between service area and MBS session.

- For ETWS, in existing specification, SIB6 contains an ETWS primary notification and SIB7 contains an ETWS secondary notification. The requirements of ETWS defined in TS 22.268 is as following:

- 5.2 Duration of delivery time Duration of the delivery time for PLMN operators is the time from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network, to the time that the Warning Notification is successfully delivered to the UEs. Provisioning of delivery of Primary and Secondary Notification may be required: - Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area even under congestion situation. - Secondary Notification is delivered to the users in the Notification Area even under congestion situation. Note 1: UEs that are out of coverage or switched off are not considered in the requirements. Note 2: Secondary Notification may not always be generated as it depends on the Warning Notification Provider’s policy. Note 3: Primary Notification may not always be generated (i.e. the warning may start with a Secondary Notification). - 5.3 Information element and volume The following are the requirements from the perspective of information element and amount of data. Both Primary and Secondary Notification shall: - support at least 2 types of emergency events, which are Earthquake and Tsunami; - be able to indicate the preferred UE behaviours when receiving Warning Notification, (e.g. whether to display text in the foreground, whether to ring a buzzer, whether to vibrate); - be distinguishable from notifications generated for the purpose of testing, training and other notification services; - be sent in an optimized type and amount of data, for example, a text with a certain length, by considering the delivery platforms for ETWS. Primary Notification shall: - convey data which is small enough to be sent quickly on the network. - convey small amount of data to indicate the imminent occurrence of Earthquake and Tsunami, etc. Secondary Notification may: - convey a large amount of data in order to deliver text, audio to instruct what to do / where to get help, graphical data such as a map indicating the route from present position to evacuation site, time table of food distribution. Note: The amount of data to be sent within a Primary Notification would be a few bytes to achieve quick information delivery. - 5.4 Priority Requirements from the perspective of priority are as follows: - Primary Notification has higher priority than Secondary Notification. - Notifications shall be able to be sequenced by the PLMN according to priority of notification in case that Primary Notification and Secondary Notification should exist at the same time in PLMN.

- Based on above, primary notification is more important, and regarding service area information for ETWS, it could be included in SIB6 if supported ETWS finally.

Proposal 2: It is proposed to include service area information for ETWS in SIB6 if supported ETWS finally.

## 2.2 Service area indication

Regarding the service area indication, we agreed that at least the following geographical area formats to model service area can be further considered:

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

- The option 1 (circles) is a simple and similar scheme to TN coverage information introduced in NTN R18 to assist TN neighbour cell measurements. For service area indication, we could consider to introduce a new IE (e.g. IntendedServiceAreaInfo-r19) as follows to provide intended service area ID, service area reference location and distance radius information to UE. Then UE receive the broadcast service based on whether in the service area or whether interested in the service.

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

- Option 2 (Geographical area information, e.g. via polygons, to better approximate the intended shape of service area) could provide a more precise service area than option 1. However, the overhead is obviously higher than option 1. For example, polygon shape description needs a list of coordinate locations, the more location, the more accurate, which would introduce larger overhead.

- Observation 1: Option 2 has overhead issue due to multiple location points for geographical area information (e.g. polygon).

- Proposal 3: Option 1 (circles) seems to be a simple solution to provide service area information. And a new IE (e.g. IntendedServiceAreaInfo-r19) is introduced as following:

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

- In addition to the above 2 options, we could also consider to taking advantage of mapped cell ID introduced in NTN R17. As existing mechanism, UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB, consisting of the following information [2]:

- List of MBS frequencies UE is receiving or interested to receive, sorted in decreasing order of interest;

- Priority between the reception of all listed MBS frequencies and the reception of any unicast bearer and multicast MRB;

- List of MBS broadcast services the UE is receiving or interested to receive, in case SIB20 is provided for PCell or SCell.

- Explicitly, the MBS Interest Indication is related to frequency level, which may not be applied in NTN system well. In R17 NTN, we introduced mapped cell ID, gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation. We could leverage the mapped cell ID to indicate intended service area in NTN. Specifically, network broadcast mapped cell ID and the corresponding area information, and provide association between mapped cell ID and service area, and maybe a new SIB needed due to information size. Then this option is also beneficial for subsequent MBS Interest Indication reporting with mapped cell granularity from UE side.

Figure 3 Association between Mapped cell and service area

- Proposal 4: In order to provide service area information, taking advantage of mapped cell ID introduced in NTN R17 could also be discussed. Specifically, network broadcast mapped cell ID and the corresponding area information, and provide association between mapped cell ID and service area.

## 2.3 UE behaviour

- We have discussed whether to limit the UE behaviour and left two issues as follows needed further check:

Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

- From our point of view, in NTN scenario, UE could be able to determine whether to establish or release the MRB for the intended service area based on its own GNSS location and satellite assistance information provided by network and/or whether interested in the broadcast service. Therefore, both the UE behaviour could be up to implementation without specification impact.

- Proposal 5: Whether to establish or release the MRB(s) associated with the intended service area when UE is not in the area or leaving the area could be up to UE implementation.

## 2.4 Country information

Per the discussion, some companies indicate that one intended service area may cross several country border, different country information need to be taken into account. However, the existing TMGI (Temporary Mobile Group Identity) used within MBMS to uniquely identify Multicast and Broadcast bearer services and the structure of TMGI is shown in the following figure:

Figure 15.2.1: Structure of TMGI [3]

And the SAI is constituted by Service Area Code (SAC) together with the PLMN-Id and the LAC (Location Area Code):

-	SAI = PLMN-Id || LAC || SAC [3]

Based on above, current TMGI and SAI have already contained MCC or PLMN ID respectively, it may be enough to represent different country and no further enhancements is needed.

Proposal 6: Current TMGI and SAI have already contained MCC or PLMN ID respectively, it may be enough to cover different country information.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following observations and proposals:

- Proposal 1: It is proposed to include service area information in MBSBroadcastConfiguration, and meanwhile provide the association between service area and MBS session.

Proposal 2: It is proposed to include service area information for ETWS in SIB6 if supported ETWS finally.

- Observation 1: Option 2 has overhead issue due to multiple location points for geographical area information (e.g. polygon).

- Proposal 3: Option 1 (circles) seems to be a simple solution to provide service area information. And a new IE (e.g. IntendedServiceAreaInfo-r19) is introduced as following:

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

- Proposal 4: In order to provide service area information, taking advantage of mapped cell ID introduced in NTN R17 could also be discussed. Specifically, network broadcast mapped cell ID and the corresponding area information, and provide association between mapped cell ID and service area.

- Proposal 5: Whether to establish or release the MRB(s) associated with the intended service area when UE is not in the area or leaving the area could be up to UE implementation.

Proposal 6: Current TMGI and SAI have already contained MCC or PLMN ID respectively, it may be enough to cover different country information.

Reference

- R2-126 NR-NTN-IoT-NTN (Sergio) EOM

- TS 22.268 V18.3.0 Public Warning System (PWS) requirements (Release 18)

- 3GPP TS 38.300 V18.0.0 NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)

- 3GPP TS 38.331 V18.1.0 NR; Radio Resource Control (RRC) protocol specification (Release 18)

- 3GPP TS 23.003 V18.4.0 Numbering, addressing and identification; (Release 18)