3GPP TSG RAN WG2 Meeting #127	R2-2406972

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.5

Source:	CMCC

Title:	Considerations on regenerative payload

Document for:	Discussion, Discussion

Introduction

- There were some discussions on regenerative payload in last RAN2 meeting, and agreements were reached as following [1]:

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In this contribution, we would like to provide some further considerations on regenerative payload based on above for NTN system.

Discussion

Common TA and kmac capture

Regarding common TA and kmac for regenerative payload, we made the agreement as following:

- 5.RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

As known the common TA and kmac are already support value zero in current TS 38.331 (as shown below).

Observation 1: Common TA and kmac are already support value zero in current specification.

From our point of view, for kmac, it is not necessary to add clarification in the stage 2 specification.

Then in section 4.2 of TS 38.213 , the calculation formula of common TA NTA,adjcommon is as following:

Delaycommont= TACommon2+ TACommonDrift2×t−tepoch+TACommonDriftVariant2×t−tepoch2

In NTN system, UE needs to pre-compensation TA based on NTN assistance information and its own GNSS, which would result in that UE may both overestimate and underestimate its TA. And with the existing common TA related parameters mentioned above, both positive and negative values are supported for NTA,adjcommon under transparent architecture whose reference point is between satellite and gNB on the ground. However, for regenerative payload with gNB on board, the reference point is most likely to be at the satellite (as in figure 1) in actual implementation, at this moment NTA,adjcommon calculation mainly depends on TACommon (i.e. ta-Common-r17 parameter) which only support positive values and 0. Then considering TA margin, it may be needed to introduce negative values for ta-Common-r17 parameter to help adjust TA better.

Figure 1: Illustration of timing relationship for regenerative payload with gNB on board

Therefore, for regenerative payload, we could just introduce negative values for ta-Common-r17 in TS 38.331 and up to network configuration, then additional clarification is not needed in stage 2 specification.

Proposal 1: For regenerative payload, introduce negative values for ta-Common-r17 in TS 38.331 and up to network configuration, then additional clarification is not needed for both common TA and kmac in stage 2 specification.

RACH-less HO

Then we agreed to support RACH-less HO for regenerative architecture, FFS optimization.

- 3.RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

The RACH-less HO is introduced in R18 NR NTN with transparent payload, and the basic functionality is as following [3]:

- The UE uses the same timing advance value at the target cell as in the source cell or timing advance value of 0. - The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell. - The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell. - The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

For regenerative payload with full gNB on board, from our perspective, the above functionality for RACH-less HO could be applied directly and further enhancement is unseen.

Proposal 2: For regenerative payload, existing RACH-less HO could be applied directly, no further enhancement is needed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on support of regenerative payload and have the following observation and proposals:

Observation 1: Common TA and kmac are already support value zero in current specification.

Proposal 1: For regenerative payload, introduce negative values for ta-Common-r17 in TS 38.331 and up to network configuration, then additional clarification is not needed for both common TA and kmac in stage 2 specification.

Proposal 2: For regenerative payload, existing RACH-less HO could be applied directly, no further enhancement is needed.

Reference

- R2-126 NR-NTN-IoT-NTN (Sergio) EOM

- R2-2403606	Regenerative NTN payload support in NR NTN Evolution	THALES, CATT, Huawei, ZTE, Inmarsat, Viasat	discussion	Rel-19	NR_NTN_Ph3-Core

- 3GPP TS 38.300 V18.2.0 NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)