3GPP TSG RAN WG2 Meeting #127	R2-2406973

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.6

Source:	CMCC

Title:	Considerations on LTE to NR mobility

Document for:	Discussion, Discussion

Introduction

- There were some discussions on LTE to NR NTN mobility in last RAN2 meeting, and agreements were reached as following [1]:

Agreements:

- For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

- For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

- SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

5.	The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

8.	Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID.

Working Assumption:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In this contribution, we would like to provide some further discussion on cell reselection enhancements from E-UTRAN TN to NR-NTN.

Discussion

Provision of the NR satellite assistance information

Per the discussion, there are 3 options to provide the NR satellite assistance information and needed to make a final decision in this meeting:

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the NeighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

Option 1 is simplest and more clear for UE. Option 2 leverage the existing SIB and just introducing a new IE, however, there may be some confusion for the UEs only support IoT-NTN. Option 3 extends the NeighSatelliteInfo has the same issue with option 2. Further, as in the current specification, the maximum number of neighbour satellite is 4, considering both IoT-NTN neighbour satellites and NR neighbour satellites information are needed, 4 neighbour satellites may not be sufficient. And the maximum number 4 is already taking the LTE SIB size into account, that is to say more satellite information is not able to include in one SIB. Therefore option 1 (i.e. a new SIB) may be better to include the NR satellite assistance information.

Proposal 1: Introduce a new SIB to include the NR satellite assistance information (option 1).

Then regarding the ntn-PolarizationDL, as in current TS 38.331, it is optional present, used to indicate polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

For LTE to NR NTN mobility, there is no essential reason to make it mandatory.

Proposal 2: Suggest ntn-PolarizationDL make optional.

Cell reselection evaluation and measurement rules

For cell reselection evaluation, a working assumption is reached as following:

Working Assumption:

- 1.	NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

In NR NTN R17, we have discussed how to prioritize TN over NTN, and some options are as follows [2]:

- Option 1: It is “hard” coded in the specification, e.g. in 38.304, that UE always prioritizes TN cell in cell reselection.

- Option 2: Indicate in system information of TN or NTN cell or both explicitly, the need to prioritize TN in the area the NTN/TN cells are covering.

- Option 3: Broadcast in system information a TN or NTN specific offset to be applied to RSRP measurement result for cell quality so that TN cell will always rank higher.

- Option 4: UE’s relaxed measurement mode is impacted to prioritize TN. For example, if UE detects TN cell UE stops any relaxed measurement mode for TN UE might have applied.

However, most companies prefer no further enhancement and legacy cell reselection prioritization (carrier frequency based) mechanism is sufficient. Agreement is reached as following [3]:

- No further enhancement on cell reselection procedure to support TN prioritization over NTN in Rel-17.

Then in NR NTN R18, we also discussed cell reselection priority between TN and NTN, in[4], it provides some solution that the priority can be configured per slicing, per service type, per mobility state, or per UE type considering different services or different mobility states. However, companies also think there is no need to introduce additional enhancements and agreements are achieved as follows [5]:

Agreements:

- We don’t introduce additional cell reselection prioritization rules for NTN vs TN in Rel-18 (e.g. per service type, per mobility state, or per UE type) on top of what specified in Rel-17

Similarly, for cell reselection evaluation between E-UTRA TN and NR NTN, legacy frequency priority scheme could also be the baseline as mentioned above for NR NTN vs. TN.

Proposal 3: Confirm the working assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

And for neighbour cell measurement, the following discussion is raised in last meeting:

Proposal 8: NR NTN neighbor cell measurement initiation is performed as in legacy, i.e. based on RRM measurement as in cell reselection from EUTRA TN to NR TN. No spec impact is needed on cell reselection procedure in TS 36.304.

-	Samsung thinks we should check this more until the next meeting. Ericsson agrees

Come back in the next meeting

In both IoT-NTN and NR NTN, we introduce time/location-based solution for neighbor cell measurement. However, both the methods are only used for UEs in a NTN cell, it is obviously not applicable for UEs camp on a LTE TN cell. And for NR TN to NR NTN cell reselection, legacy mechanism based on RRM measurement is supported during our discussion in R18 NR NTN. Therefore, for LTE TN to NR NTN cell reselection, we could just support legacy neighbour cell measurement solution.

Proposal 4: It is proposed to adopt legacy RRM based neighbor cell measurement mechanism for LTE to NR NTN cell reselection.

Then in RAN2 #125bis meeting, a working assumption about SMTC configuration is made:

Working Assumption:

- We don’t introduce multiple SMTCs in LTE

Considering that the coverage of a LTE TN cell is much smaller than a NTN cell, there would be not such many neighbour cells compared to NTN serving cell. Therefore multiple SMTCs introduced in NR NTN is not necessary for LTE TN to NR NTN cell reselection.

Proposal 5: Confirm the working assumption: we don’t introduce multiple SMTCs in LTE.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on mobility enhancements for E-UTRAN TN to NR-NTN and have the following proposals:

Proposal 1: Introduce a new SIB to include the NR satellite assistance information (option 1).

Proposal 2: Suggest ntn-PolarizationDL make optional.

Proposal 3: Confirm the working assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

Proposal 4: It is proposed to adopt legacy RRM based neighbor cell measurement mechanism for LTE to NR NTN cell reselection.

Proposal 5: Confirm the working assumption: we don’t introduce multiple SMTCs in LTE.

Reference

- R2-126 NR-NTN-IoT-NTN (Sergio) EOM

- R2-2203386_[Pre117-e][102][NTN] Idle mode open issues (ZTE)_v25_Rapporteur

- RAN2-117-e - R17 NTN-REDCAP-CE_EOM

- R2-2300732	NTN-TN Cell Reselection Enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

- R2-2301901 - Report from Break-out session on NR-NTN, IoT-NTN and RedCap