3GPP TSG-RAN WG2 Meeting #127	R2-2406975

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	8.3.2.1

Source:	CMCC

Title:	Initial simulation results for RRM measurement prediction

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In RAN2#126 meeting, there was discussion on Simulation assumptions and evaluation methodology for RRM measurement prediction. The agreements about simulation assumptions for RRM measurement prediction were achieved [1] with the following agreed evaluation scenarios to start the relevant simulations.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we would provide our initial simulation results of two high priority evaluation cases for RRM measurement prediction, including Case 2-FR1 to FR1 intra-frequency temporal domain case B and Case 4-FR2 to FR2 intra-frequency temporal domain case A.

# 2 Discussion

2.1 Simulation assumptions of the evaluation scenario

2.1.1 Description of the evaluation scenarios

Based on the discussion on AI/ML for mobility in RAN2#126, it could be observed that there are two main study goals of evaluation on AI/ML for mobility.

- 1st study goal of evaluation is to reduce measurement overhead

- 2nd study goal of evaluation is to enhance handover performance

As show above, Case2 that FR1 to FR1 intra-frequency temporal domain case B focus on Intra-cell scenario and the study goal is to reduce measurement overhead. As per the Definition of measurement reduction in agreements on simulation assumptions in RAN2#126, Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances:

MRRT= skipped measurement time instances / total measurement time instances

For Case4 that FR2 to FR2 intra-frequency temporal domain case A, it focus on Intra-cell scenario and the study goal is to enhance handover performance. As per the agreements for the evaluation exercise for 2nd study goal, there is no reduction in measurement overhead in this case.

For temporal domain prediction, there are Case A, Case B and Case B+ defined for the AI-PHY beam management in TR 38.843[2]. Here are the examples illustrated for Case A and Case B.

Figure 1: Example for Case A in [2]

Figure 2: Example for Case B in [2]

2.1.2 RRM sub case

Based on the agreements in RAN2#125bis, there are at least the following RRM sub cases for cell level measurement prediction model:

- RRM sub case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

- RRM sub case 2: To directly predict cell level results based on cell level results;

- RRM sub case 3: To directly predict cell level results based on beam level results.

In our initial simulation, we use RRM sub case 2 as starting point. The measurement results of L1 beam level RSRP are uses to derive the measurement results of L3 cell level RSRP and then the measurements of L3 cell level RSRP are used as AI/ML model input to predict L3 cell level RSRP results.

Figure 3: RRM sub case 2

2.2 Simulation results and analysis

2.2.1 Case 2-FR1 to FR1 intra-frequency temporal domain case B

In our evaluation, we simulate two sub-use cases where Measurement reduction rate is set to 1/2 and 2/3, respectively. And L1 sampling period is 40ms and the value of Measurement period is 200ms.

Table 1: The simulation parameters of Case 2

| Measurement reduction rate（MRRT） | Actual measurement time instances | Skipped measurement time instances | Illustration |
| --- | --- | --- | --- |
| 1/2 | n, n+2 | n+3 |  |
| 2/3 | n, n+3 | n+4, n+5 |  |

The simulation results of Case 2-FR1 to FR1 intra-frequency temporal domain case B is shown in Table 2.

Table 2: The simulation results of Case 2-FR1 to FR1 intra-frequency temporal domain case B

| Evaluation scenario |  | FR1 to FR1 intra-frequency temporal domain case B |  |
| --- | --- | --- | --- |
| Report parameters |  | CMCC: sub use case 1 | CMCC: sub use case 2 |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 | Option 3 |
|  | UE speed (30,60,90,120 Km/h) | 90Km/h | 90Km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A | N/A |
|  | Whether LOSsoft is modeled or not | Yes | Yes |
|  | spatial consistency option (A or B) | Procedure A | Procedure A |
|  | Number of TX beams | 8 | 8 |
|  | Number of RX beams | 2 | 2 |
|  | Measurement reduction rate(50%,…Note2) | 1/2 | 2/3 |
|  | Observation window (Note3) | N/A | N/A |
|  | Prediction window (Note3) | N/A | N/A |
|  | Any other parameters (Note 4) | N/A | N/A |
| Data Size (Number of Samples) | Training/validity | 24000, 3000 | 15600, 1950 |
|  | Testing | 3000 | 1950 |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP | Cell level RSRP |
|  | Model output (Note 6) | Cell level RSRP | Cell level RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | ResNet | ResNet |
|  | Model complexity in a number of parameters(M) | 0.024 | 0.027 |
|  | Model complexity in model size (e.g. Mbyte) | 0.093MB | 0.106MB |
|  | Computational complexity [FLOPs] | 292.66K | 578.26K |
| Metrics | Average L3 cell level RSRP difference (dBm) | 0.8062 | 1.3060 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A | N/A |

Note1: Only applicable for FR1 to FR1 inter-frequency prediction.

Note2: Only applicable for intra-frequency prediction, either temporal domain case B or spatial domain. For FR1 to FR1 inter-frequency prediction, it is fixed i.e. no measurement will be performed on the frequency carrier to be predicted

Note3: Only applicable for intra-frequency temporal domain case A. Prediction window are initially set as 200ms and 400ms for FR1 and FR2 respectively.

Note4: This could be any other parameter e.g., BS antenna configuration, UE antenna configuration, BS TX power etc.

Note5: Apart from input of RRM sub case 1,2,3, other input information e.g. L1 filtering for L1 beam measurement, UE location , information of input cells are captured here too

Note6: Apart from output of RRM sub case 1,2,3, other output e.g. information of output cells is captured here too

Based on the above simulation results, it could be observed that in the case that Measurement reduction rate is 1/2, although UE speed is high (90km/h) the prediction accuracy is kept good (less than 1dB). And Average L3 cell level RSRP difference reduces as Measurement reduction rate reduces.

Observation 1: In our simulation, in the case that Measurement reduction rate is 1/2, although UE speed is high (90km/h) the prediction accuracy is kept good (less than 1dB).

Observation 2: Average L3 cell level RSRP difference reduces as Measurement reduction rate reduces.

2.2.2 Case 4-FR2 to FR2 intra-frequency temporal domain case A

In our evaluation, we simulate three sub-use cases where the observation window is set to 3, 4 and 5 respectively and the prediction window is 1. And L1 sampling period is 40ms and the value of Measurement period is 400ms.

The simulation results of Case 4-FR2 to FR2 intra-frequency temporal domain case A is shown in Table 3. And CFD of L3 cell level RSRP difference are shown in Figure 4.

Table 3: The simulation results of Case 4-FR2 to FR2 intra-frequency temporal domain case A

| Evaluation scenario |  | FR2 to FR2 intra-frequency temporal domain case A |  |  |
| --- | --- | --- | --- | --- |
| Report parameters |  | CMCC: sub use case 1 | CMCC: sub use case 2 | CMCC: sub use case 3 |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 | Option 1 | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 | Option 3 | Option 3 |
|  | UE speed (30,60,90,120 Km/h) | 60 km/h | 60 km/h | 60 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | N/A | N/A | N/A |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | N/A | N/A | N/A |
|  | Whether LOSsoft is modeled or not | Yes | Yes | Yes |
|  | spatial consistency option (A or B) | A | A | A |
|  | Number of TX beams | 64 | 64 | 64 |
|  | Number of RX beams | 8 | 8 | 8 |
|  | Measurement reduction rate(50%,…Note2) | N/A | N/A | N/A |
|  | Observation window (Note3) | 3 | 4 | 5 |
|  | Prediction window (Note3) | 1 | 1 | 1 |
|  | Any other parameters (Note 4) | N/A | N/A | N/A |
| Data Size (Number of Samples) | Training/validity | 184140, 20460 |  |  |
|  | Testing | 41242 |  |  |
| AI/ML model input/output | Model input (Note 5) | Cell level RSRP | Cell level RSRP | Cell level RSRP |
|  | Model output(Note 6) | Cell level RSRP | Cell level RSRP | Cell level RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM | LSTM | LSTM |
|  | Model complexity in a number of parameters(M) | 1263801 |  |  |
|  | Model complexity in model size (e.g. Mbyte) | 14.499 |  |  |
|  | Computational complexity [FLOPs] | 890.82K |  |  |
| Metrics | Average L3 cell level RSRP difference (dBm) | 1.481 | 1.479 | 1.475 |
|  | Other optional KPIs (e.g., L1 beam level RSRP difference,) | N/A | N/A | N/A |

Note1: Only applicable for FR1 to FR1 inter-frequency prediction.

Note2: Only applicable for intra-frequency prediction, either temporal domain case B or spatial domain. For FR1 to FR1 inter-frequency prediction, it is fixed i.e. no measurement will be performed on the frequency carrier to be predicted

Note3: Only applicable for intra-frequency temporal domain case A. Prediction window are initially set as 200ms and 400ms for FR1 and FR2 respectively.

Note4: This could be any other parameter e.g., BS antenna configuration, UE antenna configuration, BS TX power etc.

Note5: Apart from input of RRM sub case 1,2,3, other input information e.g. L1 filtering for L1 beam measurement, UE location , information of input cells are captured here too

Note6: Apart from output of RRM sub case 1,2,3, other output e.g. information of output cells is captured here too

Figure 4: CFD of L3 cell level RSRP difference

Based on the above simulation results, it could be observed that in the case of constant prediction window, Average L3 cell level RSRP difference reduces as the observation window increase. That means it is helpful to improve the prediction accuracy that the more measurement results are used as the inputs of AI/ML model.

Observation 3: In the case of constant prediction window, Average L3 cell level RSRP difference reduces as the observation window increase. It is helpful to improve the prediction accuracy that the more measurement results are used as the inputs of AI/ML model.

However, we can also observe that the reduction of average L3 cell level RSRP difference is not significant as the observation window increase. We think it may be related to the configuration of Measurement period and UE speed in the simulation. In our simulation, a L3 cell level measurement result is generated every 400ms and UE speed is 60km/h. These may result in a relatively low correlation between L3 cell level measurements and predictions. As a result, although more measurement results are used for prediction, the improvement of prediction accuracy is limited.

Observation 4: Relatively long Measurement period and high UE speed may result in a low correlation between L3 cell level measurements and predictions. That may impact the prediction accuracy based on cell level measurement results.

2.3 Potential specification impact

Based on the above simulation results and analysis, we can observe that both intra-frequency temporal domain Case A and Case B for RRM measurement prediction based on cell level results could achieve good prediction performance. Therefore, the specification impact could be further considered to support intra-frequency temporal domain of RRM measurement prediction for measurement overhead reduction or handover performance enhancement.

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results, same with model input;

Intra-frequency temporal domain for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE should report the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE should report the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

Proposal 1: The potential specification impact could be considered for intra-frequency temporal domain for RRM measurement prediction based on cell level results.

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results, same with model input;

Proposal 2: Intra-frequency temporal domain for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE should report the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE should report the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

# 3 Conclusion

In this paper, we provide our initial simulation results of Case 2-FR1 to FR1 intra-frequency temporal domain case B and Case 4-FR2 to FR2 intra-frequency temporal domain case A, and we have the following observations and proposals:

Observation 1: In our simulation, in the case that Measurement reduction rate is 1/2, although UE speed is high (90km/h) the prediction accuracy is kept good (less than 1dB).

Observation 2: Average L3 cell level RSRP difference reduces as Measurement reduction rate reduces.

Observation 3: In the case of constant prediction window, Average L3 cell level RSRP difference reduces as the observation window increase. It is helpful to improve the prediction accuracy that the more measurement results are used as the inputs of AI/ML model.

Observation 4: Relatively long Measurement period and high UE speed may result in a low correlation between L3 cell level measurements and predictions. That may impact the prediction accuracy based on cell level measurement results.

Proposal 1: The potential specification impact could be considered for intra-frequency temporal domain for RRM measurement prediction based on cell level results.

AI/ML model input could include:

- Actual RRM measurement results, e.g., RSRP, RSRQ and SINR;

- UE assistant information, e.g., UE speed, UE location information;

AI/ML model output could include:

- Cell level RRM measurement prediction results, same with model input;

Proposal 2: Intra-frequency temporal domain for RRM measurement prediction could be supported by UE-side model or NW-side model.

- For UE-side model, UE should report the prediction results (AI/ML model output) and the time information of prediction generation to network for Handover Decision;

- For NW-side model, UE should report the RRM measurement results and UE assistant information (if needed) to network as AI/ML model input.

# Reference

- R2_126_ChairNotes_24-05-24_final, May 2024

- TR 38.843: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface