3GPP TSG-RAN WG2 Meeting #127	R2-2406976

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	8.3.4.1

Source:	CMCC

Title:	Discussion on Simulation Assumption and Methodology for RLF prediction

WID/SID:				FS_NR_AIML_Mob

Document for:	Discussion

# 1 Introduction

In last meeting [1], RAN2 discussed the RLF/HOF prediction and achieved the following agreements:

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August.  The assumption is that we will reuse RRM simulation assumptions (where possible).

In this contribution, we would discuss on RLF specific evaluation methodology, simulation assumption and relevant metrics.

# 2 Discussion

2.1 Evaluation methodology for RLF prediction

Based on the discussion and agreements on AI/ML for mobility in RAN2#126, it could be observed that there are two sub-use cases for RLF prediction with UE-sided model:

- Sub-use case 1 (Indirect case): RLF prediction based on the temporal domain serving cell RRM measurement predictions (e.g. SINR).

- Sub-use case 2 (Direct case): Directly RLF prediction by AI/ML models.

For temporal domain prediction used in Indirect case of RLF prediction, there are Case A, Case B and Case B+ defined for AI-PHY beam management in TR 38.843[2]. Based on the examples illustrated as below, we can see that Case A where the measurements at each time instance in the observation time window as the model input and prediction in the prediction time window as the model output is applicable for temporal domain RLF prediction.

Proposal 1: Case A from the RAN1 AI/ML PHY TR38.843 could be used in the Indirect case of RLF prediction evaluation.

Figure 1: Example for Case A in [2]

Figure 2: Example for Case B in [2]

Figure 3: Example for Case B+ in [2]

For Indirect case, RLF prediction evaluation based on RRM measurement prediction, which RRM sub cases are applicable should be considered. As per the agreements in RAN2#125bis, there are at least the following RRM sub cases:

- RRM sub case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

- RRM sub case 2: To directly predict cell level results based on cell level results;

- RRM sub case 3: To directly predict cell level results based on beam level results.

For all 3 RRM sub cases, predict L3 cell level results could generated directly or derived based on L1 beam results, and the L3 cell level prediction could be used for RLF prediction. Therefore, we think all 3 RRM sub cases are applicable for RLF prediction.

Proposal 2: All 3 RRM sub cases are applicable for RLF prediction evaluation based on RRM measurement prediction.

Based on the agreements in last meeting, RLF probability within a time window or at time instance is considered as RLF prediction result, at least for direct case.

6 RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

Besides that, from our point of view, the time info of RLF occurrence (the expected RLF time) could be considered as RLF prediction result for direct case.

Proposal 3: For the model output of direct RLF prediction, the time info of RLF occurrence (the expected RLF time) could be considered for direct case.

And for indirect case, RLF probability within a time window or at time instance and the time info of RLF occurrence could be also considered as the RLF prediction results. The RLF prediction results could be the output of AI/ML model directly or derived by UE based on RRM measurement prediction. That can be up to the AI/ML model implementation for RLF prediction.

Proposal 4: For indirect case, RLF probability within a time window or at time instance and the time info of RLF occurrence could be also considered as the RLF prediction results, which could be the output of AI/ML model directly or derived by UE based on RRM measurement prediction.

Considering RLF prediction is supported only by UE sided model, we think the time info of RLF prediction generation could help network to know when UE sided model generates the RLF prediction and whether to use this RLF prediction as the assistant information to decide the optimal time of handover to avoid the unintended events. Therefore, we think the time info of RLF prediction generation should be reported to network with RLF prediction result.

Proposal 5: The time info of RLF prediction generation should be reported to network with RLF prediction result, both for direct case and indirect case.

For direct RLF prediction, the model input could be L1/L3 measurements (e.g., RSRP/RSRQ/SINR) at each time instance in observation time window, historic HOF/RLF, UE location, measurement event (e.g., A3 event) parameters and RLF related parameters (e.g., T310), etc. Due to the diversity of the model input, it is proposed that the model input should be reported by companies.

Proposal 6: The model input of direct case should be reported by companies in the Evaluation of RLF prediction due to the diversity of the model input.

2.2 Performance metrics/KPIs

As defined in TR 36.839 [3], the handover procedure is divided into 3 states as shown in Figure 5.2.1.3.1.

State 1: Before the event A3 entering condition is satisfied;

State 2: After the event A3 entering condition is satisfied but before the handover command is successfully received by the UE; and

State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE.

Figure 5.2.1.3.1: A handover failure is declared when the criterion 1) is met in state 2.

Figure 5.2.1.3.2: A handover failure is declared when the criterion 2) is met in state 2.

Furthermore, the definition of RLF and RLF performance metric, HO failure and HO failure rate and RLF are also provided in TR 36.839 [3].

Definition 1: The occurrence of RLF can be categorized into two distinctive states: state 1 and state 2 of the handover process.

Definition 2: The RLF performance metric is defined as: the average number of RLF occurrences per UE per second. RLF performance in states 1 and 2 are logged separately.

Definition 3: A handover failure is counted if a RLF occurs in state 2, or a PDCCH failure is detected in state 2 or state 3.

Definition 4: The handover failure rate is defined as: Handover failure rate = (number of handover failures) / (Total number of handover attempts).

We think the definitions can be reused as performance metrics for study on AI/ML aided mobility.

Proposal 7: The definitions of RLF performance metric and HO failure rate in TR 36.839 can be reused as performance metrics for RLF prediction.

For indirect option, RLF is predicted based on the temporal domain serving cell measurement predictions (e.g. SINR), thus, the performance metrics/KPIs defined in RRM measurement prediction can be reused, e.g. the difference between the predicted measurement and the actual measurement.

For direct RLF prediction, AI/ML model can predict the RLF probability within a time window or at time instance. For the prediction accuracy of the RLF occurrence, the typical ML classification metrics (e.g. precision, recall or F1-score) could be used.

Refer to [4], there is a classical formula to interpret the Precision, Recall or F1-score:

Table 1

| Predicted event Baseline event | Positive | Negative |
| --- | --- | --- |
| Positive | TP | FN |
| Negative | FP | TN |

In Table 1, there are 4 cases defined as follows:

- TP (True Positives): the number of Predicted Positives that were correct, i.e. an event is predicted and an event occurs in baseline

- FN (False Negatives): the number of Predicted Negatives that were incorrect, i.e. no event is predicted but an event occurs in baseline

- FP (False Positives): the number of Predicted Positives that were incorrect, i.e. an event is predicted but no event occurs in baseline

- TN (True Negatives): the number of Predicted Negatives that were correct, i.e. no event is predicted and no event occurs in baseline

The accuracy formulas are as follows based on the above definitions:

- Precision = TP / (TP+FP)

- Recall = TP / (TP+FN)

- F1 score = 2 * Precision * Recall / (Precision+ Recall)

Figure 4. Example for Precision and Recall

Proposal 8: The following performance metrics/KPIs could be considered for RLF prediction:

- The performance metrics/KPIs defined for RRM measurement prediction can be reused for indirect RLF prediction, e.g. the SINR difference to the actual measurement

- The prediction accuracy for the occurrence of RLF, e.g. Precision, Recall or F1-score

2.3 Simulation assumptions for RLF prediction

Based on the above analysis, we would like to provide RLF prediction specific simulation assumptions of A3 Event and RLF related parameters. And for Simulation assumptions for FR2, we could reuse the baseline of RRM simulation assumptions.

Table 2: The Baseline simulation assumptions for A3 Event and RLF related parameters

| Parameter | Value |
| --- | --- |
| L3 measurement filter | K = 4,1,0 |
| Handover criteria | Event A3, RSRP |
| a3-offset | 2dB |
| A3 TTT | 320ms 40ms |
| A3 hysteresis | 1dB |
| HO preparation time | 50ms |
| HO execution time | 40ms |
| RLM | L1 measurement period: 20ms Qin sliding window length：100ms Qout sliding window length: 200ms Qin threshold: -6dB Qout threshold: -8dB T310: 1s |

Proposal 9: RAN2 considers the above simulation assumptions of A3 Event and RLF related parameters as the baseline simulation assumptions for RLF prediction evaluation.

# 3 Conclusion

In this paper, we discuss on RLF specific evaluation methodology, simulation assumption and relevant metrics, and we have the following proposals:

Evaluation methodology for RLF prediction

Proposal 1: Case A from the RAN1 AI/ML PHY TR38.843 could be used in the Indirect case of RLF prediction evaluation.

Proposal 2: All 3 RRM sub cases are applicable for RLF prediction evaluation based on RRM measurement prediction.

Proposal 3: For the model output of direct RLF prediction, the time info of RLF occurrence (the expected RLF time) could be considered for direct case.

Proposal 4: For indirect case, RLF probability within a time window or at time instance and the time info of RLF occurrence could be also considered as the RLF prediction results, which could be the output of AI/ML model directly or derived by UE based on RRM measurement prediction.

Proposal 5: The time info of RLF prediction generation should be reported to network with RLF prediction result, both for direct case and indirect case.

Proposal 6: The model input of direct case should be reported by companies in the Evaluation of RLF prediction due to the diversity of the model input.

Performance metrics/KPIs

Proposal 7: The definitions of RLF performance metric and HO failure rate in TR 36.839 can be reused as performance metrics for RLF prediction.

Proposal 8: The following performance metrics/KPIs could be considered for RLF prediction:

- The performance metrics/KPIs defined for RRM measurement prediction can be reused for indirect RLF prediction, e.g. the SINR difference to the actual measurement

- The prediction accuracy for the occurrence of RLF, e.g. Precision, Recall or F1-score

Simulation assumptions for RLF prediction

Proposal 9: RAN2 considers the above simulation assumptions of A3 Event and RLF related parameters as the baseline simulation assumptions for RLF prediction evaluation.

# Reference

- R2_126_ChairNotes_24-05-24_final, May 2024

- TR 38.843: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- TR 36.839 Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks

- D. J. Powers, “Evaluation: From Precision, Recall and F-measure To ROC, Informedness, Markedness & Correlation”, Journal of Machine Learning Technologies, vol. 2, no.1, pp. 37-63, 2011