# 3GPP TSG-WG2 Meeting #127                                             R2-2406977

Maastricht, Netherlands, 19th – 23th August, 2024

Source: 			CMCC

Title:  		    Discussion on functionality for A-IoT

Agenda item:	8.2.2

Document for: 	Discussion

Introduction

With the past two meetings, RAN2 discussed the functionalities for A-IoT topology 1. The following progress was made.

Agreements:

- SDAP is not supported for UP protocol stack.

- PDCP layer is not needed.  FFS how to handle AS security (if needed pending SA3 dicsussion) and any other really needed functionalities.

- RLC layer is not needed.   FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size).  RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed.

- No HARQ and RLC AM

- FFS about the level of visibility required by the reader and what information is necessary for AS layer operations.

- RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2

- RRC connection management is not supported.  FFS how the resource configuration is provided to the device (if needed based on RAN1 progress)

- Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals.

- RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

- Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

- legacy NR BSR/SR is not needed for A-IoT communication.

- FFS whether further indication of device message size/status is needed

- AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

Since this meeting, RAN2 starts the discussion on topology 2. We understand that RAN2 should discuss whether the achieved agreements are feasible for topology 2 firstly, and then discuss the other issues for functionalities.

Discussion

Alignment between topology 1 and topology 2

In topology 1, the gNB reader performs inventory or read/write command to A-IoT devices. The gNB also allocates transmission resources to A-IoT radio, and forwards the inventory or read/write command feedback to core network. In topology 2, it is the UE as intermediate node to do inventory or read/write command towards A-IoT devices. The basic end to end architecture has not been decided in RAN2 or RAN3. So, for the uu interface design between intermediate UE and network is far from clear. However, apart from the interworking between the UE reader and network, the behaivers of the UE reader are totolly same as the gNB reader at A-IoT reader. The functionality for A-IoT radio betweent A-IoT devices and gNB/UE reader is common for both topology 1 and topology 2. The A-IoT radio protocol stack betweent A-IoT devices and gNB/UE reader should be kept aligned.

Proposal 1: The functionality of A-IoT radio betweent A-IoT devices and gNB/UE reader are aligned

Proposal 2: The A-IoT radio protocol stack betweent A-IoT devices and gNB/UE reader are aligned.

Information visibility to network/reader

The inventory only, command only, inventory and command in same procedure can all be supported for A-IoT, where multiple types of RA, e.g. CBRA/CFRA and 2/4-step RA may be applicated. For A-IoT radio, it is the network to allocate RA resource and data transmission resource to A-IoT radio. Random access and data transmission are both fully controlled by network. For different kinds of procedure, the resource allocation can be different. For example, if inventory procedure is triggered, the expected data are list of device IDs, the D2R data can be almost 96bits; if read command is triggered, the acknowledgement can be 1000bits at most. For effective resource scheduling, the reader at least needs to understand the type of type of the message from CN, e.g. inventory or commands to assistant resource.

Observation 1: From the network’s perspective, it is reasonable and beneficial to be aware of the type of the message from CN in order to allocate A-IoT RA resources and data transmission resources, e.g. inventory or commands.

Proposal 3: RAN2 assumes that the A-IoT message type is visible to gNB, i.e., gNB is aware of inventory, read or write.

Furthermore, the gNB should make decisions on RA type handle the procedure feasibly and effectively with some assistant information from the message. For example, if inventory procedure targets for one device, the reader can initial the CFRA; if the device IDs are contained in the A-IoT message from the CN, the reader can allocate the access resource to each device to trigger 2-step RA. So, some parameters from CN is necessary for gNB, e.g. the number of devices to assistant RA type selection. As to the Q or mask ID, we understand it can be also generated in RAN, because them are only used as access which is always under gNB control. If the Q value and mask ID/device ID are sent by the CN, they should be visible to gNB to contain in A-IoT paging message. If the Q value is not sent by CN, reader can generate Q value based on number of devices. As to how to obtain the number of devices in reader side, we can further discuss on that.

Observation 2: Awareness of device number is beneficial for network resource allocation.

Proposal 4: If the implicit or explicit device number is included in paging message from CN, the number should be visible to gNB; If no device number is included in paging message from CN, FFS how to obtain the number of devices in gNB.

Segmentation

The segmentation is introduced for the case that the data pending to transmitted is larger than the transmission resource in PHY as indicated. RAN2 considers segmentation and reassembly would add complexity to system as agreed in last meeting. So, the segmentation and reassembly should be considered unless we see the serious necessity. The maximum message size is 1000 bits based on TR 38.848. As per the evaluation assumption on A-IoT in RAN1, the message size can be considered as 20bits, 96bits, 400bits from physical layer perspective. As discussed in past meetings, companies suggested that the A-IoT paging message type shall be visible by the gNB to assistant resource allocation. With the A-IoT paging message type, the gNB can justify the type acknowledgments of the A-IoT paging message. So, the data size of acknowledgments is predictable for the gNB. For example, the A-IoT paging message type is inventory, the gNB can predict the device ID will be feedbacked as acknowledgment, the message size can be 16 bits for temporary device ID, or 96 bits for device ID. Correspondingly, 20bits or 96bits can be configured for acknowledgement data transmission.

Observation 3: gNB can predict the message size of the acknowledgement with the visibility of the A-IoT paging message type.

Proposal 5: RAN2 pending the discussion on segmentation before RAN1 has progress on the supported size of transport block.

Message size/ status indication

As concluded in last meeting, the legacy buffer status report or schduing request are neither supported. Whether further indication of device message size/status is needed are FFS.

In the A-IoT procedure, the message size and the link condition, stored energy may affect the data transmision. For the A-IoT devices with low energy and in the near of reader can be successfully transmitted if reader indicate the device with low TB size or bit rate. From network perspective, the message size can be estimated with the paging message type, as we analysis in above Proposal 3; the exact stored energy at device is invisible to reader or network, however, the rough energy conditions can be obtained with the energy harvesting time and energy storage capability (i.e., using a capacitor); as per RAN1 progress, the proximity determination is still under discussion. At least, the reader can obtain the information that whether the devices are near or not. With above consideration, the message size report is beneficial for the network resource allocation. On the other hand, the acquisition of the message size is performed before the data transmission, which may bring some complexity to the A-IoT device.

Observation 4: The message size report is beneficial for the network resource allocation.

Observation 5: The acquisition of the message size is performed before the data transmission, which may bring some complexity to the A-IoT device.

In our view, with the assumption that the message type is visible to network and the rough energy store condition of the device can be estimated by network, the gNB can always do resource allocation well. At least for this release, the size, complexity, and power consumption of IoT devices should be reduced to enable the deployment of tens or even hundreds of billion IoT devices for various applications.

Proposal 6: Message size or status reporting is not considered for A-IoT is this release.

QoS handling

In TR 38.848, the latency targets of Ambient IoT system which represents the QoS is given as:

- 5.6 Latency The one-way end-to-end maximum latency targets, as defined in TR 22.840, are: - Longer latency target: 10 seconds - Shorter latency target: 1 second A use case is assigned to a latency target according to TR 22.840. RAN WGs can refine a definition of latency suitable for their work within the above. NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only. NOTE: The one-way end-to-end maximum latency is assumed to also include query/triggering time.

Further in the revised WID [1], the latency targets are considered in RAN1 led evaluation assumptions. Up to now, RAN1 has not discussed whether the definition of latency needs to be refined. As for the one-way end-to-end maximum latency targets defined above, RAN1 may need to discuss the DL/UL data rates, the design of preamble, etc. But we have not found a clear work in RAN2 scope. Besides there has not been any other QoS requirements agreed by other WGs. To this end, RAN2 can assume all Ambient IoT services have same QoS requirements before the definition of latency is refined by RAN1 or other QoS requirements are agreed by other WGs.

Proposal 7: Before the requirement of latency is refined by RAN1 or other QoS requirements are agreed by other WGs, RAN2 can assume all Ambient IoT services have same QoS requirements.

Proximity determination

As described in the revised SID of A-IoT, the feasibility and functionality for proximity determination should be studied.

Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects).

RAN1 has start the discussion on this issue and has some preliminary progress:

Study the following schemes for proximity determination:

Option 1: If reader receives D2R transmission from the device in response to R2D transmission, then device is determined as near

FFS: Details on reception criteria (e.g. either successful or not) at reader and device

Option 2: Device is determined to be near the reader based on measurements at the reader side

FFS: Details on measurement methods

FFS: Whether/how transmit power of R2D and/or D2R is considered for proximity determination

Considering the two options on the table in RAN1, the reader judges the devices are nearby once the reader receive the response from the device or the quality of the response is higher than some threshold. The current scheme for proximity determination is not clear enough and the task for RAN2 can not be identified at least for now. So, we suggest RAN2 pending discussion on proximity determination until RAN1 or other groups have progress on that.

Proposal 8: RAN2 pending the discussion on proximity determination until other WGs have progress on it.

Protocol stack

With the above discussion, we draft the protocol stack for A-IoT as follows:

Figure.1-1 Protocol stack for A-IoT topology 1

Figure.1-2 Protocol stack for A-IoT topology 2

Proposal 9: RAN2 is suggested to capture the protocol stack in Figure 1-1, 1-2 for A-IoT topology 1 and 2.

Conclusion

According to the above discussion, the following observations and proposals are given:

Observation 1: From the network’s perspective, it is reasonable and beneficial to be aware of the type of the message from CN in order to allocate A-IoT RA resources and data transmission resources, e.g. inventory or commands.

Observation 2: Awareness of device number is beneficial for network resource allocation.

Observation 3: gNB can predict the message size of the acknowledgement with the visibility of the A-IoT paging message type.

Observation 4: The message size report is beneficial for the network resource allocation.

Observation 5: The acquisition of the message size is performed before the data transmission, which may bring some complexity to the A-IoT device.

Proposal 1: The functionality of A-IoT radio betweent A-IoT devices and gNB/UE reader are aligned

Proposal 2: The A-IoT radio protocol stack betweent A-IoT devices and gNB/UE reader are aligned.

Proposal 3: RAN2 assumes that the A-IoT message type is visible to gNB, i.e., gNB is aware of inventory, read or write.

Proposal 4: If the implicit or explicit device number is included in paging message from CN, the number should be visible to gNB; If no device number is included in paging message from CN, FFS how to obtain the number of devices in gNB.

Proposal 5: RAN2 pending the discussion on segmentation before RAN1 has progress on the supported size of transport block.

Proposal 6: Message size or status reporting is not considered for A-IoT is this release.

Proposal 7: Before the requirement of latency is refined by RAN1 or other QoS requirements are agreed by other WGs, RAN2 can assume all Ambient IoT services have same QoS requirements.

Proposal 8: RAN2 pending the discussion on proximity determination until other WGs have progress on it.

Proposal 9: RAN2 is suggested to capture the protocol stack in Figure 1-1, 1-2 for A-IoT topology 1 and 2.

Figure.1-1 Protocol stack for A-IoT topology 1

Figure.1-2 Protocol stack for A-IoT topology 2

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR