3GPP TSG-RAN WG2 Meeting #127	R2-2406978

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

# Agenda item:	8.4.4

# Source:	CMCC

# Title:	Discussion on LP-WUS operation in CONNECTED mode

# Document for:	Discussion, Decision

- Introduction

In RAN2#126 meeting, the following agreements for LP-WUS/WUR in RRC_ CONNNECTED mode were made [1]:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we will discuss the potential RAN2 impacts with LP-WUS/WUR in RRC_CONNECTED state.

- Discussion

  - LP-WUS use option

Details of Option 1-1

In last meeting, RAN2 reached the consensus on Option 1-1 that the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer, and RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. In this section, we will discuss the detail configuration and restrictions of Option 1-1 taking DCP as reference.

Following is the DCP configuration specified in TS 38.331[2]:

DCP-Config-r16 ::= SEQUENCE { ps-RNTI-r16 RNTI-Value, ps-Offset-r16 INTEGER (1..120), sizeDCI-2-6-r16 INTEGER (1..maxDCI-2-6-Size-r16), ps-PositionDCI-2-6-r16 INTEGER (0..maxDCI-2-6-Size-1-r16), ps-WakeUp-r16 ENUMERATED {true} OPTIONAL, -- Need S ps-TransmitPeriodicL1-RSRP-r16 ENUMERATED {true} OPTIONAL, -- Need S ps-TransmitOtherPeriodicCSI-r16 ENUMERATED {true} OPTIONAL -- Need S }

ps-RNTI is the RNTI value for scrambling CRC of DCI format 2-6, while for LP-WUS, UE doesn’t monitor related RNTI but a LP sequence, therefore, RNTI is not needed. As to the LP sequence, whether it should be defined in Option 1-1 specific configuration as DCP can be decided by RAN1.

ps-Offset is used to indicated the UE the LP-WUS monitoring occasion before start of  the drx-onDurationTimer, it’s also needed for Option 1-1.

ps-WakeUp indicates UE’s default behaviour when if DCP is not detected outside active time, similar mechanism can also be applied for Option 1-1.

ps-PositionDCI-2-6 indicates UE’s starting position in DCI 2-6, and when Option1-1 is used, the UE needs to know the starting position in the LP sequence too.

As to PeriodciL1-RSRP and ps-TranmsitOtherPeriodcCSI, they are used to indicate the UE’s default behaviour on periodic L1-RSRP and CSI reporting when the drx-onDurationTimer is not started. This mechanism can be applied in Option 1-1.

Proposal 1: Configuration of Option 1-1 can include the following contents:

- Offset to indicate the UE the LP-WUS monitoring occasion before start of  the drx-onDurationTimer;

- UE’s starting position in the LP sequence;

- UE’s default behaviour when LP-WUS is not detected outside active time;

- UE’s default behaviour on periodic L1-RSRP and CSI reporting when the drx-onDurationTimer is not started.

Proposal 2: Whether LP-WUS related LS sequence configuration is included in Option 1-1 specific configuration is decided in RAN1.

Besides, DCP is only configured on the PCell when DC is configured. It’s not clear whether similar restriction is used for Option 1-1, since LP-WUS options for UE with CA configured is discussed in RAN1, which is also related to RAN1’s discussion on whether LR and MR is deployed in the same carrier or not, to avoid duplicated discussion, RAN2 can suspend the related discussion and wait for RAN1’s progress.

Proposal 3: RAN2 is asked to suspend the discussion on LP-WUS operations for UE with CA configured and wait for RAN1’s progress.

Other Options

Option 1-2

Compared with Option 1-1, Option 2 provides more PDCCH monitor or data transmission opportunities and it can avoid unexpected transmission latency due to C-DRX cycle. Option 1-2 is more suitable to the services without fixed periodicity, e.g. FTP services.

Observation 1: Option 1-2 provides more PDCCH monitoring or data transmission opportunities and less transmission latency, and it is suitable for the services without fixed periodicity, e.g. FTP services.

As agreed in RAN1, Option 1-2-1 should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX, if it is adopted. In other words, except the fixed location before drx-onDurationTimer, the UE may monitor LP-WUS in the whole drx-off period with duty-cycled mode, while for Option 1-2-2, UE will monitor LP-WUS irrespective of C-DRX configuration. For Option 1-2-1 and 1-2-2, a new “active time” is triggered different from drx active time, therefore, new timer(s) for the active time triggered by LP-WUS should be introduced. Figure 2 and Figure 3 are the examples for Option 1-2-1 and Option 1-2-2.

In SI phase, two kinds of LP-WUS operation were studied: “continuous” and “duty-cycled” [3]. Based on the WID, At least duty-cycled monitoring of LP-WUS is supported, to reduce the LP-WUS monitoring power consumption, it is preferred that UE monitor LP-WUS with “duty-cycled” mode. And similar discussion can be applied to Option 1-2-2, “duty-cycled” LP-WUS monitoring is sufficient. Figure 2 and Figure 3 are the examples for Option 1-2-1 and Option 1-2-2.

Figure 2: Example for Option 1-2-1

Figure 3: Example for Option 1-2-2

Option 1-3

Both Option 1-1 and Option 1-2 are used to trigger the MR into active time for PDCCH monitoring,  which means that with only Option 1-1 or Option 1-2, once MR is activated, the UE need to always monitor all PDCCH occasions. While Option 1-3 works inside UE’s active time, with LP-WUS indication, UE can skip some PDCCH occasions monitoring for further power saving. Option 1-3 is more suitable for XR service, which the service is periodic, but there is jitter for each packet, and LP-WUS can be used to jitter handling in this case. Figure 4 is the example for Option 1-3.

Observation 2: Option 1-3 is suitable for XR service, which the service is periodic, but there is jitter for each packet, and LP-WUS can be used to jitter handling in this case.

Figure 4: Example for Option 1-3

Based on the analysis above, different options are used for different use cases. Therefore, RAN2 can start with the LP-WUS use options identified by RAN1.

For Option 1-2 and Option 1-3, duty-cycle mode LP-WUS operation is preferred. Besides, it can be considered to introduce offset between LP-WUS monitor occasion and the first PDCCH monitor occasion of the active time and PDCCH monitoring duration for Option 1-2 and Option 1-3.

What’s more, as mentioned above, Option 1-1, 1-2 and Option 1-3 can be applied before active time and inside active time, therefore, Option 1-3 can be configured together with Option 1-1 or Option 1-2-2.

Proposal 4: For Option 1-2 and Option 1-3, duty-cycled LP-WUS operation is preferred.

Proposal 5: For Option 1-2 and Option 1-3, RAN2 can discuss whether to introduce the following parameters:

- Offset between LP-WUS monitor occasion and the first PDCCH monitor occasion of the active time

- PDCCH monitoring duration

- New timers for the MR’s active time triggered by LP-WUS

Proposal 6: Option 1-3 can be configured together with Option 1-1 or Option 1-2.

  - Configuration and (de)activation of LP-WUS monitoring

In RAN1#116 meeting, gNB RRC signalling based LP-WUS enable/disable is agreed and whether additional options on top of it is for further study [4].

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Considering the motivation of using LP-WUS in CONNECTED mode is to reduce the PDCCH monitoring power consumption while guarantee the latency performance, it is not necessary to adapt the on/off of LP-WUS monitoring frequently based on fast L1/L2 enable/disable signalling. Besides, UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring. For example, when a UE is in lower battery status, UE can report to gNB that it prefers to activate the LP-WUS monitoring function. As to the condition based activation/deactivation mechanism, similar performance can be achieved by proper network configuration based on UE reported information, including UE assistant information and SR/BSR. Therefore, in our understanding, Option 3 and Option 4 can be achieved by network implementation.

Proposal 7: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 8: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 9: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 10: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

  - Co-existence with other power saving mechanisms

Co-existence with DCP

In last meeting, RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE, while Option 1-2 and Option 1-3 is more flexible to trigger PDCCH monitoring in drx off period or within DRX active time, therefore, there’s no need to configure DCP and LP-WUS for RRC_CONNECTED PDCCH monitoring simultaneously.

Proposal 11: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

Co-existence with PDCCH monitoring skipping

For PDCCH skipping/SSSG switching, we think it can be combined with LP-WUS options on the table. For example, PDCCH skipping can be used as the indication of stop of PDCCH monitoring as in Figure 1. That is if WUR detects the LP-WUS, MR starts to monitor PDCCH until receives the PDCCH skipping indication.

Figure 5. LP-WUS coexistence with PDCCH skipping

Proposal 12: LP-WUS procedure and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously.

- Conclusions

In this contribution, we discuss the LP-WUS operation in RRC_CONNECTED mode, and the following proposal and observations are made.

Proposals:

Proposal 1: Configuration of Option 1-1 can include the following contents:

- Offset to indicate the UE the LP-WUS monitoring occasion before start of  the drx-onDurationTimer;

- UE’s starting position in the LP sequence;

- UE’s default behaviour when LP-WUS is not detected outside active time;

- UE’s default behaviour on periodic L1-RSRP and CSI reporting when the drx-onDurationTimer is not started.

Proposal 2: Whether LP-WUS related LS sequence configuration is included in Option 1-1 specific configuration is decided in RAN1.

Proposal 3: RAN2 is asked to suspend the discussion on LP-WUS operations for UE with CA configured and wait for RAN1’s progress.

Proposal 4: For Option 1-2 and Option 1-3, duty-cycled LP-WUS operation is preferred.

Proposal 5: For Option 1-2 and Option 1-3, RAN2 can discuss whether to introduce the following parameters:

- Offset between LP-WUS monitor occasion and the first PDCCH monitor occasion of the active time

- PDCCH monitoring duration

- New timers for the MR’s active time triggered by LP-WUS

Proposal 6: Option 1-3 can be configured together with Option 1-1 or Option 1-2.

Proposal 7: RRC signalling based approach is used to enable LP-WUS monitoring in RRC_CONNECTED.

Proposal 8: Once the UE receives LP-WUS monitoring configuration, it start LP-WUS monitoring based on the configuration.

Proposal 9: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring. LP-WUS monitoring is only enabled(activated) or disable(deactivated) by RRC signalling.

Proposal 10: UE assistance information can be introduced to assist gNB to enable/disable the LP-WUS monitoring.

Proposal 11: There’s no need to configure or activate DCP and any LP-WUS options for RRC_CONNECTED PDCCH monitoring simultaneously.

Proposal 12: LP-WUS procedure and Rel-17 PDCCH skipping/SSSG switching can be configured or activated to one UE simultaneously.

Observations:

Observation 1: Option 1-2 provides more PDCCH monitoring or data transmission opportunities and less transmission latency, and it is suitable for the services without fixed periodicity, e.g. FTP services.

Observation 2: Option 1-3 is suitable for XR service, which the service is periodic, but there is jitter for each packet, and LP-WUS can be used to jitter handling in this case.

- References

- Chair notes RAN2#126;

- TS 38.331; Radio Resource Control (RRC) protocol specification; (Release 18);

- TR 38.869; Study on low-power wake up signal and receiver for NR; (Release 18);

- Chair notes RAN1#116.