3GPP TSG-RAN WG2 Meeting #127	R2-2406979

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.2

Source:	CMCC

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion

Introduction

In the RAN#102 meeting, the new WI of enhancements of network energy savings for NR was set up [1], including on-demand SSB Scell operation, on-demand SIB1 for UE in idle/inactive mode and the adaptation of common signal/channel transmission. The related objective of on-demand SSB is listed below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we provide our views on on-demand SSB Scell operation for UEs in RRC_CONNECTED mode configured with CA.

Discussion

To enable reasonable UE battery consumption when CA is configured, an activation/deactivation mechanism of Cells is supported, therefore, for a UE’s SCell of a CA cell group, there could be several state: activated state, deactivated state and during SCell activation procedure. SCell can be activated or deactivated by MAC CE from PCell.

In legacy, for a deactivated SCell, taking FR1 as an example, if during the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command the UE has sent a valid measurement report for the SCell being activated and the SSB measured remains detectable, SCell is a known [3], otherwise SCell is unknown. And the activation delay for known SCell and unknown SCell is different, the activation delay of unknown SCell is longer than that of known SCell based on TS 38.133 [2].

Observation 1: In legacy, the activation delay of unknown Scell is longer than that of known SCell.

When it comes on-demand SSB of a SCell, it means on-demand SSB can be activated in different SCell state.

For the discussion of on-demand SSB Scell operation, the following agreements about scenarios were made about scenarios in RAN1#116 meeting [3]:

Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission

And in RAN1#116bis meeting [4], the scenarios are further clarified:

Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Based on the Scenarios identified, RAN1 further discussed the way to active on-demand SSB in RAN1#117[5]:

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable.

In scenario#2, SCell is configured to a UE but the UE hasn’t received SCell activation command, the NES SCell may be an unknown SCell at this stage. Once the on-demand SSB is triggered, with proper configuration, it can turn an unknown SCell to a known SCell, with less activation latency. Besides, with on-demand SSB triggered, the network may have more L3 measurement results, and avoid blind SCell activation. In this scenario, a separate DL indication of on-demand SSB transmission is needed.

Observation 2: On-demand SSB triggered before SCell activation may have the benefits of reducing SCell activation latency and avoiding blind SCell activation.

Proposal 1: For Scenario#2, a separate DL signalling, i.e. MAC CE from SCell activation command is needed to indicate the transmission  of on-demand SSB.

In Scenario#2A, there’s no SSB or always -on SSB with sparse periodicity on SCell  before SCell activation command, therefore, the SCell may be an unknown SCell. And the UE expects that on-demand SSB transmission starts at the time UE receives SCell activation signalling. Compared with Scenario#2, the SCell cannot acquire quality of unknown SCell, and this may lead to blind SCell activation.

Observation 3: On-demand SSB triggered when UE receives SCell activation  command may lead to blind SCell activation.

Proposal 2: For Scenario#2A, SCell Activation command, including RRC and MAC CE can be enhanced to indicate on-demand SSB transmission.

For Scenario#3A and 3B, on-demand SSB is triggered after SCell activation. Since Scenario#2A is supported and the first transmission occasion (i.e. instance A) of on-demand SSB burst can be after UE received SCell activation signalling and indicated by SCell activation signalling, there is no motivation to support Scenario#3A. For Scenario#3B, it is applicable to the case that after SCell activation completion, gNB (e.g. when the traffic load/the UE number is relatively small) may decide to turn off on-demand SSB for network power saving, and on-demand SSB may be triggered again.

Proposal 3: On-demand SSB SCell operation in Scenario #3A is not supported.

For Scenario#3B and Case 1, UE may loss AGC, synchronization and beam tracking after a long period with no SSB. When gNB detects new DL or UL traffic, on-demand SSB can be indicated before data transmission for AGC, synchronization and L1-RSRP measurement. The delay of these procedure may be long, the required SSB bursts and the precision of synchronization need evaluation. There may be impact of on-demand SSB on measurement metrics of RAN4, a LS can be sent to RAN4 for further study.

For Scenario#3B and Case 2, always-on SSB has a sparse periodicity. Since UE may not perform AGC or synchronization properly based on always-on SSB, on-demand SSB can also be indicated before data transmission for AGC and synchronization. The delay of these procedure is shorter compared with Case 1.

Besides, considering different UEs may operate in different SCell states, on-demand SSB transmission for one UE in Scenario#2A will impact rate matching and UL resource validation of another UE in Scenario#3B. This is because SSB transmission is cell level but indication of on-demand SSB is UE-specific, the UE in Scenario#3B does not know about the on-demand SSB transmission triggered for other UEs. To avoid negative impact on UEs in Scenario#3B, gNB can inform UEs in Scenario#3B about the transmission of on-demand SSB for other UEs. From this perspective, on-demand SSB SCell operation in Scenario #3B shall be supported. If on-demand SSB for other UEs lasts for a short time, rate matching issue can be solved by gNB scheduling, UE-specific DCI can be adapted to indicate on-demand SSB for one UE in Scenario#3B. To further reduce the delay and signaling overhead of resuming data transmission, UE-specific DCI can indicate on-demand SSB transmission meanwhile scheduling data transmission, so that the UE can perform fine synchronization before data transmission.

Proposal 4: On-demand SSB SCell operation in Scenario #3B and Case #1/Case #2 is supported.

Proposal 5: In scenario#3B, a separate DL signalling (e.g. group common DCI or UE-specific common DCI)  from SCell activation command is needed to indicate the transmission  of on-demand SSB.

When the capacity of PCell is not enough and UE detects heavy UL traffic, SCell activation can be requested by UE, and it’s realized by UE’s SR/BSR sent to the network and SCell activation command from the network. ExisitingS Cell activation mechanism can be used as a reference for on-demand SSB triggering.

To trigger the on-demand SSB transmission in the SCell, SR/BSR can be used for implicit trigger of on-demand SSB transmission. With existing SR/BSR, there’s no indication of UE’s preference of SCell to transmit on-demand SSB, therefore, enhanced explicit WUS signaling, for example dedicated RRC message or enhanced SR can be considered for triggering on-demand SSB.

Proposal 6: The following methods can be considered for on-demand SSB transmission triggering:

- Implicit indication by SR/BSR to trigger on-demand SSB;

- Introduce explicit WUS sigalling like dedicated RRC message or enhanced SR to trigger on-demand SSB.

In SCell activation mechanism, though UE may transmit SR/BSR, it’s the gNB to make the final decision which SCell is activated and indicate to the UE in SCell activation command. Similar, it’s better to have an gNB indication/confirmation to the UE to indicate which SCell’s SSB is activated, and this also helps to the UE’s measurement on that SCell.

Proposal 7: It’s gNB decision whether on-demand SSB is transmitted in a SCell.

Conclusions

In this contribution, we discussed the applicable scenario, of on-demand SSB in SCell for connected UEs, and the following proposals and observations are made.

Proposals:

Proposal 1: For Scenario#2, a separate DL signalling, i.e. MAC CE from SCell activation command is needed to indicate the transmission  of on-demand SSB.

Proposal 2: For Scenario#2A, SCell Activation command, including RRC and MAC CE can be enhanced to indicate on-demand SSB transmission.

Proposal 3: On-demand SSB SCell operation in Scenario #3A is not supported.

Proposal 4: On-demand SSB SCell operation in Scenario #3B and Case #1/Case #2 is supported.

Proposal 5: In scenario#3B, a separate DL signalling (e.g. group common DCI or UE-specific common DCI)  from SCell activation command is needed to indicate the transmission  of on-demand SSB.

Proposal 6: The following methods can be considered for on-demand SSB transmission triggering:

- Implicit indication by SR/BSR to trigger on-demand SSB;

- Introduce explicit WUS signalling like dedicated RRC message or enhanced SR to trigger on-demand SSB.

Proposal 7: It’s gNB decision whether on-demand SSB is transmitted in a SCell.

Observations:

Observation 1: In legacy, the activation delay of unknown Scell is longer than that of known SCell.

Observation 2: On-demand SSB triggered before SCell activation may have the benefits of reducing SCell activation latency and avoiding blind SCell activation.

Observation 3: On-demand SSB triggered when UE receives SCell activation  command may lead to blind SCell activation.

References

- RP-240170 Revised WID for Enhancements of network energy savings for NR; Ericsson;

- 3GPP TS 38.133 V18.4.0; Requirements for support of radio resource management (Release 18);

- Report of RAN1#116 meeting;

- Report of RAN1#116bis meeting;

- Report of RAN1#117 meeting.