3GPP TSG-RAN WG2 Meeting #127	R2-2406980

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.3

Source:	CMCC

Title:	Discussion on on-demand SIB1

Document for:	Discussion

Introduction

In RAN2#126 meeting, the scenario of on-demand SIB1 and the procedure to request SIB1 were discussed, followings are the related agreements and FFS [1]:

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

In this contribution, we provide our views on the scenario of on-demand SIB1 and related procedures.

Discussion

Scenario discussion priority

In RAN2#125bis meeting, RAN2 agreed to start with scenario 1a (Cell A SIB assisted intra-cell WUS, and WUS and SIB1 is sent to/from NES cell), while other scenarios are not excluded. And in last RAN2 meeting, RAN2 agreed to wait for RAN1’s progress whether to support scenario 3. Following is the agreements made in RAN1#117 meeting [2]:

Agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

For the cases identified in RAN1 is defined as following [3]:

Agreement For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A

Based on RAN1’s agreements,RAN1 has consensus with RAN2 on the priority on the scenario that on-demand SIB1 acquisition with NES cell and Cell A involved (Scenario 1a identified by RAN2). And the scenario that only Cell A involved on-demand SIB1 acquisition procedure is prioritized, too. But Scenario 3 identified by RAN2 with only  NES cell involved is not prioritized in some sense. To avoid non-effective discussion, RAN 2 should keep the same priority on the scenarios for on-demand SIB1 acquisition as RAN1.

Observation 1: RAN1 prioritize the scenarios to acquire on-demand SIB1 with only Cell A involved and with both Cell A and NES cell involved.

Proposa1 1: Discussion on Scenario 3 with only NES cell involved for on-demand SIB1 acquisition is deprioritized in RAN2.

Proposal 2: Discussion on the scenario that only Cell A involved for on-demand SIB1 acquisition (Case 3 identified by RAN1) is prioritized in RAN2 as RAN1 does.

Scenario 1a

Based on the last two meetings’ agreements, the WUS configuration is associated with PCI and frequency of a NES Cell to enable UEs identify for which cell the WUS configuration is used to acquire on-demand SIB1. As Msg1 based approach is agreed, therefore, some PRACH resources will be reserved for on-demand SIB1 by the NES, however, it’s not clear whether one WUS configuration can be associated with multiple NES cells.

For the scenario that more than one Cell A may provide configuration for the same NES cell, one WUS configuration associated with multiple NES cell will reduce the signalling overhead of Cell A, and it has no effect on on-demand SIB1 acquisition for NES UE.

For the scenario that the same Cell A may assist more than one NES Cells, the signalling overhead of Cell A is reduced as the former case, but since the same WUS configuration is used, it may result in two adjacent NES cells receive WUS from the same UE and transmit on-demand SIB1, and this may reduce the NES gain. But in our opinion, it can be avoided by proper configuration, i.e., one WUS configuration is not shared by two adjacent NES cells.

Based on the discussion above, we think WUS configuration can be area specific configured.

Proposal 3: WUS configuration can be area specific or cell specific configured.

Currently, RAN2 only agreed to support Msg1-based on-demand SIB1 acquisition, while for Msg-3 based on-demand SIB1 acquisition is still FFS. In legacy, on-demand SI can be requested via Msg1 based procedure or Msg3 based procedure. Though Msg3 based procedure is to serve the case that the same RACH resource is allocated for several SI request, and Msg3 based procedure can work as supplement to indicate the actual SI requested by the UE. When it comes to on-demand SIB1, though it is only used for the single informantion_SIB1, if only Msg1 based on-demand procedure is supported for SIB1 acquisition, the network needs to allocate specific resources for on-demand SIB1 requesting all the time, which introduce extra limitation to the network . Therefore, Msg 3 based on-demand procedure can be considered for on-demand SIB1 acquisition.

Proposal 4: Msg3 based on-demand procedure can be supported for on-demand SIB1 acquisition procedure.

Besides, in last meeting, only cell reselection to NES-cell with SIB1 on-demand transmission is discussed. It is if UE chooses the NES cell using legacy intra-frequency/inter-frequency cell re-selection procedure (as baseline), the UE triggers WUS transmission, and then, if the UE acquire SIB1 successfully and if the NES cell is a suitable cell, the UE camps on the NES cell and then monitor paging and SI updating as legacy. But for cell selection, it’s not clear.

Regarding the issue on cell selection procedure with NES cell, in our understanding, allowing UE select to NES during cell selection procedure (especially for initial cell selection case) may result in significant extra delay, as is shown in Figure 1.

Figure 1. An example for extra delay on OD-SIB1 procedure during cell selection

Simultaneously, during the previous discussion on both RAN1 and RAN2, the latency of the initial cell selection procedure is considered to be a critical issue. Therefore, from this perspective, it is recommended that R19 NES UE shall not select to the NES cell during cell selection procedure (especially for initial cell selection case), so that it can quickly switch to a cell that is broadcasting SIB1 and the latency of cell-selection can be ensured.

Proposal 5: Support R19 NES UE not to select the NES cell during cell selection procedure (especially for initial cell selection case).

Regarding the discussion on cell reselection procedure with NES cell, it is agreed to reuse the legacy cell reselection procedure as baseline. From this perspective, R19 NES UE can request the SIB1 of all the configured NES cells according to its needs. However, if the number of UE or the number of NES cells configured in a single UE is relatively large, a significant transmission overhead will be caused by the legacy cell reselection procedure, which can result in a large extra energy consumption from both gNB and UE side. Thus, the enhancement of legacy cell reselection procedure can be considered, e.g., the R19 NES UE can preferentially reselect to non-NES cell that meet the criteria of reselection.

Proposal 6: Enhancement of legacy cell reselection procedure can be considered, e.g., the R19 NES UE can preferentially reselect to non-NES cell that meet the criteria of reselection.

To avoid UE access to a NES cell in the initial cell selection, and Besides, UE should identify whether a cell is NES cell. Besides, RAN2 agreed that if the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. In this case, UE also need to differentiate NES cell and non-NES cell. As to the mechanism, since it is discussed in RAN1, RAN2 can wait for RAN1’s progress to avoid duplicate discussion.

Proposal 7:UE should be indicated whether a cell is a NES cell or not, and RAN2 can wait for RAN1’s further progress on the indication mechanism.

Only Cell A involved scenario (Case 3 in RAN1)

In this scenario, UE acquires WUS configuration of NES cell from Cell A, sends WUS to Cell A and acquires on-demand SIB from Cell A. With this way, the NES gain of NES cell can be ensured, but the signalling overhead and power consumption of Cell A may be increased. And some agreements made for Scenario 1a can be reused here:

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Proposal 8: For the scenario of acquiring on-demand SIB1 with only Cell A involved, the following agreements made for Scenario 1a can be reused:

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Conclusions

In this contribution, we analyse the scenario and procedure of on-demand SIB1 acquisition, following are the proposals and observations:

Observation 1: RAN1 prioritize the scenarios to acquire on-demand SIB1 with only Cell A involved and with both Cell A and NES cell involved.

Proposa1 1: Discussion on Scenario 3 with only NES cell involved for on-demand SIB1 acquisition is deprioritized in RAN2.

Proposal 2: Discussion on the scenario that only Cell A involved for on-demand SIB1 acquisition (Case 3 identified by RAN1) is prioritized in RAN2 as RAN1 does.

Proposal 3: WUS configuration can be area specific or cell specific configured.

Proposal 4: Msg3 based on-demand procedure can be supported for on-demand SIB1 acquisition procedure.

Proposal 5: Support R19 NES UE not to select the NES cell during cell selection procedure (especially for initial cell selection case).

Proposal 6: Enhancement of legacy cell reselection procedure can be considered, e.g., the R19 NES UE can preferentially reselect to non-NES cell that meet the criteria of reselection.

Proposal 7:UE should be indicated whether a cell is a NES cell or not, and RAN2 can wait for RAN1’s further progress on the indication mechanism.

Proposal 8: For the scenario of acquiring on-demand SIB1 with only Cell A involved, the following agreements made for Scenario 1a can be reused:

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

References

- Report of RAN2 #126 meeting;

- Report of RAN1 #117 meeting.