3GPP TSG-RAN WG2 Meeting #127	R2-2406981

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.4

Source:	CMCC

Title:	Discussion on adaptation of common signal/channel transmissions

Document for:	Discussion

Introduction

In the RAN#102 meeting, the new WI of enhancements of network energy savings for NR was set up [1], , one of the objectives is to adapt the common signal/channel transmission, which has been discussed during R18 SI and proved to be beneficial for energy saving, the related objective is as following,

3. Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In the RAN2#125bis meeting, only Paging occasion adaptation is discussed, and following are the agreements [2]:

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, we provide our view on common signalling adaptation.

Discussion

Adaptation of Paging occasion

In last meeting, RAN2 agreed the following two direction for the adaptation of paging occasion in time domain:

a) bundle paging frames

b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

Paging Frame bundling

In 5G system, a PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent due to multiple beams, and the PDCCH monitoring occasions for paging are determined according to pagingSearchSpace. One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO, therefore, a PF can consist of several radio frames.

Observation 1: A PO can consist of multiple time slots due to beam sweeping.

Observation 2: A PF can consist of several radio frames due to beam sweeping and multiple POs in a PF.

Therefore, it should be discuss for PF bundling, there could be two possible understanding as shown in Figure 1:

Case 1: the Nth bundled PF may start from the last PO of (N-1)th bundled PF;

Case 2:the Nth bundled PF starts from the next radio frame of (N-1)the bundled PF.

Figure 1 Potential PF bundling mechanisms

Compared the two cases, Case 1 may achieve more energy saving gain, since with the same PF/PO configuration, the network can transmit Paging message with less time, and longer sleep can be expected.

Proposal 1: RAN2 should clarify which case is the PF bundling discussed in Rel-19 NES:

Case 1: the Nth bundled PF may start from the last PO of (N-1)th bundled PF;

Case 2:the Nth bundled PF starts from the next radio frame of (N-1)the bundled PF.

If PF bundling is supported or configured in the network, new PO/PF calculation formular is needed. And for Rel-19 NES UE, it needs to know whether to use legacy PO/PF calculation formular or new formular, and it can be achieved by network indication in system information.

Proposal 2: Indication in system information can be introduced for the UE to identify whether to use legacy PO/PF calculation formular or Rel-19 formular.

Compatibility of legacy RRC_IDLE/RRC_INACTIVE UE

To minimize the impact of paging occasion adaptation to legacy UEs, there’s two options on the table:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

For Option1, a barring mechanism should be designed for barring legacy UE only, while Rel-19 NES UEs can camp on the NES cell with paging adaptation. In Rel-18 NES discussion, a new IE cellBarredNES is introduced in SIB1 for the similar consideration. The same way can be reused, a new IE to not bar Rel-19 NES UEs can be introduced in SIB1.

Besides, there’re several NES techniques under discussion in Rel-19, and the other Rel-19 NES techniques may have impact to legacy UEs，whether a unified barring mechanism is needed should be further discussed.

Proposal 3: A new IE to not bar Rel-19 NES UEs can be introduced in SIB1, similar to Rel-18 cellBarredNES.

For Option 2, separate paging resources for legacy UEs and NES UEs is needed, and in our view, it depends on RAN1 discussion on separate  paging resources, and RAN2 can wait for RAN1’s progress on this.

Proposal 4: Whether  Separate paging resources for legacy UE and NES UE is decided by RAN1 and RAN2 can wait for RAN1’s progress.

Adaptation of SSB

For SSB adaptation in time domain, the following agreement is agreed during RAN1#116-bis meeting [3].

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized

For SSB adaptation, the first issue is whether SSB adaptation can only applicable for NCD-SSB/SSB that is not a CD-SSB while CD-SSB is always transmitted in a normal periodicity. To solve this, we perform an evaluation on NES gain for this mechanism. The settings for baseline and SSB adaptation mode are shown as below:

- Baseline:

- Both CD-SSB and corresponding SIB1 are transmitted per 20ms

- PRACH is transmitted periodically with configuration index 5 and the frequency bandwidth is 12 RB.

- NCD-SSB is transmitted per 20 ms, NCD-SSB is FDM transmitted with CD-SSB.

- NCD-SSB adaptation mode:

- Both CD-SSB and corresponding SIB1 are transmitted per 20ms

- PRACH is transmitted periodically with configuration index 5 and the frequency bandwidth is 12 RB.

- NCD-SSB is transmitted per 160 ms, NCD-SSB is FDM transmitted with CD-SSB.

- Note that the assumption for cell load is empty load, and other parameters come from SLS parameter settings in [3]. Figure 1 and Figure 2 show an example for CD-SSB/SIB1/PRACH transmission pattern in TDD and FDD structure, respectively

- Figure 1. An example for CD-SSB/SIB1/PRACH transmission pattern for baseline in TDD structure

- Figure 2. An example for CD-SSB/SIB1/PRACH transmission pattern for baseline in FDD structure

For NES gain on NCD-SSB adaptation, Table 1 shows the evaluation result for different BS type in [4]. It can be observed that only 0.7%~2.7% NES gain can be obtained for Cell A in various BS types and TDD/FDD structures. Since the benefit is relatively limited, we do not support SSB adaptation is only applicable for NCD-SSB/SSB that is not a CD-SSB while CD-SSB is always transmitted in normal periodicity.

- Table 1. NES gain on NCD-SSB adaptation for different BS type in [4]

| BS category and structure | Cat 1 BS, TDD | Cat 1 BS, FDD | Cat 2 BS, TDD | Cat 2 BS, FDD |
| --- | --- | --- | --- | --- |
| NES gain (%) | 0.7% | 1.6% | 0.7% | 2.7% |

Observation 3: For SSB adaptation that is only applicable for NCD-SSB while CD-SSB is always transmitted in normal periodicity, 0.7%~2.7% NES gain can be obtained in various BS types and TDD/FDD structures.

- Note that the assumption for cell load is empty load.

Proposal 4: Do not support SSB adaptation that is only applicable for NCD-SSB/SSB that is not a CD-SSB while a CD-SSB is always transmitted in normal periodicity.

As for R19 NES-capable UE, it may perform cell search procedure based on long-period SSB transmission with some additional requirement, e.g., adding more assumption for SSB periodicity, or relaxing the total detection time during the cell search procedure.

Note that although the agreement makes a distinction on cell properties for R19 NES-capable UE, the corresponding cell can be a PCell, SCell or camped cell for another UE.

Observation 4: Additional requirement is needed if R19 NES-capable UE can perform initial cell selection on SSB-adaptable cell. E.g., adding assumption for SSB periodicity, or relaxing the total detection time for initial cell selection.

Simultaneously, apart from the usage for initial cell selection, whether the SSB-adaptable cell can be configured as a serving cell or camped cell for both R19 NES-capable UE and legacy UE is worth noticing. To be specific:

- For R19 NES-capable UE, the SSB-adaptable cell can be configured as a serving cell (include non-CA case, PCell and SCell) or camped cell (e.g. via cell reselection/selection).

- For legacy UE, since the SSB cannot be used for system information acquisition, the SSB-adaptable cell can only be configured as a SCell.

Proposal 5: The SSB-adaptable cell can be configured as:

- A serving cell (include non-CA case, PCell and SCell) or camped cell for R19 NES-capable UE.

- A SCell for legacy UE.

Adaptation of PRACH

In last meeting, RAN2 agreed that for RACH adaptation in spatial domain, RAN2 counts on RAN1 evaluation and conclusion. Following are the time-domain related agreements made in RAN1#117 meeting:

Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

Based on the agreements above, there will be additional RACH resources is supported, therefore, RAN2 needs to study how to configure and enable the additional RACH resources.

Proposal 6: RAN2 needs to study how to configure and enable the additional RACH resources.

Besides, since two types of random access procedure are supported: 4-step RA type with MSG1 and 2-step RA type with MSGA and both types of RA procedure support contention-based random access (CBRA) and contention-free random access (CFRA), we think PRACH adaptation can be applied in the above types of RA procedure.

Proposal 7: PRACH adaptation can be applied to 4-step RA and 2-step RA.

Proposal 8: PRACH adaptation can be applied to CFRA and CBRA.

Conclusions

In this contribution, we analyse the adaption of common signal/channels, include SSB, PRACH and paging adaptation, and the following proposals and observation are made:

Proposals:

Proposal 1: RAN2 should clarify which case is the PF bundling discussed in Rel-19 NES:

Case 1: the Nth bundled PF may start from the last PO of (N-1)th bundled PF;

Case 2:the Nth bundled PF starts from the next radio frame of (N-1)the bundled PF.

Proposal 2: A new IE to not bar Rel-19 NES UEs can be introduced in SIB1, similar to Rel-18 cellBarredNES.

Proposal 3: Whether  Separate paging resources for legacy UE and NES UE is decides by RAN1 and RAN2 can wait for RAN1’s progress.

Proposal 4: Do not support SSB adaptation that is only applicable for NCD-SSB/SSB that is not a CD-SSB while a CD-SSB is always transmitted in normal periodicity.

Proposal 5: The SSB-adaptable cell can be configured as:

- A serving cell (include non-CA case, PCell and SCell) or camped cell for R19 NES-capable UE.

- A SCell for legacy UE.

Proposal 6: PRACH adaptation shall not change the spatial information for RO (mapping of the association period and association pattern period).

Proposal 7: Support the following schemes for PRACH adaptation in time domain,

- Opt 1: There is one basic prach-ConfigurationIndex configured without adaption and another prach-ConfigurationIndex configured for adaption;

- Opt 2: Adaption between two prach-ConfigurationIndex;

- Opt 3: Introduce a Cell DRX-wise cycle for PRACH

Observations:

Observation 1: A PO can consist of multiple time slots due to beam sweeping.

Observation 2: A PF can consist of several radio frames due to beam sweeping and multiple POs in a PF.

Observation 3: For SSB adaptation that is only applicable for NCD-SSB while CD-SSB is always transmitted in normal periodicity, 0.7%~2.7% NES gain can be obtained in various BS types and TDD/FDD structures.

- Note that the assumption for cell load is empty load.

References

- RP-240170 Revised WID for Enhancements of network energy savings for NR; Ericsson;

- Report of RAN2#125bis meeting;

- Report of RAN1#116bis meeting;

- 3GPP TR 38.864, Study on network energy savings for NR.