3GPP TSG-RAN WG2 Meeting #127	R2-2406982

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#126 meeting, the scenario, procedures of inter-CU LTM were discussed, followings are the related agreements [1]:

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In this paper, we further discuss the open issues of NR SA scenario and potential issues of NSA case.

- Discussion

  - Security information updating

In RAN2’s discussion on security information updating, there’re several approaches and RAN2 sent LS to SA3. Followings are the reply[2] and new WID from SA3[3]:

LS reply:

# Overall description SA3 thanks RAN2 for their LS R2-2404037 on security handling for inter-CU LTM in non-DC cases. SA3 discussed the proposed solutions in the LS but are not yet in a position to provide any feedback. SA3 agreed the attached WID (S3-242401) to work on this topic and will provide feedback to RAN2 as SA3 makes any progress. SA3 kindly requests RAN2 to keep SA3 informed of any progress that RAN2 makes on this topic. # 2 Actions To RAN WG2: ACTION: SA3 kindly asks 3GPP RAN2 to take the above information into consideration

New WID on security aspects of NR mobility enhancement:

# Objective The objective of this feature is to support the inter-CU LTM in 5G systems. This work item covers: WT1: Security procedure to support inter-CU LTM: - Case 1: When CU is acting as MN when DC is not configured - Case 2: NR-DC is configured and CU is acting as SN and MCG is unchanged - Case 3: NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Subsequent LTM mobility procedures between cell switches NOTE 1: SA3 work depends on the work progress in RAN WGs. NOTE 2: SA3 Work Tasks of this WID may be added/deleted/updated based on the revision to the RAN WID (NR mobility enhancements Phase 4).

Since RAN2 has informed SA3 the potential solutions proposed by RAN2,  and the final decision depends on SA3, for inter-CU LTM security updating issues, RAN2 continues to pending the discussion and waits for SA3’s progress.

Proposal 1:RAN2 waits for SA3’s further progress on security issues.

  - RRC configuration of inter-CU LTM

In last meeting, it was agreed that The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. In R18, the RRC configuration of a intra-CU LTM candidates are included in the IE LTM-config, composed of reference configuration, candidate cell’s configuration, L1 measurement configuration and TCI-state configuration, etc.

    - Reference configuration

For reference configuration, there’s only one for all candidate cells of intra-CU LTM, considering that all the candidates belong to the same CU, and it may be easy to achieve a common configuration as a reference. Except Subsequent-LTM, reference configuration is also introduced for Rel-18 S-CPAC, in which only one reference configuration is used for candidate cells belonging to different candidate SCGs. In S-CPAC, The MN may select one of the candidate SN(s) and requests providing the SCG reference configuration as part of the SN Addition procedure. Once obtained, the MN provides the SCG reference configuration to other candidate SN(s)[4].

Observation 1: Only one reference configuration is used in Rel-18 intra-CU LTM and S-CPAC.

When it comes to R19 inter-CU LTM, the LTM candidate cells configured to the UE may belong to different CUs, which is similar to Rel-18 S-CPAC. But differently, in CPAC case, MN is not changed as an anchor node, though the reference configuration is provided by candidate SN, the reference configuration may be provided based on the source SN configuration, while for inter-CU LTM, there’s no such anchor node.

If only one reference configuration as Rel-18 is inherited, the reference may generated by the source gNB-CU, or requested from one of the candidate gNB-CUs. With the former way, the commonality of the reference configuration among candidate cells may be ensured via heavy Xn signalling between the source gNB-CU and candidate gNB CUs, while with the latter way, the reference configuration may be not suitable to other candidate cells.

Observation 2: If only one reference configuration is provided by the source gNB-CU, to ensure the commonality, heavy Xn signalling overhead cannot avoid.

Observation 3: If the only one reference configuration is provided by one candidate gNB-CU, the commonality cannot be ensured.

Based on the two observations above, we prefer to use multiple reference configurations for inter-CU LTM case, but it is not to restrict that the reference configuration is per CU configured. And with multiple reference configurations provided to the UE, the association between reference configuration and candidate cell should be introduced for the UE to identify which reference configuration to be applied when it is switched to one cell.

Proposal 2 : Multiple reference configurations can be provided to UEs for inter-CU LTM.

Proposal 3: It’s not to restrict that the reference configuration is per CU configured.

Proposal 4: The association between reference configuration and candidate cell is introduced in inter-CU LTM scenario.

As to the generation or decision node of reference configuration, similar principle to Rel-18 S-CPAC can be applied. The source gNB-CU can generate one (or multiple) reference configuration(s) or request one (or multiple) candidate gNB-CU(s) to provide reference configuration(s). Once decided or obtained, the source gNB-CU provides the reference configuration to other candidate gNB-CUs. What’s more, whether all reference configurations are provided to candidate gNB-CUs or only associated reference configuration is provided can be discussed in RAN3.

Proposal 5: The source gNB-CU provides the reference configuration to candidate gNB-CU to generate delta configuration or complete configuration.

Proposal 6: Whether all reference configurations are provided to candidate gNB-CUs or only associated reference configuration is provided can be discussed in RAN3.

    - Support of mixture of inter-CU and intra-CU LTMs

In the first discussion of inter-CU LTM, RAN2 reached the consensus on UE behaviour in LTM cell switch execution phase as following [5]:

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

And RAN2 agreed to support the mixed configuration of inter-CU and intra-CU LTMs to the UEs. To perform the behaviour agreed above, the UE needs to identify whether the LTM to be executed is an intra-CU LTM or inter-CU LTM. There are several approaches to achieve this:

- Based on the enhanced MAC CE carrying security information;

- Introduce a new ID like NoResetID for UE’s identification.

For the first approach, it’s related to security key updating mechanism, which is under discussion in SA3. As replied by SA3, there’s a new WID to the  LTM related security issues, and the discussion in SA3 and RAN2 is  in parallel. To avoid RAN’s progress delayed by SA3’s input, it’s better to design RAN2 isolated solution, not affected by SA3. Therefore, we prefer to introduce a new Rel-19 ID like NoResetID for the UE to decide whether PDCP re-establishment and security key updating is needed.

Observation 4: There’s parallel discussion of LTM related issue in SA3 with RAN2, which may delay RAN2’s progress of other issues than security.

Proposal 7: Introduce a new Rel-19 ID, if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update.

    - UE-based TA measurement

In Rel-18 intra-CU LTM, except L2 handing, the UE’s execution of UE-based TA measurement is also based on the related ID comparison of the serving cell and candidate cell. In the current spec, it’s UE implementation on how to measure TA when this way is configured by the network, and there’s no restrictions on the scenarios  in which UE can be configured with UE-based TA measurement. In other words, whether to configure UE-based TA measurement to the UE is a kind of network implementation, maybe it is decided based on the prior information in the serving cell.

Observation 5: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is kind of network implementation.

When it comes to Rel-19 inter-CU LTM, we think the same principle can be applied, the serving cell can configure UE-based TA measurement to the UE. If UE cannot access to the target cell successfully, it can fallback to legacy RACH procedure.

Proposal 8: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

  - Stage 2 signalling procedure

Basic procedure has been discussed in the last two meetings, but there’re still some open issues:

- Inter-CU LTM preparation

In RAN2#126 meeting, RAN2 agreed that the preparation of inter-CU LTM configuration is initiated by the source gNB-CU. Since the UE can be configured with multiple LTM candidate cells, and it’s possible that some of the candidate cells belong to the same candidate gNB-CU. One open issue is whether  a single HO request can be used to initiate the preparation of  candidate cells belonging to the same candidate gNB-CU. Similar case can be found in CHO. In TS 38.300, it is described as “A CHO request message is sent for each candidate cell.”  We think this can be applied in inter-CU LTM.

Proposal 9: LTM request message(in HO request) is sent for each candidate cells, no matter whether they belong to the same gNB-CU or not.

- LTM configuration transfer

In Rel-18 intra-CU LTM, all candidate cells belong to the same CU, therefore, there’s no need to transfer LTM configurations from serving cell to target cell. However, it’s different for inter-CU LTM. The LTM related configuration should be shared among candidate cells, or at least the target cell. When LTM is successfully executed, Handover Success procedure over Xn is reused, the source gNB-CU will have knowledge that UE has accessed to the target gNB-CU. Therefore, the serving gNB-CU can transfer LTM configuration after it collects all candidate cells’ configuration or after UE’s success handover to the target gNB-CU.

Proposal 10: Serving gNB-CU transmits LTM configuration after it collects all candidate cells’ configuration or after UE’s success handover to the target gNB-CU.

- LTM data forwarding

With supporting of  Rel-19 inter-CU LTM, UE will be served with the different CUs after cell switch, and data forwarding between gNBs is needed. Based on the current LTM procedure, the data forwarding address is only exchanged in the LTM preparation by the initial serving gNB-CU with candidate gNB-CU s. Once LTM executed,  the actual serving gNB-CU has no knowledge of candidate gNB-CU’s data forwarding address. Therefore, data forwarding addresses information should be indicated to candidate gNB-CUs. Similar to the previous discussion, this can be performed in LTM preparation during which all candidate gNB-CUs will be indicated data forwarding address information or performed after every LTM cell switch and only the target gNB-CU is indicated.

Proposal 11: The serving gNB-CU needs to indicate data forwarding address information of candidate gNB-CUs, and this can be performed in LTM preparation phase or after every LTM cell switch.

- LTM configuration release

As we have agreed to support subsequent LTM, that’s to say UE will maintain the LTM configuration after one LTM cell switch. If needed, the network can indicate the UE to release LTM related configuration.

Proposal 12: UE maintains the LTM configuration unless it is indicated to release by the network.

  - Inter-CU LTM in NSA scenario

There’re two sub cases of inter-CU LTM in NSA scenario:

- Inter-CU LTM of MN with SN released or unchanged

- Inter-CU LTM of SN

Inter-CU LTM of MN with SN released or unchanged

Inter-CU LTM of MN with SN released or unchanged is a kind of extension of inter-CU LTM in SA scenario, both of the two cases are to switch UE’s Pcell/MCG, while the difference is that in this scenario, the UE may have a secondary cell group after cell switch. Therefore, when UE’s Pcell/MCG changes, it should be determined that whether the source SCG is kept, changed or released. In current L3 inter-node mobility with SCG, it’s the target cell to decide whether to keep, change or release the source SCG [4]. The similar principle should also be applied to Case 3. And if the target gNB decides to keep the source SCG, it should perform SN addition procedure when it is requested by the source cell and provides SCG configuration as part of LTM candidate configuration.

Proposal 13: It’s the inter-CU LTM candidate cell/gNB to decide whether to keep or release the source SCG.

Proposal 14: If the candidate gNB decides to keep the source SCG, it performs SN addition procedure once requested by the source node and provides SCG configuration as part of LTM candidate configuration.

Besides, for the source CU/MCG with SCG, if it decides to trigger the LTM cell switch procedure for a UE, it should release its SCG.

Proposal 15: Once source CU/ MCG decides to trigger the LTM cell switch procedure for a UE, it releases its SCG.

The procedure for this case is depicted in Figure 2:

Figure 2：Procedure of Inter-CU MCG LTM with SCG

Inter-CU SCG LTM

This case is to realize PsCell of SCG switch to a inter-CU/gNB cell based on L1 measurement results. In legacy L3 mobility, the similar procedure is SN change [6], which can be triggered by both MN and SN. Hence, similarly, inter-CU SCG LTM can also be initiated by MN/MCG or SN/SCG.

Proposal 16: Inter-CU SCG LTM can also be initiated by MN/MCG or SN/SCG.

Proposal 17: L3 SN change procedure can be taken as the baseline for inter-CU SCG LTM.

Inter-CU SCG LTM initiated by MN/MCG

If the inter-CU SCG LTM is initiated by MN, it’s rational that MN to initiate candidate cells request for SN switch, provide the related configuration to the UE, and send LTM cell switch command  to the UE. And these can be realized based on Rel-15 MN initiated SN change procedure, as shown in Figure 3:

Figure 3: Procedure of Inter-CU SCG LTM initiated by MN/MCG

Proposal 18: For Inter-CU SCG LTM initiated by MN, LTM configuration and  LTM cell switch command are send by MN to the UE.

Inter-CU SCG LTM initiated by SN/SCG

For Inter-CU SCG LTM initiated by MN/MCG, the procedure is initiated by SN, and MN may indicate the selected candidate PsCell(s) to SN. Therefore, both MN and SN has candidate PsCell information. Besides, both MN and SN can configure measurement to the UE, and UE only reports the related results to the node which provides configuration.

Figure 4: Procedure of Inter-CU SCG LTM initiated by SN/SCG

Observation 6: Both MN and SN have candidate PsCell information.

Observation 7: Both MN and SN can configure measurement to the UE and have related results.

Based on the analysis above, one issue should discussed is whether both MN and SN can trigger inter-CU SCG LTM or send LTM cell switch command to the UE.

Proposal 19: RAN2 to discuss whether both MN and SN can trigger inter-CU SCG LTM or send LTM cell switch command to the UE if inter-CU SCG LTM is triggered by SN.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Proposal 1:RAN2 waits for SA3’s further progress on security issues.

Observation 1: Only one reference configuration is used in Rel-18 intra-CU LTM and S-CPAC.

Observation 2: If only one reference configuration is provided by the source gNB-CU, to ensure the commonality, heavy Xn signalling overhead cannot avoid.

Observation 3: If the only one reference configuration is provided by one candidate gNB-CU, the commonality cannot be ensured.

Proposal 2 : Multiple reference configurations can be provided to UEs for inter-CU LTM.

Proposal 3: It’s not to restrict that the reference configuration is per CU configured.

Proposal 4: The association between reference configuration and candidate cell is introduced in inter-CU LTM scenario.

Proposal 5: The source gNB-CU provides the reference configuration to candidate gNB-CU to generate delta configuration or complete configuration.

Proposal 6: Whether all reference configurations are provided to candidate gNB-CUs or only associated reference configuration is provided can be discussed in RAN3.

Observation 4: There’s parallel discussion of LTM related issue in SA3 with RAN2, which may delay RAN2’s progress of other issues than security.

Proposal 7: Introduce a new Rel-19 ID, if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update.

Observation 5: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is kind of network implementation.

Proposal 8: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

Proposal 9: LTM request message(in HO request) is sent for each candidate cells, no matter whether they belong to the same gNB-CU or not.

Proposal 10: Serving gNB-CU transmits LTM configuration after it collects all candidate cells’ configuration or after UE’s success handover to the target gNB-CU.

Proposal 11: The serving gNB-CU needs to indicate data forwarding address information of candidate gNB-CUs, and this can be performed in LTM preparation phase or after every LTM cell switch.

Proposal 12: UE maintains the LTM configuration unless it is indicated to release by the network.

Proposal 13: It’s the inter-CU LTM candidate cell/gNB to decide whether to keep or release the source SCG.

Proposal 14: If the candidate gNB decides to keep the source SCG, it performs SN addition procedure once requested by the source node and provides SCG configuration as part of LTM candidate configuration.

Proposal 15: Once source CU/ MCG decides to trigger the LTM cell switch procedure for a UE, it releases its SCG.

Proposal 16: Inter-CU SCG LTM can also be initiated by MN/MCG or SN/SCG.

Proposal 17: L3 SN change procedure can be taken as the baseline for inter-CU SCG LTM.

Proposal 18: For Inter-CU SCG LTM initiated by MN, LTM configuration and  LTM cell switch command are send by MN to the UE.

Observation 6: Both MN and SN have candidate PsCell information.

Observation 7: Both MN and SN can configure measurement to the UE and have related results.

Proposal 19: RAN2 to discuss whether both MN and SN can trigger inter-CU SCG LTM or send LTM cell switch command to the UE if inter-CU SCG LTM is triggered by SN.

- References

- Chair notes RAN2#126;

- S3-242400; Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases; Source SA3; To WG2; CC WG3;

- S3-242401; New WID on security aspects of NR mobility enhancement; SA3#116; Samsung, OPPO;

- 3GPP TS 37.340 V18.0.0; Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2 (Release 18).

- Chair notes RAN2#125bis.