3GPP TSG-RAN WG2 Meeting #127	R2-2406983

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

# Agenda item:	8.11.3

# Source:	CMCC

# Title:	Discussion on SBFD related issues

# Document for:	Discussion, Decision

- Introduction

In RAN#104 meeting, the WID of SBFD is revised to support RRC_IDLE/INACTIVE/CONNECTED UEs. In this contribution, we will discuss the potential issues to support SBFD.

- Discussion

SBFD band indication

Based on RAN1’s discussion on semi-static indication of frequency locations of SBFD subbands, following is the related agreements made in RAN1#116 meeting [2]:

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

Observation 1: For RRC_CONNECTED UE, cell-specific configuration on time domain indication is supported, while cell-specific frequency domain location indication is made as working assumption.

In our understanding, frequency location of SBFD subbands is to indicate the SBFD frequency resources in the cell, and based on the revised WID, SBFD operation to support random access in SBFD symbols is also agreed to be supported by UEs RRC_IDLE/INACTIVE mode. Therefore, the time and frequency location of SBFD subbands should be indicated as cell-specific configuration. And to ensure the access of RRC_IDLE/IANCTIVE SBFD awareness UE, it should be indicted in SIB1. Furthermore, in legacy, system information can be transmitted in RRC message, and the same principle can be applied to SBFD time and frequency configuration.

Proposal 1: Cell-specific frequency domain indication of SBFD subbands should is supported to serve UEs with different RRC states.

Proposal 2: At least cell-specific time and frequency domain indication of SBFD subbands is indicated in SIB.

Proposal 3: UE dedicated RRC signalling can be used to indicate cell-specific time and frequency domain indication of SBFD band in additional.

Regarding the FFS on UE-specific configuration, the motivation of additional support of UE-specific configuration on time locations of DL/UL SBFD subbands is not clear to us, since allowing UE dynamically changing its SBFD time location is just like dynamic SBFD, which is beyond the WID scope. Furthermore, the motivation of additional support of UE-specific configuration on frequency location of UL subband is also not clear to us. Instead, additional support of UE-specific configuration on frequency location of DL subband(s) can be considered.

Considering two UEs (e.g., UE1 and UE2) may suffer from different UE-UE CLI levels due to its interference environment, their actual usable DL resources subject to tolerable UE-UE CLI are different. If both cell-specific and UE-specific subband configuration are supported, and the bandwidth of the UE-specific DL subband can be shrink to fit the UE-UE CLI level, i.e., UE-specific DL subband configuration may provide extra available DL RB for RBG-based RA. Based on the above discussion, it is preferred to consider additional support of UE-specific configuration on frequency location of DL subband(s).

Proposal 4: UE-specific configuration of the frequency location of DL subband(s) can be supported.

Access control

SBFD feature is introduced to reduce the issues as reduced coverage, increased latency and reduced capacity caused by the limited time duration for the uplink in TDD, with more uplink opportunities can be awared by SBFD-aware UEs. There’s no extra effects to legacy UEs, hence, the legacy access control mechanism can be reused for SBFD-aware UEs. And for a SBFD supporting gNB, there’s no need to restrict the access of non-SBFD aware UEs.

Proposal 5: There’s no need to introduce new access control mechanism for SBFD supporting gNBs.

Early indication and UE capability

For a SBFD-aware UE camping on a SBFD supporting gNB, it’s better to schedule the SBFD-aware UE on SBFD subbands for extra scheduling gains. Therefore, the network should recognize SBFD-aware UEs. To achieve this, it can be implemented by early indication like RedCap and UE capability.

Considering there’s two SBFD random access operation identified by RAN1 as following[2], but it’s not limited for the UE to support both two configuration options[3]. whether separate UE capability for different  RACH configuration supporting mechanism is needed can be further discussed based on progress.

RAN1#116 Agreement For random access operation for SBFD-aware UEs in RRC CONNECTED state, at least consider the following options: - Option 1: Use one single RACH configuration with possible enhancement - The ROs within UL subband in SBFD symbols can be valid for SBFD-aware UE - FFS: Further details - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration - The ROs within UL subband in SBFD symbols configured by the additional RACH configuration can be valid for SBFD-aware UE - FFS: Further details RAN1#116bis Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

Proposal 6: Early indication of SBFD-aware UE can be supported.

Proposal 7: A new capability for SBFD supporting can be introduced, but whether it is need to further clarify the capability to support different RACH configuration options can be further discussed.

Mobility

Since SBFD operation is supported by RRC_IDLE/INACTIVE/CONNECTED UEs, in our opinion, mobility procedures like cell selection/reselection needs some enhancements to ensure SBFD awareness UE to stay in SBFD supporting cells.

Proposal 8: Legacy mobility procedures may be enhanced to ensure SBFD-aware UE to stay in SBFD supporting cells.

- Conclusions

In this contribution, we discuss the issues to support SBFD operations, and the following proposal and observations are made:

Observations:

Observation 1: For RRC_CONNECTED UE, cell-specific configuration on time domain indication is supported, while cell-specific frequency domain location indication is made as working assumption.

Proposals:

Proposal 1: Cell-specific frequency domain indication of SBFD subbands should is supported to serve UEs with different RRC states.

Proposal 2: At least cell-specific time and frequency domain indication of SBFD subbands is indicated in SIB.

Proposal 3: UE dedicated RRC signalling can be used to indicate cell-specific time and frequency domain indication of SBFD band in additional.

Proposal 4: UE-specific configuration of the frequency location of DL subband(s) can be supported.

Proposal 5: There’s no need to introduce new access control mechanism for SBFD supporting gNBs.

Proposal 6: Early indication of SBFD-aware UE can be supported.

Proposal 7: A new capability for SBFD supporting can be introduced, but whether it is need to further clarify the capability to support different RACH configuration options can be further discussed.

Proposal 8: Legacy mobility procedures may be enhanced to ensure SBFD awareness UE to stay in SBFD supporting cells.

- References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD);

- Chair notes RAN1#116;

- Chair notes RAN1#116bis;