3GPP TSG-RAN WG2 Meeting #127                     	                                        R2-2406984

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.7.5

Source: 	CMCC

Title:  	Discussion on the RLC Enhancements for XR

WID/SID:	         NR_XR_Ph3-Core

Document for:	Discussion and Decision

Introduction

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In the last RAN2 #126 meeting, the RLC enhancements has been discussed and the following related agreements have been approved.

In this contribution, we provide our view on the RLC re-transmission related enhancements.

Discussion

Avoidance of unnecessary re-transmission

In the legacy RLC specification [1], the data which has exceeded the the packet delay budgets (PDB) is still retransmitted to receiving side of AM RLC entity. The AM RLC entity never gives up retransmissions till the maximum number of retransmissions reached, which is not efficient for the system.

In the last RAN2 meeting,  two approaches have been selected as the alternative options to enhance the RLC AM mechanism for avoidance of the unnecessary retransmissions:

Option 1: Tx initiated approach

Option 2: Rx initiated approach

We will discuss the details of the Tx and Rx initiated approach and compare these two options in the following.

Tx and Rx initiated approach

Option 1: Tx initiated approach

Abandoning the retransmissions of an RLC SDU or an RLC SDU segment in the transmission side of RLC AM could be triggered by the discard indications received from PDCP layer or a new timer expired.

The transmitting side of AM RLC entity sends report to the receiving side to indicate the RLC SN gap when the transmitting side abandons the retransmission of an obsolete RLC SDU or an RLC SDU segment. The RLC SN gap report may use RLC data PDU or RLC control PDU to report the sets of SN or SN range associated with the abandonded retransmissions of RLC SDUs.

When the receiving side of AM RLC entity receives the RLC SN gap report, it discards the buffered AMD PDUs or segments of AMD PDUs associated with SNs contained in RLC SN gap report. To simplify the reception procedure, the receiving side of AM RLC entity may tread the discarded AMD PDU or segment of AMD PDU as complete received AMD PDU in the reception buffer. And the State variables and timers in receiving window should be updated according to the action as when an AMD PDU is placed in the reception buffer in legacy RLC specification [1]. No new action is needed for handling the RLC SN gap report except discarding the AMD PDUs or segments of AMD PDU associated with SN contained in RLC SN gap report and the receiving side could consider the positively acknowledged associated with discarded RLC SDU when constructing the STATUS report.

The Tx initiated discard procedure for XR with short PDB is illustrated in the Figure 1.

Figure 1. Discard procedure of retransmission of an RLC SDU or an RLC SDU segment for XR with short PDB

Option 2: Rx initiated approach

As discussed by some companies, the Rx initiated approach could be triggered by a new timer. When the new timer expired, the receiving side of RLC AM entity discards the obsolete SDUs and advance the receiving window. The receiving side of RLC AM entity inform the transmitting side with the STATUS report, where the SN of abandoned SDUs is set to ACK_SN. The transmitting side stops retransmitting the SDUs discarded by the receiving side.

The comparison of Tx and Rx initiated approach

As listed above, both Tx and Rx initiated mechanism for RLC AM enhancement could work to avoid the unnecessary re-transmission. The main difference of Tx and Rx initiated approach is which entity is in charge of discovering the obsolete SDUs and triggering the discard procedure of retransmission of RLC SDUs or RLC SDU segments.

We focus on the UL traffic case in following.

In Tx initiated approach, the UE as transmitting side of RLC AM entity could directly acquire the remaining time of an SDU from the upper layer in XR service. It is more natural and accurate that transmitting side of RLC AM entity considers the remaining time of an SDU and triggers the discard procedure timely.

Observation 1: In Tx initiated approach, it is more natural and accurate that transmitting side of RLC AM entity considers the remaining time of an SDU from the upper layer in XR service and triggers the discard procedure timely.

In Rx initiated approach, the UE as transmitting side has to wait the report sent from the gNB then stop the retransmissions of obsolete SDUs based on the received report, which could bring the delay. The gNB as receiver could perceive the remaining time below a threshold from DSR but it is not accurate. If the UE receive the discard report quite early from gNB e.g. based on the inaccurate reported remaining time, and then terminate the retransmission of obsolete SDUs which may cause more results in failure.

Observation 2: In Rx initiated approach, the gNB as receiver could perceive the remaining time below a threshold from DSR but it is not accurate. If the UE receives the discard report early from gNB e.g. based on the inaccurate reported remaining time, and then terminate the retransmission of obsolete SDUs which may cause more results in failure.

In Rx initiated approach, some companies proposed that a timer may be introduced in RLC receiver based on the t-Reordering. In this way, RLC receiver needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. Thus the RLC enable to advance its receiving window accordingly while the PDCP timer t-Reordering expires and the PDCP receiving window moves.  This method seems complexity for RLC to maintain these timers between two layers (PDCP and RLC) concurrently. If the UE as transmitting side could not receive the discard report from gNB timely e.g. only based on the t-Reordering timer, the retransmissions of obsolete SDUs may not be stopped and introduce some resource waste.

Observation 3: In Rx initiated approach, RLC receiving side needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. It seems be complexity for RLC to maintain these timers between two layers (PDCP and RLC) concurrently.

Besides for Rx initiated approach, some companies proposed to introduce the interaction between the PDCP receiving side and RLC receiving side. The PDCP receiving side may indicate the obsolete PDCP SN to the RLC receiving side, and RLC receiving abandons these SDUs based on the PDCP SN.  This method is under the prerequisite that the RLC could fetch the PDCP SN directly which may not compliance with current specification.

Therefore the Tx initiated approach may be more accurate and simple to avoid the unnecessary re-transmission in RLC AM mechanism.

Proposal 1: RAN2 to adopt the Tx initiated approach as RLC AM enhancement to avoid the unnecessary re-transmission.

Proposal 2: Abandoning the retransmissions of an RLC SDU or an RLC SDU segment in the transmission side of RLC AM could be triggered by the indications received from PDCP layer or a new timer expired.

Proposal 3: The SN gap report from transmitting side could be used to notify the receiving RLC side for the obsolete SDUs. The RLC SN gap report may include the indications for the sets of SN or possibly SN range associated with the abandoned retransmission of RLC SDUs or RLC SDU segments.

Proposal 4: When the receiving side of AM RLC entity receives the RLC SN gap report, it discards the buffered AMD PDUs or segment of AMD PDUs associated with SNs contained in RLC SN gap reporting and considers the positively acknowledged associated with discarded RLC SDU when constructing the STATUS report.

The RLC transmitting side may receive the indication from upper layer (e.g. PDCP) when these two timer discardTimer or discardTimerForLowImportance has expired in PDCP [3].  The RLC AM entity could abandoned a particular retransmissions of RLC SDU based on the discard indication and the maximum number of retransmissions may not be reached.  The indication from RLC to upper layer indicates the RLC failure (max retransmission has been reached), which may lead Radio Link Failure (RLF) and bring the interruption [2]. Therefore the indication from RLC to upper layer, which indicates max retransmission has been reached as RLC failure, shall not be triggered due to the retransmissions discarding.

Proposal 5: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Timely RLC retransmission

In the last meeting,  the following solutions were proposed in RAN2 to achieve the timely retransmissions.

- Option 1: Retransmission based on enhanced status report

- Option 2: Retransmission based on enhanced polling

- Option 3: Autonomous retransmission

Solutions for timely RLC retransmission

- Option 1: Retransmission based on enhanced status report

In RLC specification [1], when the RLC receiving side detects the transmission failure of an AMD PDU, it may delay trigger the status report, e.g.waiting the timer t-Reassembly expires, to ensure HARQ reordering or delay transmit the status report, e.g.waiting the timer t-StatusProhibit expires, to ensure not frequently transmit the status report.

If a new timer for XR service set for early transmission of status report, which may not wait for the t-Reassembly expires, it may achieve the similar effort like blind early retransmission. It seems be unnecessary to set another duplicated timer except t-Reassembly for the status report trigger mechanism in order to ensure the timely RLC retransmission. For XR service, the data with low PDB could be transmitted in the particular RLC entity/LCH and it could depend on the network to configure timers of RLC entity properly, e.g. set smaller value for these timers e.g. t-Reassembly  and t-StatusProhibit  to reduce the delay caused by transmitting status report.

Proposal 6: For XR service, the XR data with low PDB could be transmitted in the particular RLC entity/LCH and it could depend on the network to configure the timers t-Reassembly  and t-StatusProhibit properly to reduce the delay caused by transmitting status report.

- Option 2: Retransmission based on enhanced polling

The polling also could be delay triggered, e.g. waiting for the number of transmitted RLC SDUs without polling is larger than the pollPDU or the total data volume of transmitted RLC SDUs without polling is larger than pollByte, or delay transmitted, e.g. waiting the timer t-PollRetransmit expires, to ensure not frequently retransmit the polling.

In [4] as below, the timer t-PollRetransmit is used by the transmitting side of an AM RLC entity in order to retransmit a poll. The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling. The additional values could be introduced for short PBD of XR traffic, including [1,2,3,4]in milliseconds value, which is simple way to implement the timely retransmission.

T-PollRetransmit ::= ENUMERATED { ms1,ms2,ms3,ms4,ms5, ms10, ms15, ms20, ms25, ms30, ms35, ms40, ms45, ms50, ms55, ms60, ms65, ms70, ms75, ms80, ms85, ms90, ms95, ms100, ms105, ms110, ms115, ms120, ms125, ms130, ms135, ms140, ms145, ms150, ms155, ms160, ms165, ms170, ms175, ms180, ms185, ms190, ms195, ms200, ms205, ms210, ms215, ms220, ms225, ms230, ms235, ms240, ms245, ms250, ms300, ms350, ms400, ms450, ms500, ms800, ms1000, ms2000, ms4000, ms1-v1610, ms2-v1610, ms3-v1610, ms4-v1610, spare1}

Observation 4: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 7: The additional values could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

- Option 3: Autonomous retransmission

Early retransmission could be triggered even the results of previous transmission or retransmission of RLC SDUs or RLC SDU segments are not received from the peer AM RLC entity.  For RLC transmission entity,  the following mechanism could be considered to trigger the early retransmission. When the new trigger is received, the RLC PDUs or the segments of the RLC PDU could be early retransmitted to reduce the delay of retransmissions of RLC SDUs and RLC SDU segments.

- Option 1: A new indication from PDCP could be used to indicate the remaining short PDB is below a threshold.

- Option 2: A new timer could be introduced associated with a particular RLC PDU to trigger the retransmission action base on the remaining short PDB below a threshold.

- Option 3: A new indication from MAC to indicate the reception failure of a particular RLC PDU to trigger the retransmission action.

- Proposal 8: The following mechanism could be considered to trigger the early retransmission:

- Option 1: A new indication from PDCP could be used to indicate that the remaining short PDB of RLC SDU is below a threshold to trigger the retransmission action.

- Option 2: A new timer base on the remaining short PDB could be introduced associated with a particular RLC SDU to trigger the retransmission action.

- Option 3: A new indication from MAC to indicate the reception failure of a particular RLC PDU to trigger the retransmission action.

Conclusion

In this contribution, we discussed the RLC enhancements related issues. We propose:

Observation 1: In Tx initiated approach, it is more natural and accurate that transmitting side of RLC AM entity considers the remaining time of an SDU from the upper layer in XR service and triggers the discard procedure timely.

Observation 2: In Rx initiated approach, the gNB as receiver could perceive the remaining time below a threshold from DSR but it is not accurate. If the UE receives the discard report early from gNB e.g. based on the inaccurate reported remaining time, and then terminate the retransmission of obsolete SDUs which may cause more results in failure.

Observation 3: In Rx initiated approach, RLC receiving side needs to monitor the status of PDCP timer t-Reordering and start a new timer in RLC accordingly. It seems be complexity for RLC to maintain these timers between two layers (PDCP and RLC) concurrently.

Proposal 1: RAN2 to adopt the Tx initiated approach as RLC AM enhancement to avoid the unnecessary re-transmission.

Proposal 2: Abandoning the retransmissions of an RLC SDU or an RLC SDU segment in the transmission side of RLC AM could be triggered by the indications received from PDCP layer or a new timer expired.

Proposal 3: The SN gap report from transmitting side could be used to notify the receiving RLC side for the obsolete SDUs. The RLC SN gap report may include the indications for the sets of SN or possibly SN range associated with the abandoned retransmission of RLC SDUs or RLC SDU segments.

Proposal 4: When the receiving side of AM RLC entity receives the RLC SN gap report, it discards the buffered AMD PDUs or segment of AMD PDUs associated with SNs contained in RLC SN gap reporting and considers the positively acknowledged associated with discarded RLC SDU when constructing the STATUS report.

Proposal 5: The indication for RLC failure from RLC to upper layer shall not be triggered due to the retransmissions discarding.

Proposal 6: For XR service, the XR data with low PDB could be transmitted in the particular RLC entity/LCH and it could depend on the network to configure the timers t-Reassembly  and t-StatusProhibit properly to reduce the delay caused by transmitting status report.

Observation 4: The typical end-to-end latency requirement for the Mobile Metaverse for immersive gaming and live shows is in the range of  [5~20] ms. Considering the short PDB, the value below 5ms for the timer t-PollRetransmit should be added to reduce the delay of retransmitting the polling.

Proposal 7: The additional values could be introduced for short PBD of XR traffic, including [1,2,3,4] in milliseconds value.

- Proposal 8: The following mechanism could be considered to trigger the early retransmission:

- Option 1: A new indication from PDCP could be used to indicate that the remaining short PDB of RLC SDU is below a threshold to trigger the retransmission action.

- Option 2: A new timer base on the remaining short PDB could be introduced associated with a particular RLC SDU to trigger the retransmission action.

- Option 3: A new indication from MAC to indicate the reception failure of a particular RLC PDU to trigger the retransmission action.

Reference

- 3GPP TS 38.322 V18.0.0 (2023-12), Radio Link Control (RLC) protocol specification (Release 18).

- 3GPP TS 38.300 V18.1.0 (2024-03), NR and NG-RAN Overall Description;Stage 2(Release 18).

- 3GPP TS 38.323 V18.1.0 (2024-03), Packet Data Convergence Protocol (PDCP) specification (Release 18).

- 3GPP TS 38.331 V18.1.0 (2024-03),Radio Resource Control (RRC) protocol specification(Release 18)