3GPP TSG-RAN WG2 Meeting #127	R2-2406986

Maastricht, Negherlands, 19 - 23 August, 2024

Agenda Item:	8.10.5

Source:	CMCC, CATT, Ericsson, ZTE, Huawei, HiSilicon

Title:	MHI Enhancement for SCG Deactivation/Activation

WID: 	NR_ENDC_SON_MDT_Ph4-Core

Document for:	Discussion

# 1 Introduction

In last RAN2#126 meeting, potential MHI Enhancement for SCG Deactivation/Activation was discussed but no agreements achieved [1]:

R2-2405019 MHI Enhancement for SCG Deactivation/Activation CMCC, CATT, Ericsson, ZTE, Huawei, HiSilicon discussion Rel-19 NR_ENDC_SON_MDT_Ph4-Core - QC thinks that the NW knows how long time the SCG was activated. Samsung wonders what the NW does with this info. CMCC explains that if the time of activation is low, the NW could have skipped configuring the SCG completely. Samsung thinks the NW knows this without a UE report. CMCC thinks the NW report is optional. Nokia agrees with Samsung. Huawei highlights that we are talking about a mobility case and with mobility the NW cannot easily know. CATT thinks that the NW can change how much it activates/deactivates the SCG based on this info. QC, Samsung, and Nokia are not convinced of the use case. Postponed

The proponents deemed that UE needs to report the information of SCG activation/deactivation to help the network understand the actual utilization of dual connectivity and further optimize the configuration of dual connectivity. However, opponents insisted that the NW knows related information and UE’s reporting was unnecessary.

In this contribution, we further discuss the necessity of MHI Enhancement for SCG Deactivation/ Activation and provide our proposals.

# 2 Discussion

SCG activation and deactivation is introduced in Rel-17 to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

On the other hand, the Mobility History Information (MHI) for Standalone has been supported in Rel-16, where UE records the mobility history information of maximum of 16 most recently visited primary cells or time spent in any cell selection state and/or camped on any cell state in NR or E-UTRA. Then in Rel-17, MHI enhancement for Dual Connectivity is introduced, where the mobility history information of maxPSCellHistory most recently visited primary secondary cell group cells and time spent in the PSCell across all the primary cells are included. The most recently visited cell is stored first in the list. The list includes cells visited in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states for NR and RRC_IDLE and RRC_CONNECTED for E-UTRA.

According to the information that UE reported in MobilityHistoryReport, i.e., the visited cell id and time spent in the cell, the network could understand coverage of the wireless network and adjust the mobility related parameters, e.g., A3-offset, hysteresis, timeToTrigger. So that, the problem of ping pong handover or handover failure could be mitigated. Similarly, for SCG of Dual Connectivity, the cell id of the PSCell and time spent in the current PSCell while being connected to the current PCell are included in the visitedPSCellInfoList of MHI. Based on the MHI of SCG, the MN/SN could understand the coverage of the SN, enable fast DC setup, and adjust SCG mobility parameters. So that the frequent ping pong PSCell change or PSCell change failure could be mitigated.

Currently, the MHI of SCG has no information about SCG activation and deactivation, since now only PSCell id and time spent in the PSCell are recorded by the UE.

Observation 1: No information about SCG activation or deactivation is recorded by the UE in current MHI.

In our understanding, including the information of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity. The detailed information could be the time of SCG activation, or percentage of time that SCG activation. For instance, if the time or percentage of time that SCG activation is quite low, it means that the Dual Connectivity is possibly not so necessary for the UE, and the network might need not configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. On the other hand, if the time or percentage of time that SCG activation is quite high, it possibly means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficiently.

Observation 2: Including the information of SCG activation and/or deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Another issue needs discussion is that how the network obtains the information of SCG activation/deactivation, one option is that network entity (MN or SN) collects the information by itself, another option is that UE includes the related information in existing MHI. Similarly, it was discussed on how to transfer SN related MHI information in Rel-17, and both the network solution and the UE solution were discussed and accepted. In [2], companies showed views on whether to report PSCell related MHI. Some companies concerned about the UE’s memory and deemed that PSCell cannot be changed in the RRC_INACTIVE and RRC_IDLE state at the UE, and any transition of the PSCell is completely known at the serving gNB. This is true for the RRC_CONNECTED state UE, the PSCell info is completely known by the NW. But for the last connected state, the NW cannot maintain the PSCell info because the NW will remove all the UE context while UE leaving RRC_CONNECTED state. Since the UE can record at most 16 entries of PCells, it is quite possible that MHI can include the UE state transition scenarios.

Following shows one example about the possible change of UE states and the corresponding information recorded by NW in UE History Information (UHI) and recorded by UE in MHI respectively:

In Phase 1, the UE works in RRC_CONNECTED state and in the coverage of PCell 1. Initially the UE is configured with PSCell A, and after 30s, it is configured with PSCell B for 20s. Then the UE returns to standalone for 10s. It is observed that, the information recorded in MHI and UHI are the same.

Figure 1: MHI and UHI for UE in RRC_CONNECTED

In Phase 2, the UE goes to RRC_IDLE state for 120s and still in the coverage of PCell 1. It is observed that, the information of 120s in idle is only recorded in the MHI of UE, but such information cannot be recorded by NW, because all the UE context has been removed by the NW.

Figure 2: MHI and UHI for UE in RRC_IDLE

In Phase 3, the UE goes back to RRC_CONNECTED state and configured with PCell 1 and PSCell A for another 60s. It is observed that, the information of the another 60s in RRC_CONNECTED is recorded by UE in MHI with the same entry of PCell 1. But for NW, it has no idea that the same UE comes back, and it will start recording the mobility information with a new UE, i.e. UE2, in its UHI.

Figure 3: MHI and UHI for UE when state transition happens

Therefore, only the UE maintains all the mobility history information for both RRC_CONNECTED state and RRC_IDLE state, the NW only keeps the information of UE in RRC_CONNECTED state. For the UHI of NW, two entries are created for the same UE when UE comes back to RRC_CONNECTED, and the NW could not link the two entries of the same UE for DC optimization, since NW has no idea about the PSCell information neither in previous RRC_CONNECTED state nor in RRC_IDLE state.

Observation 3: Only UE could record the whole mobility history information, including cells visited in RRC_IDLE, and RRC_CONNECTED states. The network could only record the mobility information when UE is in RRC_CONNECTED state, and the information of RRC_IDLE is lost.

Observation 4: NW could not link the Mobility Information of UE when UE transits to RRC_IDLE and comes back to RRC_CONNECTED.

Therefore, the mobility information in NW side is insufficient for the optimization of dual connectivity configuration, and it is proposed that:

Proposal: Include in MHI the information of SCG activation/deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation.

# 3 Conclusion

This contribution provides our considerations on the MHI enhancement for SCG activation and deactivation, to help network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, following observations and proposal are made:

Observation 1: No information about SCG activation or deactivation is recorded by the UE in current MHI.

Observation 2: Including the information of SCG activation and/or deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Observation 3: Only UE could record the whole mobility history information, including cells visited in RRC_IDLE, and RRC_CONNECTED states. The network could only record the mobility information when UE is in RRC_CONNECTED state, and the information of RRC_IDLE is lost.

Observation 4: NW could not link the Mobility Information of UE when UE transits to RRC_IDLE and comes back to RRC_CONNECTED.

Proposal: Include in MHI the information of SCG activation/deactivation, e.g., the time of SCG activation, or percentage of time that SCG activation.

# Reference

[1]	Draft_R2_126_Meeting_Report_v1

[2]	R2-2107825, Report of [Post114-e][852][SON_MDT] Modeling aspects related to information required by SN/SCG, CATT