3GPP TSG-RAN2 Meeting #127			R2-2406987

Maastricht, Netherlands, Aug 19th-23rd, 2024

Agenda item:	8.2.4

Title:	Further consideration on Ambient IoT random access

Source:	CMCC

Document for:	Discussion

- Introduction

In the last meeting [1], RAN2 discussed the procedure of both 4-step and 2-step CBRA as well as CFRA, and leaving several FFS as follows,

4-step CBRA - A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information. 2-step CBRA - A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information - A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed CFRA - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

Based on former discussion and conclusion, this contribution will discuss how A-IoT reader and device determines to trigger CBRA or CFRA, then discussion on details for CBRA and CFRA, and finally some discussion on remaining issue of RA will be offered.

- Discussion

  - Indication of A-IoT Random Access

In the last meeting, it seems that the common understanding is that A-IoT random access requires an A-IoT trigger message, i.e., A-IoT Msg0. However, the last meeting does not have the opportunity to discussion A-IoT Msg0.

Hence, it is proposed to introduce an A-IoT Msg0 to indication the start of A-IoT random access, whether it’s CBRA or CFRA. Obviously, A-IoT Msg0 should at least include device selection information and RA resource indication for A-IoT device. Considering an A-IoT access round consisting of multiple access occasions and that A-IoT device can have a significant SFO, A-IoT Msg0 may also provide timing information for synchronization.

Synchronization can be a different from device selection information and RA resource indication, since reader may need to send synchronization message before every access occasion for A-IoT device. While device selection and RA resource indication may be (semi-)static.

Observation 1: A-IoT paging message captures device selection information and resource indication, but synchronization for A-IoT device before access occasion is absent.

With the introduction of A-IoT paging message that can provide device selection information and resource indication in TR for TR 38.769. RAN2 can introduce a supplement A-IoT Msg0 to provide at least synchronization for A-IoT device.

Proposal 1: Introduce a D2R A-IoT Msg0 to indicate the start of access occasion and provide synchronization for A-IoT device. Can discuss what other indication should be captured, e.g., RA type.

For Information, QueryRep message in EPC C1G2 servers the similar purpose.

  - Determination of CBRA and CFRA

As its terminology suggests, the difference between CBRA and CFRA is whether there will be contention or not. Hence it can be transferred into whether the A-IoT reader has enough prior information to identified all A-IoT devices, no matter where that information comes from, e.g., CN or application.

If reader is sure that there will be only 1 device to conduct random access, then it should be CFRA with or without device specified information since there will not be any device to contend against target device. And if there will be multiple devices, only if reader knows enough information to distinguish each of them and then allocate resources to each device, reader can issue a CFRA procedure for them, otherwise CBRA should be issued.

Proposal 2a: If upper layer does not indicate RA type, CBRA will be issued by reader as default.

Proposal 2b: CFRA should be issued if and only if.

- Prior information from upper layer confirms that there will only be ONE A-IoT device.

- Prior information from upper layer is enough to distinguish every related A-IoT device, and reader will allocate dedicated resources for every related A-IoT device.

- Upper layer indicates

And device should be informed about whether the random-access procedure is contention-based or contention free for resource selection. Hence this indication can be captured at least in A-IoT paging message. Can further discuss its explicit (by RA type) or implicit (by resource allocation)

Proposal 2c: Reader can indicate CBRA or CFRA for device in A-IoT paging message.

The priori information here can at least be an ID of a single A-IoT device, or multiple IDs of A-IoT devices, or mask that can serve the same purpose. The priori information can be provided at least from CN.

The other issue is, if reader has already detected a collision, reader should signal device to stop sending the rest of its ID to save energy, which is important for device B since it may use its stored energy. Also, it’s more time efficient to end a collided access occasion early and start the next occasion.

For access round, it would also be more time efficient to end an access round with too many empty or collided access occasion earlier and start a new access round with suitable parameters.

Proposal 3: For time and device energy efficiency consideration, reader can signal an early end of access occasion and access round to device if collision is detected.

The ending signaling here can be further discussed, whether to introduce a new message or reuse existing message.

  - Determination of 2-step CBRA and 3-step CBRA

The main difference of 2-step and 3-step CBRA are whether to use random ID to solve contention. Random ID is a relatively shorter ID compared to device ID. From our perspective, random ID is more efficient to solve contention for a large amount number of device due to its shorter length and early detection of collision, while device ID may have a common initial sequence.

But for significantly less devices, 2-step CFBA may be more efficient.

Observation 2: Random ID is more efficient in both energy and time to solve large scale contention since it’s shorter than device ID and can detect collision earlier than device ID.

Another advantage of 3-step CBRA is that if reader would like to use a random ID for further reader-oriented A-IoT paging, reader can issue a 3-step CBRA and get that random ID. Currently reader cannot assign an ID-like IE for device, especially when device ID is not visible or encrypted to RAN.

Proposal 4a: If upper layer does not indicate CBRA type, 3-step CBRA will be issued by reader as default.

Proposal 4b: 3-step CBRA should be issued if

- There are a considerable number of A-IoT devices selected. Can further discuss exact number.

- Reader would like a short ID for target device for further use.

- Upper layer indicates.

It can be further discussed on how many devices is ‘a considerable number’ for 3-step CBRA. Currently there is no conclusion on the length of random ID and device ID, which has impact on the decision of 3-step or 2-step, RAN2 can revisit this issue with soiled conclusions.

  - 3-step CBRA details

CBRA includes an A-IoT Msg1 where device will send a random ID. Considering that in slotted-ALOHA, which is the baseline for A-IoT RA, device will also randomly select an access occasion within access round to send its random ID. There will be difference on whether device selects its access occasion based on its random ID or not.

If A-IoT device select access occasion directly based on its random ID, e.g., using a truncated random ID as the index of access occasion. The probability that more than 1 tags has the same random ID and choose the same access occasion is,

PID_Collided_Coupled=1-i=1n1-i-1m=1-m!mn∙m-n!≅1- e-n∙(n-1)2m

Where n is the number of A-IoT devices, m is the number of access occasions with access round, and x is the length of random ID, and m<2x .

If the selection of access occasion and generation of random ID is decoupled, the former mentioned probability is,

PID_Collided_Decoupled=1-(1-1m∙2x)n-1≅1- e-n-1m∙2x

To avoid collision, PID_Collided should be as small as possible, hence the selection of access occasion and generation of random ID should be decoupled, i.e., Ambient IoT use different (random) numbers to select access occasion and generate its random ID.

Proposal 5: RAN2 to agree the working assumption that the ID in 3-step CBRA Msg1 should be independent to access occasion selection.

If not, as shown in PID_Collided_Coupled, with a short ID, it can be less possible for UE to consider contention resolution as successful if Msg2 including the same random ID, since there will be more tags with the same ID in the same access occasion.

As to the ID send to reader in Msg1, considering that device can use (partial) device ID as the random seed for RNG (random number generator) to generate a new ID, which is not that different to the random ID. Hence the actual issue here is that whether A-IoT device can use its original device ID as the ID in Msg1.

The most concerned issue of original partial device ID is whether it has enough discriminability or not since the assumption is that Msg2 will resolve most contentions. Same kind of device may have same partial ID based on current electronic product coding scheme.

And therefore, Msg1 should be randomized regardless of device group selected. Using the original device ID cannot guarantee that. Taking a 96bit SGTIN Electronic Product Code as an example, which is one of most popular standards for RFID, 58 of 96bit is pre-defined and only 38bit presenting Serial is device specified.

| Scheme | SGTIN-96 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| URI Template | urn:epc:tag:sgtin-96:F.C.I.S |  |  |  |  |  |
| Total Bits | 96 |  |  |  |  |  |
| Logical Segment | EPC Header | Filter | Partition | GS1 Company Prefix (*) | Indicator / Item Reference | Serial |
| Logical Segment Bit Count | 8 | 3 | 3 | 20-40 | 24-4 | 38 |
| Coding Segment | EPC Header | Filter | GTIN |  |  | Serial |
| URI portion |  | F | C.I |  |  | S |
| Coding Segment Bit Count | 8 | 3 | 47 |  |  | 38 |

Table 1. SGTIN-96 coding table

Observation 3: Since device ID of A-IoT device may consist of mostly predefined segments and values, device of the same company or same variety can have same partial sequence in their device ID.

Proposal 6: RAN2 to agree to use random ID only in 3-step CBRA Msg1, which may be generated based on device ID.

The upcoming issue is the length of this random ID. The length of random ID should be at least capable of distinguish all inventoried devices. For example, if there are n tags, the length of random ID x should be at least greater or equal than log2n. If length of random ID is not long enough, reader may issue multiple rounds of inventory. And if the random ID is too long, the device requires more energy to send it. Hence it would be more flexible for reader to configure the length of random ID in Msg1. If reader knows roughly about how many devices will be in the access round via prior information, reader itself can choose a suitable ID length of CBRA Msg1 based on the former probability formula and then configured in to device.

And if not, device can send a default length of random ID. Consider UHF RFID protocol which uses RN16 as device temporary ID, the random ID in Msg1 has not additional requirements and 16 is enough.

Observation 4: The ID length of Msg1 should be sufficient to accommodate all device in access round while being as short as possible to minimize tag energy consumption.

Proposal 7a: A-IoT reader can configure ID length of 3-step CBRA Msg1 to device, can be captured in A-IoT paging message.

If reader doesn’t configure the ID length, device should use a pre-defined default length. Since the length of ID is subject to the number of devices, the device density specified in TR 28.840 are less than 2.5 per m2 for all use cases. And the maximum typical ISD is 500 m, the device amount can be about 2×106 for one dense use case. If the PID_Collided_Decoupled is supposed to be less than 10-5, the random ID length should be 16 or longer.

For other probabilities, the following table can be a reference, please note that if access occasions and device amount are close, the main influence factor here is the probability only.

| Recommended Random ID Length | Probability of ID Collision (Decoupled) | Number of Access Occasions |
| --- | --- | --- |
| 16 | 10-5 | Same with device |
| 13 | 10-4 | Same with device |
| 10 | 10-3 | Same with device |

Table 2. Recommended Random ID Length

Proposal 7b: If A-IoT reader doesn’t configure ID length of CBRA Msg1, A-IoT device use a default length for 3-step CBRA Msg1, whose baseline is no more than 16.

  - 2-step CBRA details

The main purpose for 2-step CBRA here is to let device send its ID directly with more time and energy efficiency. Therefor the ID in 2-step CBRA Msg1 must be unique for every A-IoT device (e.g., EPC ID, MAC address), unless the reader is not supposed to find the A-IoT device again after this 2-step CBRA access round.

However, this device ID issue is not within RAN2’s scope, for further discussion, it can be considered as a work assumption.

Proposal 8: RAN2 to agree the working assumption that device ID in both 3-step CBRA Msg3 and 2-step CBRA A-IoT Msg1 should be unique for every A-IoT device at least in their lifespan.

And with this unique device ID, whether an additional random ID is need is doubtful. Obviously, the additional random ID is not for device identification since reader already has its device ID. It may act as a short ID for the same device but partial device ID can serve the same purpose as well.

For other purpose, e.g., CRC checking or encryption, they are not within RAN2’s scope as well.

Proposal 9: 2-step CBRA Msg1 should not contain both device ID and random ID.

For upper layer data sent together with device ID in 2-step CBRA Msg1, it can be not power efficient since it may collide with other devices. And with sending data, the length of access occasion in time domain will be expanded. Hence it would be more time efficient to send data after contention is considered as resolved, i.e., after 2-step CBRA Msg2.

Observation 5: 2-step CBRA Msg1 can be collided with other devices, sending upper layer data in Msg1 can waste device energy.

There are other difficulties for sending data with device ID. One of them is that reader would not know the existence and length of data to be sent from device. If it were supposed that device would send upper layer data in 2-step CBRA Msg1, reader would have to allocated resource enough to contain both the device ID and potential upper layer data whose length reader would never know. Without doubt, this will increase reassure and energy waste in both reader and device.

In legacy UE will send BSR to inform gNB the size of buffered data, but A-IoT device is low-complexity, it’s doubtful whether it can use BSR. For information, UHF RFID doesn’t have similar mechanism.

Observation 6: Reader cannot predict the length or existence of upper layer data in 2-step CBRA Msg1, making reader nearly impossible to allocate adequate resource to device without wasting.

In UHF RFID, reader know the size of data to be transmitter in upper layer request which will capture memory bank, pointer and length. But whether CN can provide similar information in ambient IoT is doubtful, RAN can wait for fur conclusion from other WGs.

Proposal 10: For device energy consumption and resource allocation consideration, A-IoT device should send upper layer data after contention is resolved in A-IoT random access.

As to idle slot, although reader can also end it early for higher time efficiency, but it might be helpful for device B have enough occasions to get recharged. It might be too early to decide.

  - CFRA details

The CFRA of Ambient IoT should guarantee that all devices have a dedicated, contention-free resource to each other. It’s obviously that if there is only ONE device indicated by CN, CFRA should be applied.

If there are multiple devices, reader should allocate resource to all target device. In CFRA, device will perform data transmission after A-IoT paging. The resource allocation indication (like UL grant) can be captured in A-IoT paging.

Proposal 11: For single target device CFRA, CFRA can consist of A-IoT Msg0, and subsequently A-IoT Msg1, which may include an ID and/or upper layer data.

Since RAN2 has already agreed that A-IoT paging message can indicate information about resource to be used for response and select target device. For multiple devices conducting CBRA, whether A-IoT paging can contain such resource indication will be an issue. Apparently, a long A-IoT paging is not desirable. RAN2 could also indicate another R2D A-IoT message to indicate device for CFRA.

Proposal 12: For multiple target devices CFRA, RAN2 study how to indicated dedicated resources for target device, e.g., to reuse A-IoT paging message/A-IoT Msg0 or introduce a new R2D A-IoT message.

AS to single target device, the resource indication can be captured in A-IoT paging message as agreed in the last meeting.

  - Access Failure handling

For failure handling for both 2-step and 3-step CBRA Msg1, there can be multiple reasons. Radio link failure caused by long range/low energy/interference can be a most common one, however, for RAN2’s perspective, there aren’t many effective solutions for that. One reason RAN should focus on is Msg1 failure caused by collision, i.e., multiple devices select the same occasion and their response collides.

And there are at least 2 options for Msg1 failure handling, the “Next Round Failure handling” means that the access failure should be handled in the next access round. The device will keep some information for the failure handling in next access round, e.g., failure state, SN of access occasion, since there may be an uncertain time gap between current round and the round for access failure handling. In next round failure handling, other devices will not be affected. Also, A-IoT paging should captured related information.

The “Next Occasion Failure handling” means that the access failure should be handled in the next access occasion of the same round. The device that failed to access may not need to keep any information for failure handling since there will be no time gap. But the other device which waits for next access occasion may need to wait longer since failure handling may postpone access occasions.

For the comparison, Next Round approach needs one more A-IoT paging and the failed device may record the failure state in non-volatile memory. Next Occasion failure handling may not need device to record anything but the delay for subsequent device may increase due to that no subsequent device is permitted to access until current failure handling ends. However, the overall delay is not increased since the main difference is just when to handle access failure.

These 2 approaches have no significant performance difference and benefits are on both sides. It is suggested to study both approaches in SI.

There is a third way of access failure handing, i.e., to handle it within the same access round but not immediately in the following access occasion, but in one or multiple uncertain occasions, but it would introduce another message to indicate collided devices.

Proposal 13: RAN2 to study at least the following options for both 2-step and 3-step CBRA Msg1 failure handling,

- Device attempts to re-access in the next access round.

- Device attempts to re-access in the next access occasion.

For 3-step CBRA, its Msg 3 may also failure, the main reason except radio failure is that multiple A-IoT devices select the same access occasion and use the same random ID in Msg1, making reader can only detect collision until the reception of Msg3. And device is ignorant of this procedure, so for failure handling, at least reader should inform the ACK/NAK of Msg3 reception.

As the analysis shows in Section 2.3, the probability that devices select the same access occasion and generate the same random ID is not very high, since it anyway hardly goes beyond 0.5, therefore, a NAK instead of ACK should be introduced to indicate device the failure reception of Msg3, and the absence of NAK indicates the success reception of Msg3.

Proposal 14: Introduce a R2D A-IoT message (NAK) to indicate reader’s failure reception of A-IoT Msg3, whose absence indicates otherwise.

After that, reader can reuse failure handling for CBRA Msg1 to handle Msg3 failure.

  - Potential Enhancement

Many simulation and theoretical derivation results have proven that the time efficiency of slotted-ALOHA does not exceed 0.368 when the parameters are appropriately set i.e., when the number of access occasions equals the number of A-IoT devices. Please note that the efficiency here is defined as the ratio of the number of devices successfully inventoried to the number of time slots used.

The efficiency of slot-ALOHA with different amount of device is shown in the figure below.

Figure 1: Efficiency of slotted-ALOHA with different ambient IoT device populations

It can be observed that for slotted-ALOHA, when the number of slots is equal to the number of devices to be inventoried, its efficiency is maximized at 0.368. Therefore, obtaining an accurate number of devices before random access is beneficial for efficiency enhancement.

Observation 7: With accurate device population estimation, the efficiency of slotted-ALOHA can reach its 0.368 theoretical upper limit.

Furthermore, in some other scenarios, e.g., counting the number of people in a shopping mall or park, counting vehicles in a parking lot, or inventorying in a third-party warehouse, it may be necessary to protect business or personal privacy, and the reader may not be allowed to read device information e.g. EPC ID, personal ID, vehicle identification number, in such cases, the only information that can be obtained is the population size of the devices. An accurate estimation of the device population size is also essential in these use cases.

Observation 8: In privacy concern cases, reading device ID may cause legal issues, where only ambient device population estimation can be applied.

Therefore, it is recommended to study device population estimation for the efficiency and some privacy-critical use cases.

Proposal 15: RAN2 is suggested to study device population estimation for slotted-ALOHA efficiency enhancement and some privacy-critical use cases.

Other enhancement is also for efficiency, slotted-ALOHA is a TDMA based anti-collision algorithm. TDMA algorithm can be further divided into probabilistic algorithms, presented by slotted-ALOHA, and deterministic algorithms, presented by Tree. The probabilistic algorithms are known to suffer from the starvation problem, in which even after many slots, there is a probability that some devices remain unidentified, e.g., multiple devices select the same access occasion.

While, the deterministic algorithm divides device set into multiple subsets, and repeat this procedure until there will only be one device in each set. Hence there will be no starvation problem for deterministic algorithm. The deterministic identification in Tree-based algorithm is promised by the uniqueness of device ID.

Generally, tree-based algorithm involves the reader broadcasting a prefix as the initial sequence from device ID. The devices use their built-in prefix matching circuit to determine whether the provided prefix matches. If so, the device backscatters its ID. If only one device responds, this device is considered to be successfully identified.

Therefore, as a supplement and/or enhancement of slotted-ALOHA, it’s suggested to study deterministic algorithm, presented by tree-based algorithms.

Proposal 16: RAN2 is suggested to study deterministic algorithm, presented by tree-based algorithm as a supplement or enhancement for the baseline slotted-ALOHA algorithm.

- Conclusion

This contribution discusses the A-IoT random access and provide the following,

A-IoT Msg0 aspect:

Observation 1: A-IoT paging message captures device selection information and resource indication, but synchronization for A-IoT device before access occasion is absent.

Proposal 1: Introduce a D2R A-IoT Msg0 to indicate the start of access occasion and provide synchronization for A-IoT device. Can discuss what other indication should be captured, e.g., RA type.

Determination of CBRA and CFRA aspect:

Proposal 2a: If upper layer does not indicate RA type, CBRA will be issued by reader as default.

Proposal 2b: CFRA should be issued if and only if.

- Prior information from upper layer confirms that there will only be ONE A-IoT device.

- Prior information from upper layer is enough to distinguish every related A-IoT device, and reader will allocate dedicated resources for every related A-IoT device.

- Upper layer indicates

Proposal 2c: Reader can indicate CBRA or CFRA for device in A-IoT paging message.

Proposal 3: For time and device energy efficiency consideration, reader can signal an early end of access occasion and access round to device if collision is detected.

Determination of 2-step CBRA and 3-CBRA aspect:

Observation 2: Random ID is more efficient in both energy and time to solve large scale contention since it’s shorter than device ID and can detect collision earlier than device ID.

Proposal 4a: If upper layer does not indicate CBRA type, 3-step CBRA will be issued by reader as default.

Proposal 4b: 3-step CBRA should be issued if

- There are a considerable number of A-IoT devices selected. Can further discuss exact number.

- Reader would like a short ID for target device for further use.

- Upper layer indicates.

Details on 3-step CBRA aspect:

Proposal 5: RAN2 to agree the working assumption that the ID in 3-step CBRA Msg1 should be independent to access occasion selection.

Observation 3: Since device ID of A-IoT device may consist of mostly predefined segments and values, device of the same company or same variety can have same partial sequence in their device ID.

Proposal 6: RAN2 to agree to use random ID only in 3-step CBRA Msg1, which may be generated based on device ID.

Observation 4: The ID length of Msg1 should be sufficient to accommodate all device in access round while being as short as possible to minimize tag energy consumption.

Proposal 7a: A-IoT reader can configure ID length of 3-step CBRA Msg1 to device, can be captured in A-IoT paging message.

Proposal 7b: If A-IoT reader doesn’t configure ID length of CBRA Msg1, A-IoT device use a default length for 3-step CBRA Msg1, whose baseline is no more than 16.

Details on 2-step CBRA aspect:

Proposal 8: RAN2 to agree the working assumption that device ID in both 3-step CBRA Msg3 and 2-step CBRA A-IoT Msg1 should be unique for every A-IoT device at least in their lifespan.

Proposal 9: 2-step CBRA Msg1 should not contain both device ID and random ID.

Observation 5: 2-step CBRA Msg1 can be collided with other devices, sending upper layer data in Msg1 can waste device energy.

Observation 6: Reader cannot predict the length or existence of upper layer data in 2-step CBRA Msg1, making reader nearly impossible to allocate adequate resource to device without wasting.

Proposal 10: For device energy consumption and resource allocation consideration, A-IoT device should send upper layer data after contention is resolved in A-IoT random access.

Details on CFRA aspect:

Proposal 11: For single target device CFRA, CFRA can consist of A-IoT Msg0, and subsequently A-IoT Msg1, which may include an ID and/or upper layer data.

Proposal 12: For multiple target devices CFRA, RAN2 study how to indicated dedicated resources for target device, e.g., to reuse A-IoT paging message/A-IoT Msg0 or introduce a new R2D A-IoT message.

Access failure handling aspect:

Proposal 13: RAN2 to study at least the following options for both 2-step and 3-step CBRA Msg1 failure handling,

- Device attempts to re-access in the next access round.

- Device attempts to re-access in the next access occasion.

Proposal 14: Introduce a R2D A-IoT message (NAK) to indicate reader’s failure reception of A-IoT Msg3, whose absence indicates otherwise.

Potential enhancement aspect:

Observation 7: With accurate device population estimation, the efficiency of slotted-ALOHA can reach its 0.368 theoretical upper limit.

Observation 8: In privacy concern cases, reading device ID may cause legal issues, where only ambient device population estimation can be applied.

Proposal 15: RAN2 is suggested to study device population estimation for slotted-ALOHA efficiency enhancement and some privacy-critical use cases.

Proposal 16: RAN2 is suggested to study deterministic algorithm, presented by tree-based algorithm as a supplement or enhancement for the baseline slotted-ALOHA algorithm.

- References

[1]	R2_126_ChairNotes_24-05-24_13-10_eom.docx