3GPP TSG-RAN WG2 Meeting #127			R2-2406988

Maastricht, Netherlands, Aug 19th-23rd, 2024

Agenda item:	8.7.2

Title:	Further discussion on multi-modality support for XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting [1], RAN2 agree the support of multi-modality awareness and enhancement in Rel-19 for both UL and DL. Several liaisons have been sent to relate WGs, but it requires some time before replies come back This contribution will discuss some enhancements for multi-modality in XR.

Discussion

  - Multi-modality assistance information

Based on former discussion, multi-modality assistance information mainly includes an indication of corelation among QoS flows (i.e., Multi-Modal Service ID or a similar ID) and synchronization threshold for multi-modal QoS flows. Those kinds of multi-modal assistance information are considered as static or semi-static and can be configured before actual transmission from CN. Hence no need to be reported from UE.

However, for dynamic assistance information, especially assistance information update trigger in UE side, it is reasonable for UE to report directly to gNB via RRC message, as agreed in Rel-18. It could be much faster than UE report assistance information to CN and then CN configure gNB, which may already be obsolete and cause PDU set discarding.

Considering that multi-modality flow may include a haptic flow or motion flow, which may be reported by a device with dynamic sampling rate. Dynamic sampling rate is a common technology used widely in wireless sensor for power saving, i.e., controller, mouse and key broad. Device with dynamic sampling rate can have a very low reporting rate when user stays still, but a significant higher reporting rate when user has movement. 
Such scenarios are quite common in live broadcasting, security surveillance and immersive gaming. At this time, CN can hardly predict the changes in the transmission requirements of multi-modal services, and if the UE triggers the modification of transmission parameters through the CN, there will be a time lag.

Observation 1: Multi-modality assistance information can be (semi-)static and dynamic. For dynamic information, UE should be allowed to report them to gNB directly for a less time lag.

Proposal 1: Dynamic multi-modality assistance information should be allowed to be reported by UE, e.g., synchronization threshold update triggered by dynamic sampling rate in UE side.

RAN2 can also study or wait for SA2 for further Multi-modal assistance information.

  - Discarding enhancement for multi-modality

Discarding is a common solution for capacity enhancement. For multi-modality, QoS flows of a same multi-modal service has inner dependency. If one of the flow discards PDU, the transmission of related PDU of other multi-modal flows can be meaningless. And therefore, multi-modality discarding should be considered.

In Rel-18, both timer-based and PSI-based discarding are adopted. On basis of that, when there is multi-modal QoS flow has a discarding, the discarding of other multi-modal QoS flows should triggered as well.

Proposal 2: For capacity enhancement, RAN2 is suggested to study multi-modality discarding, i.e., when one of multi-modal QoS flows has discarding triggered by timer or PSI, the rest multi-modal flows with same MMSID should also triggers discarding.

Another meaningful indication might be multi-modality integrity information. In addition to ensuring that multi-modality data is transmitted within the synchronization threshold, it may also be necessary to ensure multi-modality data integrity. That is, when the number of discarded data packets exceeds a certain proportion, there is no need to transmit the remaining multi-modality data packets. Such information might also be reported from UE as assistance information. AL-FEC may also need this mechanism.

Proposal 3: RAN2 to discuss whether to send an LS to SA2 on introduction multi-modality integrity handling indication (MMIHI).

Assembly For PSI-based discarding, it’s a bit different. If multi-modality discarding is introduced, the PSI configuration for multi-modal QoS flows with same MMSID can be considered as the same PSI. But this might be an implement issue that can be discussed in stage-3.

The other discarding related issue is emerged with the introduction of multi-modality synchronization threshold. When multi-modal data is out of synchronization, whether to discard or continue the started transmission is an open issue. For capacity consideration, discarding is preferred, but for user experience, maybe to continue the started transmission and do not send the rest is more reasonable.

Proposal 4: RAN2 is suggested to discuss how to handle multi-modality PDUs that are out of synchronization, whether to discard them or not.

Considering that discarding out of synchronization data may help the subsequent data to be sent within synchronization threshold, maybe discarding is more appreciated.

  - Synchronization enhancement for multi-modality

With the introduction of DSR, multi-modal synchronization threshold, and ongoing delay-aware LCP, it is possible for LCP based on remaining synchronization threshold. But in current DSR, the remaining time is based on PDCP discard timer. And gNB may not acquire synchronization related information.

Therefore, for synchronization enhancement in multi-modality, LCP based on remaining synchronization threshold should also be considered.

Proposal 5: RAN2 is suggested to study scheduling enhancement based on multi-modality synchronization threshold.

At the same time, UE multiplexing should also be captured, i.e., the priority of LCH in PDU multiplexing should also consider remaining synchronization threshold.

Proposal 6: RAN2 is suggested to study multiplexing and assembly enhancement based on multi-modality synchronization threshold.

One simple solution is a modification of delay-aware LCP and DSR, which may be need further progress of scheduling enhancement agenda.

Conclusion

This contribution discusses the multi-modality enhancement for XR, and offer the following,

Multi-modality assistance information aspect:

Observation 1: Multi-modality assistance information can be (semi-)static and dynamic. For dynamic information, UE should be allowed to report them to gNB directly for a less time lag.

Proposal 1: Dynamic multi-modality assistance information should be allowed to be reported by UE, e.g., synchronization threshold update triggered by dynamic sampling rate in UE side.

Discarding enhancement aspect:

Proposal 2: For capacity enhancement, RAN2 is suggested to study multi-modality discarding, i.e., when one of multi-modal QoS flows has discarding triggered by timer or PSI, the rest multi-modal flows with same MMSID should also triggers discarding.

Proposal 3: RAN2 to discuss whether to send an LS to SA2 on introduction multi-modality integrity handling indication (MMIHI).

Proposal 4: RAN2 is suggested to discuss how to handle multi-modality PDUs that are out of synchronization, whether to discard them or not.

Synchronization enhancement aspect:

Proposal 5: RAN2 is suggested to study scheduling enhancement based on multi-modality synchronization threshold.

Proposal 6: RAN2 is suggested to study multiplexing and assembly enhancement based on multi-modality synchronization threshold.

References

[1]	R2-2405706, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)