3GPP TSG-WG2 Meeting #127			R2-2406989

Maastricht, Netherlands, Aug 19th-23rd, 2024

Agenda item:	8.7.4

Title:	Further discussion on scheduling enhancement on XR

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting, RAN2 has agreed to study the delay-aware LCP enhancement and mentioned that the complexity of UE implementation should be considered. Additionally, RAN agreed to configure an additional priority for with delay-critical data, and left the following FFS,

FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH FFS whether other LCP restrictions based approaches are needed/beneficial FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

This contribution will mainly discuss the enhancement on LCP and DSR.

Discussion

  - LCP enhancement

The LCP enhancement mainly consists of two aspects: LCH prioritization and LCH restriction. Regarding LCH prioritization, the last meeting already agreed to configure additional priority for LCH with delay-critical data. Therefore, how and when to use the configured additional priority will become the focus of future work.

With the introduction of DSR, if DSR trigger threshold is configured reasonably, gNB can be aware of the volume of delay-critical data below the threshold and their remaining time. Additionally, through DSR and BSR, gNB can also obtain information about non-delay-critical data, consist of delay-tolerant data and delay critical data whose remaining time is still sufficient.

Moreover, if the priority of non-delay-critical data is increased unconditionally, it may cause a similar starvation problem for high-priority data. Therefore, it is not recommended to extend the additional priority to the entire data in LCH unless the entire LCH consists of delay-critical data.

Proposal 1: The additional priority does not apply to non-delay-critical data.

Furthermore, to minimize the impact of original LCP, the priority of delay-critical data that still has enough remaining time should also stay as the original one configured for LCH. RAN2 can further consider this issue. For example, supposing there is an LCH who consist of delay-critical data only, if additional priority applies to all delay-critical data, the original priority should be useless.

For the efficiency of resource allocation, delay-aware LCP should focus on delay-critical data whose delay budget is about exhausted.

Observation 1: For less impact on legacy QoS, additional priority should only apply to delay-critical data that is about to exhaust its delay budget.

Proposal 2: The additional priority does not apply to delay-critical data with sufficient remaining time.

As to the remaining time threshold, apparently the one used in DSR can server the same purpose. But if upper layers configure a less remaining time threshold, gNB can use the remaining time reported via DSR and the configured remaining time threshold to determine the activation of the additional priority. This approach can provide a more sophisticated delay guarantee mechanism.

For UE, a new remaining time threshold can cause more complexity, but with the introduction of multiple pairs of remaining time and buffer size in DSR. The complexity should be acceptable.

Proposal 3a: Introduce a remaining time threshold for gNB and UE to active the additional priority, i.e., the additional priority only applies to delay-critical data whose remaining time is below this threshold.

Proposal 3b: If remaining time threshold is not provided, DSR threshold can be reused.

Additionally, the additional priority should have certain constraints. Obviously, the additional priority cannot be lower than the original priority of LCH. At the same time, SRB is so important that should be exempt from the impact of XR LCP enhancement.

Proposal 4: RAN2 to agree that the additional priority should not be higher than that of any SRB.

At last, there is a bit concern on whether LCP enhancement applies to Configured Grant. The introduction of Configured Grant is aimed at reducing PDCCH overhead when transmitting services with fixed generation cycles and fixed packet sizes, e.g., voice service and some URLLC related service. If LCP also applies for CG, there is a concern that the original data in CG may have no chance to be sent with XR LCP enhancement in congestion. At this stage, it’s more reasonable to restrict LCP to using CG resource.

Proposal 5: RAN2 to agree that the LCP enhancement does not apply to CG, unless there are padding bits in CG TB.

  - DSR enhancement

At the last meeting, it was agreed to introduce DSR with multiple pairs of remaining time and buffer size. We would like a quick calcification on different remaining time and its priority, since it has significant impact on UE implementation if multiple pairs of remaining time can implicitly mean delay-critical data may also have multiple priorities.

UE complexity is a main concern in SI, and introducing multiple priorities is not helpful for it.

Proposal 6: RAN2 to clarify that multiple pairs of remaining time and buffer size has no impact on DSR triggering and LCP enhancement.

There was a debate over whether to capture PSI in DSR. Considering that the configuration of PSI is per LCH/DRB, and DSR will report LCG ID as well.

Therefore, LCG ID in DSR can serve as a rough indication of PSI, so there is no need to report PSI in the DSR. If PSI reporting is needed in DSR, it is more useful to distinguish PDU sets with different PSI within an LCG, but with suitable LCG configuration, it could be avoided.

Furthermore, since it has been determined that DSR will introduce multiple pairs of remaining time and buffer size for LCG, the size of LCG will be longer than before. Considering the impact of MAC CE overhead on capacity, RAN2 should be more cautious about extending DSR MAC CE.

Observation 2: With proper configuration, i.e., delay-critical data in LCG has the same PSI, LCG ID in DSR can be regarded as a PSI identifier since PSI is configured per LCH/DRB.

Observation 3: With the introduction of multiple pairs of remaining time and buffer size, DSR is already very long and complex, can introduce nonnegligible overhead.

Proposal 7: RAN2 to agree that PSI is not reported in the DSR.

Given the introduction of multiple pairs of remaining time and buffer size in DSR, along with the anticipated nonnegligible overhead, it is appropriate for the DSR to maintain a single trigger threshold.

If multiple trigger thresholds are introduced, then only the buffer size corresponding to the triggered threshold needs to be reported in DSR. RAN2 is suggested consider choosing either multiple trigger thresholds or multiple pairs of reported values, but not both.

Observation 4: With the introduction of multiple pairs of remaining time and buffer size, multiple triggering threshold for DSR can cause redundancy.

Another issue is the complexity of UE implementation. The introduction of multiple trigger thresholds should be approached with caution. At least based on current observations, there is no necessity to configure both multiple pairs and multiple triggers simultaneously. The UE can choose to trigger a single reported value based on multiple thresholds, or to report one of multiple pairs of measurement values based on a single threshold, considering the complexity of the UE.

Proposal 8: RAN2 to agree that at least multiple triggering thresholds and multiple pairs of remaining time and buffer size will not be configured at the same time for overhead and complexity consideration.

For non-delay-critical data, since the BSR includes all data (i.e., both delay-critical and non-delay-critical), in conjunction with the DSR, gNB has sufficient information to calculate the buffer size of non-delay- critical data. Therefore, when the BSR can be triggered as expect, reporting non-delay-critical data in the DSR is unnecessary, especially when considering the overhead.

Observation 5: BSR captures both delay-critical and non-delay-critical data, if DSR is triggered as expected, gNB can deduce the buffer size of non-delay-critical data.

Proposal 9: Non-delay-critical data is reported in BSR as legacy.

Conclusion

This contribution discusses the scheduling enhancement for XR, and provide the following,

LCP enhancement aspect

Proposal 1: The additional priority does not apply to non-delay-critical data.

Observation 1: For less impact on legacy QoS, additional priority should only apply to delay-critical data that is about to exhaust its delay budget.

Proposal 2: The additional priority does not apply to delay-critical data with sufficient remaining time.

Proposal 3a: Introduce a remaining time threshold for gNB and UE to active the additional priority, i.e., the additional priority only applies to delay-critical data whose remaining time is below this threshold.

Proposal 3b: If remaining time threshold is not provided, DSR threshold can be reused.

Proposal 4: RAN2 to agree that the additional priority should not be higher than that of any SRB.

Proposal 5: RAN2 to agree that the LCP enhancement does not apply to CG, unless there are padding bits in CG TB.

DSR enhancement aspect

Proposal 6: RAN2 to clarify that multiple pairs of remaining time and buffer size has no impact on DSR triggering and LCP enhancement.

Observation 2: With proper configuration, i.e., delay-critical data in LCG has the same PSI, LCG ID in DSR can be regarded as a PSI identifier since PSI is configured per LCH/DRB.

Observation 3: With the introduction of multiple pairs of remaining time and buffer size, DSR is already very long and complex, can introduce nonnegligible overhead.

Proposal 7: RAN2 to agree that PSI is not reported in the DSR.

Observation 4: With the introduction of multiple pairs of remaining time and buffer size, multiple triggering threshold for DSR can cause redundancy.

Proposal 8: RAN2 to agree that at least multiple triggering thresholds and multiple pairs of remaining time and buffer size will not be configured at the same time for overhead and complexity consideration.

Observation 5: BSR captures both delay-critical and non-delay-critical data, if DSR is triggered as expected, gNB can deduce the buffer size of non-delay-critical data.

Proposal 9: Non-delay-critical data is reported in BSR as legacy.

References

[1]	R2-2405706, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)